TOUCHSTONE ISSUE RESOLUTION DOCUMENT (TSIRD)

TSIRD NUMBER:	X_draft_18

ISSUE TITLE:		Standardized Pole-Residue Representation of Touchstone Data

REQUESTOR:		Arpad Muranyi, Siemens EDA

DATE SUBMITTED:	{for administrative use}

DATE REVISED:	{for administrative use}

DATE ACCEPTED:	{for administrative use}

DEFINITION OF THE ISSUE:

With the ever-increasing complexity of package and printed circuit board designs, Touchstone files used for interconnect modeling are also increasing in size.  Touchstone file sizes are often in the hundreds of Megabytes, or even Gigabytes range.  This can have a negative effect on simulation performance and makes the distribution of these files very challenging.  Consequently, there is a great need for finding ways to describe large interconnects with smaller files.

In addition, different EDA vendors use different (proprietary) algorithms to process the Touchstone file data in simulations, which can cause simulation results to be different from different EDA tools, even with identical Touchstone files.

A common method in the industry to reduce the size of Touchstone files - and thereby improve simulation performance - is to convert the large frequency table data to fitted pole-residue format.  There are numerous advantages to this method, one of which is that EDA tools can produce identical simulation results with the same pole-residue-based models.

While pole-residue fitting (also known as “rational fit” or “vector fit”) has been used for decades in the industry, this data format has not been standardized yet, which makes it EDA tool dependent.  The goal of this proposal is to standardize the pole-residue format so that all EDA tools could make use of the same pole-residue model files.

SOLUTION REQUIREMENTS:

The Touchstone specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Define three new keyword pairs to describe pole-residue data | [Begin Pole-Residue Data] / [End Pole-Residue Data], [Begin Common Poles Data] / [End Common Poles Data] and [Begin Residues Data] / [End Residues Data] |
| - Define a new keyword to document the number of pole-residue blocks | [Number of Pole-Residue Blocks] |
| - Define a new keyword pair to document the source of the pole-residue data | [Begin Pole-Residue Data Source] / [End Pole-Residue Data Source] |
| - Make the frequency table and the pole residue data mutually exclusive in the same file | Change the rules of the existing [Network Data] and [Noise Data] keywords |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

Table 2: Touchstone Keywords Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |

PROPOSED CHANGES:

Note that the existing Touchstone specification does not define the units for [Network Data] (we should probably correct that).

Consider adding a File_rev keyword to Touchstone in general…

The original Touchstone 2.0 [Mixed Mode Order] keyword could be applied to pole-residue data as well, if so desired.  However, it’s unlikely that time domain simulation (for which the fitted poles are created) can be conducted in a basis that does not represent actual node voltages and branch currents.

Another keyword pair might be invented for reciprocal models, which could further reduce the matrix size.

Change the text of the existing Touchstone 2.1 specification throughout for the [Network Data] and [Noise Data] keywords so that they would be not allowed when the [Begin Pole-Residue Data Source] and related keywords are present.

Add the following keywords to the Touchstone specification in the appropriate location:

Keyword:	[Begin Pole-Residue Data Source] / [End Pole-Residue Data Source]

Required:	The [Begin Pole-Residue Data Source] / [End Pole-Residue Data Source] keyword pair is required when the [Begin Pole-Residue Data] is present, or when [Begin Common Poles Data] and [Begin Residues Data] are present.  Otherwise, it is not permitted.

Description:	The purpose of the [Begin Pole-Residue Data Source] keyword is to document the original source Touchstone file containing the network data from which the pole-residue data was generated.

Sub-Params:	Source_file, File_date, File_revision, File_size, Company_name, Source_checksum

Usage rules:	The Source_file and File_date subparameters are required.

Each subparameter shall start on a separate line and may appear in any order.  Each subparameter and its argument shall be separated by at least one white space.  The file name argument of the Source_file subparameter is case-sensitive to support operating systems on which file names are case-sensitive.  The month argument in the File_date subparameter should be spelled out for clarity.  The File_size argument shall be provided in bytes (not kilo-bytes or mega-bytes, etc.).  Subparameters Company_Name, File_Name, File_Revision, File_Date are required.

Example:

[Begin Pole-Residue Data Source]

Company_name      NoName Corp.

Source_file       Original_matrix_formatted.s30p

File_revision     rev0.0

File_date         July 4, 2023

File_size         12345678

Source_checksum   CE114e4501d2e4E2DcEA3E17B546F339

[End Pole-Residue Data Source]

Keyword:	[Begin Data Source Conditions] / [End Data Source Conditions]

Required:	The [Begin Data Source Conditions] / [End Data Source Conditions] keyword pair is required when the [Begin Pole-Residue Data] is present, or when [Begin Common Poles Data] and [Begin Residues Data] are present.  Otherwise, it is not permitted.

Description:	The purpose of the [Begin Data Source Conditions] keyword is to document the conditions at which the original source Touchstone file was extracted from a physical layout.

Sub-Params:	Number_of_source_ports, Number_of_fitted_ports, Unfitted_port_termination, Min_frequency, Max_source_frequency, Max_fitted_frequency, Frequency_step_type, Extrapolated_DC, Source_frequency_list.

Usage rules:	These subparameters are required when the [Begin Data Source Conditions] keyword is present.  Each subparameter shall start on a separate line and may appear in any order.  Each subparameter and its argument(s) shall be separated by at least one white space.

Example:

[Begin Data Source Conditions]

Number_of_source_ports     128

Number_of_fitted_ports      12

Unfitted_port_termination    Reference

Min_frequency                0

Max_source_frequency        10e+11

Max_fitted_frequency        10e+9

Frequency_step_type          Adaptive

Extrapolated_DC              False

Source_frequency_list        0 1 10 10e+2 10e+3 10e+4 10e+5 10e+6

10e+7 10e+8 10e+9 10e+10 10e+11

[End Data Source Conditions]

Keyword:	[Number of Pole-Residue Blocks]

Required:	The [Number of Pole-Residue Blocks] keyword is required when the [Begin Pole-Residue Data] or the [Begin Residues Data] keyword is present, otherwise it is not permitted.

Description:	The purpose of this keyword is to provide a mechanism to make sure that if the total number of pole-residue blocks is less than N2, it was intentional and not because of accidental loss of data.  The data of the blocks which are not defined by the index is set to zero.

Usage rules:	The [Number of Pole-Residue Blocks] keyword shall be followed by at least one white space and a single integer number.  The value of the integer that follows this keyword shall range from N to N2, where N is the model’s number of ports given in the [Number of Ports] keyword.

Examples:

! The full matrix of a 4-port network data (.s4p file) would

! consist of 16 elements.

[Number of Pole-Residue Blocks] 16

! If all entries of the 4-port network data (.s4p file) are

! zero except its diagonal, it may be described with four

! pole-residue keywords.

[Number of Pole-Residue Blocks] 4

Keyword:	[Begin Pole-Residue Data]

Required:	The [Begin Pole-Residue Data] keyword is not permitted prior to Version 3.0 files; it is not permitted for parameter types G and H, and it is not permitted when the [Begin Common Poles Data] and [Begin Residues Data] keywords or the [Network Data] keyword are present.  Otherwise, at least N instances of the [Begin Pole-Residue Data] keyword are required optional.

Description:	This keyword contains the coefficients of the pole-residue representation of the network data.

Sub-Params:	Delay, Asymptote, Residue_at_infinity, Number_of_data_lines

Usage rules:	The [Begin Pole-Residue Data] keyword is followed by at least one white space and two integer numbers, separated by a comma, and enclosed in a pair of open and close parentheses.  The integer numbers (i, j) denote indexing to define which matrix component is described by the data that follows.  Both indices range from 1 to N, where N is the model’s number of ports given in the [Number of Ports] keyword.

When the pole-residue data is generated from Touchstone 1.0 or 1.1 Y or Z-parameters (in which case the Y or Z-parameters are normalized), the Touchstone 1.0 or 1.1 Y or Z-parameter data from which the pole-residue data is generated shall be unnormalized first by the model maker.

Unlike [Network Data] in Touchstone files, the pole-residue data is ordered by matrix components, not by frequency.  For each pole-residue component (i, j), i=1…N and j=1…N define the table of poles and residues.  Each pole-residue component (i, j) contains one or more data lines in the form of [αm ωm Am Bm], m=1…M, where M is the sum of the number of real poles and the number of complex-conjugate pairs, defined in subparameter Number_of_data_lines (see equation below).  For example, with 7 real poles and 14 complex conjugate pairs of complex poles, the number of lines M becomes 21.  αm and ωm are the real and imaginary parts of the complex poles, and Am and Bm are the real and imaginary parts of the coefficients for the pole components.  The values ½(Am - iBm) and ½(Am + iBm) are normalized residues (divided by the pole) that correspond to the pair of complex conjugate poles.  For a real pole (Bm = 0, ωm = 0), the equation below turns into Am/(1 + s/αm), where Am is the normalized residue for the real pole.  The coefficient Am is the value that the pair of complex conjugate poles or a real pole contribute to the DC value.  The unit for αm and ωm is Hz.  The unit for Am and Bm depends on the parameter type.  For S-parameters it is unitless, for Z-parameters it is ohms, and for Y-parameters it is siemens (1/ohms).

YHm(s)=12Aam-ibBm1+s/(αm+iωm)+aAm+ibBm1+s/(αm-iωm)

Duplicate real poles, or duplicate complex-conjugate pair of complex poles are not permitted in the pole-residue data provided between the [Begin Pole-Residue Data] / [End Pole-Residue Data] keyword pairs.

For stability, it is required that the real part of the pole is negative, where pole is a complex frequency at which the denominator of the above expression becomes zero.  This requirement holds for positive α, because denominator of the first summand turns into zero when (α + ω + s) = 0, which is only possible when the real part of s becomes -α and the imaginary part is -ω.  Therefore, in the .pls file we actually define the real and imaginary parts of the pole with the opposite sign, which makes them both positive numbers.

The [Begin Pole-Residue Data] / [End Pole-Residue Data] keyword pair is repeated at least N times (for a diagonal matrix) or up to N2 times (for a full matrix) in a model file, where N is given in the [Number of Ports] keyword.  Fewer than N2 [Begin Pole-Residue Data] / [End Pole-Residue Data] keyword pairs are allowed for cases when certain matrix elements are zero.  Each occurrence of this keyword pair must have a unique pair of index numbers.  For example, a 4-port model with a full matrix description would have 16 of these keyword pairs with the following indices:

[Begin Pole-Residue Data]	(1,1)

[Begin Pole-Residue Data]	(1,2)

[Begin Pole-Residue Data]	(1,3)

[Begin Pole-Residue Data]	(1,4)

[Begin Pole-Residue Data]	(2,1)

…

[Begin Pole-Residue Data]	(4,4)

Subparameter Delay is optional, but it is not permitted for Y and Z-parameters (it is only allowed for S-parameters).  In S-parameter models it may be present (or absent) within any number of [Begin Pole-Residue Data] / [End Pole-Residue Data] keyword pairs.  Its unit is seconds.

Subparameter Asymptote is optional, but it is not permitted for S-parameters (it is only allowed for Y and Z-parameters).  In Y and Z-parameter models it may be present (or absent) within any number of [Begin Pole-Residue Data] / [End Pole-Residue Data] keyword pairs.  Asymptote defines the factor K used in the asymptotic component K*(i*frequency) that linearly grows with frequency, where i = √(-1), and K = (2*π)*Linf, where Linf is an equivalent inductance for Z-parameters, or K = (2*π)*Cinf, where Cinf is an equivalent capacitance for Y-parameters.  Since frequency is defined in Hz, the coefficient K has the unit of henry/Hz for Z-parameters and farad/Hz for Y-parameters.  The unit for the asymptotic component K*(i*frequency) is ohms for Z-parameters and siemens (1/ohms) for Y-parameters.

Subparameter Residue_at_infinity is required.  It defines the value of A1m, the normalized residue for the real pole αm at infinity.  The unit for Residue_at_infinity depends on the parameter type.  For S-parameters it is unitless, for Z-parameters it is ohms, for Y-parameters it is siemens (1/ohms).

The Number_of_data_lines subparameter is required.  The value of Number_of_data_lines is equal to the number of pole-residue data lines that follow.  The value of Number_of_data_lines is not required to be the same within each [Begin Pole-Residue Data] / [End Pole-Residue Data] keyword pair in a given model file.  The Number_of_data_lines subparameter is followed by the pole-residue data lines.  Each line contains four numbers corresponding to αm, ωm, A1m and A2m, as described above.

Keyword:	[End Pole-Residue Data]

Required:	The [End Pole-Residue Data] keyword is required for each [Begin Pole-Residue Data] keyword, otherwise it is not permitted.

Description:	The [End Pole-Residue Data] keyword indicates the end of the pole-residue data.

Examples:

! This example is for S-parameter models

! for which Asymptote is not permitted

[Begin Pole-Residue Data] (1,1)

Delay = 1.26351e-09

Residue_at_infinity = 0.321123423421

Number_of_data_lines = 	35

1.60981891306855e+08	6.03830005978569e+09	-2.15363238798792e-06	1.96534688582861e-05

2.93321810887676e+09	1.91770843721616e+09	-1.05426912887832e+01	-8.82630433918342e+00

1.23990373548953e+08	4.39994357143840e+09	1.25728612812034e-05	2.13669372820529e-05

1.57193681614524e+08	3.10437931944199e+09	5.10972708034658e-05	-1.15663907003945e-05

2.83363448768380e+07	2.10218276754607e+09	3.07314704488451e-06	5.16091015967049e-06

…

5.23409852743859e+06	1.34534534593845e+07	3.07314704488451e-06	5.16091015967049e-06

[End Pole-Residue Data]

! This example is for Y or Z-parameter models

! for which Delay is not permitted

[Begin Pole-Residue Data] (1,1)

Asymptote = 0.83754e-12

Residue_at_infinity = 0.321123423421

Number_of_data_lines	 = 35

1.60981891306855e+08	6.03830005978569e+09	-2.15363238798792e-06	1.96534688582861e-05

2.93321810887676e+09	1.91770843721616e+09	-1.05426912887832e+01	-8.82630433918342e+00

1.23990373548953e+08	4.39994357143840e+09	1.25728612812034e-05	2.13669372820529e-05

1.57193681614524e+08	3.10437931944199e+09	5.10972708034658e-05	-1.15663907003945e-05

2.83363448768380e+07	2.10218276754607e+09	3.07314704488451e-06	5.16091015967049e-06

…

5.23409852743859e+06	1.34534534593845e+07	3.07314704488451e-06	5.16091015967049e-06

[End Pole-Residue Data]

Keyword:	[Begin Common Poles Data]

Required:	The [Begin Common Poles Data] keyword is not permitted prior to Version 3.0 files; it is not permitted for parameter types G and H, and it is not permitted when the [Begin Pole-Residue Data] keyword or the [Network Data] keyword is present.  Otherwise, the [Begin Common Poles Data] keyword is required optional.

Description:	This keyword contains the coefficients of the common poles from the pole-residue representation of the network data and is used together with the coefficients of the residues defined in the [Begin Residues Data] keyword.

Sub-Param:	Number_of_data_lines

Usage rules:	Only one [Begin Common Poles Data] / [End Common Poles Data] keyword pair is permitted in a model file.  Note that [Begin Common Poles Data] does not use indexing after the keyword because the set of poles is common for all matrix components.

When the pole-residue data is generated from Touchstone 1.0 or 1.1 Y or Z-parameters (in which case the Y or Z-parameters are normalized), the Touchstone 1.0 or 1.1 Y or Z-parameter data from which the pole-residue data is generated shall be unnormalized first by the model maker.

The Number_of_data_lines subparameter is required.  The value of Number_of_data_lines is equal to the number of common poles data lines that follow and shall match the value of the Number_of_data_lines subparameter in the [Begin Residues Data] keywords.  (Consequently, the value of Number_of_data_lines must be the same within each [Begin Residues Data] / [End Residues Data] keyword pair in a given model file.)  The Number_of_data_lines subparameter within the [Begin Common Poles Data] / [End Common Poles Data] keyword pair is followed by the common poles data lines.  Each line contains two numbers corresponding to αm, ωm, as described above.

Keyword:	[End Common Poles Data]

Required:	The [End Common Poles Data] keyword is required when the [Begin Common Poles Data] keyword is present, otherwise it is not permitted.

Description:	The [End Common Poles Data] keyword indicates the end of the common poles data.

Example:

[Begin Common Poles Data]

Number_of_data_lines	 = 35

1.60981891306855e+08	6.03830005978569e+09

2.93321810887676e+09	1.91770843721616e+09

1.23990373548953e+08	4.39994357143840e+09

1.57193681614524e+08	3.10437931944199e+09

2.83363448768380e+07	2.10218276754607e+09

…

5.23409852743859e+06	1.34534534593845e+07

[End Common Poles Data]

Keyword:	[Begin Residues Data]

Required:	The [Begin Residues Data] keyword is required when [Begin Common Poles Data] is present, otherwise it is not permitted.

Description:	This keyword contains the coefficients of the residues from the pole-residue representation of the network data and is used together with the coefficients of the common poles defined in the [Begin Common Poles Data] keyword.

Sub-Params:	Asymptote, Residue_at_infinity, Number_of_data_lines

Usage rules:	The [Begin Residues Data] keyword is followed by at least one white space and two integer numbers, separated by a comma, and enclosed in a pair of open and close parentheses.  The integer numbers (i, j) denote indexing to define which matrix component is described by the data that follows.  Both indices range from 1 to N, where N is the model’s number of ports given in the [Number of Ports] keyword.

When the pole-residue data is generated from Touchstone 1.0 or 1.1 Y or Z-parameters (in which case the Y or Z-parameters are normalized), the Touchstone 1.0 or 1.1 Y or Z-parameter data from which the pole-residue data is generated shall be unnormalized first by the model maker.

The [Begin Residues Data] / [End Residues Data] keyword pair is repeated at least N times (for a diagonal matrix) or up to N2 times (for a full matrix) in a model file, where N is given in the [Number of Ports] keyword.  Fewer than N2 [Begin Residues Data] / [End Residues Data] keyword pairs are allowed for cases when certain matrix elements are  zero.  Each occurrence of this keyword pair must have a unique pair of index numbers.  For example, a 4-port model with a full matrix description would have 16 of these keyword pairs in the same manner as described for the [Begin Pole-Residue Data] keyword.

Subparameter Asymptote is optional, but it is not permitted for S-parameters (it is only allowed for Y, and Z-parameters).

Subparameter Residue_at_infinity is required.  It defines the value of am, the normalized residue for the real pole αm at infinity.  The unit for Residue_at_infinity depends on the parameter type.  For S-parameters it is unitless, for Z-parameters it is ohms, and for Y-parameters it is siemens (1/ohms).

The Number_of_data_lines subparameter is required.  The value of Number_of_data_lines is equal to the number of pole-residue data lines that follow and shall match the value of Number_of_data_lines in the [Begin Common Poles Data] / [End Common Poles Data] keyword pair.  (Consequently, the value of Number_of_data_lines must be the same within each [Begin Residues Data] / [End Residues Data] keyword pair in a given model file.)  The Number_of_data_lines subparameter within the [Begin Residues Data] / [End Residues Data] keyword pair is followed by the residues data lines.  Each line contains two numbers corresponding to am and bm, as described above.

Keyword:	[End Residues Data]

Required:	The [End Residues Data] keyword is required for each [Begin Residues Data] keyword, otherwise it is not permitted.

Description:	The [End Residues Data] keyword indicates the end of the residues data.

Examples:

! This example is for S-parameter models

! in which Asymptote is not permitted

[Begin Residues Data] (1,1)

Residue_at_infinity = 0.321123423421

Number_of_data_lines	 = 35

-2.15363238798792e-06	1.96534688582861e-05

-1.05426912887832e+01	-8.82630433918342e+00

1.25728612812034e-05	2.13669372820529e-05

5.10972708034658e-05	-1.15663907003945e-05

3.07314704488451e-06	5.16091015967049e-06

…

3.07314704488451e-06	5.16091015967049e-06

[End Residues Data]

! This example is for Y or Z-parameter models

[Begin Residues Data] (1,1)

Asymptote = 0.83754e-12

Residue_at_infinity = 0.321123423421

Number_of_data_lines = 	35

-2.15363238798792e-06	1.96534688582861e-05

-1.05426912887832e+01	-8.82630433918342e+00

1.25728612812034e-05	2.13669372820529e-05

5.10972708034658e-05	-1.15663907003945e-05

3.07314704488451e-06	5.16091015967049e-06

…

3.07314704488451e-06	5.16091015967049e-06

[End Residues Data]

BACKGROUND INFORMATION/HISTORY:

The original Touchstone 2.0 [Mixed Mode Order] keyword could be applied to pole-residue data as well, if so desired.  However, it’s unlikely that time domain simulation (for which the fitted poles are created) can be conducted in a basis that does not represent actual node voltages and branch currents.

Another keyword pair might be invented for reciprocal models, which could further reduce the matrix size.

Address upper/lower matrix symmetry (as opposed to zero elements).

Technical background information:

Hm(s)=12Am-iBm1+s/(αm+iωm)+Am+iBm1+s/(αm-iωm)

Ym(s)=12am-ibm1+s/(αm+iωm)+am+ibm1+s/(αm-iωm)

There are a few reasons we (Mentor/Siemens) prefer the PLS format for S-parameters instead of the Foster format:

- It is problematic to write the expression that defines the S-parameter matrix component in the Foster form because it may require hundreds of complex and real poles.  The PLS format has a fixed number of columns (4), which are the real/imaginary part of frequency and the real/imaginary part of the coefficient).  This greatly simplifies the parser and the writer code.

- We’d like to see actual contribution from each pole (or conjugate pair) into the transfer function.  In equation (2) below the factor A shows how much a particular pole contributes into DC value.  The coefficients A, B have the same dimension as the parameters, i.e., dimensionless for S, or impedance/admittance for Z/Y, but denominator is dimensionless.  In the Foster format, denominator is measured in rad/sec therefore the factors in the numerator bear this unit as well and are not normalized.  That is, in the Foster format, the coefficients at the larger poles are typically very large as well, so that their contribution into DC equals their ratio.

- In the PLS format the pole frequencies are defined in Hz, not rad/s, similar to the sampled Touchstone data.  This simplifies interpretation of the values.  Since the denominator is dimensionless, the poles could be easily rescaled (into rad/s) without touching the coefficients, which is not possible in Foster format.

- There is no difference in the format between the real pole or a complex conjugate pair of poles (except that for a real pole the imaginary parts of the pole frequency or the coefficient are zero), but the Foster format assumes different number of summands.

- Delay multiplier could be added by defining it as a single value, e.g., an insertion loss.

Of course, the Foster format could always be converted into PLS format, and in many cases an inverse is possible as well.  The main reason was convenience and performance when reading and writing complicated PLS files.  Some of these reach tens of MB of disk space, especially for multi-port S-parameter models.  Ultimately, the spec must be convenient to the users / EDA companies and allow flexibility.

The PLS format is closely related to the Foster formula, which defines partial fraction expansion of the frequency dependence.  The Foster format defines the function of the complex frequency s as:

.			(1)

(1) is a sum of the constant component k0, the linearly growing imaginary component k1s, and the sum of partial fractions each one being a ratio of the complex coefficient and the difference between the complex frequency and the pole , which could be real or complex.

Note that (1), by its structure, doesn’t enforce realness and stability of the corresponding time domain responses.  Realness requires that the summands corresponding to the complex poles always come in complex conjugate pairs, which makes the corresponding time domain response real.  Stability means that with bounded input the system produces a bounded output.  The latter requires that the real part of the poles in (1) is negative.

The proposed PLS format enforces these conditions.  In particular, the component that corresponds to a pair of the complex conjugate poles, is represented as:

,		(2)

where and are complex numbers that define the normalized residue and the complex pole, and , are their complex conjugates.  Realness and stability are enforced by the PLS format as follows:

Every row of data contains four numbers, , which define the real and imaginary parts of the complex pole and the complex coefficient.  The first two define the complex frequency Qm as:

With that:

- Realness is enforced “by design”, because these four numbers in each data row make a pair of summands with complex-conjugate frequencies and coefficients.

- Stability is enforced by the requirement that .  Indeed, the denominator of the first summand in (2) becomes zero when , where is a mathematical definition of the complex pole.  The real part of the stable polemust be negative, which happens when .  For any complex conjugate pair of poles the value of specified in the PLS format is positive.  This is because we have the freedom to choose the sign of this parameter any way we please (since they go in complex-conjugate pairs, one of them will have a positive).  Of course, after we define positive values for  and , the values  and  can’t be chosen arbitrarily.  In case of a real pole, we have , .

We can find the Foster form or as follows:

,		(3)

where  and .

Expression (2) can define a single real pole when and , which makes and .

Conversion of parameters:

PLS to FOSTER:  ,

FOSTER to PLS:  ,