TOUCHSTONE ISSUE RESOLUTION DOCUMENT (TSIRD)

TSIRD NUMBER:	(Draft_35)

ISSUE TITLE:		Standardized Port Mapping

REQUESTOR:		Michael Mirmak, Intel Corp.

Arpad Muranyi, Siemens EDA

Walter Katz, MathWorks

Randy Wolff, Siemens EDA

DATE SUBMITTED:	{for administrative use}

DATE REVISED:	{for administrative use}

DATE ACCEPTED:	{for administrative use}

DEFINITION OF THE ISSUE:

When using Touchstone data to define models, it is essential to know how the ports of that Touchstone data set should be connected in a design.  This implies that the “port ordering” or “port mapping” information must be supplied along with or inside the Touchstone file.  Currently, this information is sometimes provided in a separate file, but increasingly more often it is included in the Touchstone file as a commented “header” section.

When a Touchstone file contains many ports, connecting its ports manually becomes a tedious and error-prone job.  Consequently, automation is highly desired.  EDA vendors developed several different formats throughout the years, but the problem is that they are all different with various features and capabilities, making parsing them difficult.  Additionally, in most cases this information is provided as a commented header section in the Touchstone file, and strictly speaking, comments are non-parsable, arbitrary text.

The goal of this proposal is to provide a standardized port mapping format for Touchstone files so that all relevant information could be included in it, and all EDA tools could parse them reliably.

SOLUTION REQUIREMENTS:

Port Mapping structures and data must be able to support:

- Reliably, hooking up a Touchstone File in a simulation.

- Automate creating a schematic symbol.

- Automate generating/verifying [Interconnect Model]s in .ibs files.

- Automate generating/verifying [EMD Model]s in .emd files.

- Automate generating/verifying [C Comp Model]s in .ibs files.

- Enable Touchstone File viewers to generate mixed-mode network parameters for differential ports.

- Report Sij Status (Measured | Simulated | TBD | Placeholder)

- Automatic generation of test probe locations for test equipment.

- Support swathing

- Ability to add new user defined parameters

The Touchstone specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Define two new keywords | [Begin Port Map] / [End Port Map] |
| - Define connections to IBIS structures | EMD, IBIS Interconnect, C_comp_model, Ts4file |
| - Define structures for connectivity as well as for data and schematic display | Physical, Logical, Diff_port, and Net structures |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

Table 2: Touchstone Keywords Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| [Begin Port Map] / [End Port Map] | New | New keywords are self-contained and only interact with the [Number of Ports] keyword |

PROPOSED CHANGES:

Add the following keywords to the Touchstone specification in the appropriate location:

Keyword:	[Begin Port Map] / [End Port Map]

Required:	Optional; illegal prior to Version 3.0.

Description:	Identifies and associates physical connection points, schematic labels, and/or measurement locations with the port numbers specified in a Touchstone file.

Touchstone data is commonly used in industry for a variety of purposes, including documentation of measured behavior of a component or circuit element, as well as for electrical modeling of behavior when connecting the modeled element to other elements (for example, to model the electrical behavior of a connector to be used in simulation when connected to electrical models of printed circuit board traces, board vias, and the like).

Except for the limited cases defined here, Touchstone file contents are only data and do not themselves imply how that data should be used in any wider context.  Touchstone data can be used for both circuit analysis (e.g., SPICE simulation using Kirchoff’s laws) and wave propagation analysis (e.g., electromagnetic field simulation using Maxwell’s/Heaviside’s equations), but the use of the Touchstone format itself is insufficient to ensure that the representation of an entire system is correct.  Care must be taken to ensure that Touchstone model data, depending on context, does not compromise the loop behavior of the overall circuit being simulated or the wave propagation of the overall system being simulated.  Depending on whether circuit behavior or wave propagation is being studied, the concepts of return path, reference node(s), and reference plane(s) may be different; not every Touchstone data set may be appropriate to use in all contexts.  How return paths and/or referencing are handled when the Touchstone data is used in simulation is entirely up to the user of the data and/or the EDA simulation software.

Sub-Params:	Bus_label, Data_usage, Group, Port, Signal_name, Source, Swathing, Symbol_leftside, Symbol_topside, Symbol_rightside, Symbol_bottomside, UD, Unit

Usage rules:	If present in the file, the [Begin Port Map] / [End Port Map] keyword pair shall immediately follow the [Number of Ports] keyword and its associated data.  Each subparameter shall start on a new line.  Only one [Begin Port Map] / [End Port Map] keyword pair is permitted per Touchstone file.

Data_usage is an optional subparameter which takes a single string argument.  The subparameter identifies that the Touchstone file represents part of a larger structure for simulation.  The permitted string arguments are listed below.  Only one Data_usage entry is permitted per [Begin Port Map] / [End Port Map] keyword pair.

“IBIS_Interconnect” – this indicates that the Touchstone data represents part of an IBIS [Interconnect Model].

“EMD” – this indicates that the Touchstone file represents part of an IBIS [EMD Model].

“C_comp_model” – this indicates that the Touchstone file represents an IBIS [C Comp Model].

“Ts4file” – this indicates that the Touchstone file represents the complex impedance of a buffer as described in an IBIS AMI file.

If Data_usage is present for a given Port Map, a Physical name-value pair (see below) shall be present for each Port in that Port Map.  Data_usage, if present, shall be the first subparameter to appear after the [Begin Port Mapping] keyword.

The Source subparameter is optional.  It is followed by a file name identifying the original file from which the Touchstone data was obtained or derived.  The file name may be an ODB++ board data base, a schematic file, a simulation model, or any other data file.  As the subparameter is purely informational, the data file is not required to be present on the computer system containing the associated Touchstone file.  The filename shall be enclosed within double quotation marks and may include spaces.  Source, if present, shall appear before any Port subparameter lines.

The Bus_label subparameter is optional if Data_usage “EMD” or “IBIS_Interconnect” is present for a given Port Map; it is prohibited otherwise.  Inclusion of the Bus_label subparameter facilitates automated generation of connectivity syntax in the calling EMD file, and/or checking the consistency between the Touchstone file and calling EMD file.  The Bus_label subparameter names collections of power or ground pins that are shorted together.  Bus_label may appear multiple times for a [Begin Port Map]/[End Port Map] pair, with each Bus_label subparameter appearing on a separate line.  These lines are placed after the last Port subparameter (see below) and before the [End Port Map] keyword.  The Bus_label subparameter is followed by the string “Physical” and a string naming the Bus_label described by the line; these strings are separated by white space.  The Bus_label Physical <name> string is followed by white space, a “(“, followed by a list of Physical terminal names separated by white space, and terminated by a “)”.  Bus_label names shall only contain alphanumeric characters, the underscore (“_”) character, the period (“.”) character, or the colon (“:”) character; the latter two have a special meaning (see below).

Bus_label can be only used for Physical terminal names associated with Ports of Type P or the references for Ports of Type P or Type S (see below).  A line termination sequence without a “)” continues the list of Physical names on the next line.  Physical terminal names shall not contain white space.  Physical terminal names used with the Bus_label subparameter may use the wildcard character “*” to efficiently combine multiple rail connections, as described below.  Bus_label subparameter lines, if present, shall appear after any Port subparameter lines but may appear in any position relative to other subparameters placed after the Port subparameter lines.953953

When used with Data_usage “EMD”, if the list of Bus_label Physical terminal names includes terminal names containing reference designators and pins (e.g., “U1.3”), these refer to individual components and pins within the associated EMD file.  Pins on the entire EMD module may be used as Bus_label Physical terminal names if they are preceded by the string “EMD” (e.g., “EMD.P1”).

The Group subparameter is optional.  The Group subparameter names collections of power or ground pins, which are shorted together.  Group may appear multiple times for a [Begin Port Map]/[End Port Map] pair, with each Group subparameter appearing on a separate line.  These lines are placed after the last Port subparameter (see below) and before the [End Port Map] keyword.  The Group subparameter is followed by the string “Physical” and a string naming the Group described by the line; these strings are separated by white space.  The Group Physical <name> string is followed by white space, a “(“, followed by a list of Physical terminal names separated by white space, and terminated by a “)”.  Group names shall only contain alphanumeric characters and the underscore (“_”) character; the period (“.”) character and the colon (“:”) character are prohibited in Group names.

Group can be only used for Physical terminal names associated with Ports of Type P or the references for Ports of Type P or Type S (see below).  A line termination sequence without a “)” continues the list of Physical names on the next line.  Physical terminal names shall not contain white space.  Physical terminal names used with the Group subparameter may use the wildcard character “*” to efficiently combine multiple rail connections, as described below.  Group subparameter lines, if present, shall appear after any Port subparameter lines but may appear in any position relative to other subparameters placed after the Port subparameter lines.  Group is prohibited with Data_usage “C_comp_model”, “EMD”, “IBIS_Interconnect”, and “Ts4file”.

The Signal_name subparameter is optional if Data_usage “EMD” is present for a given Port Map; it is prohibited otherwise.  Inclusion of the Signal_name subparameter, in combination with Bus_label, enables efficient and unambiguous linking to, or generation of, EMD files which may use either Signal_names or Bus_labels for rail connections to individual components.  Signal_name may appear multiple times for a [Begin Port Map]/[End Port Map] pair, with each Signal_name subparameter appearing on a separate line.  These lines are placed after the last Port subparameter (see below) and before the [End Port Map] keyword.  The Signal_name subparameter is followed by the string “Physical” and a string naming the collection of  described by the line; these strings are separated by white space.  The Signal_name Physical <name> string is followed by white space, a “(“, followed by a list of Physical terminal names separated by white space, and terminated by a “)”.  Signal_name names shall only contain alphanumeric characters, the underscore (“_”) character, and the colon (“:”) character; note that the latter has a special meaning (described below).  The period character (“.”) is prohibited in Bus_label names.

Signal_name shall be only used for Physical terminal names associated with Ports of Type P or the references for Ports of Type P or Type S (see below).  A line termination sequence without a “)” continues the list of Physical names on the next line.  Physical terminal names shall not contain white space.  Signal_name subparameter lines, if present, shall appear after any Port subparameter lines but may appear in any position relative to other subparameters placed after the Port subparameter lines.  Wildcards (see below) are prohibited with Signal_name.

The Swathing subparameter is optional.  It is followed by a string, defining a Schema name.  Connector companies may create Touchstone files for a portion or “slice” of a connector.  These “slices” may be combined algorithmically according to the schema to create a virtual Touchstone file that represents all the pins of the connector.  Reserved Schemas, defined in a separate specification, are supported, as are other, model-maker-defined Schema values.  The Swathing subparameter, if present, shall appear after any Port subparameter lines but may appear in any position relative to other subparameters placed after the Port subparameter lines.

The Symbol_leftside, Symbol_rightside, Symbol_topside, and Symbol_bottomside subparameters are intended to provide guidance on the placement of ports on a circuit element or symbol representing the Touchstone data in a larger schematic diagram (e.g., so that symbols may be automatically generated by an EDA tool). These subparameters shall appear on separate lines after the last Port subparameter and before the [End Port Map] keyword, but may appear in any position relative to other subparameters placed after the Port subparameter lines.

For these subparameters, their Port number arguments need not follow a particular numerical order, but the sequence of Port numbers shall correspond to the physical placement of the port numbers in the symbol in the orientation described (e.g., for Symbol_leftside, Port arguments 1, 3, 7, 10, 6 defines Port 1 as being the topmost port and Port 6 as being the bottommost port).

If any Symbol_leftside, Symbol_rightside, Symbol_topside, or Symbol_bottomside subparameters are present then any Port number shall appear once and only once in these subparameters.

Note that Symbol placement of reference terminals in a schematic is left up to the EDA software,

The Symbol_leftside subparameter is optional.  The Symbol_leftside subparameter string is followed by a list of Port numbers defining terminals to be shown on the left side of a schematic symbol, ordered as listed from top to bottom, with each number separated by commas.  Whitespace separation is optional.

The Symbol_rightside subparameter is optional.  The Symbol_rightside subparameter string is followed by a list of Port numbers defining terminals to be shown on the right side of a schematic symbol, ordered as listed from top to bottom, with each number separated by commas.  Whitespace separation is optional.

The Symbol_topside subparameter is optional.  The Symbol_topside subparameter string is followed by a list of Port numbers defining terminals to be shown on the top side of a schematic symbol, ordered as listed from left to right, with each number separated by commas.  Whitespace separation is optional.

The Symbol_bottomside subparameter is optional.  The Symbol_bottomside subparameter string is followed by a list of Port numbers defining terminals to be shown on the bottom side of a schematic symbol, ordered as listed from left to right, with each number separated by commas.  Whitespace separation is optional.

For all the symbol subparameters above, the Port number arguments need not follow a particular numerical order, but the sequence of Port numbers shall correspond to the physical placement of the port numbers in the symbol in the orientation described (e.g., for Symbol_leftside, Port arguments 1, 3, 7, 10, 6 defines Port 1 as being the topmost port and Port 6 as being the bottommost port).

If any Symbol_leftside, Symbol_rightside, Symbol_topside, or Symbol_bottomside subparameters are present then any Port number shall appear once and only once in these subparameters.

The Unit subparameter is optional in most cases (see below).  The “Unit” string shall be followed by a string declaring the unit for physical coordinates.  The only permitted arguments are “m”, “mm”, “microns”, “inches”, and “mils”, for meters, millimeters, microns, inches, and mils, respectively.  The Unit subparameter shall be present when any coordinate-based definitions are used for Physical port descriptions.  The Unit subparameter, if present, shall appear after any Port subparameter lines but may appear in any position relative to other subparameters placed after the Port subparameter lines.

The UD (for user-defined) subparameter is optional.  Note that user-defined information may be declared after the Port declarations or in Port lines.  When used after the Port declarations, “UD” is considered a subparameter and UD data shall be declared using lines, without parentheses, where an arbitrary number of strings may follow the “UD” string and whitespace.  For UD data on Port lines, the data shall follow the string “UD” and whitespace within the same pair of parentheses.  UD information in either location effectively will be ignored by EDA tools but may be used to track how the data describing the port behavior was generated, and/or how the Port data was physically measured.  The UD subparameter, if present, shall appear after any Port subparameter lines but may appear in any position relative to other subparameters placed after the Port subparameter lines.

The Port subparameter is required.  The [Begin Port Map] keyword shall be followed by as many Port subparameter lines as the number of ports defined by the [Number of Ports] keyword.  The entire content of each Port subparameter shall be on a single line.  Note that a Port for connectivity and measurement purposes always denotes a pair of terminals; the reference terminal may be explicitly identified for those purposes using the syntax defined below.

The Port subparameter shall be followed by whitespace and an integer number, indicating which port is described by the line.  Note that the listed Port numbers shall include all positive, non-zero integers from one (“1”) to N inclusive, where N is equal to the Number of Ports entry in the same Touchstone file.    Ports may be listed in any order so long as no Port number is duplicated.  The integer port number is followed by whitespace, then (<name> <value>) pairs, hereafter called “name-value pairs”, enclosed in parentheses as shown.  In addition, the character “-“ may be used before specific name-value pairs as described below.  Unless otherwise noted (e.g., when “Data_usage” is present, the Physical name-value pair shall also be present), all Port name-value pairs are optional.  Names used on the Port lines are case-sensitive.

The strings below are reserved and shall not be used as values in name-value pairs for the Port subparameter:

Diff_port

Logical

Net

Physical

Probe

Side

Type

UD

The Port subparameter string and its associated Port number shall appear first, at the beginning of each line.  Name-value pairs may appear in any order thereafter.

Several name-value pairs refer to the port as a unit.  These name-value pairs may only appear once per Port line.  These are:

Diff_port

Side

Type

UD

Several name-value pairs define properties of individual terminals; they apply to terminals within the port, not to the port as a whole.  These name-value pairs may appear twice per Port line; if one of these name-value pairs appears twice for a specific port, one of them shall be preceded by the “-“ character.  The terminal-oriented names are:

Logical

Net

Physical

Probe

The “-“ symbol is used to associate a terminal-oriented name-value pair with the reference terminal of the Port (see the “Referencing” section below).  The “-“ symbol may also be used with a Physical Group of terminals as detailed above.  Use of these name-value pairs without the “-“ symbol indicates the pair refers to the measured terminal, not its reference.

An example of the structure of the Port subparameter is shown below:

Port n (name value) (name value)  -(name value)  (name value) -(name value)…

where n is the non-zero positive integer port number, and name is one of the reserved Port names listed above.

The port-oriented names Diff_port, Side, and Type shall not appear in a name-value pair identified with “-“.

Note that the “-“ symbol shall be followed without whitespace by the “(“ character in its associated name-value pair.  The parentheses defining a name-value pair may be separated from other parts of the Port line with whitespace.

Name-Value Pair Syntax Examples:

Port 1 (Physical U7.1)

Port 2 (Type S) (Physical U7.2)

Port 3 (Physical U7.4) (Logical DQS+)  (Diff_port 4)  -(Physical U7.3)

Port 4 (Physical U7.5) (Logical DQS-)   (Diff_port 3)  -(Physical U7.6)

Port Name-Value Pair Rules

Note that all name-value pairs described below are optional.  None of the name-value pairs described below preclude the use of any other name-value pair for the same Port assignment (i.e., for the same Port number).

Diff_port: 	The Diff_port value identifies the given Port as one-half of a differential pair and defines the Port number of the complementary port.  Polarity is not directly indicated, since that will be established by how the ports are used in simulation.  Each port declared as a Diff_port shall have a corresponding (complementary) Diff_port declaration.  Note that Diff_port is primarily used for data visualization where, for example, plotting of differential or mixed-mode data for direct, convenient comparison against an industry specification may be automatically performed by an EDA tool.  The Diff_port value refers to the port as a whole.

Logical: 	The Logical value identifies how the Touchstone ports may be used in a schematic symbol, to imply a specific function or usage across different contexts.  For example, a bipolar junction transistor terminal may use the Logical value “emitter” to identify the usage or behavior of the terminal; this is distinct from Physical values (e.g., “B1”), which may have no relation to the functionality of the specific terminal.  Note that connectivity between components in larger structures (e.g., a simulation netlist) is not defined by Logical values.  The Logical name-value pair refers to an individual terminal of the associated Port and is expected to be used primarily as a label.

Net: 		The Net value identifies a collection of terminals for generation of simulation or schematic topologies.  All ports that have the same Net value are part of the same electrical signal path (this does not mean they share the same terminal, as in a short); note that this is called an “extended net” in some industry contexts.  Net value identifiers and Physical value identifiers (described below) are the primary mechanisms by which connectivity information between the ports of a Touchstone data set used as a model and its surroundings (e.g., other Touchstone files, SPICE component descriptions, IBIS packages, etc.) may be provided.  Nevertheless, Touchstone files do not name the IBIS-related files, SPICE netlists, etc. that describe the components to which the ports of the Touchstone data set may be connected.  Instead, the connectivity is defined in the external calling file (e.g., the SPICE netlist, the [Interconnect Model] keyword of an IBIS file, the [EMD Model] keyword of an EMD file, the Ts4file parameter in an AMI file, etc.) that instantiates the Touchstone file with its appropriate syntax.  The information contained in the Touchstone port mapping section facilitates automated generation of such connectivity syntax in the calling files, and/or checking the consistency between the Touchstone file and calling files.  The Net name-value pair refers to an individual terminal of the associated Port.

Physical: 	The value for the name “Physical” is a string that describes the physical location of a Port terminal for use in connection to other devices, circuit elements, etc. (for example, pin name “B1” on a connector).  This string could be used, for instance, in a circuit simulator netlist to determine how the circuit element described by the Touchstone data is to be connected to other terminals.  The value following the Physical name is not intended to identify the function of the terminal (the “Logical” name is intended assigning functional labels to terminals).  A Physical name-value pair refers to an individual terminal of the associated Port.

To clearly identify terminals on different components, a syntax using the period (".") character is commonly used in industry and is maintained here.  This is most common when referring to specific pins on components defined in a PCB schematic or netlist.  The component is identified using a short alphanumeric string, with the component usually beginning with a capitalized letter referring to a common class of component.  For example, connectors may begin with the letter "J", while integrated circuits may begin with the letter "U".  Specific instances of a component will be numbered (e.g., "U2"), and the complete component reference called a "reference designator" or "refdes" ("designator" is used in this specification for clarity).  An individual pin name would follow the reference designator and the period character, as in "U2.5".

Outside of the schematic context, similar syntax is used, particularly in relation to IBIS Interconnect supply rail naming (e.g., "Gnd_clamp_ref.5", where "5" is a pin name) or physical locations of IBIS Interconnect structures (e.g., "pin.AB2").  Similar syntax is used for IBIS EMD structures, such as EMD Designator Pins (“U23.5”).

A single name may be used to refer to a collection of Physical terminals on Type P ports (or for references of Type P or Type S ports), through Group or Bus_label, as defined below.  Note that Bus_label is only permitted with IBIS-related Data_usage “EMD” and “IBIS_Interconnect”, while Group is a similar concept that only applies to non-IBIS Touchstone structures.  The syntax here for both is nearly identical: the string "Bus_label" or "Group" is followed by a colon (":") character, and then the name of the collection of EMD pins or Physical terminals, respectively.  Reference designators may be used when the Bus_label or Group name only refers to a group of pins on a specific component.  In these cases, the "Bus_label" or "Group" string is separated from the reference designator and the actual name string by the colon character.

The rules defined below can be followed to ensure correct connectivity, including through automation, for the circuit described by the Touchstone network data using Physical name-value pairs to external IBIS [Interconnect Model]s, IBIS [EMD Model]s, IBIS [C Comp Model]s, and IBIS AMI Ts4file analog circuits.  The value provided in the various <name>, <pin_name>, and <designator> fields shown below shall match the value used in the IBIS or EMD file referencing this Touchstone file.  For this reason, if Data_usage is present for a given Port Map, a Physical name-value pair shall be present for all Ports in that Port Map.  Note that EMD files may be generated from the Touchstone data through automation using the Port Map.

The asterisk character (“*”) may be used as a wildcard, but only in three specific contexts:

- As part of the Bus_label or Group subparameter, to represent multiple component designators with the same pins on each component shorted together (e.g., “Group Physical GND_Ux (*.2 *.11)”).

- As part of a Physical name in a Port line, in a Port Map with Data_usage “EMD” where Bus_labels or Signal_names are used for rails (Ports identified as Type P, or references for Ports of Type P or Type S).  As with Group, the wildcard is used in place of a designator to show that, across all components, all pins sharing a Bus_label or Signal_name are shorted together.

- As part of the Bus_label or Signal_name subparameter, used in place of a designator to show that, across all components, all pins sharing a Bus_label or Signal_name are shorted together.

Allowed Physical names for Data_usage “C_comp_model”:

- Buffer_I/O (this is required for C_comp_model)

- Buffer_I

- Pullup_ref

- Pulldown_ref

- Power_clamp_ref

- Gnd_clamp_ref

- Ext_ref

Note that Bus_label, Group, Signal_name, and wildcards are prohibited for use with Data_usage “C_comp_model”.

Rules for Data_usage “EMD”:

For Type S Ports, the Physical name shall follow one of the structures listed below:

- an EMD <pin_name>

- <designator>.<pin_name>

For Type P Ports and references for Type P and Type S Ports, the Physical name shall follow one of the formats listed below.

- Bus_label:<Designator>:<Bus_label name>

- Bus_label:EMD:<Bus_label name>

- Signal_name:<Designator>:<Signal_name>

- Signal_name:EMD:<Signal_name>

- Bus_label:*:<Bus_label name>

- Pin_name:EMD.<Pin_name>

Note that the period (“.”) character is used in reference to pin names, while the colon (“:”) character is used with Bus_labels and Signal_names.  A Bus_label can be used within a single designator or, when the wildcard (“*”) is present, across all designators.  If Bus_label is used without designator information, the Bus_label is assumed to be associated in the corresponding EMD file with the EMD pin interface.  Note that Group is prohibited for use with Data_usage “EMD”.

Bus_label and Signal_name strings are used with Data_usage “EMD” to ensure unambiguous connections between subsets of larger rail networks.  As defined in IBIS, EMD connections to rail networks may be defined by names in the “Bus_label” column of the [EMD Pin List] and [Designator Pin List] keywords; where such Bus_label names are absent in these EMD keywords, the Signal_name column entries are used to define the rail network.  Such rail network definitions may occur at the level of the entire EMD module or at the level of the individual components within the module.  Within Port Maps used with EMD, the Bus_label subparameter is used to define new collections of rail connections, but also to connect to existing ones defined in the associated EMD.  To distinguish between these, the separate “Bus_label” parameter, with a Bus_label name followed by an EMD reference designator and the colon (“:”) character, is used to associate EMD Bus_labels with Ports using the same syntax as used in the Physical name-value pairs in the Port lines.  Similarly, the “Signal_name” string is used both in the Port lines as well as the separate “Signal_name” parameters, using either a designator followed by an EMD Signal_name, separated by a colon (“:”) character; or using the string “EMD” followed by a Bus_label name for the entire EMD module.   See Example 15 for a detailed illustration.

Rules for Data_usage “IBIS_Interconnect”:

For Type S Ports, the Physical name shall follow one of the structures listed below:

- pin.<pin_name>

- pad.<pin_name>

- buf.<pin_name>

These associate signal connection points (pin, die pad, buffer) with a pin name.  Note that Group, Signal_name, and wildcards are prohibited for use with Data_usage “IBIS_Interconnect”.

For Type P Ports, and for references of Type P or Type S Ports, the Physical name for individual pins and die pads shall be, respectively, in the form:

- pin.<pin_name>

- pad.<pin_name>

Individual rail connections to the buffer shall be in one of the following formats:

- Pullup_ref.<pin_name>

- Pulldown_ref.<pin_name>

- Power_clamp_ref.<pin_name>

- Gnd_clamp_ref.<pin_name>

- Ext_ref

- A_gnd

Rail connections on Type P Ports or the references of Type P or Type S Ports using Bus_labels follow the naming conventions below.

- Pullup_ref:Bus_label:<Bus_label name>

- Pulldown_ref:Bus_label:<Bus_label name>

- Power_clamp_ref:Bus_label:<Bus_label name>

- Gnd_clamp_ref:Bus_label:<Bus_label name>

- pin:Bus_label:<Bus_label name>

- pad:Bus_label:<Bus_label name>

- buf:Bus_label:<Bus_label name>

As with EMD, for IBIS Interconnect, the period (“.”) character is used in reference to pin names, while the colon (“:”) character is used in reference to Bus_labels.

Rules for Data_usage “Ts4file”:

Files identified as Data_usage “Ts4file” shall contain no more and no fewer than four (4) ports.  The usage and connectivity are defined by Ts4file parameters in the calling IBIS .ami file.  No terminal identified with “-“ is permitted for a file identified as Data_usage “Ts4file”.  By IBIS rules, Ts4file circuit references are assumed to be A_gnd.  Note that Bus_label, Group, Signal_name, and wildcards are prohibited for use with Data_usage “Ts4file”.

If Data_usage is not present, Physical rail-connected terminals of Ports of Type P, or references for Ports of Type P or Type S, may be collected into Groups.  Here, the string “Group” precedes a string identifier as part of a Physical name-value pair on the Port line (this includes Physical name-value pairs preceded by “-“).  A matching string identifier shall appear as part of the Group subparameter after the Port lines.  Note that Group is not supported for name-value types other than Physical.

Probe:	The Probe value identifies the location of the terminal where the Port data is captured, either through simulation or lab measurement.  Specifically, this is a set of coordinates identifying the terminal location.  The value argument to the “Probe” name shall be an X:Y:Z string, with each value, representing a coordinate, separated from other coordinates by the colon (“:”) character.  Whitespace separating the coordinate values and the colon character is permitted.  X and Y values shall be numeric (floating point) values and are assumed to define distances.  Because PCB system descriptions in Touchstone files may involve either a physical distance (e.g., the top of a connector on a PCB) Z may be a numeric (floating point) value to describe a distance, or to describe a layer within a PCB stackup, the Z value may be a non-negative integer or a string without whitespace.  The coordinate origin is arbitrary and not stated in the Touchstone file, but all coordinates used in a single Touchstone file port map shall use the same origin.  Note that Probe values may refer to a measurement fixture or system not described in the Touchstone file data.  This fixture or system may be distinct from any terminal information in Net and/or Physical used for connectivity to other components.

Probe coordinate values have no meaning for the purposes of connectivity, unlike Net and Physical values.

Side: 	The Side value identifies a collection of ports, to help define a functional orientation of the ports relative to other ports on the same component; this is a way to collect Logical functions according to Physical associations.  Interconnects described by Touchstone often have two sides; for instance, a connector linking a mainboard and daughterboard.  Ports on such a connector may therefore have Side values of “mainboard” and “daughterboard”.  A memory DQ route may have a Controller Side, and separate Sides for each memory device connected to that route.  Similarly, a cable assembly for a car may have four Sides (e.g., CPU, Brake, Engine, and Camera).  Note that, like Logical values, values used with Side should not be used for establishing or cross-checking physical connectivity; the Side value is primarily intended as a label.  The Side value refers to the port as a whole.

Type: 	The value following the name “Type” shall be either the S or the P character.  S stands for Signal, while P stands for Power (in other words, rail).  If not specified, the Type default is S.  The Type name-value pair refers to the port as a whole.

UD: 	For UD data on Port lines, the data shall follow the string UD and whitespace within the same pair of parentheses.  UD information effectively will be ignored by EDA tools, but may be used to track how the data describing the port behavior was generated, and/or how the Port data was physically measured.

Rules for EMD

To support automated generation of EMD files from Touchstone data featuring rail structures, several specific subparameters must be used within [Begin Port Map]/[End Port Map].

- The Bus_label subparameter must be used to declare common rail connections which will appear explicitly under the Bus_label column as part of [EMD Pin List] and/or [Designator Pin List] in the created EMD file

- The Signal_name subparameter must be used for any common rail connections where Bus_label is not used (per EMD rules, where Bus_label is not present, rail connections can be declared using common Signal_name entries)

- Port lines must include the “Type P” declaration and Physical name-value pairs, where the values include Bus_label, Signal_name, and Pin_name strings and the colon character (“:”) per the rules below

- Port lines must also include Net name-value pairs where the value used shall match the argument to any Pin_name subparameter present, per the rules below

The fundamental concepts are:

- Bus_labels on Port lines declare rail names and the reference designators of the component featuring that rail; the Bus_label subparameter declares each reference designator and pin associated with the given Bus_label.

- Signal_names on Port lines declare rail names for entire EMD modules; the Signal_name subparameter declares the pin associated with the given Signal_name.  Both the Signal_name in the Physical name-value pair and the Signal_name subparameter use the string “EMD” where Bus_labels would feature a reference designator.

- Net name-value pairs shall match the Bus_label value or Signal_name value provided for the same Port.

The formatting of the Bus_label and Signal_name values for the Physical name use the following rules:

The “Physical” name-value pair may contain “Signal_name” or “Bus_label” only for Type P ports or for references of Type P or Type S ports.  The same <Signal_name> value is prohibited to appear in the “+” terminal definition of one port and the “-“ terminal definition of another port, or in both terminal definitions of the same port.

The “Net” name-value pair is required for each “Physical” name-value pair which contains “Pin_name” or “Bus_label” to define a <Signal_name> that includes that Pin_name or Bus_label.  The same <Signal_name> value may appear in multiple “Net” name-value pairs of multiple Pin_name or Bus_label definitions, but the same <Signal_name> value shall not be used for Pin_name and Bus_label definitions simultaneously.  In addition, the same <Signal_name> value is prohibited to appear in the “Net” name-value pair of a “+” terminal of one port and the “Net” name-value pair of a “-“ terminal of another port, or in both “Net” name-value pair definitions of the same port.  When a “Physical” name-value pair is defined for the reference terminal of the port, a corresponding “Net” name-value pair must also be provided.

“Physical” name-value pairs with different definition types shall not use the same <Signal_name>, i.e., Signal_name, Bus_label and Pin_name definitions must be associated with different <Signal_name>s.

In summary, to be consistent with the overlap exclusion rules on pg. 420 of the IBIS 8.0 specification, port maps shall not make overlapping references to <RailPinName>s, <Bus_label>s and <Signal_name>s on the port lines.  For example, if a certain port uses a “Pin_name” reference to a certain <RailPinName>, another port shall not use a “Bus_label” or “Signal_name” reference that includes that <RailPinName> in its list of <RailPinName>s.  Or, if a certain port uses a “Bus_label” reference to a collection of <RailPinName>s, another port shall not use a “Signal_name” reference that includes that <Bus_label> or a “Pin_name” reference to an individual <RailPinName> which is a member of that <Bus_label>.

Port Line rules for EMD:

Port <integer> (Type P) (Physical Signal_name:<Designator>:<Signal_name>

Port <integer> (Type P) (Physical Signal_name:EMD:<Signal_name>) (Net <Signal_name>)

Port <integer> (Type P) (Physical Pin_name:EMD.<Pin_name>)(Net <Signal_name>)

Port <integer> (Type P) (Physical Bus_label:<Designator>:<Bus_label>)(Net VDD)

Signal_name subparameter rules:

Signal_name EMD:<Signal_name> (EMD.<pin name> EMD.<Pin_name>)

Signal_name <Designator>:<Signal_name> (<Designator>.<Pin_name> < Designator>.<Pin_name>)

Note that the <Designator> values that appear on a given Signal_name line shall all match.

Bus_label subparameter rules:

Bus_label <Designator>:<Bus_label> (<Designator>.<Bus_label> <Designator>.<Bus_label>)

Bus_label EMD:<Bus_label> (<Designator>.<Bus_label> <Designator>.<Bus_label>)

- Bus_label:<Designator>:<Bus_label name>

- Bus_label:EMD:<Bus_label name>

- Bus_label:*:<Bus_label name>

Note that the <Designator> values that appear on a given Bus_label line shall all match.

Bus_label EMD:<xyz> is allowed, but not with wildcards.

Pin_name subparameter rules:

- Pin_name:EMD.<Pin_name>

- Pin_name:<Designator>.<Pin_name>

For Type P ports or for references of Type P or Type S ports, a Signal_name subparameter line is required for each <Signal_name> defined in the “Net” name-value pair, or the “Physical” name-value pair that contains “Signal_name”.

For Type S ports, a Signal_name subparameter line is not required (shall it be prohibited?) for the “+” terminal of the port.

Each Signal_name subparameter shall start on a new line.  The Signal_name subparameter is followed by a whitespace, “<EMD | Designator>:<Signal_name>”, whitespace, an open parenthesis, a list of “<EMD | Designator>.<RailPinName>” separated by whitespace, and a close parenthesis.  The list of <RailPinName>s may span multiple lines if necessary.

The list of <RailPinName>s must be unique on each Signal_name subparameter line, i.e., the same <RailPinName> is not allowed to appear on any other Signal_name or Bus_label subparameter lines.

The “<EMD | Designator>” entries within a Signal_name subparameter line must be identical.

The “<EMD | Designator>:<Signal_name>” entry of the Signal_name subparameter must be unique, i.e., none of the other Signal_name subparameters shall have the same “<EMD | Designator>:<Signal_name>”.

Bus_label subparameter rules:

For Type P ports or for references of Type P or Type S ports, a Bus_label subparameter line is required for each <Bus_label> defined in the “Physical” name-value pair that contains “Bus_label”.  Each Bus_label subparameter shall start on a new line.  The Bus_label subparameter is followed by a whitespace, “<EMD | Designator>:<Bus_label>”, whitespace, an open parenthesis, a list of “<EMD | Designator>.<RailPinName>” separated by whitespace and a close parenthesis.  The list of <RailPinName>-s may span multiple lines if necessary.

The list of <RailPinName>s must be unique on each Bus_label subparameter line, i.e., the same <RailPinName> is not allowed to appear on any other Bus_label or Signal_name subparameter line.

The “<EMD | Designator>” entries within a Bus_label subparameter line must be identical.

Wildcard rules:

The <Designator> entry in the “Physical” name-value pair may be an asterisk character (“*”) to serve as a wildcard when “Physical” is followed by “Signal_name” or “Bus_label”.  In this case, all pins that use <Signal_name> or <Bus_label name> will be shorted together by an ideal short across all components.  When “Physical” is followed by “Pin_name”, the wildcard character is prohibited.

Referencing

Name-value pairs for Logical, Net, Physical, or Probe preceded with a “-“ character define the reference terminal for that Port.  In the case of Net and Physical, this defines how the reference terminal should be connected.  In the case of Probe, this is the reference terminal for how the data was measured.  For Logical declarations, the reference terminal identification is simply an informational label.  The value in these reference terminal name-value pairs may be different for each port or may be the same for some ports (in any combination) or may be the same for all ports.  This notation helps model makers document how the probes or ports were set up in the lab measurements or extraction software simulations.

In the case of Net and Physical declarations, if the “-“ symbol is not present for a port, the reference terminal for that port will be assumed to be connected to the simulator global reference node (“A_gnd” as defined in IBIS) by simulation tools for the contexts where this is relevant (e.g., SPICE circuit simulation).  Note that the string “A_gnd” shall not be used for the non-reference terminal of a Port in Net or Physical name-value pairs.

A_gnd may not appear in Logical or Probe declarations.  As Logical declarations are effectively functional labels, a reference terminal does not have a specific meaning for Logical declarations (e.g., a Logical reference terminal may be useful for schematic symbol documentation, but must not be used for connectivity to other components).  Similarly, the reference terminal may be omitted for Probe declarations, though this may compromise reproducibility of the provided data (e.g., if real-world measurements were to be conducted on the component from which the Touchstone data was generated).

Diff_port does not identify polarity or referencing; it may therefore be declared for a Port where the “-“ symbol is used.  There is no conflict using both syntax features, as Diff_port is intended for use in visualization of data.  EDA tools are responsible for providing clear controls to users regarding connectivity and data display when Diff_port and the “-“ symbol are present for the same Port(s).  Finally, Diff_port is not a declaration that mixed-mode data is present.

The reference scheme defined here is intended to both document how data was collected and, in part, help determine how the data can most appropriately be used in a larger system, particularly for simulation purposes.  Many EDA simulation software packages permit Touchstone data to be used by encapsulating the data in a SPICE circuit using one of three referencing schemes: N, 2N, or N+1, where N is the number of non-reference terminals in the Touchstone data set.  These three schemes correspond, respectively, to: referencing of all ports to the ideal simulator terminal (A_gnd for IBIS, or 0 in many SPICE implementations); referencing of each port to its own separate individual terminal; and referencing all the ports to a single terminal.  Note that the latter two schemes collapse into the first scheme if the reference terminals are the simulator global reference node.  As can be seen, the Port Map referencing approach defined here permits Touchstone data collection with unambiguous referencing, and can inform correct usage of that data in combination with other circuit models and data sets.

Examples

Example 1:   5 signals of a cable connected from a CPU to a sensor.

| Each of these ports uses A_gnd as a reference for the Physical and Net declarations.

[Begin Port Map]

Port 1 (Physical A.1)(Side CPU)(Net 1)

Port 2 (Physical A.2)(Side CPU)(Net 2)

Port 3 (Physical A.3)(Side CPU)(Net 3)

Port 4 (Physical A.4)(Side CPU)(Net 4)

Port 5 (Physical A.5)(Side CPU)(Net 5)

Port 6 (Physical B.1)(Side Sensor)(Net 1)

Port 7 (Physical B.2)(Side Sensor)(Net 2)

Port 8 (Physical B.3)(Side Sensor)(Net 3)

Port 9 (Physical B.4)(Side Sensor)(Net 4)

Port 10 (Physical B.5)(Side Sensor)(Net 5)

Symbol_leftside 1, 2, 3, 4, 5

Symbol_rightside 6, 7, 8, 9, 10

[End Port Map]

Example 2a:   Description of the measurement of a bipolar junction transistor, common emitter.

| Note that this describes how a set of S-parameters were collected to characterize a

| device for use in RF applications.  The name-value pairs establish this as a 2-port,

| 3-terminal device.

[Begin Port Map]

Port 1 ( Logical Base )      -( Logical Emitter ) (Probe 32.5:16.0:6.0)  -( Probe 7.0:10.0:2.0 )

Port 2 ( Logical Collector ) -( Logical Emitter ) (Probe 29.5:13.0:3.0)  -( Probe 7.0:10.0:2.0 )

[End Port Map]

Example 2b:   Description of the measurement of a bipolar junction transistor, common collector.

| Note that this describes how a set of S-parameters were collected to characterize a

| device for use in RF applications.  The name-value pairs used establish this as a

| 2-port, 3-terminal device.

[Begin Port Map]

Port 1 ( Logical Base )    -( Logical Collector ) (Probe 32.5:16.0:6.0) -( Probe 29.5:13.0:3.0)

Port 2 ( Logical Emitter ) -( Logical Collector ) (Probe 7.0:10.0:2.0) -( Probe 29.5:13.0:3.0)

[End Port Map]

Example 2c:   Description of the measurement of a bipolar junction transistor, common base.

| Note that this describes how a set of S-parameters were collected to characterize a

| device for use in RF applications.  The name-value pairs used establish this as a

| 2-port, 3-terminal device.

[Begin Port Map]

Port 1 ( Logical Collector ) -( Logical Base ) (Probe 29.5:13.0:3.0)  -(Probe 32.5:16.0:6.0)

Port 2 ( Logical Emitter )   -( Logical Base ) (Probe 7.0:10.0:2.0) -(Probe 32.5:16.0:6.0)

[End Port Map]

Example 2d:   Model data for a bipolar junction transistor with symbol information for a schematic.

| Here, all three terminals are represented using Physical declarations with respect to

| the assumed ideal reference (A_gnd).  The [Number of Ports] field is 3.  The number of

| terminals in a symbol could be 3 or 4 depending on whether the reference terminal is

| shown.

[Begin Port Map]

Port 1 (Logical Base)      (Physical 1) (Probe 32.5:16.0:6.0)

Port 2 (Logical Collector) (Physical 2) (Probe 29.5:13.0:3.0)

Port 3 (Logical Emitter)   (Physical 3) (Probe 7.0:10.0:2.0)

Symbol_leftside 1

Symbol_rightside 2 3

[End Port Map]

Example 2e: Measurement data for a bipolar junction transistor with single reference, and with user-defined data after Port definitions.

| Here, all three terminals are represented using Physical declarations with respect to a

| separate reference terminal.  The [Number of Ports] field is 3; the assumed reference

| for use in simulation would be A_gnd

.

[Begin Port Map]

Port 1 (Physical 1) (Logical Base)      -(Physical 4) (Side B) (Probe 32.5:16.0:6.0) -(Probe 0.0:20.5:1)

Port 2 (Physical 2) (Logical Collector) -(Physical 4) (Side C) (Probe 29.5:13.0:3.0) -(Probe 0.0:20.5:1)

Port 3 (Physical 3) (Logical Emitter)   -(Physical 4) (Side E) (Probe 7.0:10.0:2.0) -(Probe 0.0:20.5:1)

UD SijStatus 1 2 Measured

[End Port Map]

Example 2f: Measurement data for a bipolar junction transistor with single reference, and with user-defined data identified in-line with the Port definitions.

| Here, all three terminals are represented using Physical declarations of ports with

| respect to a separate reference terminal.

[Begin Port Map]

Port 1 (Physical 1)(Logical Base) -(Physical probe_ref)(Side B) (Probe 32.5:16.0:6.0) -(Probe 0.0:20.5:1) (UD SijStatus Measured)

Port 2 (Physical 2)(Logical Collector) -(Physical probe_ref)(Side C) (Probe 29.5:13.0:3.0) -(Probe 0.0:20.5:1) (UD SijStatus Measured)

Port 3 (Physical 3)(Logical Emitter)  -(Physical probe_ref)(Side E) (Probe 7.0:10.0:2.0) -(Probe 0.0:20.5:1)

[End Port Map]

Example 3a:   Data for an 8” coplanar wave guide (one signal net with two terminals, and three references).

[Begin Port Map]

Port 1 ( Physical A3 ) (Net 1) (Side Left)  -( Physical H7 )

Port 2 ( Physical A4 ) (Net 1) (Side Right) -( Physical Group:GND_R )

Group Physical GND_R (C6 D8)

[End Port Map]

Example 3b:   An 8” coplanar wave guide (one signal net with  two references).

[Begin Port Map]

Port 1 (Physical A3) (Net 1) (Side Left)  -( Physical Group:GND_L )

Port 2 (Physical A4) (Net 1) (Side Right) -( Physical Group:GND_R )

Group Physical GND_L (E7 F8)

Group Physical GND_R (C6 D8)

[End Port Map]

Example 4:  Single-ended connection between components U1.7 and U3.5 on a PCB.

[Begin Port Map]

Port 1 (Physical U1.7)(Side U1)(Net DQ3)(Logical CPU)

Port 2 (Physical U3.5)(Side U3)(Net DQ3)(Logical SDRAM)

[End Port Map]

Example 5:  IBIS [EMD Model] of a 4-bit DQ nibble in a 2-rank DIMM, as shown in Figure 1 below.

[Begin Port Map]

Data_usage "EMD"

Port 1 (Physical  20)   (Side EMD) (Net DQ0) (Logical DQ0)

Port 2 (Physical  21)   (Side EMD) (Net DQ1) (Logical DQ1)

Port 3 (Physical  22)   (Side EMD) (Net DQ2) (Logical DQ2)

Port 4 (Physical  23)   (Side EMD) (Net DQ3) (Logical DQ3)

Port 5 (Physical  24)   (Side EMD) (Net DQS+)(Logical DQS+)      (Diff_port 6)

Port 6 (Physical  25)   (Side EMD) (Net DQS-)(Logical DQS-)      (Diff_port 5)

Port 7 (Physical  U1.A2)(Side mem1)(Net DQ0) (Logical mem1_DQ0)

Port 8 (Physical  U1.A3)(Side mem1)(Net DQ1) (Logical mem1_DQ1)

Port 9 (Physical  U1.B2)(Side mem1)(Net DQ2) (Logical mem1_DQ2)

Port 10 (Physical U1.B3)(Side mem1)(Net DQ3) (Logical mem1_DQ3)

Port 11 (Physical U1.A4)(Side mem1)(Net DQS+)(Logical mem1_DQS+) (Diff_port 12)

Port 12 (Physical U1.B4)(Side mem1)(Net DQS-)(Logical mem1_DQS-) (Diff_port 11)

Port 13 (Physical U2.A2)(Side mem2)(Net DQ0) (Logical mem2_DQ0)

Port 14 (Physical U2.A3)(Side mem2)(Net DQ1) (Logical mem2_DQ1)

Port 15 (Physical U2.B2)(Side mem2)(Net DQ2) (Logical mem2_DQ2)

Port 16 (Physical U2.B3)(Side mem2)(Net DQ3) (Logical mem2_DQ3)

Port 17 (Physical U2.A4)(Side mem2)(Net DQS+)(Logical mem2_DQS+) (Diff_port 18)

Port 18 (Physical U2.B4)(Side mem2)(Net DQS-)(Logical mem2_DQS-) (Diff_port 17)

| The parameters below will generate a symbol

| where DQS+ and DQS- are presented at the top of the left and right

| sides of the symbol

Symbol_leftside  5, 6, 1, 2, 3, 4

Symbol_rightside 11, 12, 7, 8, 9, 10, 17, 18, 13, 14, 15, 16

[End Port Map]

Figure 1 - Dual-Rank DIMM with EMD

Example 6:  4-port Touchstone file for a differential buffer modeled using IBIS AMI.

[Begin Port Map]

Data_usage "Ts4file"

Port 1 (Physical A.1)  (Side Die) (Net 1) (Diff_port 2)

Port 2 (Physical A.2)  (Side Die) (Net 2) (Diff_port 1)

Port 3 (Physical B.1)  (Side Pin) (Net 1) (Diff_port 4)

Port 4 (Physical B.2)  (Side Pin) (Net 2) (Diff_port 3)

[End Port Map]

Example 7:  C_comp Model for an input buffer IBIS model.

[Begin Port Map]

Data_usage "C_comp_model"

Port 1 (Type S) (Physical Buffer_I)

Port 2 (Type S) (Physical Buffer_I/O)

| Buffer_I/O is required for all C_comp_models

| A_gnd is the default reference for the Physical declarations

[End Port Map]

Example 8: An incomplete example, for an EMD model representing a multi-die packaged component, using Bus_label.

[Begin Port Map]

Usage_model "EMD"

Port 1  (Type S) (Side Die1)  (Physical U1.94)  (Net A0) –( Physical Bus_label:GND_BGA )

Port 2  (Type S) (Side Die2)  (Physical U2.94)  (Net A0) –( Physical Bus_label:GND_BGA )

Port 3  (Type S) (Side Die3)  (Physical U3.94)  (Net A0) –( Physical Bus_label:GND_BGA )

Port 4  (Type S) (Side EMDpin)(Physical EMD.K2) (Net A0) –( Physical Bus_label:GND_BGA )

…

Port 7  (Type S) (Physical U1.95)(Side Die1)  (Net A1) –( Physical Bus_label:GND_BGA )

Port 8  (Type S) (Physical U2.95)(Side Die2)  (Net A1) –( Physical Bus_label:GND_BGA )

Port 9  (Type S) (Physical U3.95)(Side Die3)  (Net A1) –( Physical Bus_label:GND_BGA )

Port 10 (Type S) (Physical J2)   (Side EMDpin)(Net A1) –( Physical Bus_label:GND_BGA )

…

Port 121 (Type S)(Side Die1)  (Physical U1.71)(Net CLK_c) –(Physical Bus_label:GND_BGA) (Diff_port 123)

Port 122 (Type S)(Side EMDpin)(Physical E2)   (Net CLK_c) -(Physical Bus_label:GND_BGA) (Diff_port 124)

Port 123 (Type S)(Side Die1)  (Physical U1.70)(Net CLK_t) –(Physical Bus_label:GND_BGA) (Diff_port 121)

Port 124 (Type S)(Side EMDpin)(Physical D2)   (Net CLK_t) -(Physical Bus_label:GND_BGA) (Diff_port 122)

…

Port 163 (Type P) (Physical Bus_label:U1:VDD_U1)  (Net VDD) (Side Die1)   –(Physical Bus_label:GND_BGA)

Port 164 (Type P) (Physical Bus_label:U2:VDD_U2)  (Net VDD) (Side Die2)   –(Physical Bus_label:GND_BGA)

Port 165 (Type P) (Physical Bus_label:U3:VDD_U3)  (Net VDD) (Side Die3)   –(Physical Bus_label:GND_BGA)

Port 166 (Type P) (Physical Bus_label:VDD_BGA) (Net VDD) (Side EMDpin) –(Physical Bus_label:GND_BGA)

Bus_label U1:VDD_U1 (U1.4  U1.8  U1.17  U1.20  U1.33  U1.36  U1.40  U1.45  U1.49  U1.52)

Bus_label U2:VDD_U2 (U2.4  U2.8  U2.17  U2.20  U2.33  U2.36  U2.40  U2.45  U2.49  U2.52)

Bus_label U3:VDD_U3 (U3.4  U3.8  U3.17  U3.20  U3.33  U3.36  U3.40  U3.45  U3.49  U3.52)

Bus_label VDD_BGA ( EMD.A3 EMD.A4 EMD.A7 EMD.AB5 EMD.AB9 EMD.AB11 EMD.AB13 EMD.AC7 EMD.B1 EMD.B5 )

Bus_label GND_BGA ( EMD.A5 EMD.A9 EMD.A11 EMD.A14 EMD.AA5 EMD.AA13 EMD.AC5 EMD.AC9 EMD.AC11 EMD.AC14 )

[End Port Map]

Example 9: An incomplete example for a multi-die packaged component using Group, including wildcards.

[Begin Port Map]

Port 1   (Type S) (Physical U1.94) (Net A0) (Side Die1)   –(Physical Group:GND_Ux)

Port 2   (Type S) (Physical U2.94) (Net A0) (Side Die2)   –(Physical Group:GND_Ux)

Port 3   (Type S) (Physical U3.94) (Net A0) (Side Die3)   –(Physical Group:GND_Ux)

Port 4   (Type S) (Physical K2)    (Net A0) (Side Pin)    –(Physical Group:GND_BGA)

…

Port 7   (Type S) (Physical U1.95) (Net A1) (Side Die1)   –(Physical Group:GND_Ux)

Port 8   (Type S) (Physical U2.95) (Net A1) (Side Die2)   –(Physical Group:GND_Ux)

Port 9   (Type S) (Physical U3.95) (Net A1) (Side Die3)   –(Physical Group:GND_Ux)

Port 10  (Type S) (Physical J2)    (Net A1) (Side Pin)    –(Physical Group:GND_BGA)

…

Port 121 (Type S)(Side Die1)(Physical U1.71)(Net CLK_c) –(Physical Group:GND_Ux) (Diff_port 123)

Port 122 (Type S)(Side Pin) (Physical E2)   (Net CLK_c) –(Physical Group:GND_BGA)(Diff_port 124)

Port 123 (Type S)(Side Die1)(Physical U1.70)(Net CLK_t) –(Physical Group:GND_Ux) (Diff_port 121)

Port 124 (Type S)(Side Pin) (Physical D2)   (Net CLK_t) -(Physical Group:GND_BGA)(Diff_port 122)

…

Port 163 (Type P) (Side Die1)(Physical Group:VDD_U1)  (Net VDD) –(Physical Group:GND_Ux)

Port 164 (Type P) (Side Die2)(Physical Group:VDD_U2)  (Net VDD) –(Physical Group:GND_Ux)

Port 165 (Type P) (Side Die3)(Physical Group:VDD_U3)  (Net VDD) –(Physical Group:GND_Ux)

Port 166 (Type P) (Side Pin) (Physical Group:VDD_BGA) (Net VDD) –(Physical Group:GND_BGA)

…

Group Physical VDD_U1 ( U1.4  U1.8  U1.17  U1.20  U1.33  U1.36  U1.40  U1.45  U1.49  U1.52 )

Group Physical VDD_U2 ( U2.4  U2.8  U2.17  U2.20  U2.33  U2.36  U2.40  U2.45  U2.49  U2.52 )

Group Physical VDD_U3 ( U3.4  U3.8  U3.17  U3.20  U3.33  U3.36  U3.40  U3.45  U3.49  U3.52 )

Group Physical VDD_BGA ( A3 A4 A7 AB5 AB9 AB11 AB13 AC7 B1 B5 )

…

Group Physical GND_Ux  ( *.2    *.11   *.15    *.23    *.38    *.42     *.43    *.47    *.50     *.54 )

Group Physical GND_BGA ( A5 A9 A11 A14 AA5 AA13 AC5 AC9 AC11 AC14 )

[End Port Map]

Example 10: An EMD model example with a single reference for all ports.  The corresponding IBIS EMD example, “DQ1_Victim”, may be found in IBIS 8.0, in section 13.7, on page 424.

Note that the EMD interface only makes connections with the odd-numbered Ports below; the even-numbered Ports will be terminated through the Unused_port_terminations subparameter.

[Begin Port Map]

Data_usage "EMD"

Port 1  (Type S) (Side DIMM_pin) (Physical A1)   (Net DQ0)

Port 2  (Type S) (Side DIMM_pin) (Physical A5)   (Net DQ4)

Port 3  (Type S) (Side DIMM_pin) (Physical A2)   (Net DQ1)

Port 4  (Type S) (Side DIMM_pin) (Physical A6)   (Net DQ5)

Port 5  (Type S) (Side DIMM_pin) (Physical A3)   (Net DQ2)

Port 6  (Type S) (Side DIMM_pin) (Physical A7)   (Net DQ6)

Port 7  (Type S) (Side DIMM_pin) (Physical A4)   (Net DQ3)

Port 8  (Type S) (Side DIMM_pin) (Physical A8)   (Net DQ7)

Port 9  (Type S) (Side U1_pin)   (Physical U1.5) (Net DQ0)

Port 10 (Type S) (Side U1_pin)   (Physical U1.9) (Net DQ4)

Port 11 (Type S) (Side U1_pin)   (Physical U1.6) (Net DQ1)

Port 12 (Type S) (Side U1_pin)   (Physical U1.10)(Net DQ5)

Port 13 (Type S) (Side U1_pin)   (Physical U1.7) (Net DQ2)

Port 14 (Type S) (Side U1_pin)   (Physical U1.11)(Net DQ6)

Port 15 (Type S) (Side U1_pin)   (Physical U1.8) (Net DQ3)

Port 16 (Type S) (Side U1_pin)   (Physical U1.12)(Net DQ7)

Port 17 (Type S) (Side U2_pin)   (Physical U2.5) (Net DQ0)

Port 18 (Type S) (Side U2_pin)   (Physical U2.9) (Net DQ4)

Port 19 (Type S) (Side U2_pin)   (Physical U2.6) (Net DQ1)

Port 20 (Type S) (Side U2_pin)   (Physical U2.10)(Net DQ5)

Port 21 (Type S) (Side U2_pin)   (Physical U2.7) (Net DQ2)

Port 22 (Type S) (Side U2_pin)   (Physical U2.11)(Net DQ6)

Port 23 (Type S) (Side U2_pin)   (Physical U2.8) (Net DQ3)

Port 24 (Type S) (Side U2_pin)   (Physical U2.12)(Net DQ7)

[End Port Map]

Example 11: An EMD model example with a single reference for all ports using A_gnd (This is currently the referencing method supported by EMD in IBIS through version 8.0.  While legal in EMD files, this approach does not define any EMD or Designator ground pins, leaving the Designator ground pins floating.).

[Begin Port Map]

Port 1   (Type S)(Physical U1.94)(Side Die1)  (Net A0)

Port 2   (Type S)(Physical U2.94)(Side Die2)  (Net A0)

Port 3   (Type S)(Physical U3.94)(Side Die3)  (Net A0)

Port 4   (Type S)(Physical K2)   (Side EMDpin)(Net A0)

…

Port 7   (Type S)(Physical U1.95)(Side Die1)  (Net A1)

Port 8   (Type S)(Physical U2.95)(Side Die2)  (Net A1)

Port 9   (Type S)(Physical U3.95)(Side Die3)  (Net A1)

Port 10  (Type S)(Physical J2)   (Side EMDpin)(Net A1)

…

Port 121 (Type S)(Physical U1.71)(Side Die1)  (Net CLK_c)(Diff_port 123)

Port 122 (Type S)(Physical E2)   (Side EMDpin)(Net CLK_c)(Diff_port 124)

Port 123 (Type S)(Physical U1.70)(Side Die1)  (Net CLK_t)(Diff_port 121)

Port 124 (Type S)(Physical D2)   (Side EMDpin)(Net CLK_t)(Diff_port 122)

…

Port 163 (Type P)(Physical U1.VDD)(Side Die1)  (Net VDD)

Port 164 (Type P)(Physical U2.VDD)(Side Die2)  (Net VDD)

Port 165 (Type P)(Physical U3.VDD)(Side Die3)  (Net VDD)

Port 166 (Type P)(Physical VDD)   (Side EMDpin)(Net VDD)

[End Port Map]

Example 12: An EMD model example with a single reference for all ports using A_gnd and additional ports to connect the EMD and Designator ground pins.  (This is legal in EMD files and provides a complete definition for all EMD and Designator ground pins).

[Begin Port Map]

Data_usage "EMD"

Port 1   (Type S)(Physical U1.94)(Side Die1)  (Net A0)

Port 2   (Type S)(Physical U2.94)(Side Die2)  (Net A0)

Port 3   (Type S)(Physical U3.94)(Side Die3)  (Net A0)

Port 4   (Type S)(Physical K2)   (Side EMDpin)(Net A0)

…

Port 7   (Type S)(Physical U1.95)(Side Die1)  (Net A1)

Port 8   (Type S)(Physical U2.95)(Side Die2)  (Net A1)

Port 9   (Type S)(Physical U3.95)(Side Die3)  (Net A1)

Port 10  (Type S)(Physical J2)   (Side EMDpin)(Net A1)

…

Port 121 (Type S)(Physical U1.71)(Side Die1)  (Net CLK_c)(Diff_port 123)

Port 122 (Type S)(Physical E2)   (Side EMDpin)(Net CLK_c)(Diff_port 124)

Port 123 (Type S)(Physical U1.70)(Side Die1)  (Net CLK_t)(Diff_port 121)

Port 124 (Type S)(Physical D2)   (Side EMDpin)(Net CLK_t)(Diff_port 122)

…

Port 163 (Type P)(Physical U1.VDD)(Side Die1)  (Net VDD)

Port 164 (Type P)(Physical U2.VDD)(Side Die2)  (Net VDD)

Port 165 (Type P)(Physical U3.VDD)(Side Die3)  (Net VDD)

Port 166 (Type P)(Physical VDD)   (Side EMDpin)(Net VDD)

…

Port 200 (Type P)(Physical U1.GND)(Side Die1)  (Net GND)

Port 201 (Type P)(Physical U2.GND)(Side Die2)  (Net GND)

Port 202 (Type P)(Physical U3.GND)(Side Die3)  (Net GND)

Port 203 (Type P)(Physical GND)   (Side EMDpin)(Net GND)

[End Port Map]

Example 13: An IBIS package [Interconnect Model] example with pin rail grouping for a model between the pins and buffers.  The corresponding IBIS Interconnect example may be found in IBIS 8.0, in section 11.2, on page 376 (“Example 2b”).

[Begin Port Map]

Data_usage "IBIS_Interconnect"

Port 1   (Type S) (Physical pin.A1)            (Side Pin) (Net DQ1)

Port 2   (Type S) (Physical pin.A2)            (Side Pin) (Net DQ2)

Port 3   (Type S) (Physical pin.A3)            (Side Pin) (Net DQ3)

Port 4   (Type S) (Physical pin.D1)            (Side Pin) (Net DQS+) (Diff_port 5)

Port 5   (Type S) (Physical pin.D2)            (Side Pin) (Net DQS-) (Diff_port 4)

Port 6   (Type P) (Physical pin:Bus_label:VDD) (Side Pin) (Net VDD)

Port 7   (Type P) (Physical pin:Bus_label:VSS) (Side Pin) (Net VSS)

Port 8   (Type S) (Physical buf.A1)            (Side Buf) (Net DQ1)

Port 9   (Type S) (Physical buf.A2)            (Side Buf) (Net DQ2)

Port 10  (Type S) (Physical buf.A3)            (Side Buf) (Net DQ3)

Port 11  (Type S) (Physical buf.D1)            (Side Buf) (Net DQS+) (Diff_port 12)

Port 12  (Type S) (Physical buf.D2)            (Side Buf) (Net DQS-) (Diff_port 11)

Port 13  (Type P) (Physical buf:Bus_label:VDD) (Side Buf) (Net VDD)

Port 14  (Type P) (Physical buf:Bus_label:VSS) (Side Buf) (Net VSS)

[End Port Map]

Example 14: An IBIS [Interconnect Model] example using pin and buffer names and terminals without grouping (the model is between pin and buffer terminals).  The corresponding IBIS Interconnect example may be found in IBIS 8.0, in section 11.2, on pages 382 and 383 (“Example 10”).

[Begin Port Map]

Data_usage "IBIS_Interconnect"

|

Port 1   (Type S) (Physical pin.A1) (Side Pin_IO)      (Net DQ0)

Port 2   (Type S) (Physical buf.A1) (Side Buffer_IO)   (Net DQ0)

Port 3   (Type S) (Physical pin.A2) (Side Pin_IO)      (Net DQ1)

Port 4   (Type S) (Physical buf.A2) (Side Buffer_IO)   (Net DQ1)

Port 5   (Type S) (Physical pin.A3) (Side Pin_IO)      (Net DQ2)

Port 6   (Type S) (Physical buf.A3) (Side Buffer_IO)   (Net DQ2)

|

[End Port Map]

Example 15: An EMD model example for a power delivery network, with a single reference for all ports, using Bus_label and Signal_name.  The corresponding IBIS EMD example may be found in IBIS 8.0, in section 13.7, on page 421.

[Begin Port Map]

Data_usage "EMD"

Port 1  (Type P) (Physical Signal_name:EMD:VDD) (Net VDD)

Port 2  (Type P) (Physical EMD:G1)              (Net VSS)

Port 3  (Type P) (Physical Bus_label:U1:VDD1)   (Net VDD)

Port 4  (Type P) (Physical Bus_label:U1:VDD2)   (Net VDD)

Port 5  (Type P) (Physical Signal_name:U1:VSS)  (Net VSS)

Port 6  (Type P) (Physical Bus_label:U2:VDD1)   (Net VDD)

Port 7  (Type P) (Physical Bus_label:U2:VDD2)   (Net VDD)

Port 8  (Type P) (Physical Signal_name:U2:VSS)  (Net VSS)

Signal_name Physical EMD:VDD  (EMD.P1 EMD.P2)

Signal_name Physical U1:VSS   (U1.3 U1.4)

Signal_name Physical U2:VSS   (U2.3 U2.4)

Bus_label   Physical U1:VDD1 Signal_name:U1:VDD  (U1.1)

Bus_label   Physical U1:VDD2 Signal_name:U1:VDD  (U1.2)

Bus_label   Physical U1:VDD1 Signal_name:U2:VDD  (U2.1)

Bus_label   Physical U1:VDD2 Signal_name:U2:VDD  (U2.2)

[End Port Map]