BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

ISSUE TITLE:   	Using IBIS-AMI Leaf Labels in List Parameters

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:	{date you sent the document}

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

In the Section 10A, "AMI Parameter Definition File Structure ", the branch "Labels" is used to assign names to columns in a "Table".

The suggestion is to use "Labels" in a similar way to assign names to entries in a "List". Values in a List often are numbers that do not help the User identify the meaning of each List value. The EDA Tool may use the value of each Labels field as a Tool Tip for each value in the List, or the User may select a value from the List by selecting from a list of Labels.

An example of using Labels

(Process (Usage In) (Type Integer)(

(List –2 –1 0 1 2)(Default 0)

(Labels “X Slow” “Slow” “Typ” “Fast” “X Fast”)

(Description “Process corner used in the algorithmic model”)

)

ANY OTHER BACKGROUND INFORMATION:

{These documents will be archived, so use this section to add any detail that is not part of the section above or the changed text itself , but should not be lost.}

Replace this following text

List <default value> <value> <value> <value> ... <value>

This defines a discrete set of values from which the user may select one value

With

List <default value> <value> <value> <value> ... <value>

This defines a discrete set of values from which the user may select one value.

Labels is an optional leaf within List parameters and it is followed by a String entry for each entry in the List. The number of entries in Labels must be the same as the number of entries in List. The nth entry in Lables shall correspond to the nth entry in List.  Quoted null entries are not permitted. All entries in Labels shall be unique, except that if two entries in List are the same, then the corresponding Labels entries must also be the same.

(Labels <"label1"> <"label2"> <"label3"> ...)

Example:

(Strength (Usage In) (Type Integer)(Description “Strength of Driver”)

(List 0 1 2 3 4)(Default 2)

(Labels “Extra Weak” “Weak” “Nominal” “Strong” “Extra Strong”))