BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD ID#:            TBD

ISSUE TITLE:     Clarify definition of Tx_Dj

REQUESTER:     Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:  TBD November 4, 2014

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

The definition of the AMI Reserved Parameter Tx_Dj is inconsistent with the other notes for Tx_Dj. This BIRD modifies the definition of Tx_Dj to be consistent with the notes. A common understanding of Deterministic Jitter is that it contains both Data Dependent Jitter (DDJ) and Bounded Uncorrelated Jitter (BUJ). In Other Notes:

Time(n) = n * bit_time + 2.0 * Tx_Dj * rand()

clearly defines BUJ, and has no Data Dependent component. We should therefore define Tx_Dj to only include BUJ.

I also took the liberty of changing uncorrelated bounded jitter to Bounded Uncorrelated Jitter (BUJ).

Replace the following definition on page 208:

Definition:           The worst case half the peak to peak variation at the transmitter implemented by the EDA tool by modifying the stimulus input or by post processing the simulation results. Tx_Dj shall include all deterministic and uncorrelated bounded jitter that is not accounted for by Tx_DCD, and Tx_Sj. Entries are assumed to be in units of seconds when declared as Type Float.

With

Definition:           The worst case half the peak to peak variation at the transmitter implemented by the EDA tool by modifying the stimulus input or by post processing the simulation results. Tx_Dj shall include all Bounded Uncorrelated Jitter (BUJ) that is not accounted for by Tx_DCD, and Tx_Sj. Entries are assumed to be in units of seconds when declared as Type Float.