BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	Draft 5 March 17, 2015

ISSUE TITLE:   		C_comp Model Using IBIS-ISS

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

Randy Wolff, Micron Technology, Inc.

DATE SUBMITTED:	{date you sent the original document, for new BIRDs}

DATE REVISED:	{date(s) you sent any revisions to the document}

DATE ACCEPTED BY IBIS OPEN FORUM:

STATEMENT OF THE ISSUE:

The current C_comp model is either a single capacitance or optionally up to four capacitors attached to a [Model]’s power and ground reference terminals.  This simple C_comp model is not accurate enough for high speed buffers. This BIRD enhances IBIS to allow an alternative C_comp Model using an IBIS-ISS subcircuit or Touchstone file.  An enhanced C_comp Model would allow modeling of effects such as frequency and voltage dependencies.

STATEMENT OF THE RESOLVED SPECIFICATIONS:

A new keyword [C_comp Model] shall be positioned after [C Comp Corner]. In the tree diagram under [Model] and after [C Comp Corner], add:

│         ├── [C_comp Model]				Param, File_TS, File_ISS

│         │							Number_of_Terminals, Terminal

After the keyword [C Comp Model] add:

Keywords:	[C_comp Model], [End C_comp Model]

Required:	No

Description:	Defines an enhanced C_comp model referenced in an external file using either the Touchstone or IBIS-ISS languages. The [C_comp Model] has ports compatible with the [Model] keyword or has ports compatible with the [Model] keyword plus additional ports connecting between the [Model] and the pad and/or at the input buffer.

Sub-Params:	Param, File_TS, File_ISS, Number_of_Terminals, Terminal

Usage Rules:	If [C_comp Model] is present, it overrides [C Comp Corner] or any other C_comp* representations.

Other Notes:

IBIS-ISS interconnect models allow for on-die interconnect circuits between the IBIS buffer and the Die Pad. A [C_comp Model] may also contain series elements modeling on-die interconnect between the buffer and the Die Pad.  Use of an on-die interconnect model and/or [C_comp Model] with series elements in the signal path shall assume that V-T and I-V curves are measured at the A_signal terminal of the [Model].  If there is no on-die interconnect model, then this is the “Pad” in legacy IBIS models. If there is on-die interconnect, and an associated on-die interconnect model, then the I-V and V-T curves are assumed to be measured at the node between the [Model] and on-die interconnect.

- If the V-T and I-V curves are measured in a test fixture that includes the on-die interconnect, package and test fixture itself, then all of the on-die interconnect, package and test fixture shall be de-embedded to give the V-T and I-V curves as if generated using the following simulation method.

- If the V-T and I-V curves are generated by SPICE simulation it shall be assumed that the SPICE model shall include the [C_comp Model] and that the terminal of the SPICE model shall connect the Device Under Test (DUT, Figure 15) directly to the Test Fixture. When using the [C_comp Model], the [Model] may not use L_dut, R_dut and C_dut.

The effect of series elements in a [C_comp Model] may need to be de-embedded from I-V waveform data in the [Model].

Number_of_Terminals rules:

Subparameter Definitions:The Number_of_Terminals subparameter is required and defines the number of terminals associated with the C_comp Model. The subparameter name is followed by a single integer argument greater than zero on the same line, separated from the subparameter name by whitespace.  Only one Number_of_Terminals subparameter may appear for a given [C_comp Model] keyword.

Unless noted below, no subparameter requires the presence of any other subparameter.  Each subparameter is optional.

Param:

The Param subparameter is optional and only legal for File_ISS references.  Param shall be followed by four arguments: a string argument, param_name, which is the name of the parameter to be passed into the IBIS-ISS subcircuit; and three numerical values or three string values (surrounded by double quotes) located in the typ, min, and max columns.  Several Param lines are permitted as long as each of the param_name entries is unique within that [C_comp Model] keyword.  Each Param line shall have a typ entry.  Either or both the min and max entries may be NA, in which cases the typ entry is used.  The typ, min, and max parameters are, by default, associated with the corner_name Typ, Min, and Max files and their corresponding circuit_names.  However, the EDA tool is expected to support passing any of the Param typ, min, or max values, as selected by the User or EDA tool, into any File_ISS corner_name file.  The Param values associated with any param_name shall all be numerical or all string values (or NA).  If possible, the Param min and max values should represent slow and fast interconnect conditions.  Because of parameter interactions, this may not always be possible.

Other Notes:  The numerical value rules follow the scaling conventions in Section 3, GENERAL SYNTAX RULES AND GUIDELINES.  The EDA tool is responsible for translating IBIS specified parameters into IBIS-ISS parameters.  For example, 1 megohm, represented as 1M in Param would be converted to 1meg (1x is not recommended) in IBIS-ISS.  The value 1Kohm is 1 ohm in IBIS and would therefore be passed into IBIS-ISS as 1 ohm, even though 1K is 1 kilohm in IBIS-ISS.  Quoted string parameters are converted to the string parameter syntax in IBIS-ISS.  For example, the Param value “typ.s2p” is converted to str(‘typ.s2p’) in IBIS-ISS.

The base unit of frequency is hertz, and the base unit of length is meter.  Values can be passed in terms of other base units of length if scaling conversions are added to the IBIS-ISS .subckt definition. For example, the intended value of 10 mils might be entered as the Param value of 10 if the conversion to 10 mils is done through multiplication within the .subckt.

Examples:

| Param      param_name typ       min       max

Param        abc        2m        1m        2m

Param        def        4k        NA        NA

Param        ts_file    “typ.s2p” “min.s2p” “max.s2p” | used in IBIS-ISS

File_TS:

Either File_TS or File_ISS is required for a [C_comp Model]/[End C_comp Model] group.  File_TS is followed by three entries for typ, min, and max file names.  The typ entry is required and shall point to a Touchstone file located in the same directory as the .ibs file and representing typical conditions.  The min and max entries may point to the same file or other files representing minimum (slow) and maximum (fast) models or contain NA. If the entry is NA, the typical file entry shall be used.

Examples:

| file_type  typ             min             max

File_TS      c_comp_typ.s8p  c_comp_min.s8p  c_comp_max.s8p

| file_type  typ             min             max

File_TS      c_comp_typ.s4p  c_comp_min.s4p  NA

File_ISS:

Either File_TS or File_ISS is required for a [C_comp Model]/[End C_comp Model] group.  The File_ISS subparameter is followed by three string arguments consisting of corner_name, file_name, and circuit_name (.subckt name). The IBIS-ISS (.iss) file shall be located in the same directory as the .ibs file.  The corner_name shall be Typ, Min, or Max.  File_ISS for the Typ corner_name is required, and File_ISS for the Min and Max corner_names are optional.  If present, each File_ISS shall have a unique corner_name.  If File_ISS for either the Min or Max corner_name is missing, the File_ISS for the Typ corner_name shall be used to describe the missing corner_name file reference.  The Min and Max file_names should represent slow and fast model conditions respectively.

Examples:

| file_type  corner_name  file_name       circuit_name (.subckt name)

File_ISS     Typ          c_comp.iss      c_comp_typ

File_ISS     Min          c_comp.iss      c_comp_min | in same file as net.sp

File_ISS     Max          c_comp_max.iss  c_comp_max | in separate file

Number_of_Terminals:

The Number_of_Terminals subparameter is required and defines the number of terminals associated with the [C_comp Model]. The subparameter name is followed by a single integer argument equal to or greater than two on the same line, separated from the subparameter name by whitespace.  Only one Number_of_Terminals subparameter may appear for a given [C_comp Model] keyword.

Terminal:

Two or more Terminal subparameters may appear under a given [C_comp Model] keyword.  At least one signal and one reference Terminal subparameter is required.  Each Terminal record contains information on a single terminal of an IBIS-ISS subckt (or Touchstone file).

The Terminal subparameter is followed by two arguments: Terminal_number and Terminal_Location.

Terminal_number shall be a positive, non-zero integer and less than or equal to the number of terminals in the Number_of_Terminals argument. The same Terminal_number shall not appear more than once for a given [C_comp Model].  All Terminals must be present for a given [C_comp Model].

A [C_comp Model] can either replace C_comp by connecting a single terminal of the [C_comp Model] at the same location that the [Model]’s C_comp connects (see Figure X), or it can replace C_comp with a model containing series elements (see Figure Y).  In this case, the [C_comp Model] will require a terminal at the output of the buffer (A_signal) and a terminal at an internal buffer node (A_signal_I).

Figure X

Figure Y

Some Input and/or I/O buffers contain series elements between the Die Pad and the input buffer circuit that may provide isolation or filtering.  If it is desired to view the analog input waveform at the input buffer, the [C_comp Model] can contain the terminal A_receive.  The terminal A_receive is analogous to the terminal my_receive of an [External Model] as seen in Figure 24.

Terminal_Location is a string and shall have one of the values A_signal, A_signal_I, A_signal_pos, A_signal_pos_I, A_signal_neg, A_signal_neg_I, A_receive, A_puref, A_pdref, A_pcref, A_gcref or A_extref.

- A_signal indicates this terminal connects to the [Model]’s signal terminal. This is the location that is either at the Die Pad, or there may optionally be on-die interconnect between this terminal and the Die Pad.

- A_signal_I is used when there is a series component to the [C_comp Model].

- A_signal_pos indicates this terminal connects to the positive (non-inverting) side of a differential [Model]’s signal terminal. This is the location that is either at the Die Pad, or there may optionally be on-die interconnect between this terminal and the Die Pad.

- A_signal_pos_I is used when there is a series component to the differential [C_comp Model].

- A_signal_neg indicates this terminal connects to the complementary (Inverting) side of the [Model]’s signal terminal. This is the location that is either at the Die Pad, or there may optionally be on-die interconnect between this terminal and the Die Pad.

- A_signal_neg_I is used when there is a series component to the differential [C_comp Model].

- A_receive is used when there is a series component to the input buffer.

- A_puref indicates this terminal connects to the [Model]’s pullup reference.

- A_pdref indicates this terminal connects to the [Model]’s pulldown reference.

- A_pcref indicates this terminal connects to the [Model]’s power clamp reference.

- A_gcref indicates this terminal connects to the [Model]’s ground clamp reference.

- A_extref indicates this terminal connects to the [Model]’s external reference.

Other Notes:

For a [C_comp Model] using File_TS with N ports, N is either determined from the N in the .sNp file name extension for a Touchstone 1 file or from the [Number of Ports] record in a Touchstone 2 file. The [Number of Terminals] in the [C_comp Model] shall be N+1.  Terminal rules are described below:

- The EDA tool shall use the Terminal_Location specified in the Terminal “N+1” record as the reference node for each of the N ports.

- Terminal/Port Mapping

  - Terminal              Port

  - 1                              1

  - 2                              2

  - …

  - N                             N

  - N+1		reference

- All Ports shall be connected through the Terminal record.

- It shall be an error if Terminal N+1 is not specified to a Buffer terminal that is not on a connection to a Signal_name that is POWER or GND

Examples:

[C_comp Model]

File_ISS  Typ  A.iss  A

Param  Value  C  1pF  2pF  0.5pF

Number_of_Terminals 2

Terminal 1 A_signal

Terminal 2 A_gcref

[End C_comp Model]

[C_comp Model]

File_TS  C_typ.s2p  C_min.s2p  C_max.s2p

Number_of_Terminals 3

Terminal 1 A_signal_I

Terminal 2 A_signal

Terminal 3 A_gcref

[End C_comp Model]

[C_comp Model]

File_ISS  Typ  B.iss  B

Number_of_Terminals 7

Terminal 1 A_signal_I

Terminal 2 A_signal

Terminal 3 A_puref

Terminal 4 A_pdref

Terminal 5 A_pcref

Terminal 6 A_gcref

Terminal 7 A_receive

[End C_comp Model]

[C_comp Model]

File_TS  C_typ.s4p  NA  NA

Number_of_Terminals 5

Terminal 1 A_signal_pos_I

Terminal 2 A_signal_neg_I

Terminal 3 A_signal_pos

Terminal 4 A_signal_neg

Terminal 5 A_gcref

[End C_comp Model]

________________________________________________________

The following section should be appended to the end of the IBIS document.

RULES OF PRECEDENCE

The EDA tool shall either use C_comp or [C_comp Model], but not both. The user and EDA tool may assume that the [C_comp Model] is more accurate than C_comp.

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

Inadequacies with the C_Comp* model have been known for a long time.  Some presentations on C_comp improvements include:

http://www.eda.org/ibis/summits/oct04/mirmak2.pdf

http://www.eda.org/ibis/summits/jan15/wolff.pdf

BIRD79 was an effort to improve C_comp modeling: http://www.eda.org/ibis/birds/bird79.txt.

ANY OTHER BACKGROUND INFORMATION:

{These documents will be archived, so use this section to add any detail that is not part of the section above or the changed text itself , but should not be lost.}