BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	(for administrative use)

ISSUE TITLE:   		[Pin Reference]

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:	(Draft 3_ross, May 17, 2016)

DATE REVISED:	(for administrative use)

DATE ACCEPTED:	(for administrative use)

DEFINITION OF THE ISSUE:

All measurements (“IBIS Data”) that are used to generate voltage values for IBIS subparameters within the [Model], [Model Spec], [Submodel Spec], and [Receiver Thresholds] keywords and are relative to a test fixture reference node or a simulator reference node.

IBIS defines the derivation of “IBIS Data” consisting of I-V, V-T, ISSO and voltage thresholds for a device under test. For I-V, voltages are defined as measured across the associated [Pullup], [Pulldown], [POWER Clamp], and [GND Clamp] elements. For V-T, ISSO and voltage thresholds, IBIS defines the reference node used to measure these voltages. IBIS contemplates the use of these models with the buffer supplied by specific rail voltages prescribed by the [Voltage Range], [Pullup Reference], [Pulldown Reference], [POWER Clamp Reference], [GND Clamp Reference], and [External Reference] (“[*** Reference]”) keywords. These voltages are measured relative to the test fixture reference.

During a simulation that uses IBIS Models, the IBIS specification is not clear what node should be used as the reference node for these voltages. This is not an issue when the simulator supplies rail voltages (“***_ref”) to a model relative to the simulator reference node that are same as the reference voltages (“[*** Reference]”) supplied to the buffer when generating the IBIS Data. Some EDA tools use the terminal of the IBIS model that has a [*** Reference]=0.0V as the reference node for measurements. As a result, when a model is simulated with voltages applied to the models' rail terminals (relative to a EDA tool reference node) other than the prescribed values, it is not defined in the specification how to compare the voltages at the buffer I/O (pin) terminal with the thresholds that were generated relative to the test fixture reference.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Allow an EDA tool to use IBIS model threshold in simulations where the voltages applied to a model rail terminals are not the same as the voltages applied to the rail terminals of the model when the “IBIS Data” is generated. |  |

(Enumerate each requirement in the table above, adding rows as needed.)

SUMMARY OF PROPOSED CHANGES:

This BIRD address this confusion by specifying supply pinwhose voltage the EDA tool can use to adjust the voltage measurement at the model I/O terminal that can be compared with the model thresholds.

For review purposes, the proposed changes are summarized as follows:

Table 2: IBIS Keywords, Subparameters,  AMI Reserved_Parameters, and AMI functions  Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| New [Component] section [Pin Reference] keyword | New | It is requiredthat there is a [Pin Mapping] section in the [Component] |

PROPOSED CHANGES:

Add to section 5 after [Pin Mapping]:

Keyword:	[Pin Reference]

Required:	No

Description:	This keyword defines for an I/O pin_name under the [Pin] keyword a bus_label entry to be used as reference node for voltage measurements at the I/O pin.

Sub-Params:	bus_label

Usage Rules:	For each pin_name listed, the simulation node at the bus_label shall be used as the reference node of measurements at the pin_name node when comparing simulation results with model thresholds.

The [Pin Reference] keyword is followed by the column heading bus_label.  Under the [Pin Reference] keyword each line of data consists of two columns for the pin_name entry and bus_label entry.  Each pin_name pin must exist under the [Pin] keyword.

Only pin_name pins that are for Input*, Output*, or I/O* models described later under Model_type need to be listed.  However, the listing of any pin_name is optional.

The [Pin Mapping] keyword, where

bus_label entries are defined, must exist.

** (Additional rules already exits in [Pin Mapping] and/or will be captured in another BIRD, not here)

Other Notes:	If a pin_name in the component section does not have an entry under the [Pin Reference] keyword, and there is a model_name on that pin that is not NC, POWER or GND, then the EDA tool must choose a reference node for simulation results at the pin_name and rail terminals of the model.  Some EDA tools use simulator Node 0 for this reference. Some EDA tools use the rail terminal that has a reference voltage [* Reference] ([Pullup Reference], [Pulldown Reference], [POWER Clamp Reference], [GND Clamp Reference], [Pullup Reference], and [External Reference]).  The [Pin Reference] keyword allows the model maker to describe the node.

When analyzing the waveforms at the buffer to compare them to such things as Vinl, Vinh, Vmeas and Receiver Thresholds, the voltage at the I/O Pin relative to the EDA tool reference node, must be adjusted by the difference of the voltage at the [Pin Reference] pin_name term relative to the EDA tool reference node and the value of the [* Reference] for the corresponding bus_label terminal.

Note that the “Reference_supply” in the [Receiver Thresholds] section may not be the same rail bus_label as the bus_label in the [Pin Reference] for that buffer. The equation for the adjusted Vth must also supply this correction to the voltage at the Reference_supply terminal relative to the EDA tool reference node.

Note, the appropriate reference supply should be a bus_label terminal whose corresponding [* Reference] value is based on the dominant internal electrical circuitry.  For some technologies, the appropriate reference, such as PECL and Open_source configurations, the appropriate reference should be the most positive terminal.  For Bipolar (TTL) technology, the most negative terminal or the one that is closest to GND may be the appropriate value.  For some technologies, the threshold values can depend on two rails.  So the model developer should choose the one that would impact threshold and Vref and Vmeas values.

Normally the [Pin Reference] terminal is in a bus_label that is supplied to one of the [* Reference] terminals of the model, however there may be models that none of the [* Reference] terminals of the model are used as the reference node of the test fixture. For example, this may occur on RS232, and PECL buffers where none the [* Reference] voltages are 0.0 V relative to the test fixture reference. This may only occur if there is a pin with a signal_name (note using signal_name purposely here) that is used as the test fixture reference. By definition of the data derivation methods described in this document for Devices Under Test, this voltage is 0.0 V relative to the test fixture reference. In this case the voltage at the I/O Pin relative to the EDA tool reference node, must be adjusted by the voltage at the [Pin Reference] node (note the [Pin Reference] node is not a terminal of the model) relative to the EDA tool reference node.

Example:

[Component]     Reference_Node_Example

| ...

|

[Pin]  signal_name        model_name        R_pin     L_pin     C_pin

|

1      IO_CMOS_3_3        IO_CMOS_3_3

2      IO_CMOS_3_3-PC_5_0 IO_CMOS_3_3-PC_5_0

3      IO_OPEN_SOURCE_3_3 IO_OPEN_SOURCE_3_3

4      IO_TTL_5_0         IO_TTL_5_0

|

5      IO_ECL_0_0         IO_ECL_0_0

6      IO_PECL_5_0        IO_PECL_5_0

7      IO_PECL_2_0_M3_2   IO_PECL_2_0-M3_2

|

8      OUT_RS232         OUT_RS232_10_0-M10_0

9      IN_RS232_5_0      IN_RS232_5_0

|

| Power Rail Voltages for Examples in [Pin Mapping]

|

20     VCC_10_0          POWER    | 10.0 V

21     VCC_5_0           POWER    | 5.0 V

22     VCC_3_3           POWER    | 3.3 V

23     VCC_2_0           POWER    | 2.0 V

24     VEE_0_0           GND      | 0.0 V

25     VSS_M3_2          POWER    | -3.2 V

26     VSS_M5_2          POWER    | -5.2 V

27     VSS_M10_0         POWER    | -10.0 V

|

[Pin Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref

|

1             VEE_0_0      VCC_3_3                              | IO_CMOS_3_3

2             VEE_0_0      VCC_3_3    VEE_0_0       VCC_5_0     | IO_CMOS_3_3-_PC_5_0

|

3             VEE_0_0      VCC_3_3    VEE_0_0       VCC_3_3     | IO_OPEN_SOURCE_3_3

|

4             VEE_0_0      VCC_5_0                              | IO_TTL_5_0

|

5             VEE_0_0      VEE_0_0    VSS_M5_2      VEE_0_0     | IO_ECL_0_0

6             VCC_5_0      VCC_5_0    VEE_0_0       VCC_5_0     | IO_PECL_5_0

7             VCC_2_0      VCC_2_0    VSS_M3_2      VCC_2_0     | IO_PECL_2_0-M3_2

|

8             VSS_M10_0    VCC_10_0                             | OUT_RS232_10_0-M10_0

9             VEE_0_0      VCC_5_0                              | IN_RS232_5_0

|

20            NC           VCC_10_0   | POWER   10.0 V

21            NC           VCC_5_0    | POWER    5.0 V

22            NC           VCC_3_3    | POWER    3.3 V

23            NC           VCC_2_0    | POWER    2.0 V

24            VEE_0_0      NC         | GND      0.0 V

25            NC           VSS_M3_2   | POWER   -3.2 V

26            NC           VSS_M5_2   | POWER   -5.2 V

27            NC           VSS_M10_0  | POWER  -10.0 V

|

[Pin Reference] bus_label

|

| For CMOS, the pulldown_ref and pullup_ref are equally dominant for thresholds

|

1               VEE_0_0   | VCC_3_3 could also be chosen

2               VEE_0_0   | VCC_3_3 could also be chosen

|

| For IO_Open Source models the pullup_ref entry is dominant for thresholds

|

3               VCC_3_3   | VCC_3_3 is the dominant reference

|

| For TTL, the pulldown_ref is the dominant reference

|

4               VEE_0_0   | VEE_0_0 is the dominant reference

|

| For ECL/PECL, the most positive voltage (pullup_ref = pulldown_ref) is dominant

|

5               VEE_0_0

6               VCC_5_0

7               VCC_2_0

|

| For normal OUT_ RS232 with +/- voltages generated by charge pumps, either

| the pulldown_ref or the pullup_ref are equally dominant

|

8               VSS_M10_0 | VCC_10_0 is equally dominant for RS232 Output

9               VEE_0_0   | VCC_5_0 is equally dominant for CMOS Input

|

BACKGROUND INFORMATION/HISTORY:

Walter Katz gave a presentation “Receiver_Thresholds Assume GND=0.0V=Node 0” in the April 19, 2016 IBIS-ATM meeting, describing this issue.

Bob Ross gave a presentation "[Pin Reference] Cases regarding dominant internal electrical circuitry dependencies at the May 3, 2016 IBIS-ATM meeting that showed the pullup reference dominance for ECL/PECL, and showed that CMOS has equal dominance between pulldown and pullup references