BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	(Draft 2)

ISSUE TITLE:   		Rx Deterministic Noise Support for AMI

REQUESTOR:     	Michael Mirmak, Intel Corp.

DATE SUBMITTED:

DATE REVISED:

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

IBIS 6.1 defines separate deterministic (uniform) and random (Gaussian) jitter Reserved Parameters for Tx and Rx devices as Tx_Dj, Tx_Rj, Rx_Dj and Rx_Rj, respectively.  However, the receiver noise Reserved Parameter Rx_Noise covers only random (Gaussian) noise.  A more complete definition of receiver noise would include a parameter for deterministic (uniform) noise, and would ideally use names for both noise parameters that parallel the naming convention for jitter.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Deterministic (uniform) Rx Noise must be supported by IBIS-AMI, separately from the existing Gaussian random Rx Noise parameter. |  |
| - The existing Rx_Noise Reserved Parameter must be clarified as referring to Gaussian random noise, as is already done for Tx_Rj and Rx_Rj. |  |
| - The ranges for the Gaussian and uniform random multipliers of the noise value in the equations to be used by EDA tools should be clearly stated. |  |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

Table 2: IBIS Keywords, Subparameters,  AMI Reserved_Parameters, and AMI functions  Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| The Reserved Parameter Rx_BoundedUniform is defined | New |  |
| The Reserved Parameter Rx_Gaussian is added as an alternate name for Rx_Noise. | Modified | This is a modification of the existing parameter Rx_Noise, to clarify its relationship to the new Reserved Parameter Rx_BoundedUniform |
| The range for gaussian_rand() and the meaning of the Rx_Noise equation is clarified. | Other | The text of the Rx_Noise parameter definition is clarified to define the range for gaussian_rand(), with the output and input to the equations made explicit. |

PROPOSED CHANGES:

The definition of Rx_Noise on page 228 of the existing IBIS 6.1 specification should be changed from:

Parameter:	Rx_Noise

Required:	No, and illegal before AMI_Version 6.0

Direction:	Rx

Descriptors:

Usage:		Info, Out, Dep

Type:		Float

Format:		Value, List, Range, Corner, Increment, Steps

Default:	<numeric_literal>

Description:	<string>

Definition:	The standard deviation, in volts, of a white Gaussian random process, which is to be added by the EDA tool to the signal measured at the sampling latch of a receiver.

Usage Rules:	If Rx_Noise is Usage Out, then the EDA tool shall use the value returned by Rx AMI_Init if Rx AMI_GetWave is not used. If Rx AMI_GetWave is used, then the EDA tool may apply the value returned by each AMI_GetWave call to the waveform returned by that call to AMI_GetWave, or use the average value of Rx_Noise returned by all calls to AMI_GetWave (after Ignore_Bits), or the value of Rx_Noise returned by the last call to AMI_GetWave.

Other Notes:	Time is calculated as follows:

wave(t) = wave(t) + Rx_Noise * gaussian_rand()

Where wave(t) is the waveform returned by Rx AMI_GetWave.

Example:

(Rx_Noise (Usage Info) (Value 0.010) (Type Float)

(Description "Rx amplitude noise at sampling latch in volts."))

… to:

Parameter:	Rx_Noise, Rx_Gaussian

Required:	No, and Rx_Noise is illegal before AMI_Version 6.0; Rx_Gaussian is illegal before AMI_Version 6.2

Direction:	Rx

Descriptors:

Usage:		Info, Out, Dep

Type:		Float

Format:		Value, List, Range, Corner, Increment, Steps

Default:	<numeric_literal>

Description:	<string>

Definition:	The standard deviation, in volts, of a white Gaussian random process, which is to be added by the EDA tool to the signal measured at the sampling latch of a receiver.

Usage Rules:	If Rx_Noise is Usage Out, then the EDA tool shall use the value returned by Rx AMI_Init if Rx AMI_GetWave is not used. If Rx AMI_GetWave is used, then the EDA tool may apply the value returned by each AMI_GetWave call to the waveform returned by that call to AMI_GetWave, or use the average value of Rx_Noise returned by all calls to AMI_GetWave (after Ignore_Bits), or the value of Rx_Noise returned by the last call to AMI_GetWave.

Other Notes:	The output voltage waveform is calculated as follows:

Output_wave(t) = wave(t) + Rx_Noise * gaussian_rand()

Where wave(t) is the waveform returned by Rx AMI_GetWave and gaussian_rand() is a function that returns floating point numbers between -inf and +inf.  The distribution of these numbers shall be a white Gaussian distribution centered at 0.0 with a standard deviation of 1.0.

Rx_Gaussian is permitted as an alternate name for Rx_Noise in AMI_Version 6.2 and higher.

Example:

(Rx_Noise (Usage Info) (Value 0.010) (Type Float)

(Description "Rx amplitude noise at sampling latch in volts."))

Immediately after, a new Reserved Parameter should be added:

Parameter:	Rx_BoundedUniform

Required:	No, and illegal before AMI_Version 6.2

Direction:	Rx

Descriptors:

Usage:		Info, Out, Dep

Type:		Float

Format:		Value, List, Range, Corner, Increment, Steps

Default:	<numeric_literal>

Description:	<string>

Definition:	The worst-case half peak-to-peak variation, in volts, of a uniform random process which is to be added by the EDA tool to the signal measured at the sampling latch of a receiver.

Usage Rules:	If Rx_BoundedUniform is Usage Out, then the EDA tool shall use the value returned by Rx AMI_Init if Rx AMI_GetWave is not used. If Rx AMI_GetWave is used, then the EDA tool may apply the value returned by each AMI_GetWave call to the waveform returned by that call to AMI_GetWave, or use the average value of Rx_BoundedUniform returned by all calls to AMI_GetWave (after Ignore_Bits), or the value of Rx_BoundedUniform returned by the last call to AMI_GetWave.

Other Notes:	The output voltage waveform is calculated as follows:

Output_wave(t) = wave(t) + 2 * Rx_BoundedUniform * rand()

Where wave(t) is the waveform returned by Rx AMI_GetWave and rand() is a function that returns floating point numbers between -0.5 and +0.5 with white uniform distribution.

Example:

(Rx_BoundedUniform (Usage Info) (Value 0.010) (Type Float)

(Description "Rx deterministic amplitude noise at sampling latch in volts."))

BACKGROUND INFORMATION/HISTORY:

This was submitted by Michael Mirmak of Intel Corp. as a draft for review by the IBIS Advanced Technology Modeling Task Group on November 10, 2016.

Draft 2 changes the names of Rx_Dn and Rx_Rn to Rx_BoundedUniform and Rx_Gaussian, at the suggestion of participants in the IBIS-ATM Task Group.