BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	197.3_draft_1

ISSUE TITLE:   		New AMI Reserved Parameter DC_Offset

REQUESTOR:     	Walter Katz, SiSoft,

Ambrish Varma, Cadence Design Systems,

Randy Wolff, Micron Techology,

Justin Butterfield, Micron Technology

DATE SUBMITTED:	November 27, 2018

DATE REVISED:	December 4, 2018, January 15, 2019

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

AMI modeling is now being applied to single ended channels (e.g. DDR5). The current input to AMI_Init is an Impulse Response. This forces all AMI simulations to be centered around the mid-level of the signal of a single ended port.  A DLL may need to know the single ended voltage levels (e.g. to handle saturation in a DFE summer). This BIRD proposes a new AMI Reserved Parameter DC_Offset, which is the mid value of the beginning and end of the step response of the channel.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Allow the EDA tool to convey to the model the mid-point of the steady state high and low voltages found during analog model characterization of single ended signals. |  |

SUMMARY OF PROPOSED CHANGES:

Add new AMI Reserved Parameter DC_Offset.

PROPOSED CHANGES:

Parameter:	DC_Offset

Required:	No, and illegal before AMI_Version 7.0

Direction:	Rx

Descriptors:

Usage:                   In

Type:                     Float

Format:                  Value

Default:                 <numeric_literal>

Description:           <string>

Definition:	The mean value of the steady state high and low voltage of the channel at the Rx pad.

Usage Rules:	If the impulse response was generated by differentiating the step response, then the value of DC_Offset should be the same as the average of the step response initial and final voltages.

The AMI_Init function can use this value to determine the voltages inside of the model.

It is also assumed that the waveform input to the Rx AMI_GetWave function is the waveform minus this DC_Offset. The Rx AMI_GetWave function can choose to construct the single ended waveform by adding DC_Offset to the input waveform. The waveform output of the Rx AMI_GetWave shall be adjusted so that the EDA tool can add DC_Offset to get the voltage of the waveform at the slicer (aka latch, decision point) of the single ended port.

Other Notes:	It is the responsibility of the EDA tool to determine the DC_Offset. The EDA tool may use any method to do this.

Example:

DC_Offset (Usage In) (Type Float) (Value 0.5)

(Description “The EDA tool is responsible for determining the DC_Offset value to input to the executable model.”)

BACKGROUND INFORMATION/HISTORY:

Typographical updates made in BIRD197.1, based on feedback from Open Forum and ATM review.

BIRD197.2 contains additional editorial changes.

BIRD197.3 contains editorial changes to the verbiage related to the usage of the words “single ended”.