BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER:

ISSUE TITLE:   		New GetWave API for Clock Forwarding Modeling

REQUESTOR:     	Fangyi Rao, Keysight Technologies

Wendem Beyene, Intel

DATE SUBMITTED:

DATE REVISED:

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

In DDR systems, DQ Rx uses DQS Rx output signal as a forwarded clock to clock the slicer and the DFE. To model such clock forwarding functionality, the DQ Rx model needs two input waveforms, one for DQ, and the other for DQS. This BIRD proposes a new API AMI_GetWave2 and a new AMI Reserved Parameter GetWave2_Exists to enable modeling of clock forwading.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Allow the Rx model to model clock forwarding. |  |

SUMMARY OF PROPOSED CHANGES:

Add new API AMI_GetWave2 and new AMI Reserved Parameter GetWave2_Exists

PROPOSED CHANGES:

Parameter:	GetWave2_Exists

Required:	No, and illegal before AMI_Version 7.2

Direction:	Rx

Descriptors:

Usage:                   Info

Type:                     Boolean

Format:                 Value

Default:                 <Boolean_literal>

Description:          <string>

Definition:	Tells the EDA tool whether the AMI_GetWave2 is implemented in this model.

Usage Rules:	If omitted, the default is “False”. If Init_Returns_Impulse is set to “False”, then GetWave_Exists or GetWave2_Exists or both shall be set to “True”.

Other Notes:

If both GetWave_Exists and GetWave2_Exists are “True”, then model consumer (including EDA tool) decides whether to use AMI_GetWave or AMI_GetWave2 in simulation.

Example:

(GetWave2_Exists (Usage Info) (Type Boolean) (Default True)

(Description "Model implements AMI_GetWave2 function")

)

(GetWave2_Exists (Usage Info) (Type Boolean) (Value True)

(Description "Model implements AMI_GetWave2 function")

)

Function: AMI_GetWave2

Required: No

Declaration: long AMI_GetWave2 (double *wave,

double *wave_clk,

long wave_size,

double *clock_times,

char ** AMI_parameters_out,

void * AMI_memory)

Arguments:

wave

Same as for AMI_GetWave.  See AMI_GetWave.

wave_clk

“wave_clk” points to a memory location where a uniformly sampled vector of a time domain waveform is stored.  The sample spacing and the number of samples of the “wave_clk” waveform are the same as those of the waveform of the AMI_GetWave2 argument “wave”, respectively. The waveform pointed to by the “wave_clk” argument is input.  The algorithmic model can use the waveform to control the timing of sampling and equalization on the waveform of the AMI_GetWave2 argument “wave”. If the sampling timing is controlled by “wave_clk”, the algorithmic model is expected to derive values in the AMI_GetWave2 argument “clock_times” from “wave_clk”. Note that the algorithmic model can internally process the waveform of “wave_clk” before it is used.

The EDA tool provides the waveform of “wave_clk”. In the case of DQ Rx model in a DDR channel, the input waveform of “wave_clk” is the DLL output waveform of the DQS Rx that provides the forwarded clock to that DQ Rx.

wave_size

Same as for AMI_GetWave.  See AMI_GetWave.

clock_times

Same as for AMI_GetWave.  See AMI_GetWave.

AMI_parameters_out

Same as for AMI_GetWave.  See AMI_GetWave.

AMI_memory

Same as for AMI_GetWave.  See AMI_GetWave.

ReturnValue

Same as for AMI_GetWave.  See AMI_GetWave.

Note:

AMI_GetWave2 shall be implemented only by Rx models that need to model clock forwarding. In DDR systems, DQ Rx models can implement AMI_GetWave2. DQS, command-address, conotrol and clock Rx models shall not implement AMI_GetWave2 because these signals do not use forwarded clock.

BACKGROUND INFORMATION/HISTORY: