BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER:

ISSUE TITLE:   		New GetWave API for Clock Forwarding Modeling

REQUESTOR:     	Fangyi Rao, Keysight Technologies

Wendem Beyene, Intel

DATE SUBMITTED:

DATE REVISED:

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

In DDR systems, DQ Rx uses DQS Rx output signal as a forwarded clock to clock the slicer and the DFE. To model such clock forwarding functionality, the DQ Rx model needs two input waveforms, one for DQ, and the other for DQS. This BIRD proposes a new API AMI_GetWave2 and a new AMI Reserved Parameter GetWave2_Exists to enable modeling of clock forwading.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Allow the Rx model to model clock forwarding. |  |

SUMMARY OF PROPOSED CHANGES:

Add new API AMI_GetWave2 after AMI_GetWave and new AMI Reserved Parameter GetWave2_Exists after GetWave_Exists in the IBIS Spec.

PROPOSED CHANGES:

Parameter:	GetWave2_Exists

Required:	No, and illegal before AMI_Version 7.1

Direction:	Rx

Descriptors:

Usage:                   Info

Type:                     Boolean

Format:                 Value

Default:                 <Boolean_literal>

Description:          <string>

Definition:	Tells the EDA tool whether the AMI_GetWave2 is implemented in this model.

Usage Rules:	If omitted, the default is “False”. If Init_Returns_Impulse is set to “False”, then GetWave_Exists or GetWave2_Exists or both shall be set to “True”.

Other Notes:

If both GetWave_Exists and GetWave2_Exists are “True”, then model consumer (including EDA tool) decides whether to use AMI_GetWave or AMI_GetWave2 in simulation.

Example:

(GetWave2_Exists (Usage Info) (Type Boolean) (Default True)

(Description "Model implements AMI_GetWave2 function")

)

(GetWave2_Exists (Usage Info) (Type Boolean) (Value True)

(Description "Model implements AMI_GetWave2 function")

)

Function: AMI_GetWave2

Required: No

Declaration: long AMI_GetWave2 (double *wave,

double *wave_clk,

long wave_size,

double *clock_times,

char **AMI_parameters_out,

void *AMI_memory)

Arguments:

wave

Same as for AMI_GetWave.  See AMI_GetWave.

wave_clk

“wave_clk” points to a memory location where a uniformly sampled vector of a time domain waveform is stored.  The sample spacing and the number of samples of the “wave_clk” waveform are the same as those of the waveform of the AMI_GetWave2 argument “wave”, respectively. The waveform pointed to by the “wave_clk” argument is input.  The algorithmic model can use the waveform to control the timing of sampling and equalization on the waveform of the AMI_GetWave2 argument “wave”. If the sampling timing is controlled by “wave_clk”, the algorithmic model is expected to derive values in the AMI_GetWave2 argument “clock_times” from “wave_clk”. Note that the algorithmic model can internally process the waveform of “wave_clk” before it is used.

The EDA tool provides the waveform of “wave_clk”. In the case of DQ Rx model in a DDR channel, the input waveform of “wave_clk” is the algorithmic model’s output waveform of the DQS Rx that provides the forwarded clock to that DQ Rx.

wave_size

Same as for AMI_GetWave.  See AMI_GetWave.

clock_times

Same as for AMI_GetWave.  See AMI_GetWave.

AMI_parameters_out

Same as for AMI_GetWave.  See AMI_GetWave.

AMI_memory

Same as for AMI_GetWave.  See AMI_GetWave.

ReturnValue

Same as for AMI_GetWave.  See AMI_GetWave.

Note:

AMI_GetWave2 shall be implemented only by Rx models that need to model clock forwarding. In DDR systems, for example, DQ Rx models may implement AMI_GetWave2. DQS, command-address, control and clock Rx models shall not implement AMI_GetWave2 because these signals do not use forwarded clock.

Statements referring to AMI_GetWave in the current Spec also apply to AMI_GetWave2 except in the following sections.

10.2.3 Definition of AMI_GetWave

10.4 Definition of GetWave_Exists

10.4 Definition of Use_Init_Output

When applying statements regarding AMI_GetWave in the current Spec to AMI_GetWave2, the waveform mentioned in these statemetns refers to the waveform of the AMI_GetWave2 argument “wave”. The waveform of the AMI_GetWave2 argument “wave_clk” is provided by the EDA tool when calling AMI_GetWave2 as stated in the AMI_GetWave2 definition.

A time domain simulation flow that utilizes the DQ Rx AMI_GetWave2 function is illustrated in the figure below.

+

-

…

…

DQ0

DQ Tx DLL

DQS Tx DLL

DQ Tx DLL

DQ7

DQS0

+

-

…

…

DQ8

DQ Tx DLL

DQS Tx DLL

DQ Tx DLL

DQ15

DQS1

DQ0

DQ7

+

-

+

-

DQS0

DQ8

DQS1

DQ15

…

…

…

…

DQS Rx DLL

DQ Rx

GetWave2

DQ Rx

GetWave2

DQS Rx DLL

DQ Rx

GetWave2

DQ Rx

GetWave2

1

1

1

1

1

1

2

2

3

3

3

3

Step 1: compute analog channel output according to the current Spec (crosstalk taken into account)

Step 2: compute output of all DQS Rx DLLs according to the current Spec

Step 3: compute output of all DQ Rx DLLs

Channel

BACKGROUND INFORMATION/HISTORY: