BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER:	154.1

ISSUE TITLE:   	Using IBIS-AMI Leaf List_Tip in List Parameters

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:	November 16, 2012

DATE REVISED:	May 24, 2013

DATE ACCEPTED BY IBIS OPEN FORUM:  June 7, 2013

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

Values in a List often are numbers that do not help the User identify the meaning of each List value. The EDA Tool may use the value of each List_Tip field as a Tool Tip for each value in the List.

An example of using List_Tip

(Process (Usage In) (Type Integer)

(List –2 –1 0 1 2)(Default 0)

(List_Tip "X Slow" "Slow" "Typ" "Fast" "X Fast")

(Description "Process corner used in the algorithmic model")

)

ANY OTHER BACKGROUND INFORMATION:

Approved during the IBIS Open Forum teleconference on June 7, 2013.

Modify the Parameter Rules Summary on Pages 174-175 as shown:

PARAMETER RULES SUMMARY

The features of a model described in a parameter definition file are called AMI parameters, and are grouped into the branches Reserved Parameters and Model_Specific Parameters.  AMI parameters are themselves branches, but may only contain leaves and not other branches.

Branches may define AMI parameters and/or other branches.  A branch which contains one or more sub-branches may only contain the (Description <string>) leaf in addition to the sub-branches.  Each sub-branch of a branch must have a unique name.

All AMI parameter branches shall contain leaf entries formatted as follows:

(parameter_name

(Usage <usage>)

(Type <data_type>)

({Format} <data_format> <data>)

(List_Tip)			| only with ({Format} List) as discussed below

(Default <value>)

(Description <string>))

AMI parameter branches shall contain the leaves Type, Usage, and any of the following leaves:

Default

<data_format> or Format <data_format>

List_Tip		| only with List as discussed below

All leaves of the parameter definition file shall begin with one of the following reserved words:

Type

Usage

Description

Default

<data_format> or Format <data_format>

List_Tip		                       | only with List as discussed below

Replace this following text

List <default value> <value> <value> <value> ... <value>

This defines a discrete set of values from which the user may select one value

With

List <default value> <value> <value> <value> ... <value>

This defines a discrete set of values from which the user may select one value.

List_Tip <”default_entry”><”entry”><”entry”><”entry”>…<”entry”>

This is an optional leaf of a parameter with Format List and it is followed by a String entry for each entry in the List. The number of entries in List_Tip must be the same as the number of entries in List. The nth entry in List_Tip shall correspond to the nth entry in List.  Quoted null entries are not permitted. All entries in List_Tip shall be unique, except that if two entries in List are the same, then the corresponding List_Tip entries must also be the same.  List is required for List_Tip to be entered, and the word Format before List_Tip as in (Format List_Tip ,,,) is not allowed.

Example:

(Strength (Usage In) (Type Integer)(Description "Strength of Driver")

(List 0 1 2 3 4)(Default 2)

(List_Tip "Extra Weak" "Weak" "Nominal" "Strong" "Extra Strong"))