BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	184.2

ISSUE TITLE:   		Model_name and Signal_name Restriction for POWER and GND Pins

REQUESTOR:     	Bob Ross, Teraspeed Labs

DATE SUBMITTED:	September 1, 2016

DATE REVISED:	September 16, 2016; December 6, 2016

DATE ACCEPTED:          January 6, 2017

DEFINITION OF THE ISSUE:

This BIRD adds a new requirement to [Pin] keyword Usage Rules that "If any pin has model_name POWER, then all other pins with the same signal_name as this pin must have model_name POWER. If any pin has model_name GND, then all other pins with the same signal_name as this pin must have model_name GND."

BIRD180.X deals with an omission in the IBIS 6.1 specification where there is no statement regarding uniqueness of pin names in the first column of a [Pin] section. This BIRD does not include this rule to keep the issues separate.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Keep editorial changes in BIRD180.X for [Pin] Usage Rules | Must Shall, add CIRCUITCALL |
| - Delete "which shall not be repeated within the same [Pin] keyword for a [Component]" to allow two technical issues to be considered separately as BIRD180.X and this BIRD | Covered in BIRD180.X, deleted here due to possible interest in BIRD125.1, which proposes a method for using duplicate [Pin Numbers] |
| - Add "If two POWER or GND pins have the same signal_name, they must have the same model_name" | New Rule |
| - If both BIRD180.X and this BIRD are approved, the content can be merged (Or the BIRDs can be merged into one BIRD) | All other changes in BIRD180.X and this BIRD should track each other |

(Enumerate each requirement in the table above, adding rows as needed.)

SUMMARY OF PROPOSED CHANGES:

[Pin] keyword Usage Rules adds some editorial text and also adds a new rule listed below.

Table 2: IBIS Keywords, Subparameters,  AMI Reserved_Parameters, and AMI functions  Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| [Pin] keyword Usage Rules | Modified | Retain editorial changes of BIRD180.X Delete "which shall not be repeated within the same [Pin] keyword for a [Component] in BIRD180.X" Add "If any pin has model_name POWER, then all other pins with the same signal_name as this pin must have model_name POWER. If any pin has model_name GND, then all other pins with the same signal_name as this pin must have model_name GND." " |

PROPOSED CHANGES:

Change:

Keyword: [Pin]

Required: Yes

Description: Associates the component’s I/O models to its various external pin names and signal names.

Sub-Params: signal_name, model_name, R_pin, L_pin, C_pin

Usage Rules: All pins on a component must be specified. The first column must contain the pin name. The second column, signal_name, gives the data book name for the signal on that pin. The third column, model_name, maps a pin to a specific I/O buffer model or model selector name. Each model_name must have a corresponding model or model selector name listed in a [Model] or [Model Selector] keyword below, unless it is a reserved model name (POWER, GND, or NC).

to:

Keyword: [Pin]

Required: Yes

Description: Associates the component’s I/O models to its various external pin names and signal names.

Sub-Params: signal_name, model_name, R_pin, L_pin, C_pin

Usage Rules: For a full component description, all pins on a component should be specified. The first column shall contain the pin name.  (The pin name column is also referred to as pin_name elsewhere in this document). The second column, signal_name, gives the data book name for the signal on that pin. The third column, model_name, maps a pin to a specific I/O buffer model or model selector name. Each model_name shall have a corresponding model or model selector name listed in a [Model] or [Model Selector] keyword below, unless it is a reserved model name (POWER, GND, CIRCUITCALL, or NC).

If a pin has model_name POWER, then all other pins with the same signal_name as this pin shall have model_name POWER. If a pin has model_name GND, then all other pins with the same signal_name as this pin shall have model_name GND.

Note, where in conflict, the changes in this BIRD override the changes in BIRD180 with the exception that the change in the sentence below approved in BIRD180 is retained:

The first column shall contain the pin name, which shall not be repeated within the same [Pin] keyword for a [Component].

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

The new statement is added as a new "paragraph' after both signal_name and model_name column entries have been defined in the previous paragraph.  This also highlights a change for the parser developer.

This new rule (not specified or enforced in existing versions of the IBIS Specification is targeted to POWER and GND model_name entries to avoid possible conflicts with the same signal_name used for both a POWER and GND pin.  However, the new rule can apply to all pins because data books do use different signal_names for each non-POWER or non-GND pins

The addition is to apply the rule only to POWER or GND pins.  This simplifies parser checking, and the more general statement is not needed for the pending Interconnect Specification that uses only pin_names for I/O pins.  Note, in a commercial model based on data sheet information, there would not be similar signal_names for I/O pins.

ANY OTHER BACKGROUND INFORMATION:

Walter Katz proposed the language in the ATM Task Group in the BUSLabelBIRD_1.docx, July 12, 2016 (it may have been proposed elsewhere or earlier).  Walter could be listed as the primary author.

A revised language was proposed by Walter to Bob Ross on August 30, 2016.  It clarifies the intent and broadens the rule so that if any pin has a model_name POWER, then its signal_name must not be used for any other pins except those with model_name POWER.  Similarly for model_name GND pins.  The earlier statement could have been interpreted to not exclude NC or even I/O model_names sharing the same signal_name as a POWER or GND pin.

BIRD184.1 changes "any" to "a" in the added paragraph.

BIRD184.2 makes editorial corrections submitted by Michael Mirmak and also discussed at the December 6, 2016 ATM meeting. BIRD184.2 was accepted by the IBIS Open Forum January 6, 2017.