BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	188.1

ISSUE TITLE:   		Expanded Rx Noise Support for AMI

REQUESTOR:     	Michael Mirmak, Intel Corporation

DATE SUBMITTED:	 December 13, 2016

DATE REVISED:	 January 17, 2017

DATE ACCEPTED:	 February 17, 2017

DEFINITION OF THE ISSUE:

IBIS 6.1 defines separate uniform and random (Gaussian) jitter Reserved Parameters for Tx and Rx devices as Tx_Dj, Tx_Rj, Rx_Dj and Rx_Rj, respectively.  However, the receiver noise Reserved Parameter Rx_Noise covers only random (unbounded Gaussian) noise.  A more complete definition of receiver noise would include a parameter for bounded uniform noise, and would ideally use names for both noise parameters that parallel the naming convention for jitter.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Bounded (uniform) Rx Noise must be supported by IBIS-AMI, separately from the existing Gaussian random Rx Noise parameter. |  |
| - The existing Rx_Noise Reserved Parameter must be clarified as referring to unbounded Gaussian random noise, as is already done for Tx_Rj and Rx_Rj. |  |
| - The ranges for the unbounded Gaussian and bounded uniform random multipliers of the noise value in the equations to be used by EDA tools should be clearly stated. |  |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

Table 2: IBIS Keywords, Subparameters,  AMI Reserved_Parameters, and AMI functions  Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| The Reserved Parameter Rx_UniformNoise is defined | New |  |
| The Reserved Parameter Rx_GaussianNoise is added as an alternate name for Rx_Noise. | Modified | This is a modification of the existing parameter Rx_Noise, to clarify its relationship to the new Reserved Parameter Rx_UniformNoise |
| The range for gaussian_rand() and the meaning of the Rx_Noise equation are clarified. | Other | The text of the Rx_Noise parameter definition is clarified to define the range for gaussian_rand(), with the output and input to the equations made explicit. |

PROPOSED CHANGES:

The definition of Rx_Noise on page 228 of the existing IBIS 6.1 specification should be changed from:

Parameter:	Rx_Noise

Required:	No, and illegal before AMI_Version 6.0

Direction:	Rx

Descriptors:

Usage:		Info, Out, Dep

Type:		Float

Format:		Value, List, Range, Corner, Increment, Steps

Default:	<numeric_literal>

Description:	<string>

Definition:	The standard deviation, in volts, of a white Gaussian random process, which is to be added by the EDA tool to the signal measured at the sampling latch of a receiver.

Usage Rules:	If Rx_Noise is Usage Out, then the EDA tool shall use the value returned by Rx AMI_Init if Rx AMI_GetWave is not used. If Rx AMI_GetWave is used, then the EDA tool may apply the value returned by each AMI_GetWave call to the waveform returned by that call to AMI_GetWave, or use the average value of Rx_Noise returned by all calls to AMI_GetWave (after Ignore_Bits), or the value of Rx_Noise returned by the last call to AMI_GetWave.

Other Notes:	Time is calculated as follows:

wave(t) = wave(t) + Rx_Noise * gaussian_rand()

Where wave(t) is the waveform returned by Rx AMI_GetWave.

Example:

(Rx_Noise (Usage Info) (Value 0.010) (Type Float)

(Description "Rx amplitude noise at sampling latch in volts."))

… to:

Parameter:	Rx_Noise, Rx_GaussianNoise

Required:	No, and Rx_Noise is illegal before AMI_Version 6.0; Rx_GaussianNoise is illegal before AMI_Version 6.2

Direction:	Rx

Descriptors:

Usage:		Info, Out, Dep

Type:		Float

Format:		Value, List, Range, Corner, Increment, Steps

Default:	<numeric_literal>

Description:	<string>

Definition:	The standard deviation, in volts, of an unbounded white Gaussian random process, which is to be added by the EDA tool to the signal measured at the sampling latch of a receiver.

Usage Rules:	If Rx_Noise is Usage Out, then the EDA tool shall use the value returned by Rx AMI_Init if Rx AMI_GetWave is not used. If Rx AMI_GetWave is used, then the EDA tool may apply the value returned by each AMI_GetWave call to the waveform returned by that call to AMI_GetWave, or use the average value of Rx_Noise returned by all calls to AMI_GetWave (after Ignore_Bits), or the value of Rx_Noise returned by the last call to AMI_GetWave.

Other Notes:	The output voltage waveform is calculated as follows:

Output_wave(t) = wave(t) + Rx_Noise * gaussian_rand()

where wave(t) is the waveform returned by Rx AMI_GetWave and gaussian_rand() is a function that returns floating point numbers between -inf and +inf.  The distribution of these numbers shall be a white Gaussian distribution centered at 0.0 with a standard deviation of 1.0.

Rx_GaussianNoise is permitted and recommended as an equivalent name for Rx_Noise in AMI_Version 6.2 and higher.

Example:

(Rx_Noise (Usage Info) (Value 0.010) (Type Float)

(Description "Rx amplitude noise at sampling latch in volts."))

Immediately after, a new Reserved Parameter should be added:

Parameter:	Rx_UniformNoise

Required:	No, and illegal before AMI_Version 6.2

Direction:	Rx

Descriptors:

Usage:		Info, Out, Dep

Type:		Float

Format:		Value, List, Range, Corner, Increment, Steps

Default:	<numeric_literal>

Description:	<string>

Definition:	The worst-case half peak-to-peak variation, in volts, of a bounded uniform random process which is to be added by the EDA tool to the signal measured at the sampling latch of a receiver.

Usage Rules:	If Rx_UniformNoise is Usage Out, then the EDA tool shall use the value returned by Rx AMI_Init if Rx AMI_GetWave is not used. If Rx AMI_GetWave is used, then the EDA tool may apply the value returned by each AMI_GetWave call to the waveform returned by that call to AMI_GetWave, or use the average value of Rx_UniformNoise returned by all calls to AMI_GetWave (after Ignore_Bits), or the value of Rx_UniformNoise returned by the last call to AMI_GetWave.

Other Notes:	The output voltage waveform is calculated as follows:

Output_wave(t) = wave(t) + 2 * Rx_UniformNoise * rand()

where wave(t) is the waveform returned by Rx AMI_GetWave and rand() is a function that returns floating point numbers between -0.5 and +0.5 with white uniform distribution.

Example:

(Rx_UniformNoise (Usage Info) (Value 0.010) (Type Float)

(Description "Rx uniform amplitude noise at sampling latch in
                       volts."))

BACKGROUND INFORMATION/HISTORY:

This was submitted by Michael Mirmak of Intel Corp. as a draft for review by the IBIS Advanced Technology Modeling Task Group on November 10, 2016.

Draft 2 changes the names of Rx_Dn and Rx_Rn to Rx_BoundedUniform and Rx_Gaussian, at the suggestion of participants in the IBIS-ATM Task Group.

Draft 3 changes the names of Rx_BoundedUniform and Rx_Gaussian to Rx_BoundedUniformNoise and Rx_UnboundedGaussianNoise, respectively, at the suggestion of participants in the IBIS-ATM Task Group.

Draft 4 changes the names of Rx_BoundedUniform and Rx_Gaussian to Rx_UniformNoise and Rx_GaussianNoise, respectively, at the suggestion of participants in the IBIS-ATM Task Group.  This draft was submitted to the IBIS Open Forum and became BIRD 188 on December 13, 2016.

BIRD188.1 was submitted on January 17, 2017, to clarify the use of Rx_GaussianNoise as the recommended name in place of Rx_Noise, to correct capitalization in the sentence fragments following equations, and to improve indentation of an AMI syntax example. It was transmitted by Michael Mirmak of Intel Corp. December 16, 2016 but not posted at that time due to an oversight. BIRD188 without these changes was accepted January 6, 2017, necessitating a revote for the BIRD188.1 update.