BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	197.7

ISSUE TITLE:   		New AMI Reserved Parameter DC_Offset

REQUESTOR:     	Walter Katz, SiSoft,

Ambrish Varma, Cadence Design Systems,

Randy Wolff, Micron Technology,

Justin Butterfield, Micron Technology,

Fangyi Rao, Keysight Technologies

DATE SUBMITTED:	November 27, 2018

DATE REVISED:	December 4, 2018, January 15, 2019, June 25, 2019, July 23, 2019, October 15, 2019, December 3, 2019, January 7, 2020

DATE ACCEPTED:	February 21, 2020

DEFINITION OF THE ISSUE:

AMI modeling is now being applied to NRZ single-ended channels (e.g. DDR5). The current input to AMI_Init is an Impulse Response. This forces all AMI simulations to be centered around the mid-level of the signal of a single-ended port.  A receiver (Rx) DLL may need to know the input single-ended voltage levels (e.g. to handle saturation in a DFE summer). This BIRD proposes one new AMI Reserved Parameter DC_Offset to address these issues.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Allow the Rx model to recover the single-ended signal at the Rx input. |  |

SUMMARY OF PROPOSED CHANGES:

Add new AMI Reserved Parameter DC_Offset

PROPOSED CHANGES:

Parameter:	DC_Offset

Required:	No, and illegal before AMI_Version 7.1

Direction:	Rx

Descriptors:

Usage:                   In

Type:                     Float

Format:                 Value

Default:                 <numeric_literal>

Description:          <string>

Definition:	The input value of DC_Offset is the mean value of the steady state high and low voltages of the analog channel step response at the Rx pad.

Usage Rules:	If the impulse response was generated by differentiating the analog channel step response, then the input value of DC_Offset should be the same as the average of the step response initial and final voltages.

It is assumed that the waveform input to the Rx AMI_GetWave function is the physical Rx input waveform minus the input value of this DC_Offset. The Rx AMI_GetWave function may choose to reconstruct the physical input waveform by adding the input value of DC_Offset to the input waveform.

The Rx AMI_GetWave output waveform returned by the AMI model shall swing around zero volts.

Other Notes:

The EDA tool ignores the DC_Offset value specified in the .ami file.  It is the responsibility of the EDA tool to determine the input value of DC_Offset.  The EDA tool may use any method to do this. The EDA tool may use any method to do this.  The EDA tool may use the input value of DC_Offset to post process data returned by the AMI model to graphically compare the waveform output of Rx AMI_GetWave to the input waveform without the DC_Offset subtracted.

  - For the example below, assume that the EDA tool determines the DC_Offset Value is 0.1 V and then passes it to the executable model

  - Rx AMI_GetWave returns the output waveform in the range from -0.5 V to 0.5 V

  - The EDA tool may shift the output waveform by the DC_Offset Value of 0.1 V to a range from -0.4 V to 0.6 V

Example:

DC_Offset (Usage In) (Type Float) (Value 0.0)

(Description “The EDA tool is responsible for determining the input value sent to the executable model.”)

BACKGROUND INFORMATION/HISTORY:

Typographical updates made in BIRD197.1, based on feedback from Open Forum and ATM review.

BIRD197.2 contains additional editorial changes.

BIRD197.3 contains editorial changes to the verbiage related to the usage of the words “single-ended”.

BIRD197.4 changes the DC_Offset Usage from In to In or InOut.  NRZ_Threshold parameter is added.

BIRD197.5 changes the DC_Offset Usage from In or InOut to In.  NRZ_Threshold parameter is removed.

BIRD197.6 has some editorial additions regarding the example and Other Notes.

BIRD197.7 has more editorial corrections to the example and Other Notes.