BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	201.1

ISSUE TITLE:   		Back-channel Statistical Optimization

REQUESTOR:     	Walter Katz, Signal Integrity Software

DATE SUBMITTED:	January 7, 2020

DATE REVISED:	June 2, 2020

DATE ACCEPTED:	July 17, 2020 (Superseded by BIRD215)

DEFINITION OF THE ISSUE:

IC vendors would like back-channel to support training in the Statistical flow iterating between the Tx and Rx doing optimization based on impulse responses.

This BIRD defines one new BCI reserved parameter and how it determines what the EDA tool needs to do to support training in the Statistical Flow.

This BIRD also defines one new executable model function AMI_Impulse. After the initial calls to Tx and Rx AMI_Init, and if Statistical Training is enabled, the EDA tool shall repeatably call the Tx and Rx AMI_Impulse functions until the Rx AMI_Impulse returns BCI_State Converged.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

| Requirement | Notes |
| --- | --- |
| - Enable back-channel link training messages between the Tx and Rx executable models to enable the Rx or Tx executable model to control the equalization of the Tx or Rx during statistical simulations. |  |
| - Allow the user and tool to know when statistical link training has ended, and normal operation has begun. |  |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| New AMI Reserved Parameter BCI_Training_Mode | All are new AMI Parameters | All affect the operation of the AMI function AMI_Impulse. |
| New executable function AMI_Impulse | New |  |

PROPOSED CHANGES:

ADD TO SECTION 10.2.3 AFTER Function AMI_Init and above Function AMI_GetWave:

Function:	AMI_Impulse

Required:	Yes

Declaration:	long AMI_Impulse (double *impulse_matrix,

char *BCI_parameters_in,

char **BCI_parameters_out,

char **AMI_parameters_out,

void *AMI_memory)

Arguments:

impulse_matrix

Same as impulse_matrix argument defined under AMI_Init above.

Note that since both AMI_Init and AMI_Impulse modify the impulse_matrix in place, the EDA tool could maintain the original impulse_matrix and use different memory for the impulse_matrix input to the AMI_Init and AMI_Impulse functions.

Note that the AMI_Impulse function uses the number_of_rows, aggressors, sample_interval, and bit_time that were passed to the AMI_Init call.

BCI_parameters_in

The BCI_parameters_in argument is a pointer to a string.  This pointer is returned in the BCI_parameter_out argument by a previous call to an AMI_Impulse function in another executable model in the channel.  Memory for the string is allocated and de-allocated by the previous call to an AMI_Impulse function in another executable model in the channel.  The string must be formatted as defined by the BCI_Protocol. On the first call to the primary Tx AMI_Impulse function, this pointer shall be the Null pointer (0).

BCI_parameters_out

The BCI_parameters_out argument is a pointer to a string pointer.  Memory for the string is allocated and de-allocated by the algorithmic model. The model returns a pointer to the string as the contents of this argument. The string must be formatted as defined by the BCI_Protocol.

The EDA tool must initialize the memory content at this address to zero (null pointer) prior to calling the AMI_Impulse function, so that after the execution of the function it can determine whether or not the function returned a valid string pointer at that address.

AMI_parameters_out

Same as AMI_Parameters_out argument defined under AMI_Init above.

AMI_memory

This is the memory pointer which was allocated during the AMI_Init call.

Return Value

1 for success

0 for failure

Algorithmic models shall return a failure code (0) if and only if the function call fails due to a program execution error.  In all other cases the return code shall be "success" (1), even if the function cannot operate properly due to some functional problems.  For example, if a function includes a CDR which is unable to get into a stable mode, the function shall still return a success code (1).  Examples for returning a failure code (0) may include an invalid data type, a null pointer during run time, or anything that prevents the successful execution of the model’s code.

The authors of Algorithmic Models are encouraged to provide feedback to the EDA tool’s users through the various available messaging options about any difficulties the model encounters during execution, regardless of what the value of the function’s return code is.

ADD SECTION SUBTITLES FOR REPEATERS

Page 262 ABOVE “The time-domain simulation flow for a Repeater link shown in Figure 41 is defined below.”

10.8.2 REPEATER TIME DOMAIN SIMULATION FLOW FOR A REPEATER LINK

Page 264, ABOVE “The statistical simulation flow for a Repeater link shown in Figure 41 is defined below.”

10.8.3 REPEATER STATISTICAL DOMAIN SIMULATION FLOW FOR A REPEATER LINK

REPLACE:

## AMI Reserved Parameter Definitions For Link Training Communications

In this section, the parameters BCI_Protocol, BCI_State, BCI_ID, BCI_Message_Interval_UI and BCI_Training_UI are documented, to enable link training communication.  These Reserved Parameters are in the AMI file and positioned under the Reserved_Parameters branch.

WITH

## AMI Reserved Parameter Definitions For Link Training Communications

With the information provided in this section, IC vendors and EDA tool vendors can develop models that support Back Channel Training and the enhancements EDA tools will need to support these models.  The following Reserved Parameters are in the AMI file and positioned under the Reserved_Parameters branch.

ADD AFTER  Parameter BCI_Training_UI SECTION AND ABOVE 10.9.1:

Parameter:	BCI_Training_Mode

Required:	No, and illegal before AMI_Version 7.1

Direction:	Rx, Tx

Descriptors:

Usage:		In

Type:		String

Format:		Value, List

Default:	<string literal>

Description:	<string>

Definition:	This parameter tells the EDA tool if the model supports Statistical Optimization Only, Time Domain Optimization Only, or is “Both” (both Statistical and Time Domain Optimization). The only allowed values of BCI_Training_Mode are “Impulse”, “GetWave”, or “Both”. If “Both” is present, then “Impulse” and “GetWave” shall also be present.

Allowed Formats:

(Value “Impulse”)

(Value “GetWave”)

(List “Impulse”)

(List “GetWave”)

(List “Impulse” “GetWave”)

(List “Impulse” “GetWave” “Both”)

Illegal Formats:

(Value “Both”)

(List “Both” “Impulse”)

(List “Both” “GetWave”)

Usage Rules: The user/EDA tool can only choose a BCI_Training_Mode value if it is available on both the Tx and the Rx (and must be set the same for Tx and Rx). If BCI_Training_Mode is not present, then it shall default to “GetWave”.

To run a BCI Statistical simulation, the Tx and Rx model (or the primary Tx, terminal Rx, and all repeater Rx and Tx models) must have BCI_Training_Mode as either “Impulse” or “Both”.

To run a BCI Time Domain simulation, the Tx and Rx model (or the primary Tx, terminal Rx, and all repeater Rx and Tx models) must have BCI_Training_Mode as either “GetWave” or “Both”.

Example:

(BCI_Training_Mode (Usage In)(Type String)(List “Both” “Impulse” “GetWave”)

(Description "This Device supports Back-channel Statistical and Time Domain Optimization”))

AFTER SECTION 10.9.2, ADD NEW SECTIONS 10.9.3 AND 10.9.4

10.9.3 STATISTICAL TRAINING/ANALYSIS FLOW FOR CHANNELS WITH NO REPEATER

The EDA tool shall make the following calls to the Tx and Rx AMI_Init and AMI_Impulse functions if (BCI_Training_Mode “Impulse” or “Both”).

- Tx AMI_Init is called with

  - (BCI_State “Training”) (BCI_Protocol “<name>”) (BCI_ID “<my_ ID>”) (BCI_Training_Mode “Both” or ”Impulse”)

  - If (BCI_Training_Mode  “GetWave”) follow the flow in Section 10.9.1.

  - Impulse Matrix contains Impulse Response of channel

  - If the Tx executable model does not implement the BCI_Protocol and BCI_Training_Mode, it returns “Error” in BCI_State.

- Rx AMI_Init is called with

  - (BCI_State “Training”) (BCI_Protocol “<name>”) (BCI_ID “<my_ID>”) (BCI_Training_Mode “Both” or ”Impulse”)

  - Impulse Matrix contains Impulse Response output of Tx AMI_Init

  - If the Rx executable model does not implement the BCI_Protocol and BCI_Training_Mode, it returns “Error” in BCI_State.

  - The EDA tool may analyze the results of Rx AMI_Init.

- Tx AMI_Impulse is called with the same impulse_matrix used in the call to Tx AMI_Init. The value of BCI_parameters_in shall be Null (0) on the first call to Tx AMI_Impulse and the value of BCI_parameters_out of the previous call to Rx AMI_Impulse on subsequent calls to Tx AMI_Impulse.

- The Rx AMI_Impulse is called using the impulse_matrix output of Tx AMI_Impulse. The value of BCI_parameters_in shall be set to the value of BCI_parameters_out of the previous call to Tx AMI_Impulse.

- Steps 3 and 4 are repeated until the Rx AMI_Impulse returns AMI Reserved Parameter BCI_State “Converged”, “Failed”, or “Error”

  - “Converged”: The impulse_matrix and AMI_parameters_out returned by the receiver AMI_Impulse function are used by EDA tool to complete the simulation. The EDA tool may proceed with Time Domain simulation.

  - “Fail”: This tells the EDA tool that the training failed to converge. The EDA tool can terminate the simulation or proceed using the outputs of AMI_Impulse or AMI_Init to complete the simulation at its own risk. The EDA tool may proceed with Time Domain simulation.

  - “Error”: Tells the EDA tool that an error was detected that prevented optimization to continue. The EDA tool can terminate the simulation or proceed using the outputs of AMI_Impulse or AMI_Init to complete the simulation at its own risk. The EDA tool may proceed with Time Domain simulation.

- If the User selected to do Time Domain simulation after Impulse training

  - If BCI_Training_Mode is “Impulse” the Tx and Rx AMI_GetWave shall not do training and shall return BCI_State “Off”.

  - If BCI_Training_Mode is “Both” the time-domain training outlined in section 10.9.1 is invoked.

10.9.4 STATISTICAL TRAINING/ANALYSIS FLOW FOR CHANNELS WITH REPEATERS

- It is required that all AMI models from the primary Tx to the terminal Rx support (BCI_Training_Mode “Impulse” or “Both”).

- The AMI_Init flow is identical to the flow defined for the statistical simulation flow for a Repeater link shown in Figure 41 in Section 10.8.2 and documented in Section 10.8.3.

- This same flow is repeated with the calls to AMI_Init replaced by calls to AMI_Impulse.

- The BCI training terminates when BCI_State returns “Converged”, “Fail”, or “Error” in the AMI_parameters_out of the terminal Rx AMI_Impulse function.

Note that the EDA tool shall set the value of BCI_parameters_in to the value of BCI_parameters_out of the previous call to an AMI_Impulse function in the channel.

CHANGE SECTION NUMBER FROM SECTION 10.9.3 TO 10.9.5 AND ADD BCI_Training_Mode to TABLES 33-35:

### Summary Tables for Usage, Type and Format

Table 33 – General Rules and Allowable Usage for BCI Reserved Parameters

| Reserved Parameter | General Rules |  | Allowable Usage |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required | Default2,4 | Info | In | Out | Dep1 | InOut |
| BCI_Message_Interval_UI | Yes3 | -- | X |  |  |  |  |
| BCI_ID | Yes3 | -- |  | X |  |  |  |
| BCI_Protocol | No | -- |  | X |  |  |  |
| BCI_State | Yes3 | -- |  |  |  |  | X |
| BCI_Training_UI | Yes3 | -- |  | X |  |  |  |
| BCI_Training_Mode | No5 | “GetWave” |  | X |  |  |  |
| Notes: - Illegal for AMI_Version 6.0 and earlier - “Default” in this context means “behavior if Reserved Parameter is absent” - Required if BCI_Protocol is present - “--" means that an entry must be provided if the parameter is present; no default is assumed or permitted - Illegal for AMI_Version 7.0 and earlier, required if BCI_Protocol supports Statistical Training |  |  |  |  |  |  |  |

Table 34 – Allowable Data Types for BCI Reserved Parameters

| Reserved Parameter | Data Type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Float | UI | Integer | String | Boolean |
| BCI_Message_Interval_UI |  |  | X |  |  |
| BCI_ID |  |  |  | X |  |
| BCI_Protocol |  |  |  | X |  |
| BCI_State |  |  |  | X |  |
| BCI_Training_UI |  |  | X |  |  |
| BCI_Training_Mode |  |  |  | X |  |

Table 35 – Allowable Data Formats for BCI Reserved Parameters

| Reserved Parameter | Data Format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Value | Range | Corner | List | Increment | Steps | Gaussian | Both-Dirac | DjRj | Table |
| BCI_Message_Interval_UI | X |  |  |  |  |  |  |  |  |  |
| BCI_ID | X |  |  |  |  |  |  |  |  |  |
| BCI_Protocol | X |  |  | X |  |  |  |  |  |  |
| BCI_State |  |  |  | X |  |  |  |  |  |  |
| BCI_Training_UI | X |  |  |  |  |  |  |  |  |  |
| BCI_Training_Mode | X |  |  | X |  |  |  |  |  |  |

ADD TO TABLE 39 ABOVE BCI_Training_UI IN SECTION 10.12 FOR FIRST SUPPORTED AMI VERSION:

BCI_Training_Mode	7.1

BACKGROUND INFORMATION/HISTORY:

BIRD201

Contributor Michael Mirmak, Intel Corp

BIRD201.1

Technical clarifications and organizational changes to declare where content is positioned