BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	207

ISSUE TITLE:   		New AMI Reserved Parameters Component_Name and Signal_Name

REQUESTOR:     	Randy Wolff, Micron Technology

DATE SUBMITTED:	July 29, 2020

DATE REVISED:

DATE ACCEPTED:	October 9, 2020

DEFINITION OF THE ISSUE:

IBIS-AMI executable models can make use of Component name and signal_name to identify a unique Tx or Rx buffer on a die.  With both names passed into the executable model, it can use look-up tables to model unique buffer characteristics, such as a buffer-specific delay for DQ to DQS timing.  Adding these Reserved Parameters will enable new modeling techniques for clock-forwarded architectures.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Add the capability of passing the Component name into an IBIS-AMI executable model. |  |
| - Add the capability of passing the signal_name into an IBIS-AMI executable model. |  |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

Table 2: IBIS Keywords, Subparameters,  AMI Reserved_Parameters, and AMI functions Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| AMI Reserved Parameter Component_Name | New |  |
| AMI Reserved Parameter Signal_Name | New |  |

PROPOSED CHANGES:

Parameter:	Component_Name

Required:	No, and illegal before AMI_Version 7.1

Direction:	Rx, Tx

Descriptors:

Usage:                   In

Type:                     String

Format:                  Value

Default:                 <string_literal>

Description:           <string>

Definition:	Name of the IBIS [Component] keyword that is being used.

Usage Rules: The Value specified in the .ami file is ignored. The EDA tool must pass the name of the IBIS [Component] keyword that is being instantiated by the EDA tool through the input parameter strings to the AMI_Resolve and AMI_Init functions as the value of this parameter.

If the EDA tool is used in such a way that the name of the IBIS [Component] keyword is not available during simulation, then the EDA tool shall pass in an empty string ("").

Component_Name must be present if Signal_Name is present.  Component_Name must be absent if Signal_Name is absent.

Other Notes:

Example:

(Component_Name (Usage In) (Type String) (Value "placeholder")

(Description "The name of the instantiated IBIS Component")

)

Parameter:	Signal_Name

Required:	No, and illegal before AMI_Version 7.1

Direction:	Rx, Tx

Descriptors:

Usage:                   In

Type:                     String

Format:                  Value

Default:                 <string_literal>

Description:           <string>

Definition:	Name of the IBIS [Pin] keyword’s signal_name sub-parameter that is being used.

Usage Rules: The Value specified in the .ami file is ignored. The EDA tool must pass the name of the IBIS [Pin] keyword’s signal_name sub-parameter that is being instantiated by the EDA tool through the input parameter strings to AMI_Resolve and AMI_Init functions as the value of this parameter.

If the EDA tool is used in such a way that the name of the IBIS [Pin] keyword’s signal_name is not available during simulation, then the EDA tool shall pass in an empty string ("").

Signal_Name must be present if Component_Name is present.  Signal_Name must be absent if Component_Name is absent.

Other Notes:

Examples:

(Signal_Name (Usage In) (Type String) (Value "placeholder")

(Description "The name of the instantiated IBIS Pin’s signal_name sub-

parameter")

)

BACKGROUND INFORMATION/HISTORY: