BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER:		225

ISSUE TITLE:		Clarification for bus_label rules

REQUESTOR:		Arpad Muranyi, Weston Beal, Randy Wolff, Siemens EDA

DATE SUBMITTED:	July 11, 2023

DATE REVISED:

DATE ACCEPTED:	September 15, 2023

DEFINITION OF THE ISSUE:

The error message E5810 issued by the IBIS parser states that: “IBIS 7.0 compatibility is required for IBIS files referenced by EMD”, but the IBIS specification doesn’t spell out such a requirement.  Since the reason and intent of this error message is correct, we need to add this requirement to the IBIS specification.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Define the missing requirement |  |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:  Add a definition for the missing requirement.

PROPOSED CHANGES:

Change the following paragraph on pg. 378 in IBIS 7.2 from:

The fourth column (bus_label) is optional for rail pins (signal_type POWER or GND). The bus_label entry is a name assigned to a subset of the pins with a rail signal_name. As its name implies, bus_label entries are not required to be unique for each row. However, all pins that have the same bus_label must have the same signal_name. If the bus_label column is not specified for signal_type POWER or GND, then the bus_label shall be assumed to be the signal_name.

to:

The fourth column (bus_label) is optional for rail pins (signal_type POWER or GND).  The bus_label entry is a name assigned to a subset of the pins with a rail signal_name.  As its name implies, bus_label entries are not required to be unique for each row.  However, all pins that have the same bus_label must have the same signal_name within EMD files as well as within any IBIS [Component] referenced in [EMD Parts].  If the bus_label column is not specified for signal_type POWER or GND, then the bus_label shall be assumed to be the signal_name.

BACKGROUND INFORMATION/HISTORY:

TBD