BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	147.3

ISSUE TITLE:   		Back-channel Support

REQUESTOR:     	Bob Miller, Broadcom, Ltd

Ambrish Varma, Cadence Design Systems, Inc;

Walter Katz, Signal Integrity Software, Inc;

Kumar Keshavan, Cadence Design Systems, Inc;

Ken Willis, Cadence Design Systems, Inc

DATE SUBMITTED:	October 18, 2011

DATE REVISED:	September 1, 2016, October 11, 2016, October 13, 2016

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

Link training communication is required for PCI Express, IEEE 802.3,USB,  Fibre Channel and other emerging serial link standards. This communication ‘provides a mechanism through which the receiver can tune the transmitter equalizer to optimize performance’ [1].  These mechanisms employ a reliable “back-channel” to support administrative link training communication between the transmitter and receiver SerDes.

Broadcom wants the IBIS standard to be able to define standardized Back Channel Interface (BCI) Protocols so that different IC Vendors can write IBIS-AMI models that can communicate with each other, without requiring any support from EDA Vendor tools.

This BIRD defines how link training communications can be standardized in the IBIS specification. This BIRD expressly does not attempt at this time to define specific standard protocols which utilize these definitions.

[1] Section 5, IEEE Std 802.3.SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

| Requirement | Notes |
| --- | --- |
| - Enable back-channel link training messages between the Tx and Rx executable models to enable the Rx executable model to control the equalization of the Tx during time domain (AMI_GetWave) simulations. | Back-channel messages are implemented via file I/O in the simulation’s working directory instead of parameter string passing via the EDA tool. |
| - Support back-channel messages between the Tx and Rx executable models in channels that have Repeater(s) to enable the Downstream (primary) Rx executable model to control the equalization of the upstream Tx and Rx executable models during time domain (AMI_GetWave) simulations. | A lightweight communication scheme supports multi-hop channel optimization, the details of which may be defined in specific future protocols. |
| - Does not require the EDA tool to make any changes to support these communications. | There are minor changes the EDA tool can make to improve the user experience, but these changes are not required since they can be accommodated in either the .ami files, or some extra setup required by the user. |
| - Allow the user and tool to know when link training has ended and normal operation has begun. | EDA support can facilitate user awareness of successful training but is not required. |
| - Support both private and published link training protocols. | Protocols might be published within IBIS or elsewhere later. |
| - Provide all channel executable model instances with a unique file namespace for back-channel communication. | If the EDA tool does not directly facilitate the namespace selection, the user may select namespaces which are compatible with a specific EDA tool via the models’ ami files. |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| New AMI Reserved_Parameters BCI_Protocol BCI_State BCI_ID BCI_GetWave_Block_UI BCI_Training_UI | All are new AMI Parameters | All affect the operation of the AMI functions AMI_Init, AMI_GetWave |

PROPOSED CHANGES:

## Introduction (Section 10.1)

(Insert before

‘This section defines how the components of an algorithmic model are specified in an IBIS file.’)

There are scenarios when a receiver and transmitter circuits do not have prior information of their analog channel. Advanced models can perform link training communication to tune the transmitter equalizer parameters for optimized performance and adapt to the signature of any analog channel. This is done when transmitter tap parameters are re-configurable and receivers help them to be configured. Advanced communication specifications such as PCI express, USB, Fibre Channel, and IEEE 802.3 define link training protocols for transmitters and receivers. If both the transmitter and receiver AMI executable models support the same link training protocol (Back-channel Protocol), the EDA tool will facilitate the communication between the executable models enabling link training. Another name for Link Training in the industry is Auto-Negotiation.

A Link Training algorithm can either emulate what the silicon is actually doing, or it can use channel analysis methods to determine the optimal Tx equalization settings. This ability will also allow Rx AMI models to determine the Tx equalizations settings for channels that do not have automatic link training capabilities.

Channels with Repeaters will require that the Downstream Rx be able to control all upstream equalization.

Communications between the Rx and Tx executable models are in messages that both the Rx and Tx executable models understand, and the EDA tool does not need to understand. These agreed upon messages are called a Back-channel Protocol. This specification does not describe the details of the Back-channel Protocol but only a method to make the communication work.

This specification describes an underlying mechanism for the AMI .ami file and the executable model to allow information to be transferred from the Tx to the Rx and from the Rx to the Tx without requiring the EDA tool to understand the content of this information, or even for the EDA tool to know that back-channel communications is occurring.

With the information provided in this specification, IC Vendors can develop models that support Back Channel Training in current IBIS AMI EDA tools.

ADD A SECTION 10.8 after Section 10.7 AND (RENUMBER LATER SECTIONS AS NECESSARY)

## 10.8 AMI Reserved Parameter DEFINITIONs For Link training Communications

In this section, the parameters BCI_Protocol, BCI_State, BCI_ID, BCI_GetWave_Block_UI and BCI_Training_UI are documented to enable link training communication.  These Reserved Parameters are in the AMI file and positioned under the Reserved_Parameters branch.

Parameter:	BCI_Protocol

Required:	No, and illegal before AMI_Version 7.0

Direction:	Rx, Tx

Descriptors:

Usage:		In

Type:		String

Format:		Value, List

Default:	<string literal>

Description:	<string>

Definition:	This parameter contains the name (or names) of Back-channel Protocol(s) that the model supports.  This parameter tells the model which Back-channel Protocol is being used for the training process. The BCI_Protocol defines the back-channel message files and BCI data contained therein that is read and/or generated by each call to each executable model.

Usage Rules: Both the transmitter and receiver for a given channel must have identical settings for the BCI_Protocol parameter for link training to be enabled. Both the transmitter and receiver for a given channel must have GetWave_Exists = True for link training to be enabled.

BCI_Protocol must be present if the model supports any BCI protocol

Other Notes:	A BCI_Protocol may be private or approved by the IBIS Open Forum.  Protocol names beginning with the prefix "IBIS” are reserved for protocols approved by the IBIS Open Forum.

BCI_Protocol names beginning with “IBIS” are reserved for future protocols adopted and published. Names for private and independently-specified published protocols should contain character strings sufficiently unique to avoid conflicts with other independently-named protocols.

Example:

(BCI_Protocol (Usage In)(Type String)(Value "Company_xyz")

(Description "This Device supports Back-channel Protocol Company_xyz. For private protocols, we suggest that the name should begin with a company name to help keep private protocol names unique. Protocols officially adopted by the IBIS Open Forum would begin with IBIS."))

Parameter:   	BCI_ID

Required:	No, and illegal before AMI_Version 7.0

Direction:        Rx, Tx

Descriptors:

Usage:                   In

Type:                     String

Format:                  Value

Default:                 <string literal>

Description:           <string>

Definition:	The EDA tool is responsible for recognizing this parameter name and replacing the value declared in the .ami file with a string that contains a unique alphanumeric identifier, optionally pre-pended with a “path string”. The algorithmic model is responsible for using BCI_ID as the base name and path for any data files that the model creates, either for use as temporary storage or for recording output data in accordance with the BCI_Protocol.  All model instances in a channel between and including the upstream Tx and downstream Rx shall share a unique BCI_ID. Each concurrent channel (as in a crosstalk simulation) has its own BCI_ID. The use of BCI_ID helps guarantee that multiple channels do not mix up data as a result of collisions between temporary or permanent file names.

The “path string” portion of the BCI_ID string is constructed and interpreted according to the following rules  (similar to those of the Reserved_Parameter “DLL_Path”):

The contents of the “path string” concatenated into BCI_ID can either be an absolute path, or a path relative to the current working directory of the process running the executable model.  In this string, the path separator is the forward slash "/".  Back slashes “\” are not allowed.  The model is responsible for making any OS-specific adjustments (for example, replacing forward slashes "/" with backslashes "\") if necessary. The last character of the “path string” shall be a forward slash “/”.

Usage Rules: To access a supporting file using BCI_ID, the executable model should create a file name by creating a string consisting of the value of BCI_ID, convert forward slashes “/” to backslashes “\” on operating systems that require a backslash “\” as a path separator, and then appending additional characters as specified in BCI_Protocol to create the name of the file.  If the EDA tool chooses to make the current working directory the directory where files are created and accessed under the BCI_Protocol, then the “path string” portion of BCI_ID can be “./”,  the absolute path to this directory, or omitted entirely. If the EDA tool uses BCI_ID to specify a directory other than the current working directory, the directory must exist and be read/write accessible to the executable models.

BCI_ID must be present if BCI_Protocol is present.  BCI_ID must be absent if BCI_Protocol is absent.

Other Notes:  A BCI_Protocol may define one, two (e.g. one per direction) or any number of BCI message files with the same BCI_ID prefix to be used by the channel Tx and Rx executable models to support the required back-channel optimization.

If the EDA tool does not modify/uniquify BCI_ID, the user must select and insert an appropriate matching string in the ami files of each executable model participating in the channel’s  BCI_Protocol  to manually provide a functioning file namespace.

Example:

(BCI_ID (Usage In) (Type String) (Value "../dll_scratch_dir/channel1")

(Description "Models may create/read/write/delete files in ‘../dll_scratch_dir’ with names beginning with ‘channel1’"))

Parameter:	BCI_State

Required:	No, and illegal before AMI_Version 7.0

Direction:	Rx, Tx

Descriptors:

Usage:		InOut

Type:		String

Format:		List  (“Off” ”Training” “Converged” “Failed” “Error”)

Default:	<string_literal>

Description:	<string>

Definition:	The user sets the value of BCI_State to either “Off” or ”Training” on the calls to the Tx and Rx AMI_Init. The values of BCI_State sent to the Tx and Rx executable models shall be the same for both the Tx and Rx AMI_Init.

Usage Rules: 	If the BCI_State is “Off” on the calls to Tx and Rx AMI_Init, both the Tx and Rx executable models will not read or generate files in the BCI_ID namespace. The values of BCI_Protocol, BCI_GetWave_Block_UI or BCI_Training_UI shall be ignored by the executable models. Executable models  receiving BCI_State “Off” and subsequently returning BCI_State shall return BCI_State “Off”.

If the BCI_State is “Training” on the calls to Tx and Rx AMI_Init, both the Tx and Rx executable models will read and/or write files in the BCI_ID namespace per the BCI_Protocol. The values of BCI_Protocol, BCI_ID, BCI_GetWave_Block_UI and BCI_Training_UI are required. The Rx AMI_GetWave calls shall return a value in BCI_State of either “Training”, “Converged”,  ”Failed” or “Error”. If theTx AMI_GetWave returns a value  in BCI_State, it shall also be either “Training”, “Converged”,  ”Failed” or “Error”; “Training”, “Converged” , and “Failed” should reflect the Rx state per the BCI_Protocol.

The EDA tool shall consider the value of  BCI_State returned by the terminating Rx executable model to be the definitive BCI_Protocol training state. However, any executable model in the channel, upon returning a BCI_State value of “Error”, may thereby signal that a BCI_Protocol has failed due to a mis-communication under the BCI_Protocol.

If the returned value is “Training”, then the Tx and Rx AMI_GetWave will continue to read and/or modify BCI_ID files per the BCI_Protocol.

If the returned value is “Converged”, then the Tx and Rx AMI_GetWave may continue to read and/or modify the BCI_ID files per the BCI_Protocol. However, it is implied that no further adaptation is performed under the BCI_Protocol and the EDA tool may complete the simulation/analysis starting with this waveform.

If the returned value is “Failed” the Rx AMI_GetWave function indicates a condition that it was not able to converge in its search algorithm.  Then the Tx and Rx AMI_GetWave may continue to read and/or modify the BCI_ID files per the BCI_Protocol. However, it is implied that no further adaptation is performed under the BCI_Protocol and the EDA tool may complete the simulation/analysis starting with this waveform.

If the returned Tx or Rx value is “Error”, the executable model indicating “Error” is unable to understand the messages according to the BCI_Protocol. The Tx and/or Rx AMI_GetWave will stop reading and/or modifying the BCI_ID files. The EDA tool may communicate a protocol error to the user and complete the simulation/analysis starting with this waveform.

BCI_State must be present if BCI_Protocol is present.  BCI_State must be absent if BCI_Protocol is absent.

Other Notes:

Training and co-optimization is done by Rx models using one or more Tx equalization exploration algorithms. The Rx model may have Model Specific parameters that allow the user to choose which exploration algorithm to use.

During “Training”, the EDA tool may supply a “training” stimulus pattern defined by the user. While not required, the Back Channel Protocol will likely specify the pattern that should be used.

Example:

(BCI_State (Usage InOut)(Type String)

(List “Off” ”Training” “Converged” “Failed” “Error”))

Parameter:	BCI_GetWave_Block_UI

Required:	No, and illegal before AMI_Version 7.0

Direction:	Rx

Descriptors:

Usage:		Info

Type:		UI

Format:		Value

Default:	<numeric_literal>

Description:	<string >

Definition:	This Rx parameter tells the EDA tool the recommended number of UI in each AMI_GetWave call to be used in Time Domain simulations.

Usage Rules: The wave_size passed to AMI_GetWave would be the value of BCI_GetWave_Block_UI*bit_time/sample_interval.

BCI_GetWave_Block_UI must be present if BCI_Protocol is present.  BCI_GetWave_Block_UI must be absent if BCI_Protocol is absent.

Other Notes:	 This parameter allows a BCI_Protocol to define the number of training bits (“dwell time”) between BCI messages, which necessarily must occur at most once per AMI_GetWave call. This may be necessary in some protocols or rigorous channel simulations to enforce bit-by-bit emulation.

Example:

(BCI_GetWave_Block_UI(Usage Info) (Type UI) (Value 2000)

(Description "AMI_GetWave blocks should contain 2000 UI”))

Parameter:	BCI_Training_UI

Required:	No, and illegal before AMI_Version 7.0

Direction:	Rx

Descriptors:

Usage:		In

Type:		UI

Format:		Value

Default:		<numeric_literal>

Description:	<string>

Definition:	Tells the EDA tool how long the time variant model may take to complete training.

Usage Rules:	This parameter is meant for Rx models that support BCI Training.  The value in this field tells the EDA tool and the Rx AMI_GetWave function how many bits of the AMI_GetWave output should be used for training.

BCI_Training_UI must be present if BCI_Protocol is present.  BCI_Training_UI must be absent if BCI_Protocol is absent.

Other Notes: 	If an EDA tool does not use BCI_Training_UI or BCI_State to determine when it can start analysis of the optimized waveform generated by the Rx AMI_GetWave, the user (or .ami file) should set Ignore_Bits to the same value as BCI_Training_UI.

Examples:

(BCI_Training_UI (Usage In) (Type UI) (Value 100000)

(Description "BCI training may require 100000 UI")

Table YY1 – General Rules and Allowable Usage for General Reserved Parameters

| Reserved Parameter | General Rules |  | Allowable Usage |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required | Default | Info | In | Out | Dep1 | InOut |
| BCI_GetWave_Block_UI | No, Yes if BCI_Protocol is present | -- | X |  |  |  |  |
| BCI_ID | No, Yes if BCI_Protocol is present | -- |  | X |  |  |  |
| BCI_Protocol | No, Yes to support BCI protocol | -- |  | X |  |  |  |
| BCI_State | No, Yes if BCI_Protocol is present | -- |  |  |  |  | X |
| BCI_Training_UI | No, Yes if BCI_Protocol is present | -- |  | X |  |  |  |

- Illegal for AMI_Version 6.0 and earlier

Table YY2 – Allowable Data Types for General Reserved Parameters

| Reserved Parameter | Data Type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Float | UI | Integer | String | Boolean |
| BCI_GetWave_Block_UI |  | X |  |  |  |
| BCI_ID |  |  |  | X |  |
| BCI_Protocol |  |  |  | X |  |
| BCI_State |  |  |  | X |  |
| BCI_Training_UI |  | X |  |  |  |

Table YY3 – Allowable Data Formats for General Reserved Parameters

| Reserved Parameter | Data Format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Value | Range | Corner | List | Increment | Steps | Gaussian | Dual-Dirac | DjRj | Table |
| BCI_GetWave_Block_UI | X |  |  |  |  |  |  |  |  |  |
| BCI_ID | X |  |  |  |  |  |  |  |  |  |
| BCI_Protocol | X |  |  | X |  |  |  |  |  |  |
| BCI_State |  |  |  | X |  |  |  |  |  |  |
| BCI_Training_UI | X |  |  |  |  |  |  |  |  |  |

Training/Analysis Flow for Channels with No Repeater

The EDA tool shall make the following calls to the Tx and Rx AMI_Init, AMI_Init  and AMI_GetWave functions:

- Tx AMI_Init is called with

  - (BCI_State “Training”) (BCI_Protocol “<name>“) (BCI_ID  “<my_ ID> “)

  - If the Tx executable model does not implement the BCI_Protocol, it returns “Error” in BCI_State.

  - The Tx may write a message file in the BCI_ID namespace under BCI_Protocol.

- Rx AMI_Init is called with

  - (BCI_State “Training”) (BCI_Protocol “<name>“) (BCI_ID “<my_ID> “) (BCI_Training_UI <# Training Bits>)

  - If the Rx executable model does not implement BCI_Protocol, it returns “Error” in BCI_State.

  - The Rx may read, write, modify and/or delete message files in the BCI_ID namespace under BCI_Protocol.

- Tx AMI_GetWave is called with the stimulus pattern. The Tx may read, write, modify and/or delete message files in BCI_namespace under BCI_Protocol.

- Rx AMI_GetWave is called with the waveform output of Tx AMI_GetWave convolved with the IR of the channel. The Rx may read, write, modify and/or delete message files under BCI_Protocol.

- Steps 3 and 4 are repeated until the EDA tool stops the simulation.

  - The EDA tool should start processing the output of Rx AMI_GetWave after Ignore_Bits and either:

after BCI_Training_UI, or

when the Rx AMI_GetWave function returns BCI_State “Converged” or “Failed” or either the Tx or Rx executable model returns “Error”.

Note that the EDA tool does not need to perform any operations specifically assisting the BCI communication between the Tx and the Rx executable models beyond passing the BCI parameters to both executable models on AMI_Init.

Training/Analysis Flow for Channels with One Repeater

The EDA tool shall make the following calls to the Upstream Tx, Repeater Rx, Repeater Tx, Downstream Rx AMI_Init, AMI_Init  and AMI_GetWave functions:

- Upstream Tx AMI_Init is called with

  - (BCI_State “Training”) (BCI_Protocol “<name>“) (BCI_ “<my_ID>“)

  - If the executable model does not implement the BCI_Protocol, it returns “Error” in BCI_State

  - The executable model may write a message file in the BCI_ID namespace under BCI_Protocol.

- Repeater Rx AMI_Init is called with

  - (BCI_State “Training”) (BCI_Protocol “<name>“) (BCI_ID “<my_ID>“)

  - If the executable model does not implement the BCI_Protocol, it returns “Error” in BCI_State

  - The executable model may read, write, modify and/or delete message files in the BCI_ID namespace under BCI_Protocol.

- Repeater Tx AMI_Init is called with

  - (BCI_State “Training”) (BCI_Protocol “<name>“) (BCI_ID “<my_ID>“)

  - If the executable model does not implement the BCI_Protocol, it returns “Error” in BCI_State

  - The executable model may read, write, modify and/or delete message files in the BCI_ID namespace under BCI_Protocol.

- Downstream Rx AMI_Init is called with

  - (BCI_State “Training”) (BCI_Protocol “<name>“) (BCI_ID “<my_ID>“)  (BCI_Training_UI <# Training Bits>)

  - If the executable model does not implement the BCI_Protocol, it returns “Error” in BCI_State

  - The executable model may read, write, modify and/or delete message files in the BCI_ID namespace under BCI_Protocol.

- Upstream Tx AMI_GetWave is called with the stimulus pattern. The executable model may read, write, modify and/or delete message files in the BCI_ID namespace under BCI_Protocol.

- Repeater Rx AMI_GetWave is called with the waveform output of the Upstream Tx AMI_GetWave modified by the Upstream Channel Impulse Response. The executable model may read, write, modify and/or delete message files in the BCI_ID namespace under BCI_Protocol.

- Repeater Tx AMI_GetWave is called with the waveform output of the Repeater Rx AMI_GetWave. The executable model may read, write, modify and/or delete message files in the BCI_ID namespace under BCI_Protocol.

- Downstream Rx AMI_GetWave is called with the waveform output of the Repeater Tx AMI_GetWave modified by the Downstream Channel Impulse Response. The executable model may read, write, modify and/or delete message files in the BCI_ID namespace under BCI_Protocol

- Steps 5 through 8 are repeated until the EDA tool stops the simulation.

  - The EDA tool should start processing the output of Rx AMI_GetWave after Ignore_Bits and either:

after BCI_Training_UI, or

when the downstream Rx AMI_GetWave function returns BCI_State “Converged” or “Failed” or any executable model in the channel returns “Error”.

Note that it is the responsibility of the BCI _Protocol to define the BCI message files and contents therein so that each executable model in the channel can determine its role/position in the channel optimization.

BACKGROUND INFORMATION/HISTORY:

Link training capability was initially developed by Sigrity (now Cadence Design Systems) and Snowbush (IP division of Gennum). It was deemed desirable to bring this capability to the IBIS standard in order to encourage other SerDes IP suppliers to enable link training functionality for their IP as well.

BIRD 147 has been discussed extensively in the Advanced Technology Modeling Task Group, with a number of updated draft versions posted in the group’s work archive. The changes made for BIRD 147.1 are substantial.

BIRD147.2 based on ATM meeting discussion and subsequent email discussion, this version changes the requirement rules and clarifies that BCI_Protocol is required to support BCI. All four other BCI_* parameters are required.  The default of 1000 for BCI_GetWave_Block_UI has been removed.  Also, a rule is added that it is not illegal to have any BCI_* parameter without the others. This makes the rule and its checking simple and removes a default that the EDA tool may not necessarily implement.  The Table YYI Required entries are modified to show the Required dependence on BCI_Protocol, as stated for each of the other BCI_* parameters.

The BCI_Protocol parameter Other Notes and Example were modified to state that IBIS Specification would not list the approved IBIS protocols, but that we still suggest names of official IBIS protocols start with IBIS, and private protocols start with the company name.

A future mechanism where the parser checks for names listed in a file under www.ibis.org could be used to test if an IBIS_XXX protocol name is official.

BIRD147.3 expresses more clearly in BCI_Protocol Other Notes private and IBIS prefixed protocol names per email input.