BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	197.1

ISSUE TITLE:   		New AMI Reserved Parameter DC_Offset

REQUESTOR:     	Walter Katz, SiSoft,

Ambrish Varma, Cadence Design Systems,

Randy Wolff, Micron Techology,

Justin Butterfield, Micron Technology

DATE SUBMITTED:	November 27, 2018

DATE REVISED:	December 4, 2018

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

AMI modeling is now being applied to singled ended channels (e.g. DDR5). The current input to AMI_Init is an Impulse Response. The forces all AMI simulations to be centered around the mid-level of the single ended signal. A DLL may need to know the singled ended voltage levels (e.g. to handle saturation in a DFE sumer). This BIRD proposes a new AMI Reserved Parameter DC_Offset which is the singled ended voltage that is the mid value of the beginning and end of the step response of the channel.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Allow the EDA tool to convey to the model the mid-point of the steady state high and low voltages found during analog model characterization of single-ended signals.” |  |

SUMMARY OF PROPOSED CHANGES:

Add new AMI Reserved Parameter DC_Offset.

PROPOSED CHANGES:

Parameter:   	DC_Offset

Required:	No, and illegal before AMI_Version 7.x

Direction:        Rx

Descriptors:

Usage:                   In

Type:                     Float

Format:                  Value

Default:                 <numeric_literal>

Description:           <string>

Definition:	The mean value of the steady state high and low voltage of the channel at the Rx pad. If the impulse response was generated by differentiating the step response, then the value of DC_Offset should be the same as the average of the step response beginning and end voltage.

The AMI_Init function can use this value to determine the single ended voltages inside of the model.

It is also assumed that the waveform input to the Rx AMI_GetWave function is the single ended waveform minus this DC_Offset. The Rx AMI_GetWave function can choose to construct the singled ended waveform by adding DC_Offset to the input waveform. The waveform output of the Rx AMI_GetWave shall be adjusted so that the EDA tool can add DC_Offset to get the single ended voltage of the waveform at the slicer (aka latch, decision point).

Usage Rules: It is the responsibility of the EDA tool to determine the DC_Offset. The EDA tool may use any method to do this.

Example:

(DC_Offset (Usage In) (Type Float) (Value .5)

(Description "The EDA tool is responsible for determining the DC_Offset value to input to the executable model”)

BACKGROUND INFORMATION/HISTORY:

Typograchical updates made in BIRD197.1, based on feedback from Open Forum and ATM review.