BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	197.2

ISSUE TITLE:   		New AMI Reserved Parameter DC_Offset

REQUESTOR:     	Walter Katz, SiSoft,

Ambrish Varma, Cadence Design Systems,

Randy Wolff, Micron Techology,

Justin Butterfield, Micron Technology

DATE SUBMITTED:	November 27, 2018

DATE REVISED:	December 4, 2018, January 15, 2019

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

AMI modeling is now being applied to singled ended channels (e.g. DDR5). The current input to AMI_Init is an Impulse Response. The forces all AMI simulations to be centered around the mid-level of the single ended signal. A DLL may need to know the singled ended voltage levels (e.g. to handle saturation in a DFE summer). This BIRD proposes a new AMI Reserved Parameter DC_Offset, which is the singled ended voltage that is the mid value of the beginning and end of the step response of the channel.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Allow the EDA tool to convey to the model the mid-point of the steady state high and low voltages found during analog model characterization of single-ended signals. |  |

SUMMARY OF PROPOSED CHANGES:

Add new AMI Reserved Parameter DC_Offset.

PROPOSED CHANGES:

Parameter:	DC_Offset

Required:	No, and illegal before AMI_Version 7.0

Direction:	Rx

Descriptors:

Usage:                   In

Type:                     Float

Format:                  Value

Default:                 <numeric_literal>

Description:           <string>

Definition:	The mean value of the steady state high and low voltage of the channel at the Rx pad.

Usage Rules:	If the impulse response was generated by differentiating the step response, then the value of DC_Offset should be the same as the average of the step response initial and final voltages.

The AMI_Init function can use this value to determine the single ended voltages inside of the model.

It is also assumed that the waveform input to the Rx AMI_GetWave function is the single ended waveform minus this DC_Offset. The Rx AMI_GetWave function can choose to construct the singled ended waveform by adding DC_Offset to the input waveform. The waveform output of the Rx AMI_GetWave shall be adjusted so that the EDA tool can add DC_Offset to get the single ended voltage of the waveform at the slicer (aka latch, decision point).

Other Notes:	It is the responsibility of the EDA tool to determine the DC_Offset. The EDA tool may use any method to do this.

Example:

DC_Offset (Usage In) (Type Float) (Value 0.5)

(Description “The EDA tool is responsible for determining the DC_Offset value to input to the executable model.”)

BACKGROUND INFORMATION/HISTORY:

Typographical updates made in BIRD197.1, based on feedback from Open Forum and ATM review.

BIRD197.2 contains additional editorial changes.