BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	TBD

ISSUE TITLE:   		Clarification when two different I/O pin_names have the same

signal_name

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:

DATE REVISED:

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

There is no rule that prevents two different pin_names having the same signal_name. The following passes IBISCHK7 V7.0.0:

[Pin]  signal_name            model_name   R_pin         L_pin        C_pin

CP22     MA7          ADD_CMD

CP22x   MA7          DQ

This is a confusing situation. This BIRD will require that if two or more pins have the same signal_name they shall have the same model_name, and that there shall be one instance of that model_name that is connected to all of the pins with that same signal_name. This BIRD shall also clarify the package model that shall be used to connect these pins to the model instance.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - New rule that requires that two or more pins with the same signal_name shall have the same model name. |  |
| - Define package models shall connect the pins with the same signal name to the I/O buffer model. |  |

PROPOSED CHANGES:

Replace the following paragraph on page 29

If a pin has a model_name POWER, then all other pins with the same signal_name as this pin shall

have model_name POWER. If a pin has model_name GND, then all other pins with the same

signal_name as this pin shall have model_name GND.

With

With a pin has a model_name that is not CIRCUITCALL or NC then all other pins with the same

signal_name as this pin shall have the same model_name.

After the following bullet item on page 35.

- If an *_I/O pin_name does not appear on a terminal line in any Interconnect Model in an Interconnect Model Group, then the EDA tool should use any other existing package model in this document.

Add

- If two or more I/O pin_names have the same signal_name, and one of them is in any Interconnect Model in an Interconnect Model Group, then all of them shall be in the interconnect model.

BACKGROUND INFORMATION/HISTORY: