Presented at the IBIS Interconnect Meeting on October 21, 2015

The logic of connecting supply interconnect models between package model pad nodes and on-die model pad nodes is straightforward, and can be described by discussing the following 6 cases:

Side A                   Side B

- Pad_name          pad_name

- Pad_name          bus_label

- Pad_name          signal_name

- Bus_label            bus_label

- Bus_label            signal_name

- Signal_name      signal_name

Note that the logic for the three cases exclude are handle by switching the Side A and Side B conditions:

- Bus_label            pad_name

- Signal_name      bus_label

- Signal_name      signal_name

Side A                   Side B

- Pad_name          pad_name

  - Simple one to one mapping

- Pad_name          bus_label

  - All A Pad_names have a bus_label

  - Connect the terminal for each bus_label on B to the terminals on A that have the same bus_label

- Pad_name          signal_name

  - All A Pad_names have a signal_name

  - Connect the terminal for each signal_name on B to the terminals on A that have the same signal_name

- Bus_label            bus_label

  - Simple one to one mapping

- Bus_label            signal_name

  - All A Pad_names have a signal_name

  - Connect the terminal for each signal_name on B to the terminals on A that have the same signal_name

- Signal_name      signal_name

  - Simple one to one mapping

This logic is self-evident, I do not think it is a requirement that is be in the specification. If we decide to put this logic in the specification, then this would have to be converted to English that is clear.

I point out that there is a requirement that there are one or more Side A models that need to be able to map all of the supply terminals to one or more Side B models. Model makers can create Side A and Side B models that are easy to match and marry. Model makers can create Side A and Side B models that can be extremely difficult (and sometimes impossible) to match and marry. For example an impossible set of models:

Side A has one model that has pad names VDD1, VDD2, and VDD3

There are two Side B models:

- Side B model 1 has pad names VDD1, VDD2

- Side B model 2 has pad names VDD2, VDD3