RDIMM Example Figures and Example Syntax for BIRD202

Randy Wolff

Micron Technology

4/17/2020

RDIMM Example Figures

Figure X shows a DDR4 Registered DIMM containing DRAM components labeled by designators U1, U2, U4, U5 (front side) and U7-U11 (back side, not seen) and a Register component labeled by designator U3.

Also shown is pre-register Net A07 connecting from an EMD Pin to a Designator Pin of designator U3 and post-register net BA07 connecting from a Designator Pin of designator U3 to Designator Pins of designators U4, U5, U7, and U8 as well as termination resistor RN13 connecting to the VTT rail.

Figure X

Figure Y, a zoomed in area of Figure X, shows an example of an extended net. The extended net A07 can be modeled two ways:

- One EMD Model defining only terminals for EMD Pin 211 and Designator Pin U3.W1.  The EMD Model contains the complete signal path of net A07, the series resistor R123, and net A07r.

- One EMD Model or multiple EMD Models contained with an EMD Set that include terminals for EMD Pin 211 and Designator Pin U3.W1 and two terminals for the pins of the series resistor.  The resistor would be assigned a designator (R123) referencing an IBIS component.

Figure Y

Figure Z, a zoomed in area of Figure X, shows an example of an internal net. The post-register net BA07 connects from the register’s Designator Pin U3.B11 to the DDR4 DRAMs’ Designator Pins U4.M8, U5.M8, U7.M8, and U8.M8 as well as to one Designator Pin of the termination resistor RN13.  RN13 terminates the signal to the VTT rail.

Figure Z

Example Syntax

| EMD Syntax Example 1 (Embedded Resistors)

| Using DDR4 RDIMM Example

[Begin EMD] DDR4_RDIMM

[Number of EMD Pins] 4

[EMD Pin List] signal_name signal_type  bus_label

203            VSS         GND

211            A07

212            VDD         POWER        VDD1

223            VTT         POWER

[End EMD Pin List]

[EMD Parts]

DDR4_Reg_253b  register.ibs   DDR4_Register

DDR4_x8_78b    dram.ibs       DDR4_8Gb_x8

[End EMD Parts]

[EMD Designator List]

U3        DDR4_Reg_253b

U4        DDR4_x8_78b

U5        DDR4_x8_78b

U7        DDR4_x8_78b

U8        DDR4_x8_78b

[End EMD Designator List]

[Designator Pin List] signal_name  signal_type  bus_label

U3.B9                 VDD          POWER        VDD1

U3.B11                BA07

U3.B12                VSS          GND

U3.V3                 VDD          POWER        VDD1

U3.W1                 A07

U3.W3                 VSS          GND

|

U4.K9                 VSS          GND

U4.M8                 BA07

U4.N9                 VDD          POWER        VDD1

|

U5.K9                 VSS          GND

U5.M8                 BA07

U5.N9                 VDD          POWER        VDD1

|

U7.K9                 VSS          GND

U7.M8                 BA07

U7.N9                 VDD          POWER        VDD1

|

U8.K9                 VSS          GND

U8.M8                 BA07

U8.N9                 VDD          POWER        VDD1

[End Designator Pin List]

[Voltage List]

VDD   1.200   1.140   1.260

VSS   0.000   0.000   0.000

VTT   0.600   0.570   0.630

[End Voltage List]

[EMD Group]    Just_One

Addr_07  NA

[End EMD Group]

[End EMD]

[EMD Set]      Addr_07

[EMD Model]     A07

File_IBIS-ISS   A07.iss       A07

Number_of_terminals = 6

1  Pin_I/O      pin_name      211

2  Pin_I/O      pin_name      U3.W1

3  Pin_Rail     bus_label     VDD1

4  Pin_Rail     signal_name   VSS

5  Pin_Rail     bus_label     U3.VDD1

6  Pin_Rail     bus_label     U3.VSS

[End EMD Model]

[EMD Model]     BA07

File_IBIS-ISS   A07.iss       BA07

Number_of_terminals = 16

1  Pin_I/O      pin_name      U3.B11

2  Pin_Rail     bus_label     U3.VDD1

3  Pin_Rail     signal_name   U3.VSS

4  Pin_I/O      pin_name      U4.M8

5  Pin_Rail     bus_label     U4.VDD1

6  Pin_Rail     signal_name   U4.VSS

7  Pin_I/O      pin_name      U5.M8

8  Pin_Rail     bus_label     U5.VDD1

9  Pin_Rail     signal_name   U5.VSS

10 Pin_I/O      pin_name      U7.M8

11 Pin_Rail     bus_label     U7.VDD1

12 Pin_Rail     signal_name   U7.VSS

13 Pin_I/O      pin_name      U8.M8

14 Pin_Rail     bus_label     U8.VDD1

15 Pin_Rail     signal_name   U8.VSS

17 Pin_Rail     bus_label     VDD1

18 Pin_Rail     signal_name   VTT

19 Pin_Rail     signal_name   VSS

[End EMD Model]

[End EMD Set]

| EMD Syntax Example 2 (External Resistors)

| Using DDR4 RDIMM Example

[Begin EMD] DDR4_RDIMM

[Number of EMD Pins] 4

[EMD Pin List] signal_name signal_type  bus_label

203            VSS         GND

211            A07

212            VDD         POWER        VDD1

223            VTT         POWER

[End EMD Pin List]

[EMD Parts]

DDR4_Reg_253b  register.ibs   DDR4_Register

DDR4_x8_78b    dram.ibs       DDR4_8Gb_x8

510-500874     resistors.ibs  RES_22OHM

510-501618     resistors.ibs  RPACK4_33OHM

[End EMD Parts]

[EMD Designator List]

U3        DDR4_Reg_253b

U4        DDR4_x8_78b

U5        DDR4_x8_78b

U7        DDR4_x8_78b

U8        DDR4_x8_78b

R123      510-500874

RN13      510-501618

[End EMD Designator List]

[Designator Pin List] signal_name  signal_type  bus_label

U3.B9                 VDD          POWER        VDD1

U3.B11                BA07

U3.B12                VSS          GND

U3.V3                 VDD          POWER        VDD1

U3.W1                 A07

U3.W3                 VSS          GND

|

U4.K9                 VSS          GND

U4.M8                 BA07

U4.N9                 VDD          POWER        VDD1

|

U5.K9                 VSS          GND

U5.M8                 BA07

U5.N9                 VDD          POWER        VDD1

|

U7.K9                 VSS          GND

U7.M8                 BA07

U7.N9                 VDD          POWER        VDD1

|

U8.K9                 VSS          GND

U8.M8                 BA07

U8.N9                 VDD          POWER        VDD1

|

R123.1                A07

R123.2                A07

RN13.2                VTT          POWER

RN13.7                BA07

[End Designator Pin List]

[Voltage List]

VDD   1.200   1.140   1.260

VSS   0.000   0.000   0.000

VTT   0.600   0.570   0.630

[End Voltage List]

[EMD Group]    Just_One

Addr_07  NA

[End EMD Group]

[End EMD]

[EMD Set]      Addr_07

[EMD Model]     A07

File_IBIS-ISS   A07.iss       A07

Number_of_terminals = 8

1  Pin_I/O      pin_name      211

2  Pin_I/O      pin_name      R123.1

3  Pin_I/O      pin_name      R123.2

4  Pin_I/O      pin_name      U3.W1

5  Pin_Rail     bus_label     VDD1

6  Pin_Rail     signal_name   VSS

7  Pin_Rail     bus_label     U3.VDD1

8  Pin_Rail     signal_name   U3.VSS

[End EMD Model]

[EMD Model]     BA07

File_IBIS-ISS   A07.iss       BA07

Number_of_terminals = 16

1  Pin_I/O      pin_name      U3.B11

2  Pin_Rail     bus_label     U3.VDD1

3  Pin_Rail     signal_name   U3.VSS

4  Pin_I/O      pin_name      U4.M8

5  Pin_Rail     bus_label     U4.VDD1

6  Pin_Rail     signal_name   U4.VSS

7  Pin_I/O      pin_name      U5.M8

8  Pin_Rail     bus_label     U5.VDD1

9  Pin_Rail     signal_name   U5.VSS

10 Pin_I/O      pin_name      U7.M8

11 Pin_Rail     bus_label     U7.VDD1

12 Pin_Rail     signal_name   U7.VSS

13 Pin_I/O      pin_name      U8.M8

14 Pin_Rail     bus_label     U8.VDD1

15 Pin_Rail     signal_name   U8.VSS

16 Pin_I/O      pin_name      RN13.7

17 Pin_Rail     bus_label     VDD1

18 Pin_Rail     pin_name      RN13.VTT

19 Pin_Rail     signal_name   VSS

[End EMD Model]

[End EMD Set]

| EMD Syntax Example 3 (External Resistors, Separate A07, A07R, and POWER Models)

| Using DDR4 RDIMM Example

|

[Begin EMD] DDR4_RDIMM

[Number of EMD Pins] 4

[EMD Pin List] signal_name signal_type  bus_label

203            VSS         GND

211            A07

212            VDD         POWER        VDD1

223            VTT         POWER

[End EMD Pin List]

[EMD Parts]

DDR4_Reg_253b  register.ibs   DDR4_Register

DDR4_x8_78b    dram.ibs       DDR4_8Gb_x8

510-500874     resistors.ibs  RES_22OHM

510-501618     resistors.ibs  RPACK4_33OHM

[End EMD Parts]

[EMD Designator List]

U3        DDR4_Reg_253b

U4        DDR4_x8_78b

U5        DDR4_x8_78b

U7        DDR4_x8_78b

U8        DDR4_x8_78b

R123      510-500874

RN13      510-501618

[End EMD Designator List]

[Designator Pin List] signal_name  signal_type  bus_label

U3.B9                 VDD          POWER        VDD1

U3.B11                BA07

U3.B12                VSS          GND

U3.V3                 VDD          POWER        VDD1

U3.W1                 A07

U3.W3                 VSS          GND

U4.K9                 VSS          GND

U4.M8                 BA07

U4.N9                 VDD          POWER        VDD1

U5.K9                 VSS          GND

U5.M8                 BA07

U5.N9                 VDD          POWER        VDD1

U7.K9                 VSS          GND

U7.M8                 BA07

U7.N9                 VDD          POWER        VDD1

U8.K9                 VSS          GND

U8.M8                 BA07

U8.N9                 VDD          POWER        VDD1

R123.1                A07

R123.2                A07

RN13.2                VTT          POWER

RN13.7                BA07

[End Designator Pin List]

[Voltage List]

VDD   1.200   1.140   1.260

VSS   0.000   0.000   0.000

VTT   0.600   0.570   0.630

[End Voltage List]

[EMD Group]    Just_One

Addr_07           NA

RIGHT_SIDE_POWER  NA

[End EMD Group]

[End EMD]

[EMD Set]      Addr_07

[EMD Model]     A07

File_IBIS-ISS   A07.iss       A07

Number_of_terminals = 3

1  Pin_I/O      pin_name      211

2  Pin_I/O      pin_name      R123.1

3  Pin_Rail     signal_name   VSS

[End EMD Model]

|

[EMD Model]     A07R

File_IBIS-ISS   A07.iss       A07R

Number_of_terminals = 3

1  Pin_I/O      pin_name      R123.2

2  Pin_I/O      pin_name      U3.W1

3  Pin_Rail     signal_name   VSS

[End EMD Model]

|

[EMD Model]     BA07

File_IBIS-ISS   A07.iss      BA07

Number_of_terminals = 16

1  Pin_I/O      pin_name      U3.B11

3  Pin_Rail     signal_name   U3.VSS

4  Pin_I/O      pin_name      U4.M8

6  Pin_Rail     signal_name   U4.VSS

7  Pin_I/O      pin_name      U5.M8

9  Pin_Rail     signal_name   U5.VSS

10 Pin_I/O      pin_name      U7.M8

12 Pin_Rail     signal_name   U7.VSS

13 Pin_I/O      pin_name      U8.M8

15 Pin_Rail     signal_name   U8.VSS

16 Pin_I/O      pin_name      RN13.7

18 Pin_Rail     pin_name      RN13.VTT

19 Pin_Rail     signal_name   VSS

[End EMD Model]

[End EMD Set]

[EMD Set]      RIGHT_SIDE_POWER

[EMD Model]     RIGHT_SIDE_VDD1_VTT_VSS

File_IBIS-ISS   rdimm_power.iss RIGHT_SIDE_VDD1_VTT_VSS

Number_of_terminals = 8

1  Pin_Rail     bus_label     VDD1

2  Pin_Rail     signal_name   VSS

3  Pin_Rail     signal_name   VTT

4  Pin_Rail     bus_label     U3.VDD1

5  Pin_Rail     signal_name   U3.VSS

6  Pin_Rail     bus_label     U4.VDD1

7  Pin_Rail     signal_name   U4.VSS

8  Pin_Rail     bus_label     U5.VDD1

9  Pin_Rail     signal_name   U5.VSS

10 Pin_Rail     bus_label     U7.VDD1

11 Pin_Rail     signal_name   U7.VSS

12 Pin_Rail     bus_label     U8.VDD1

13 Pin_Rail     signal_name   U8.VSS

14 Pin_Rail     signal_name   RN13.VTT

[End EMD Model]

[End EMD Set]