BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	Draft 11 September 10, 2014

ISSUE TITLE:   		Interconnect Modeling Using IBIS-ISS

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:	{date you sent the original document, for new BIRDs}

DATE REVISED:	{date(s) you sent any revisions to the document}

DATE ACCEPTED BY IBIS OPEN FORUM: {leave blank; for administrative use only}

STATEMENT OF THE ISSUE:

This BIRD enhances IBIS interconnect models to supterminal Broadband and Coupled package and on-die interconnect using IBIS-ISS and Touchstone models.

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

Definitions:

IBIS Interconnect modeling makes several assumptions:

- Interconnect Models can either be IBIS-ISS subckts or Touchstone Files

- If two points are “Connected” then there is either a low resistance DC electrical path between the two points, or a small insertion loss at Nyquist frequency between the two points.

- For each I/O Pin, there is a single Die Pad and single Buffer I/O that are “Connected”.

- For each POWER or GND Signal_name, all pins, die pads and buffer supply terminals that use that Signal_name are “Connected”

- The Terminals (or Terminals) of Interconnect Models are Pins, Die Pads, Buffer I/O or Buffer supply terminals.

- There is an implicit assumption that one I/O pin is connected to one Die Pad and is connected to a one buffer. Two differential I/O pins must be connected to two differential die pads and either two single ended buffers or a single true differential buffer.

- If there are joins, multiple buffer connected to a single pin then should use EMD …

- An Interconnect Model may represent a single connection between Pins and Buffers, Pins and Die Pads, or Die Pads and Buffers. An Interconnect Model may also represent multiple connections between multiple Pins and multiple Buffers, multiple Pins and multiple Die Pads, or multiple Die Pads and multiple Buffers.

ANY OTHER BACKGROUND INFORMATION:

{These documents will be archived, so use this section to add any detail that is not part of the section above or the changed text itself , but should not be lost.}

Keyword:	[Interconnect Model Selector]

Required:	No

Description:	Used to pick an interconnect model for this component.

Usage Rules:	Interconnet Models are described by IBIS-ISS subckts or Toucshstone files that are between the Pins, Die Pads and Buffers of a Component.

A component may have none, one or more [Interconnect Model]. If there are any [Interconnect Model]s, they must be listed in this section.

The section under the [Interconnect Model Selector] keyword must have two fields.  The fields must be separated by at least one white space.  The first field lists the [Interconnect Model] name (up to 40 characters long).  The second field is the name of the file containing the [Interconnect Model]. If the [Interconnect Model] is in this IBIS file, then the second field must be “*”.

The first entry under the [Interconnect Model Selector] keyword shall be considered the default by the EDA tool.

Example:

[Interconnect Model Selector]

QS-SMT-cer-8-pin-pkgs_iss *

QS-SMT-cer-8-pin-pkgs_sNp QS-SMT-cer-8-pin-pkgs_sNp.ipkg

[End Interconnect Model Selector]

Keyword:	[Interconnect Model]

Required:	No

Description:	Marks the beginning of an interconnect model description.

Usage Rules:	The length of the interconnect model name must not exceed 40 characters in length.  Blank characters are not allowed.

Example:

[Interconnect Model]     QS-SMT-cer-8-pin-pkgs_iss

Keyword:	[Manufacturer] Allow or Require

Keyword:	[Description] Allow or Require

Same requirements as in IBIS if separate file.

Keyword:	[Begin Interconnect Model] <Interconnect Model Name>

Subparameter: File_TS Typ_File Min_File Max_File Subparameter: File_ISS  Typ|Min|Max File_Name Circuit_Name

Subparameter: Param <name> Typ_Value Min_Value Max_Value

Subparameter: Unused_Terminal_Termination <resistance>

Subparameter: Number_of_Terminals <# terminals>         |  Made into a Subparameter

Subparameter:  Terminal Terminal_number Location ID {Qualifiers}Keyword: [End Interconnect Model]

Keyword: [End Interconnect Model]

Required: Yes, to end the [Interconnect Model] keyword

Description: Indicates the end of the interconnect model data.

Other Notes: In between the [Begin Interconnect Model] and [End Interconnect Model] keywords is the package model data itself. The data is any number of interfaces to either IBIS-ISS models or Touchstone files.

Example:

[End Interconnect Model]

We need a careful discussion on how Pin Mapping is used in conjunction with Terminals that have Signal_name. (MM)

We need a carefull discussion on when package models are Pre-Layout only. (Walter)

We need a carefull discussion on precedence rules if more than one model can be used to represent interconnect. (Walter)

Interaction with Circuit Call and External Circuit? (Mutually Exclusive)

Interaction with Define Package Model, or are they mutually exclusive.

Precedence Rules? Both allowed in a component but only one type can be used.

Sub-Params:	File_TS, File_ISS, Param | Other sub-params not fully documented here

Usage Rules:  Number_Of_Nodes, Terminal and either File_TS or File_ISS are required (both File_TS and File_ISS together are not permitted).

For referencing Touchstone files:

Subparameter:  File_TS Typ_File Min_File Max_File

Required: Either File_TS or File_ISS is required for a [Begin Model]/[End Model] group

Description: File_TS is followed by three entries for typ, min, and max file names.  The typical entry is required and must point to a Touchstone file located in the same directory as the .ibs file and representing typical conditions.  The minimum and maximum entries may point to the same file or other files representing minimum (slow) and maximum (fast) interconnect conditions or contain NA. If the entry is NA, the typical file entry shall be used.

Example:

| file_type  typ      min      max

File_TS      typ.s8p  min.s8p  max.s8p

or

| file_type  typ      min      max

File_TS      typ.s4p  min.s4p  NA

For referencing IBIS-ISS files:

Subparameter:  File_ISS  Typ|Min|Max File_Name Circuit_Name

Required: Either File_TS or File_ISS is required for a [Begin Model]/[End Model] group

Description: File_ISS is followed by three entries consisting of corner_name, file_name, and circuit_name (.subckt name) for that file and located in the same directory as the .ibs file.  The corner_name shall be Typ, Min, or Max.  File_ISS for the Typ corner_name is required, and File_ISS for the Min and Max corner_names are optional.  If present, each File_ISS must have a unique corner_name.  If File_ISS for either the Min or Max corner_name is missing, the File_ISS for the Typ corner_name shall be used to describe the missing corner_name file reference.  The Min and Max file_names should represent slow and fast interconnect conditions.Example:

| file_type  corner_name file_name   circuit_name (.subckt name)

File_ISS     Typ         net.iss     netlist_typ

File_ISS     Min         net.iss     netlist_min | in same file as net.sp

File_ISS     Max         net_max.iss netlist_max | in separate file

Subparameter: Param <name> Typ_Value Min_Value Max_Value

Required: No, but legal only if Language is IBIS-ISS.

Description:

The subparameter Param is optional and only legal for File_ISS references.  Param shall be followed by a param_name of the parameter to be passed into the IBIS-ISS and its numerical values or a string values (surrounded by double quotes) located in the typ, min, and max columns.  Several Param lines are permitted as long as each of the param_name entries is distinct.  Each Param line shall have a typ entry.  Either or both the min and max entries can be NA, in which cases the typ entry is used.  The typ, min, and max parameters are, by default, associated with the corner_name Typ, Min, and Max files and their corresponding circuit_names.  However, the EDA tool is expected to support passing any of the Param typ, min, or max values, as selected by the User or EDA tool, into any File_ISS corner_name file.  The Param values associated with any param_name must all be numerical or all string values (or NA).  If possible, the Param min and max values should represent slow and fast interconnect conditions.  Because of parameter interactions, this may not always be possible.

Other Notes:  The numerical value rules follow the scaling conventions in Section 3, GENERAL SYNTAX RULES AND GUIDELINES.  The EDA tool is responsible for translating IBIS specified parameters into IBIS-ISS parameters.  For example, 1 megohm, represented as 1M in Param would be converted to 1meg (1x is not recommended) in IBIS-ISS.  The value 1Kohm is 1 ohm in IBIS and would therefore be passed into IBIS-ISS as 1 ohm, even though 1K is 1 kilohm in IBIS-ISS.  Quoted string parameters are converted to the string parameter syntax in IBIS-ISS.  For example, the Param value “typ.s2p” is converted to str(‘typ.s2p’) in IBIS-ISS.

The base unit of frequency is Hertz, and the base unit of length is meter.  Values can be passed in terms of other base units of length if scaling conversions are added to the IBIS-ISS .subckt definition. For example, the intended value of 10 mils might be entered as the Param value of 10 if the conversion to 10 mils is done through multiplication within the .subckt.

Examples:

| Param      param_name typ       min       max

Param        abc        2m        1m        2m

Param        def        4k        NA        NA

Param        ts_file    “typ.s2p” “min.s2p” “max.s2p” | used in IBIS-ISS

NOTES AND QUESTIONS

Source Touchstone | IBIS-ISS is not necessary since the file format is recognized by File_TS or File_ISS.  File_ISS captures both the file_name and circuit_name for each corner.

For File_ISS, an alternative syntax could have been File_ISS_Typ, File_ISS_Min, File_ISS_Max to eliminate the corner_name column, where only File_ISS_Typ is required for file references to IBIS-ISS.

Parameter is shorted to Param (.param is legal in IBIS-ISS) to differentiate it further from Parameters in the multi-lingual syntax.  (Parameter has several meanings in IBIS/IBIS-AMI.)

File_names are not quoted to be consistent with Corner in the multi-lingual syntax.

For File_TS, all columns typ, min, and max are entered (or NA for either or both min and max) to follow the corner syntax convention used for most IBIS keywords and subparameters.  The typ entry is required, and the typ entry is used for any NA entry.  The same typ, min, max convention is used for the subparameter Param.

Entries for strings in Param are surrounded by double quotes to be consistent with string_literal Parameters in the multi-lingual syntax (or where the AMI string_literal parameter surrounded by double quotes is passed into the multi-lingual Parameters reference).  The EDA tool needs to convert string_literals into the parameter string syntax in IBIS-ISS.

FBASE and FMAX are not defined in IBIS-ISS or Touchstone, so they are not documented here as reserved names for parameters.

Interaction of Param entries was not discussed.  For example, for a T-line TD and Z0 could each have max and min entries, but the EDA tool could make available combinations of min/min, min/max, max/min or max/max for any corner .  Due to parameter interactions, some mixing of corner combinations might not be realistic. (E.g., Z0min or Z0max might not correlate with TDmin or TDmax values.)  (TDmin=sqrt(LminCmin), Z0min=sqrt(Lmin/Cmax), etc.).

How corners of File_ISS and Params are processed might be based on vendor supplied documentation.  For example some, but not all, combinations are shown below:

- One file_name for all corners, one .subckt name, and all corner settings controlled by Param settings

- One file_name, three .subckts (with internal default .param settings), additional corner settings controlled by Param settings or Param is not used

- Three file_names with the same .subckt name, but with distinct default .param settings, additional settings controlled by Param settings or Param is not used

- Three file_names with three distinct .subckt name and with distinct default .param settings, additional corner settings controlled by Param settings or Param is not used

No interpretation is given for Param typ, min, and max values.  It is possible to independently use typ, min, or max values for any of the Param names that have been defined (e.g., the max value of one parameter may be used with the min value of another parameter).

Keyword: Unused_Terminal_Termination <resistance>  (Brad)

Required:  No

Description: Defines the termination that is to be applied to the Terminals of a subckt or Touchstone file that are not being used in each [Begin Interconnect Model]/[End Interconnect Model] group.

Other Notes: If this subparameter is defined the EDA should connect the unused Terminals to GND through a <resistance> ohm resistor.

If this parameter is not defined and if Language is IBIS-ISS, then the EDA tool should connect the unused Terminals to GND through a 1Meg ohm resistor. If Language is Touchstone, then the EDA tool should connect the unused Terminals to GND through a resistor with the Touchstone File reference resistance of the Terminal.

Example:

[Unused_Terminal_Termination] 50

Keyword: Number_of_Terminals <# terminals>         |  Made into a Subparameter

Required:  Yes, for each [Begin Interconnect Model]/[End Interconnect Model] group

Description: The number of terminals of the IBIS-ISS subckt or Touchstone file.

Other Notes:

Example:

Number_of_Terminals 2

Subparameter:  Terminal Terminal_number Location ID {Qualifiers}

Required: An Interconnect Model must have Terminal subparameter records for each [Begin Interconnect Model]/[End Interconnect Model] group.

Description: Each Terminal record contains information on a terminal of an IBIS-ISS subckt (or Touchstone file).

Terminal_number must be a positive integer number greater or equal to one and less than or equal to the number of terminals [Number of Terminals], of the IBIS-ISS subckt (or Toucshtone file). Two Terminal records may not have the same Terminal_number. If a Terminal Number does not exist in any of the [Terminal] records then the terminal is unused, and should be terminated according to the Unused_Terminal_Termination_ Rules.

Location shall be Pin, Pad, Buf, Pin_Sig, Pad_Sig, Buf_Sig, Buf_PURef, Buf_PDRef, Buf_PCRef, Buf_GCRef or Buf_XRef.

- Pin indicates this terminal connected to a specific pin, ID must be a Pin_name, Model_name or Default.

- Pad indicates this terminal connected to  a specific die pad, ID must be a Pin_name, Model_name or Default.

- Buf indicates this terminal connected to a specific buffer model I/O or signal terminal, ID must be a Pin_name, Model_name or Default.

- Pin_Sig indicates that this terminal is connected to all pins that have Signal_name ID. ID must be a Signal_name on a Pin that has Model_name Power or GND. All pins that have Signal_name ID are considered shorted together at the pin side of the package model.

- Pad_Sig indicates that this terminal is connected to all die pads that have Signal_name ID. ID must be a Signal_name on a Pin that has Model_name Power or GND. All die pads that have Signal_name ID are considered shorted together at the die pad side of the package model.

- Buf_Sig indicates that this terminal is connected to all buffer model terminals Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference or External Reference that have an ID containing a Signal_name ID must be a Signal_name on a Pin that has Model_name Power or GND. All Buffer terminal nodes that have Signal_name ID are considered shorted together at the buffer side of the package model.

- Buf_PURef indicates this terminal connected to a specific buffer model pullup reference, ID must be a Pin_name, Model_name or Default.

- Buf_PDRef indicates this terminal connected to a specific buffer model pulldown reference, ID must be a Pin_name, Model_name or Default.

- Buf_PCRef indicates this terminal connected to a specific buffer model power clamp reference, ID must be a Pin_name, Model_name or Default.

- Buf_GCRef indicates this terminal connected to a specific buffer model ground clamp reference, ID must be a Pin_name, Model_name or Default.

- Buf_XRef indicates this terminal connected to  a specific buffer model external reference, ID must be a Pin_name, Model_name or Default.

ID shall be a Pin_name, Signal_name, Model_name or Default.

Qualifiers may have the values Aggressor, Model_name, Default, Inverting, Non-Inverting and Connection(n). Qualifiers are optional, there may be zero, one or several qualifiers on each Terminal record. Qualifiers may appear in any order.

- Aggressor, any Terminal may have the qualifier aggressor. It means that terminal does not have coupling from all aggressor sources, so can be treated as an aggressor and should not be treated as a victim. By default a connection is a Victim.

- Model_name, means that the ID on this terminal is a Model_name

- Default, means that the ID on this terminal must be Default.

- A terminal cannot have both Default and Model_name qualifiers.

- If a terminal is either qualifier Default or Model_name then the terminal is considered a “Pre-Layout” terminal.

- If a “Pre-Layout” terminal is connected to a differential model, then the terminal must have either the Inverting or Non-Inverting qualifier.

- All terminals that have the same Connection(n) (where n is a positive integer) are electrically connected. A single ended connection will have two terminals with Connection(n). A differential connection will have four terminals with Connection(n).` Connection(n) qualifiers are required if there are two or more Pre-Layout connections. Is a differential one connection or two connections (clarify).

- Special differential rules for Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference.

  - There can be only one terminal for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference on a true differential [External Model]. These can be referenced by either the Non-Inverting or Inverting Pin_name.

  - There may be only one terminal for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference for each side of a legacy differential model that consists of two independent single ended models. These can be referenced by either the Non-Inverting or Inverting Pin_name.

  - There may be two terminals for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference for each side of a legacy differential model that consists of two independent single ended models.

Other Notes:

More than one [Interconnect Model] may be available for a specific simulation. The EDA tool may choose any of the available models but, in general, should prefer a model that matches by Pin_name, then Model_name and finally Default.

An Interconnect Model with  File_TS with N Ports. N is either determined from the N in the .sNp file name extension for a Touchstone I file or from the [Number of Ports] record in a Touchstone II file. The [Number of Terminals] in the Interconnect Model shall be N+1. The Terminal Rules is described below:

- [

  - The EDA tool shall use the Pin_name or Signal_name specified in the Terminal “N+1” record as the reference node for each of the N ports.

  - Terminal/Port Mapping

    - Terminal              Port

    - 1                              1

    - 2                              2

    - …

    - N                             N

    - N+1		reference

  - If a Port is not connected, then it shall be terminated with a resistor to the node on Terminal N+1. The resistance shall be the Port Reference Impedance.

  - It shall be an error if Terminal N+1 is not specified to a Pin, Pad, or Buffer that is not on a connection to a Signal_name that is POWER or GND

Examples:

IBIS File

[Pin] signal_name model_name      R_pin   L_pin   C_pin

A1    DQ1         DQ

A2    DQ2         DQ

A3    DQ3         DQ

D1    DQS         DQS

D2    DQS         DQS

P1    VDD         POWER

P2    VDD         POWER

P3    VDD         POWER

P4    VDD         POWER

P5    VDD         POWER

G1    VSS         GND

G2    VSS         GND

G3    VSS         GND

G4    VSS         GND

[Diff Pin]  inv_pin  vdiff  tdelay_typ tdelay_min tdelay_max

D1          D2       NA     NA         NA         NA

[Die Supply Pads]

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

[Pin Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref ext_ref

A1            VSS          VDD        NC            NC              NC

A2            VSS          VDD        NC            NC              NC

A3            VSS          VDD        NC            NC              NC

D1            VSS          VDD        NC            NC              NC

D2            VSS          VDD        NC            NC              NC

- Single DQ (A1)

  - Terminal 1 Pin A1

  - Terminal 2 Buf A1

- Single DQS | There is a [Diff Pin] record “D1 D2 …”

  - Terminal 1 Pin D1

  - Terminal 2 Pin D2

  - Terminal 3 Buf D1

  - Terminal 4 Buf D2

- One DQ (A2) victim, two DQ (A1 and A3) aggressors

  - Terminal 1 Pin A1 Aggressor

  - Terminal 2 Buf A1 Aggressor

  - Terminal 3 Pin A2

  - Terminal 4 Buf A2

  - Terminal 5 Pin A3 Aggressor

  - Terminal 6 Buf A3 Aggressor

- Single DQ (A1) Pin to Die Pad

  - Terminal 1 Pin A1

  - Terminal 2 Pad A1

- Single ended model that can be used for all I/O pins

  - Terminal 1 Pin Default Default

  - Terminal 2 Buf Default Default

- Madel that can connect all Pins with Signal_name VDD to all Buffer supply terminals that are connected to Signal_name VDD as described in Pin_mapping. All Pins with Signal_name VDD are shorted together. All Buffer supply terminals that are connected to Signal_name VDD are shorted together

  - Terminal 1 Pin_Sig VDD

  - Terminal 2 Buf_Sig VDD

- VDD: Pins connected to board “bed spring” model, all buffers connected to VDD shorted

  - Terminal 1 Pin P1

  - Terminal 2 Pin P2

  - Terminal 3 Pin P3

  - Terminal 4 Pin P4

  - Terminal 5 Pin P5

  - Terminal 6 Buf_Sig VDD

- VDD: Interconnect between VDD Pins and individual buffer Pullup Reference.

  - Terminal 1 Pin P1

  - Terminal 2 Pin P2

  - Terminal 3 Pin P3

  - Terminal 4 Pin P4

  - Terminal 5 Pin P5

  - Terminal 6 Buf_PURef A1

  - Terminal 7 Buf_PURef A2

  - Terminal 8 Buf_PURef A3

  - Terminal 9 Buf_PURef D1

- VDD: Interconnect between VDD Pins and die VDD pads.

  - Terminal 1 Pin P1

  - Terminal 2 Pin P2

  - Terminal 3 Pin P3

  - Terminal 4 Pin P4

  - Terminal 5 Pin P5

  - Terminal 6 Pad VDD1

  - Terminal 7 Pad VDD2

  - Terminal 8 Pad VDD3

- VDD: Interconnect between die VDD pads and individual buffer Pullup Reference.

  - Terminal 1 Pad VDD1

  - Terminal 2 Pad VDD2

  - Terminal 3 Pad VDD3

  - Terminal 4 Buf_PURef A1

  - Terminal 5 Buf_PURef A2

  - Terminal 6 Buf_PURef A3

  - Terminal 7 Buf_PURef D1

- Single DQ

  - Terminal 1 Pin DQ Model_name

  - Terminal 2 Buf DQ Model_name

- Single DQS

  - Terminal 1 Pin DQS Model_name Non-Inverting

  - Terminal 2 Pin DQS Model_name Inverting

  - Terminal 3 Buf DQS Model_name Non-Inverting

  - Terminal 4 Buf DQS Model_name Inverting

- Single DQ victim, two DQ aggressors

  - Terminal 1 Pin DQ Model_name Aggressor  Connection(1)

  - Terminal 2 Buf DQ Model_name Aggressor  Connection(1)

  - Terminal 3 Pin DQ Model_name Connection(2)

  - Terminal 4 Buf DQ Model_name Connection(2)

  - Terminal 5 Pin DQ Model_name Aggressor Connection(3)

  - Terminal 6 Buf DQ Model_name Aggressor Connection(3)

- One DQ victim, two DQ aggressors, one DQS aggressor

  - Terminal 1 Pin DQ Model_name Aggressor  Connection(1)

  - Terminal 2 Buf DQ Model_name Aggressor  Connection(1)

  - Terminal 3 Pin A2

  - Terminal 4 Buf A2

  - Terminal 5 Pin DQ Model_name Aggressor Connection(2)

  - Terminal 6 Buf DQ Model_name Aggressor Connection(2)

  - Terminal 7 Pin DQS Model_name Aggressor Connection(3) Non-Inverting

  - Terminal 8 Buf DQS Model_name Aggressor Connection(3) Inverting

  - Terminal 9 Pin DQS Model_name Aggressor Connection(3) Non-Inverting

  - Terminal 10 Buf DQS Model_name Aggressor Connection(3) Inverting

- One single ended victim, two single ended aggressors, one differential aggressor

  - Terminal 1 Pin Default Default Aggressor  Connection(1)

  - Terminal 2 Buf Default Default Aggressor  Connection(1)

  - Terminal 3 Pin Default Default

  - Terminal 4 Buf Default Default

  - Terminal 5 Pin Default Default Aggressor  Connection(2)

  - Terminal 6 Buf Default Default Aggressor  Connection(2)

  - Terminal 7 Pin Default Default Aggressor Connection(3) Non-Inverting

  - Terminal 8 Buf Default Default Aggressor Connection(3) Inverting

  - Terminal 9 Pin Default Default Aggressor Connection(3) Non-Inverting

  - Terminal 10 Buf Default Model_name Aggressor Connection(3) Inverting

Arpad:  All these examples seem to go between pins and pads, or pins and buffer terminals.  It would be nice to have some examples which go between pads and buffer terminals.  Also, combinations of pin to pad and pad to buffer and pin to buffer might be nice to see if we desire to support those combinations.

Keyword:	[Die Supply Pads]

Required:	No

Description:	This begins a section in [Component] that contains one line of data for die pads supply nodes. IBIS assumes that for I/O pins (pins that have a Model_name that is not POWER, GND or NC), there is a one to one correspondence between a Pin, Die Pad and Buffer I/O. There are no such assumptions for POWER and GND pins. A POWER or GND Signal_name may have a different number of Pin nodes, die pad nodes and buffer nodes. If the model maker chooses to make separate package and on-die power distribution networks (PDN), then he must supply a list of nodes (and their associated Signal_name) that can be used to mate the package and on-die PDN models.

Sub-Params:	?

Usage Rules:	 TBD

Other Notes:	The data in this section consists of  a list of die pad node names and their corresponding Signal_names that can be used to mate package and on-die PDN networks.

Example:

[Die Supply Pads]

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

Keyword: [End Die Supply Pads]

Required: Yes.

Description: Indicates the end of the [Die Supply Pads] data.

Other Notes:

Example:

[End Die Supply Pads]

Examples

[Define Package Model]

[ISS Model Data]

[Begin ISS Model] IOA3

Language Touchstone

File Value ioA3.s2p

Number_of_Terminals 2

Terminal 1  Pin    Pin_name A3

Terminal 2  Buffer Pin_name A3

[End ISS Model]

[Begin ISS Model] IOA7

| This model uses I/O pin A7

Language Touchstone

File Value ioA7.s2p

Number_of_Terminals 2

Terminals  Pin.A7  Buf.A7

[End ISS Model]

[Begin ISS Model] IOB3C3

Language Touchstone

File Value ioB3C3.s4p

Number_of_Terminals 4

Terminal 1  Pin    Pin_name B3

Terminal 2  Buffer Pin_name B3

Terminal 3  Pin    Pin_name C3

Terminal 4  Buffer Pin_name C3

[End ISS Model]

[Begin ISS Model] IOA3

Language IBIS_ISS

File Value io.iss

Subckt io

Parameter Length Value 10.   | 10mm

Number_of_Terminals 2

Terminal 1  Pin    Pin_name A3

Terminal 2  Buffer Pin_name A3

[End ISS Model]

[Begin ISS Model] DQS

Language Touchstone

File Value DQS.s4p

Number_of_Terminals 4

Terminal 1  Pin    Model_name DQS Diff_pos

Terminal 2  Buffer Model_name DQS Diff_pos

Terminal 3  Pin    Model_name DQS Diff_neg

Terminal 4  Buffer Model_name DQS Diff_neg

[End ISS Model]

[Begin ISS Model] VDDQ

Language IBIS_ISS

File Value vddq.iss

Subckt vddq

Number_of_Terminals 2

Terminal 1  Pin    Signal_name VDDQ

Terminal 2  Buffer Signal_name VDDQ

[End ISS Model]

[Begin ISS Model] VDDQ_A3

Language IBIS_ISS

File Value vddq_a3.iss

Subckt vddq_A3

Number_of_Terminals 2

Terminal 1  Pin    Signal_name VDDQ

Terminal 2  Buffer Pin_name    A3   Pullup_Reference

[End ISS Model]

[Begin ISS Model] IOA3

Language Touchstone

File Value ioA3.s10p

Number_of_Terminals 10

Terminal 1  Pin     Pin_name   A3

Terminal 2  Buffer  Pin_name   A3

Terminal 3  Pin     Model_name DQ  NA       1 Aggressor

Terminal 4  Buffer  Model_name DQ  NA       1 Aggressor

Terminal 5  Pin     Model_name DQ  NA       2 Aggressor

Terminal 6  Buffer  Model_name DQ  NA       2 Aggressor

Terminal 7  Pin     Model_name DQS Diff_pos 3 Aggressor

Terminal 8  Buffer  Model_name DQS Diff_pos 3 Aggressor

Terminal 9  Pin     Model_name DQS Diff_neg 3 Aggressor

Terminal 10 Buffer  Model_name DQS Diff_neg 3 Aggressor

[End ISS Model]

[End ISS Model Data]

[End Package Model]