BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	Draft 19 – April 21, 2015

ISSUE TITLE:   		Interconnect Modeling Using IBIS-ISS

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:

DATE REVISED:

DATE ACCEPTED BY IBIS OPEN FORUM:

STATEMENT OF THE ISSUE:

This BIRD enhances IBIS with interconnect modeling features to support broadband and coupled package and on-die interconnect using IBIS-ISS and Touchstone data.

The BIRD also adds a keyword for buffer rail mapping, to link to the new Terminal definitions defined for buffers.

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

Definitions:

Enhanced interconnect descriptions in IBIS, called hereinafter “IBIS Interconnect Models”, rely on several assumptions:

- IBIS Interconnect Models can be described either using IBIS-ISS subcircuit files or Touchstone files.  Interconnect Model definitions may be included inside an IBIS file, but neither IBIS-ISS nor Touchstone data may be included inside an IBIS file.

- If two points in an IBIS Interconnect Model are “Linked”, then there is either a low resistance DC electrical path between the two points, or a small insertion loss at the Nyquist frequency between the two points.  For the purposes of IBIS Interconnect Models, “point” and “node” refer to identical locations.

- IBIS Components, and therefore IBIS Interconnect Models, contain terminals consisting of Pins, Die Pads, Buffer I/O Terminals, and Buffer Supply Terminals. Pins are defined under the [Pin] keyword, and may be I/O, POWER, GND, or NC.

- For each I/O Pin, there is a single, associated Die Pad and single, associated Buffer I/O Terminal.  All of these shall be considered “Linked”.

- Under [Pin], for each Signal_name associated with Model_name POWER or GND, all Pins, Die Pads and Buffer Supply Terminals that use that Signal_name are “Linked”

- IBIS assumes that each I/O [Pin] is connected to one Die Pad and one Buffer I/O Terminal. Two differential I/O pins shall be connected to two differential die pads and either two single-ended Buffer I/O Terminals or a single true differential Buffer I/O Terminal.

- If multiple Buffer Terminals (Supply or I/O) are connected to a single pin, EMD shall be used for the interconnect description.

- An Interconnect Model may describe the relationship between a single Pin and Buffer Terminal (Supply or I/O), a signle Pin and linked Die Pad, or between a single Die Pad and a Buffer Terminal (Supply or I/O). An Interconnect Model may also describe connections between multiple Pins and multiple Buffer Terminals (Supply and I/O), multiple Pins and multiple Die Pads, or multiple Die Pads and multiple Buffer Terminals (Supply and I/O).

ANY OTHER BACKGROUND INFORMATION:

Parameter is shorted to Param (.param is legal in IBIS-ISS) to differentiate it further from Parameters in the multi-lingual syntax (Parameter has several meanings in IBIS and the Algorithmic Modeling Interface.)

File_names are not quoted to be consistent with Corner in the multi-lingual syntax.

For File_TS, all columns typ, min, and max are entered (or NA for either or both min and max) to follow the corner syntax convention used for most IBIS keywords and subparameters.  The typ entry is required, and the typ entry value is used by the EDA tool for any NA entry.  The same typ, min, max convention is used for the subparameter Param.

Entries for strings in Param are surrounded by double quotes to be consistent with string_literal Parameters in the multi-lingual syntax (or where the AMI string_literal parameter surrounded by double quotes is passed into the multi-lingual Parameters reference).  The EDA tool needs to convert string_literals into the parameter string syntax in IBIS-ISS.

Interaction of Param entries was not discussed.  For example, for a transmission line, TD and Z0 could each have max and min entries, but the EDA tool could make available combinations of min/min, min/max, max/min or max/max for any corner .  Due to parameter interactions, some mixing of corner combinations might not be realistic. (E.g., Z0min or Z0max might not correlate with TDmin or TDmax values, where TDmin=sqrt(LminCmin), Z0min=sqrt(Lmin/Cmax), etc.).

How corners of File_IBIS-ISS and Params are processed might be based on vendor supplied documentation.  For example some, but not all, combinations are shown below:

- One file_name for all corners, one .subckt name, and all corner settings controlled by Param settings

- One file_name, three .subckts (with internal default .param settings), additional corner settings controlled by Param settings or Param is not used

- Three file_names with the same .subckt name, but with distinct default .param settings, additional settings controlled by Param settings or Param is not used

- Three file_names with three distinct .subckt name and with distinct default .param settings, additional corner settings controlled by Param settings or Param is not used

No interpretation is given for Param typ, min, and max values.  It is possible to independently use typ, min, or max values for any of the Param names that have been defined (e.g., the max value of one parameter may be used with the min value of another parameter).

The following keywords should be added as their own Chapter.  The current Chapter 7 should be modified with the existing text placed in a sub-section called “[PACKAGE MODEL]”.

_________________________________________________________________________

7  PACKAGE MODELING

Several types of package modeling formats are available in IBIS.  These include:

- Lumped [Component]-level models for the entire [Component], using the [Package] keyword

- Lumped [Component]-level modeling per-pin, using the [Pin] keyword

- [Package Model] (including [Alternate Package Models] and [Define Package Model])

- Interconnect Model Selectors

The lumped formats are described in the [Package] and [Pin] keyword defintions above.  The [Package Model] format is described in this chapter, while Interconnect Model Selectors are described in Chapter 13.

The order of precedence between the various formats is described in Chapter 12, “Rules of Precedence”.

…

________________________________________________________________________

13 INTERCONNECT MODEL SELECTORS

This chapter defines an advanced format for interconnect descriptions that may be used for packages as well as other types of interconnect between buffer models and pins, for signal and power path modeling purposes.

Keyword:	[Interconnect Model Selector]

Required:	No

Description:	Used to list available interconnect models for the component.

Usage Rules:	Interconnect Models are described by IBIS-ISS or Touchstone files that are between the Pins, Die Pads and Buffer Terminals (Supply and I/O) of a Component.

A component may have none, one or more than one Interconnect Model associated with it. If any Interconnect Models exist for the Component, they shall be listed in this section.  An Interconnect Model Selector is required even if only a single Interconnect Model is associated with the Component.

The section under the [Interconnect Model Selector] keyword shall have two fields per line, with each line defining the Interconnect Models associated with the Component.  The fields shall be separated by at least one white space.  The first field lists the Interconnect Model name (up to 40 characters long).  The second field is the name of the file containing the Interconnect Model. If the Interconnect Model is in this IBIS file, then the second field shall be “*”.

The file containing the Interconnect Model shall be located in the same directory as the .ibs file. The file name shall follow the rules for file names given in Section 3, "GENERAL SYNTAX RULES AND GUIDELINES".

The first entry under the [Interconnect Model Selector] keyword shall be considered the default by the EDA tool.  Each Interconnect Model name may only appear once under the [Interconnect Model Selector] keyword for a given Component.

Example:

[Interconnect Model Selector]

QS-SMT-cer-8-pin-pkgs_iss *

QS-SMT-cer-8-pin-pkgs_sNp QS-SMT-cer-8-pin-pkgs_sNp.ipkg

[End Interconnect Model Selector]

Keyword: 	[End Interconnect Model Selector]

Required: 	Yes, for each instance of the [Begin Interconnect Model Selector] keyword

Description: 	Indicates the end of the Interconnect Model selector data.

Example:

[End Interconnect Model Selector]

Keyword:	[Begin Interconnect Model]

Required:	No

Description:	Marks the beginning of an interconnect model description.

Usage Rules:	[Begin Interconnect Model] has a single argument, which is the name of the associated Interconnect Model.  The length of the Interconnect Model name shall not exceed 40 characters in length.  Blank characters are not allowed.

Example:

[Begin Interconnect Model]     QS-SMT-cer-8-pin-pkgs_iss

The following subparameters are defined:

Manufacturer

Description

Param

File_TS

File_IBIS-ISS

Unused_Terminal_Termination

Number_of_Terminals

Terminal

Unless noted below, no Interconnect Model subparameter requires the presence of any other subparameter.  Each subparameter is optional.

Manufacturer rules:

This subparameter specifies the name of the interconnect’s manufacturer.  The length of the manufacturer’s name shall not exceed 40 characters.  Blank characters are permitted.

Description rules:

This subparameter provides a concise yet easily human-readable description of what the Interconnect Model represents.  The description shall be fewer than 60 characters in length, shall fit on a single line, and may contain spaces.

Unused_Terminal_Termination rules:

This subparameter defines the termination that is to be applied by the EDA tool during simulation to the Terminals of any IBIS-ISS subcircuit or Touchstone networks that are not being used in the [Begin Interconnect Model]/[End Interconnect Model] group.  The subparameter name is followed by a single integer argument greater than zero on the same line, separated from the subparameter name by whitespace.

If this subparameter is present, the EDA should connect the unused Terminals to GND through a resistor with the value of resistance in ohms provided in the argument.

If this parameter is not defined and if Language is IBIS-ISS, then the EDA tool should connect the unused Terminals to GND through a 1 megaohm resistor. If Language is Touchstone, then the EDA tool should connect the unused Terminals to GND through a resistor with the Touchstone File reference resistance of the Terminal.

Only one Unused_Terminal_Termination subparameter may appear for a given [Begin Interconnect Model] keyword.

Number_of_Terminals rules:

The Number_of_Terminals subparameter is required and defines the number of terminals associated with the Interconnect Model. The subparameter name is followed by a single integer argument greater than zero on the same line, separated from the subparameter name by whitespace.  Only one Number_of_Terminals subparameter may appear for a given [Begin Interconnect Model] keyword.

Param rules:

The subparameter Param is optional and only legal with the File_IBIS-ISS subparameter documented below.  Param is illegal with the File_TS subparameter documented below.  Param shall be followed by three arguments: an unquoted string argument giving the name of the parameter to be passed into the IBIS-ISS, a reserved word for the parameter format, and one numerical value or one string value (surrounded by double quotes) for the parameter value to be passed into the IBIS-ISS.

The numerical value rules follow the scaling conventions in Section 3, GENERAL SYNTAX RULES AND GUIDELINES.  The EDA tool is responsible for translating IBIS specified parameters into IBIS-ISS parameters.  For example, 1 megaohm, would be represented as 1M in Param value according to The Section 3 rules, but would be converted by the EDA tool to case-insensitive 1meg (1X is not recommended) or 1E6 for IBIS-ISS use.  Quoted string parameters in IBIS are converted to the string parameter syntax in IBIS-ISS.  For example, the Param value "typ.s2p" would be converted to str('typ.s2p') in IBIS-ISS.

Examples:

| Param   name     format   value

Param     abc      Value    2m        | 2E-3 in IBIS

Param     def      Value    4k        | 4E3 in IBIS

Param     ts_file  Value    "typ_s2p" | file name string passed

| into IBIS-ISS

File_IBIS-ISS rules:

Either File_IBIS-ISS or File_IBIS-TS (documented next) is required for a [Begin Interconnect Model]/[End Interconnect Model] group.  The File_IBIS-ISS subparameter is followed by two string arguments consisting of the file_name, and circuit_name (.subckt name) for an IBIS-ISS file.  These entries are not surrounded by double quotes to be consistent with the [External Model] and [External Circuit] convention for the Corner subparameter.  The referenced file under file_name is located in the same directory as the .ibs file.

Example:

| file_type    file_name    circuit_name(.subckt name)

File_IBIS-ISS  net.iss      netlist_typ

File_TS rules:

Either File_TS or File_IBIS-ISS is required for a [Begin Interconnect Model]/[End Interconnect Model] group.  File_TS is followed by the file name entry for a Touchstone file.  The Touchstone file under file_name is located in the same directory as the .ibs file.

Example:

| file_type    file_name

File_TS        typ.s8p

Terminal rules:

One or more Terminal subparameters may appear under a given [Begin Interconnect Model] keyword.  At least one Terminal subparameter is required.  Each Terminal record contains information on a terminal of an IBIS-ISS subckt (or Touchstone file).

The Terminal subparameter is followed by at least three arguments: Terminal_number, Terminal_ID and Terminal_Location.  An unlimited number of Qualifiers may optionally follow these three required arguments.

Terminal_number shall be a positive non-zero integer and less than or equal to the number of terminals in the Number_of_Terminals argument. The same Terminal_number shall not appear more than once for a given Interconnect Model.  If any Terminals are not present for a given Interconnect Model, then those terminals are unused, and shall be terminated according to the Unused_Terminal_Termination rules.

Terminal_ID is a string using either a [Pin] name, a Signal_name, a Model_name, or “Default”, with restrictions as noted for specific Terminal_Location values.

Terminal_Location is a string, and shall have one of the values Pin_A_signal, Pad_A_signal, A_signal, Pin_Signal_name, Pad_Signal_name, A_Signal_name, A_puref, A_pdref, A_pcref, A_gcref or A_extref.

- Pin_A_signal indicates this terminal is the buffer A_signal_terminal connected to a specific pin, Terminal_ID shall be a Pin_name, Model_name or Default.

- Pad_A_signal indicates this terminal is the buffer A_signal_terminal connected to a specific die pad, Terminal_ID shall be a Pin_name, Model_name or Default.

- A_signal indicates this terminal is the buffer A_signal terminal. Terminal_ID shall be a Pin_name, Model_name or Default.

- Pin_Signal_name indicates that this terminal is connected to all pins that have Signal_name Terminal_ID. Terminal_ID shall be a Signal_name on a Pin that has Model_name Power or GND. All pins that have Signal_name Terminal_ID are considered shorted together at the pin side of the package model.

- Pad_Signal_name indicates that this terminal is connected to all die pads that have Signal_name Terminal_ID. Terminal_ID shall be a Signal_name on a Pin that has Model_name Power or GND. All die pads that have Signal_name Terminal_IDs are considered shorted together at the die pad side of the package model.

- A_Signal_name indicates that this terminal is connected to all buffer model terminals Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference or External Reference (A_puref, A_pdref, A_pcref, A_gcref or A_extref) that have an Terminal_ID containing a Signal_name Terminal_ID shall be a Signal_name on a Pin that has Model_name Power or GND. All Buffer Terminals that have Signal_name Terminal_ID are considered shorted together at the buffer side of the package model.

- A_puref indicates this terminal connected to a specific buffer model pullup reference, Terminal_ID shall be a Pin_name, Model_name or Default.

- A_pdref indicates this terminal connected to a specific buffer model pulldown reference, Terminal_ID shall be a Pin_name, Model_name or Default.

- A_pcref indicates this terminal connected to a specific buffer model power clamp reference, Terminal_ID shall be a Pin_name, Model_name or Default.

- A_gcref indicates this terminal connected to a specific buffer model ground clamp reference, Terminal_ID shall be a Pin_name, Model_name or Default.

- A_extref indicates this terminal connected to  a specific buffer model external reference, Terminal_ID shall be a Pin_name, Model_name or Default.

Qualifiers may have the values Aggressor, Model_name, Default, Inverting, Non-Inverting and Connection(n).  Qualifiers are optional; there may be zero, one, or several qualifiers for each Terminal record. Qualifiers may appear in any order.

- Aggressor: any Terminal may have the qualifier aggressor. It means that terminal does not have coupling from all aggressor sources, so can be treated as an aggressor and should not be treated as a victim. By default a connection is a Victim.

- Model_name: the Terminal_ID on this terminal is a Model_name

- Default: the Terminal_ID on this terminal shall be Default.

- A terminal cannot have both Default and Model_name qualifiers.

- If a terminal is either qualifier Default or Model_name then the terminal is considered a “Pre-Layout” terminal.

- If a “Pre-Layout” terminal is connected to a differential model, then the terminal shall use the following Terminal Locations

    - Pin_A_signal_pos

    - Pad_A_signal_pos

    - A_signal_pos

    - Pin_A_signal_neg

    - Pad_A_signal_neg

    - A_ signal_neg

- All terminals that have the same Connection(n) (where n is a positive integer) are electrically connected. A single-ended connection will have two terminals with Connection(n). A differential connection will have four terminals with Connection(n).` Connection(n) qualifiers are required if there are two or more Pre-Layout connections. Special differential rules for Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference.

  - There may be only one terminal for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference on a true differential [External Model]. These can be referenced by either the Non-Inverting or Inverting signal Pin_name.

  - There may be only one terminal for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference for each side of a legacy differential model that consists of two independent single-ended models. These can be referenced by either the Non-Inverting or Inverting signal Pin_name.

  - There may be two terminals for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference for each side of a legacy differential model that consists of two independent single-ended models.

Other Notes:

More than one Interconnect Model may be available for a specific simulation. The EDA tool may choose any of the available models but, in general, should prefer a model that matches by Pin_name, then Model_name and finally Default.

For an Interconnect Model using File_TS with N ports, N is either determined from the N in the .sNp file name extension for a Touchstone 1 file or from the [Number of Ports] record in a Touchstone 2 file. The [Number of Terminals] in the Interconnect Model shall be N+1.  Terminal rules are described below:

- The EDA tool shall use the Pin_name or Signal_name specified for the associated Terminal “N+1” entry as the reference node for each of the N ports.

- Terminal/Port Mapping

  - Terminal              Port

  - 1                              1

  - 2                              2

  - …

  - N                             N

  - N+1		reference

- If a Port is not connected, then it shall be terminated by the EDA tool with a resistor to the node on Terminal N+1. The resistance shall be the Port Reference Impedance.

- It shall be an error if Terminal N+1 is not specified to be connected to a Pin, a Pad, or a Buffer that is not part of a connection to a Signal_name that is POWER or GND

Examples:

IBIS File

[Pin] signal_name model_name      R_pin   L_pin   C_pin

A1    DQ1         DQ

A2    DQ2         DQ

A3    DQ3         DQ

D1    DQS         DQS

D2    DQS         DQS

P1    VDD         POWER

P2    VDD         POWER

P3    VDD         POWER

P4    VDD         POWER

P5    VDD         POWER

G1    VSS         GND

G2    VSS         GND

G3    VSS         GND

G4    VSS         GND

[Diff Pin]  inv_pin  vdiff  tdelay_typ tdelay_min tdelay_max

D1          D2       NA     NA         NA         NA

[Die Supply Pads]

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

[Buffer Rail Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref ext_ref

A1            VSS          VDD        NC            NC              NC

A2            VSS          VDD        NC            NC              NC

A3            VSS          VDD        NC            NC              NC

D1            VSS          VDD        NC            NC              NC

D2            VSS          VDD        NC            NC              NC

| Single DQ (A1)

Terminal 1 Pin_A_signal A1

Terminal 2 A_signal     A1

| Single DQS | There is a [Diff Pin] record “D1 D2 …”

Terminal 1 Pin_A_signal D1

Terminal 2 Pin_A_signal D2

Terminal 3 A_signal     D1

Terminal 4 A_signal     D2

| One DQ (A2) victim, two DQ (A1 and A3) aggressors

Terminal 1 Pin_A_signal A1 Aggressor

Terminal 2 A_signal     A1 Aggressor

Terminal 3 Pin_A_signal A2

Terminal 4 A_signal     A2

Terminal 5 Pin_A_signal A3 Aggressor

Terminal 6 A_signal     A3 Aggressor

|Single DQ (A1) Pin to Die Pad

Terminal 1 Pin_A_signal A1

Terminal 2 Pad_A_signal A1

| Single-ended model that can be used for all I/O pins

Terminal 1 Pin_A_signal Default Default

Terminal 2 A_signal Default Default

| Model that can connect all Pins with Signal_name VDD to all Buffer supply terminals that are connected to

| Signal_name VDD as described in Pin_mapping. All Pins with Signal_name VDD are shorted together.

| All Buffer supply terminals that are connected to Signal_name VDD are shorted together

Terminal 1 Pin_Signal_name VDD

Terminal 2 A_Signal_name   VDD

| VDD: Pins connected to board “bed spring” model, all buffer terminals connected to VDD shorted

Terminal 1 Pin_A_signal  P1

Terminal 2 Pin_A_signal  P2

Terminal 3 Pin_A_signal  P3

Terminal 4 Pin_A_signal  P4

Terminal 5 Pin_A_signal  P5

Terminal 6 A_Signal_name VDD

| VDD: Interconnect between VDD Pins and individual buffer Pullup Reference.

Terminal 1 Pin_A_signal P1

Terminal 2 Pin_A_signal P2

Terminal 3 Pin_A_signal P3

Terminal 4 Pin_A_signal P4

Terminal 5 Pin_A_signal P5

Terminal 6 A_puref      A1

Terminal 7 A_puref      A2

Terminal 8 A_puref      A3

Terminal 9 A_puref      D1

| VDD: Interconnect between VDD Pins and die VDD pads.

Terminal 1 Pin_A_signal    P1

Terminal 2 Pin_A_signal    P2

Terminal 3 Pin_A_signal    P3

Terminal 4 Pin_A_signal    P4

Terminal 5 Pin_A_signal    P5

Terminal 6 Pad_Signal_name VDD1

Terminal 7 Pad_Signal_name VDD2

Terminal 8 Pad_Signal_name VDD3

| VDD: Interconnect between die VDD pads and individual buffer Pullup Reference.

Terminal 1 Pad_Signal_name VDD1

Terminal 2 Pad_Signal_name VDD2

Terminal 3 Pad_Signal_name VDD3

Terminal 4 A_puref         A1

Terminal 5 A_puref         A2

Terminal 6 A_puref         A3

Terminal 7 A_puref         D1

| Single DQ

Terminal 1 Pin_A_signal DQ Model_name

Terminal 2 A_signal     DQ Model_name

| Single DQS

Terminal 1 Pin_A_signal_pos DQS Model_name

Terminal 2 Pin_A_signal_neg DQS Model_name

Terminal 3 A_signal_pos     DQS Model_name

Terminal 4 A_signal_neg DQS Model_name

| Single DQ victim, two DQ aggressors

Terminal 1 Pin_A_signal DQ Model_name Aggressor Connection(1)

Terminal 2 A_signal     DQ Model_name Aggressor Connection(1)

Terminal 3 Pin_A_signal DQ Model_name           Connection(2)

Terminal 4 A_signal     DQ Model_name           Connection(2)

Terminal 5 Pin_A_signal DQ Model_name Aggressor Connection(3)

Terminal 6 A_signal     DQ Model_name Aggressor Connection(3)

| One DQ victim, two DQ aggressors, one DQS aggressor

Terminal 1 Pin_A_signal     DQ  Model_name Aggressor Connection(1)

Terminal 2 A_signal         DQ  Model_name Aggressor Connection(1)

Terminal 3 Pin_A_signal     A2

Terminal 4 A_signal         A2

Terminal 5 Pin_A_signal     DQ  Model_name Aggressor Connection(2)

Terminal 6 A_signal         DQ  Model_name Aggressor Connection(2)

Terminal 7 Pin_A_signal_pos DQS Model_name Aggressor Connection(3)

Terminal 8 A_signal_pos     DQS Model_name Aggressor Connection(3)

Terminal 9 Pin_A_signal_neg DQS Model_name Aggressor Connection(3)

Terminal 10 A_signal_neg    DQS Model_name Aggressor Connection(3)

| One single-ended victim, two single-ended aggressors, one differential aggressor

Terminal 1 Pin_A_signal     Default Default Aggressor  Connection(1)

Terminal 2 A_signal         Default Default Aggressor  Connection(1)

Terminal 3 Pin_A_signal     Default Default

Terminal 4 A_signal         Default Default

Terminal 5 Pin_A_signal     Default Default Aggressor  Connection(2)

Terminal 6 A_signal         Default Default Aggressor  Connection(2)

Terminal 7 Pin_A_signal_pos Default Default Aggressor Connection(3)

Terminal 8 A_signal_pos     Default Default Aggressor Connection(3)

Terminal 9 Pin_A_signal_neg Default Default Aggressor Connection(3)

Terminal 10 A_signal_neg    Default Model_name Aggressor Connection(3)

| DQ: (A1) (Post-Layout)

Terminal 1 Pin_A_signal A1

Terminal 2 A_signal     A1

| DQS: (Post-Layout) There is a [Diff Pin] record “D1 D2 …”

Terminal 1 Pin_A_signal D1

Terminal 2 Pin_A_signal D2

Terminal 3 A_signal     D1

Terminal 4 A_signal     D2

| DQS: (Pre-Layout)

Terminal 1 Pin_A_signal_pos DQS Model_name

Terminal 2 Pin_A_signal_neg DQS Model_name

Terminal 3 A_signal_pos     DQS Model_name

Terminal 4 A_signal_neg     DQS Model_name

| VDD: Pins connected to board “bed spring” model, all buffer terminals connected to VDD shorted

Terminal 1 Pin_A_signal P1

Terminal 2 Pin_A_signal P2

Terminal 3 Pin_A_signal P3

Terminal 4 Pin_A_signal P4

Terminal 5 Pin_A_signal P5

Terminal 6 A_Signal_name VDD

| VDD: Interconnect between VDD Pins and individual buffer Pullup Reference

Terminal 1 Pin_A_signal P1

Terminal 2 Pin_A_signal P2

Terminal 3 Pin_A_signal P3

Terminal 4 Pin_A_signal P4

Terminal 5 Pin_A_signal P5

Terminal 6 A_puref A1

Terminal 7 A_puref A2

Terminal 8 A_puref A3

Terminal 9 A_puref D1

Keyword: 	[End Interconnect Model]

Required: 	Yes, for each instance of the [Begin Interconnect Model] keyword

Description: 	Indicates the end of the Interconnect Model data.

Other Notes:	Between the [Begin Interconnect Model] and [End Interconnect Model] keywords is the package model data itself. The data describes any number of interfaces to either IBIS-ISS models or Touchstone files.

Example:

[End Interconnect Model]

Keyword: 	[End Interconnect Model Selector]

Required: 	Yes, for each instance of the [Begin Interconnect Model Selector] keyword

Description: 	Indicates the end of the Interconnect Model selector data.

Example:

[End Interconnect Model Selector]

Keyword:	[Die Supply Pads]

Required:	No

Description:	This begins a section in [Component] that contains one line of data assigning die pads as supply nodes. IBIS assumes that for I/O pins (pins that have a Model_name that is not POWER, GND or NC), there is a one-to-one correspondence between a Pin, a Die Pad and the Buffer I/O connection point. There are no such assumptions for POWER and GND pins. A POWER or GND Signal_name may have a different number of Pin nodes, die pad nodes and buffer nodes. If the model maker chooses to make separate package and on-die power distribution networks (PDN), then he shall supply a list of nodes (and their associated Signal_name) that can be used to mate the package and on-die PDN models.

Sub-Params:	None

Usage Rules:	 Arguments under the [Die Supply Pads] keyword consist of two strings per line, where the strings define a die pad node name and a corresponding Signal_name, in that order.  Signal_names may appear multiple times, but die pad node names may appear only once each under the [Die Supply Pads] keyword.

Other Notes:	The data in this section consists of a list of die pad node names and their corresponding Signal_names that can be used to mate package and on-die PDN networks.

Example:

[Die Supply Pads]

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

Keyword: 	[End Die Supply Pads]

Required: 	Yes

Description: 	Indicates the end of the [Die Supply Pads] data.

Other Notes:

Example:

[End Die Supply Pads]

Examples

[Define Package Model]

[Begin Interconnect Model] IOA3

| file_name  typ      min      max

File_TS ioA3.s2p NA NA

Number_of_Terminals 2

Terminal 1  Pin_A_signal    Pin_name A3

Terminal 2  A_signal Pin_name A3

Terminal 3 Pin_Signal_name VSS

[End Interconnect Model]

[Begin Interconnect Model] IOA7

| This model uses I/O pin A7

| file_name  typ      min      max

File_TS      ioA7.s2p NA       NA

Number_of_Terminals 2

Terminal 1  Pin_A_signal    A7

Terminal 2  A_signal A7

Terminal 3 Pin_Signal_name VSS

[End Interconnect Model]

[Begin Interconnect Model] IOB3C3

| file_name  typ         min   max

File_TS      ioB3C3.s4p  NA    NA

Number_of_Terminals 4

Terminal 1  Pin_A_signal    Pin_name B3

Terminal 2  A_Signal Pin_name B3

Terminal 3  Pin_A_signal    Pin_name C3

Terminal 4  A_Signal Pin_name C3

Terminal 5 Pin_Signal_name VSS

[End Interconnect Model]

[Begin Interconnect Model] IOA3

| file_type  corner_name file_name   circuit_name (.subckt name)

File_IBIS-ISS     Typ         io.iss      io

Parameter Length Value 10.   | 10mm

Number_of_Terminals 2

Terminal 1  Pin_A_signal    Pin_name A3

Terminal 2  A_signal Pin_name A3

Terminal 3 Pin_Signal_name VSS

[End Interconnect Model]

[Begin Interconnect Model] DQS

| file_name  typ      min    max

File_TS      DQS.s4p  NA     NA

Number_of_Terminals 4

Terminal 1  Pin_A_signal    Model_name DQS Diff_pos

Terminal 2  A_signal Model_name DQS Diff_pos

Terminal 3  Pin_A_signal    Model_name DQS Diff_neg

Terminal 4  A_signal Model_name DQS Diff_neg

Terminal 5 Pin_Signal_name VSS

[End Interconnect Model]

[Begin Interconnect Model] VDDQ

| file_type  corner_name file_name   circuit_name (.subckt name)

File_IBIS-ISS     Typ         vddq.iss    vddq

Number_of_Terminals 2

Terminal 1  Pin_A_signal    Signal_name VDDQ

Terminal 2  A_signal Signal_name VDDQ

Terminal 3 Pin_Signal_name VSS

[End Interconnect Model]

[Begin Interconnect Model] VDDQ_A3

| file_type  corner_name  file_name   circuit_name (.subckt name)

File_IBIS-ISS     Typ          vddq_a3.iss vddq_A3

Number_of_Terminals 2

Terminal 1  Pin_A_signal    Signal_name VDDQ

Terminal 2  A_signal Pin_name    A3   Pullup_Reference

Terminal 3 Pin_Signal_name VSS

[End Interconnect Model]

[Begin Interconnect Model] IOA3

| file_name  typ       min      max

File_TS    ioA3.s10p   NA       NA

Number_of_Terminals 10

Terminal 1  Pin_A_signal     Pin_name   A3

Terminal 2  A_signal Pin_name   A3

Terminal 3  Pin_A_signal     Model_name DQ  NA       1 Aggressor

Terminal 4  A_signal Model_name DQ  NA       1 Aggressor

Terminal 5  Pin_A_signal     Model_name DQ  NA       2 Aggressor

Terminal 6  A_signal Model_name DQ  NA       2 Aggressor

Terminal 7  Pin_A_signal     Model_name DQS Diff_pos 3 Aggressor

Terminal 8  A_signal Model_name DQS Diff_pos 3 Aggressor

Terminal 9  Pin_A_signal     Model_name DQS Diff_neg 3 Aggressor

Terminal 10 A_signal Model_name DQS Diff_neg 3 Aggressor

Terminal 11 Pin_Signal_name VSS

[End Interconnect Model]

[End Interconnect Model Data]

[End Package Model]

________________________________________________________

The following section should be included in Chapter 5, Component Modeling.

Keyword:	[Buffer Rail Mapping]

Required:	No

Description:	Used to indicate the signal_name to which a given driver, receiver or terminator is connected.

Sub-Params:	pulldown_ref, pullup_ref, gnd_clamp_ref, power_clamp_ref, ext_ref

Usage Rules:	The [Buffer Rail Mapping] defines the connections between POWER and/or GND pins and buffer and/or terminator voltage supply references using signal_name. When [Buffer Rail Mapping] is present, then the signal_name field (second column of [Pin] records) shall indicate that all POWER and GND pins with the same signal_name are connected.

Each line shall contain either three, five or six entries. Use the reserved word NC for columns where a connection is not made.

The first column contains a pin name.  Each pin name shall match one of the pin names declared in the [Pin] section of the [Component] as a buffer or terminator.

The remaining columns correspond to the voltage supply references for the named pin.  Each [Model] supply reference is connected to a signal_name in the corresponding column.

The second column, pulldown_ref, designates the ground (GND) signal_name for the buffer or termination associated with that pin.  The signal_name under pulldown_ref is associated with the [Pulldown] I-V table for non-ECL [Model]s.  This is also the signal_name associated with the [GND Clamp] I-V table and the [Rgnd] model unless overridden by a label in the gnd_clamp_ref column.

The third column, pullup_ref, designates the power (POWER) signal_name for the buffer or termination.  The signal_name under pullup_ref is associated with the [Pullup] table for non-ECL [Model]s (for ECL models, this bus is associated with the  [Pulldown] table).  This is also the signal_name associated with the [POWER Clamp] I-V table and the [Rpower] model unless overridden by a label in the power_clamp_ref column.

The fourth and fifth columns, gnd_clamp_ref and power_clamp_ref, contain entries, if needed, to specify additional ground signal_name and power signal_name connections for clamps. Finally, the sixth column, ext_ref, contains entries to specify external reference supply signal_name connections.

There shall be no entries for pins listed under the [Pin] keyword with model_name GND, POWER and NC.

If the [Buffer Rail Mapping] keyword is present, then the supply reference connections for every pin listed under the [Pin] keyword (except POWER, GND and NC pins) shall be given.

The column length limits are:

[Buffer Rail Mapping]		5 characters max

pulldown_ref			40 characters max

pullup_ref				40 characters max

gnd_clamp_ref			40 characters max

power_clamp_ref			40 characters max

ext_ref				40 characters max

Example:

[Buffer Rail Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref  ext_ref

|

1               VSS1       VCC1      | Signal pins and their associated

2               VSS2       VCC2      | ground, power and external

|                                    | reference connections

3               VSS1       VCC1         VSSCLAMP       VCCCLAMP

4               VSS2       VCC2         VSSCLAMP       VCCCLAMP

5               VSS2       VCC2         NC             VCCCLAMP      V_EXTREF1

6               VSS2       VCC2         NC             VCCCLAMP

7               VSS2       VCC2         NC             VCCCLAMP      V_EXTREF2

8               VSSCLAMP   VCCCLAMP  | Note that normal Input, Output and I/O

|                                      buffers will need only three columns

|                                    | Some possible clamping

|                                    | connections are shown above

|                                    | for illustration purposes

|

| The following [Pin] list corresponds to the [Buffer Rail Mapping] shown above.

|

[Pin] signal_name model_name R_pin L_pin C_pin

|

1     OUT1         output_buffer1      | Output buffers

2     OUT2         output_buffer2      |

3     IO3          io_buffer1          | Input/output buffers

4     IO4          io_buffer2          |

5     SPECIAL1     ref_buffer1         | Buffers with POWER CLAMP but no

6     SPECIAL2     io_buffer_term1     | GND CLAMP I-V tables; two use

7     SPECIAL3     ref_buffer2         | external reference voltages

8     IN1          input_buffer

11    VSS1          GND

12    VSS1          GND

13    VSS1          GND

21    VSS2          GND

22    VSS2          GND

23    VSS2          GND

31    VCC1          POWER

32    VCC1          POWER

33    VCC1          POWER

41    VCC2          POWER

42    VCC2          POWER

43    VCC2          POWER

51    VSSCLAMP      GND                | Power connections for clamps

52    VCCCLAMP      POWER              |

71    V_EXTREF1     POWER              | External reference voltage pins

72    V_EXTREF2     POWER              |

________________________________________________________

The following section should be appended to the end of the IBIS document.

12 RULES OF PRECEDENCE

The sections below detail the rules of precedence to be assumed by EDA tools and model makers where multiple keywords may support similar functions.

12.1 PACKAGES

The order of precedence for package model data to be used by EDA tools in simulation is defined below, in ascending order.  If a package data format at a numerically higher position on the list is available in an IBIS or related file, that data shall be used by the EDA tool for simulation; any data present in formats numerically lower on the list shall be ignored.

- [Component]/[Package]

- [Component]/[Pin]

- [Package Model] (including [Alternate Package Models] and [Define Package Model])

- [Interconnect Model Selector]