BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	Draft 4, May 28, 2014

ISSUE TITLE:   		Interconnect Modeling Using IBIS-ISS

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:	{date you sent the original document, for new BIRDs}

DATE REVISED:	{date(s) you sent any revisions to the document}

DATE ACCEPTED BY IBIS OPEN FORUM: {leave blank; for administrative use only}

STATEMENT OF THE ISSUE:

This BIRD enhances IBIS interconnect models to supterminal Broadband and Coupled package and on-die interconnect using IBIS-ISS and Touchstone models.

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

Definitions:

IBIS Interconnect modeling makes several assumptions:

- Interconnect Models can either be IBIS-ISS subckts or Touchstone Files

- If two points are “Connected” then there is either a low resistance DC electrical path between the two points, or a small insertion loss at Nyquist frequency between the two points.

- For each I/O Pin, there is a Die Pad and Buffer I/O that are “Connected”.

- For each POWER or GND Signal_name, all pins, die pads and buffer supply terminals that use that Signal_name are “Connected”

- The Terminals (or Terminals) of Interconnect Models are Pins, Die Pads, Buffer I/O or Buffer supply terminals.

- An Interconnect Model may represent a single connection between Pins and Buffers, Pins and Die Pads, or Die Pads and Buffers. An Interconnect Model may also represent multiple connections between Pins and Buffers, Pins and Die Pads, or Die Pads and Buffers.

ANY OTHER BACKGROUND INFORMATION:

{These documents will be archived, so use this section to add any detail that is not part of the section above or the changed text itself , but should not be lost.}

Keyword:	[Interconnect Model Selector]

Required:	No

Description:	Used to pick an interconnect model for this component.

Usage Rules:	Interconnet Models are IBIS-ISS subckts or Toucshstone files that are interconnect models between the Pins, Die Pads and Buffers of a Component.

A component may have none, one or more [Interconnect Model]. If there are any [Interconnect Model]s, they must be listed in this section.

The section under the [Interconnect Model Selector] keyword must have two fields.  The fields must be separated by at least one white space.  The first field lists the [Interconnect Model] name (up to 40 characters long).  The second field is the name of the file containing the [Interconnect Model]. If the [Interconnect Model] is in this IBIS file, then the second field must be “*”.

The first entry under the [Interconnect Model Selector] keyword shall be considered the default by the EDA tool.

Example:

[Interconnect Model Selector]

QS-SMT-cer-8-pin-pkgs_iss *

QS-SMT-cer-8-pin-pkgs_sNp QS-SMT-cer-8-pin-pkgs_sNp.ipkg

[End Interconnect Model Selector]

Keyword:	[Interconnect Model]

Required:	No

Description:	Marks the beginning of an interconnect model description.

Usage Rules:	The length of the package model name must not exceed 40 characters in length.  Blank characters are not allowed.

Example:

[Interconnect Model]     QS-SMT-cer-8-pin-pkgs_iss

Keyword:	[Manufacturer] Allow or Require

Keyword:	[Description] Allow or Require

Same requirements as in IBIS if separate file.

Keyword:	[Begin Interconnect Model] <Interconnect Model Name>

Subparameter: Source <IBIS-ISS | Touchstone>

Subparameter: File  <file name>  {<file name> <file name>}

Subparameter: Subckt  <subckt name>  {< subckt name> < subckt name>}

Subparameter: Parameter <name>  <param value>  {<param value > <param value >}

Subparameter: Unused_Terminal_Termination <resistance>

Subparameter: Number_of_Terminals <# terminals>         |  Made into a Subparameter

Subparameter:  Terminal  <Field 1> <Field 2> <Field 3> <Field 4> {<Field 5> <Field 6> <Field 7>}

Subparameter:  Terminals  <Terminal 1> < Terminal 2> < Terminal 3> < Terminal 4> …

Keyword: [End Interconnect Model]

Keyword: [End Interconnect Model]

Required: Yes, to end the [Interconnect Model] keyword

Description: Indicates the end of the interconnect model data.

Other Notes: In between the [Interconnect Model] and [End Interconnect Model] keywords is the package model data itself. The data is any number of interfaces to either IBIS-ISS models or Touchstone files.

Example:

[End Interconnect Model]

We need a careful discussion on how Pin Mapping is used in conjunction with Terminals that have Signal_name.

We need a carefull discussion on when package models are Pre-Layout only.

We need a carefull discussion on precedence rules if more than one model can be used to represent interconnect.

Terminal/Terminal/Node

Reference Node in Definition of Touchstone Data

If Touchstone then # terminals =N+1 for an sNp, and reference will always be last terminal.

S2p would have terminals 1 2 Ref

Interaction with Circuit Call and External Circuit?

Interaction with Define Package Model, or are they mutually exclusive.

Precedence Rules?

Subparameter: Source <IBIS-ISS | Touchstone>

Required: Yes for each [Begin Interconnect Model]/[End Interconnect Model] group

Description: Indicates if the model is an IBIS-ISS subckt or a Touchstone file.

Other Notes:

Example:

Source IBIS-ISS

Subparameter: File  Typ|Min|Max <file name>

Required: Yes for each [Begin Model]/[End Model] group

Description: Defines the file(s) containing the model.

Other Notes: The Files must be either IBIS-ISS files or Touchstone files.

Example:

File my_file.iss

File my_file_typ.iss my_file_min.iss my_file_max.iss

Subparameter: Subckt Typ|Min|Max <subckt name>

Required: Yes if Source IBIS-ISS.

Description: Defines the subckt in the File.

Other Notes:

Example:

Subckt my_subckt

Subckt my_subckt_typ my_subckt_slow my_subckt_fast

Subparameter: Parameter <name> Typ|Min|Max <param value>

Required: No, but legal only if Language is IBIS-ISS.

Description: Defines the parameters that are to be passed into an instance of the IBIS-ISS subckt. <name> is the name of the parameter. String parameters shall be enclosed in “’”.

Other Notes: Number shall use IBIS number notation. The EDA tool is repsponsible for converint numbers using IBIS scale factors to sumbers using IBIS-ISS scale factors when instantiating subckts.

Parameters are not passed into a Touchstone file; however, there are two optional reserved parameters that are used in conjunction with Language Touchstone. They are FBASE and FMAX. They must have one value. See the IBIS-ISS manual to understand how FBASE and FMAX should be used in conjunction with Touchstone files.

Examples:

Parameter Length 11. 12. 9.

Parameter Tstonefile ‘abc.s2p’

Keyword: Unused_Terminal_Termination <resistance>

Required:  No

Description: Defines the termination that is to be applied to the Terminals of a subckt or Touchstone file that are not being used in each [Begin Interconnect Model]/[End Interconnect Model] group.

Other Notes: If this subparameter is defined the EDA should connect the unused Terminals to GND through a <resistance> ohm resistor.

If this parameter is not defined and if Language is IBIS-ISS, then the EDA tool should connect the unused Terminals to GND through a 1Meg ohm resistor. If Language is Touchstone, then the EDA tool should connect the unused Terminals to GND through a resistor with the Touchstone File reference resistance of the Terminal.

Example:

[Unused_Terminal_Termination] 50

Keyword: Number_of_Terminals <# terminals>         |  Made into a Subparameter

Required:  Yes, for each [Begin Interconnect Model]/[End Interconnect Model] group

Description: The number of terminals (terminals) of the IBIS-ISS subckt or Touchstone file.

Other Notes:

Example:

Number_of_Terminals 2

Subparameter:  Terminal <Field 1> <Field 2> <Field 3> <Field 4> {<Field 5> <Field 6> <Field 7>}

Required: An Interconnect Model must either have Terminal subparameter records, or one or more Terminals subparameter records for each [Begin Interconnect Model]/[End Interconnect Model] group. The subparameters Terminal and Terminals are mutually exclusive.

Description: Each Terminal record contains information on a terminal (or terminal) of an IBIS-ISS subckt (or Touchstone file).

The logic here requires VERY careful review!

<Field 1>  Terminal Number. This must be an integer number greater or equal to 1 and less than or equal to the number of terminals ([Number of Terminals], of the IBIS-ISS subckt (or Toucshtone file). Two [Terminal] records may not have the same Terminal Number. If a Terminal Number does not exist in any of the [Terminal] records then the terminal is unused, and should be terminated according to the Unused_Terminal_Termination_ Rules.

<Field 2> Shall be either Pin Pad or Buffer. This describes if the Terminal is at a Pin, Die Pad, or at the Buffer.

<Field 3> Shall be either Pin_name, Signal_name, Model_name or Default.

<Field 4>  If <Field 3> is Pin_name, Signal_name or Model_name then <Field 4> shall be either a legal Pin_name, Signal_name or  Model_name respectively. If <Field 3> is Default then <Field 4> shall be NA.

<Field 5> If the connection on this terminal is to a Buffer signal terminal then <Field 3> is Model_name or Default <Field 5> shall be either Diff_pos, Diff_neg or SE, otherwise it shall be NA. If the connection on this terminal is to a Buffer supply terminal, then <Field 5> shall be either Pullup_Reference, Pulldown_Reference , Power_Clamp_Reference , Ground_Clamp_Reference  or External_Reference.

<Field 6> Is the channel number. A channel is either a single connection or two connections that form a differential pair.  If there is only one channel in the model then <Field 6> may be NA.

<Field 7> Shall contain Aggressor or NA. If NA, then this channel accurately represents that channel through path and crosstalk from all of the other channels. If Aggressor than the channel only accurately represents the crosstalk to all of the NA (Victim) channels.

Other Notes:

Fields at then end of a Terminal record that are NA are optional

If <Field 2> is Buffer, and the terminal connects to ths

Example:

Terminal 1 Pin    Pin_name    M8    NA       NA NA

Terminal 1 Pin    Pin_name    M8

Terminal 1 Pad    Pin_name    M8

Terminal 1 Buffer Pin_name    M8

Terminal 1 Buffer Pin_name    M8    Pullup_Reference

Terminal 1 Pin    Model_name  DQ

Terminal 1 Pin    Model_name  DQS   Diff_pos

Terminal 1 Pin    Default     NA    SE

Terminal 1 Pin    Model name  DQ    SE

Terminal 1 Buffer Pin_name    M8    NA       2

Terminal 1 Buffer Pin_name    M8    NA       3  Aggressor

Terminal 1 Buffer Signal_name VDDQ

Subparameter:  Terminals <Terminal 1> < Terminal 2> < Terminal 3> < Terminal 4> …

Required: An Interconnect Model must either have Terminal subparameter records, or one or more Terminals subparameter records for each [Begin Interconnect Model]/[End Interconnect Model] group. The subparameters Terminal and Terminals are mutually exclusive.

Description:     Defines the node names associated with the Terminals (Terminals) of an IBIS-ISS subckt or a Toushtone file

Usage Rules:   <terminal> names correspond to either a Component Pin, Component Die Pad, or Buffer Terminal. The order of the <terminals> shall be the order of the Terminals of the IBIS-ISS subckt or Touchstone file. There may be more than one Terminals keyword in a [Begin Interconnect Model]/[End Interconnect Model] group. If there are more than one Terminals record the subsequent Terminals records after the first Terminals record shall be considered continuation lines, and the <terminal>s shall be concatenated.

Note that the <terminal> names are used to associate nodes in the package model subckt instance, and are not the names of the nodes within the subckt itself. Unconnected terminals shall be named “NC” The following are the terminal naming rules for connected terminals. The Terminals subparameter is not as general as the Terminal subparameter. It does not allow associating terminals with Victim/Aggressor, Model Name or Default.

Signal (I/O) Terminal Naming Rules

Pin

Pin:<Pin_name>

Die Pad

Pad:<Pin_name>

Buffer (I/O)

Buf:<Pin_name>

Supply (POWER and GND) Terminal Naming Rules

Pin

Pin:<Pin_name>

Pin_SIG:<Signal_name>

Die Pad

Pad:<die_supply_pad_name>

Pin_SIG:<Signal_name>

Buffer

Buf_PUR:<Pin_name>		| Pullup Reference

Buf_PDR:<Pin_name>		| Pulldown Reference

Buf_PCR:<Pin_name>		| Power Clamp Reference

Buf_GCR:<Pin_name>	| Power Clamp Reference

Buf_EXTREF:<Pin_name>>	| External reference voltage terminal

Buf_SIG.<Signal_name>	| Supply (Power/Ground) Signal Name

Examples:

Terminals Pin:A7 Buf:A7

Terminals Pin:A7 Pad:A7

Terminals Pad:A7 Buf:A7

Terminals Pin_SIG:VDD Buf_SIG:VDD

Terminals Pin_SIG:VDD Pin_SIG:VSS Buf_PUR:A7 Buf_PDR:A7

Keyword:	[Die Supply Pads]

Required:	No

Description:	This begins a section in [Component] that contains one line of data for die pads supply nodes. IBIS assumes that for I/O pins (pins that have a Model_name that is not POWER, GND or NC), there is a one to one correspondence between a Pin, Die Pad and Buffer I/O. There are no such assumptions for POWER and GND pins. A POWER or GND Signal_name may have a different number of Pin nodes, die pad nodes and buffer nodes. If the model maker chooses to make separate package and on-die power distribution networks (PDN), then he must supply a list of nodes (and their associated Signal_name) that can be used to mate the package and on-die PDN models.

Sub-Params:	?

Usage Rules:	 TBD

Other Notes:	The data in this section consists of  a list of die pad node names and their corresponding Signal_names that can be used to mate package and on-die PDN networks.

Example:

[Die Supply Pads]

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

Keyword: [End Die Supply Pads]

Required: Yes.

Description: Indicates the end of the [Die Supply Pads] data.

Other Notes:

Example:

[End Die Supply Pads]

Examples

[Define Package Model]

[ISS Model Data]

[Begin ISS Model] IOA3

Language Touchstone

File Value ioA3.s2p

Number_of_Terminals 2

Terminal 1  Pin    Pin_name A3

Terminal 2  Buffer Pin_name A3

Terminals Pin:A3 Buf:A3

Nodes Pin:A3 Buf:A3

[End ISS Model]

[Begin ISS Model] IOA7

| This model uses I/O pin A7

Language Touchstone

File Value ioA7.s2p

Number_of_Terminals 2

Terminals  Pin.A7  Buf.A7

[End ISS Model]

[Begin ISS Model] IOB3C3

Language Touchstone

File Value ioB3C3.s4p

Number_of_Terminals 4

Terminal 1  Pin    Pin_name B3

Terminal 2  Buffer Pin_name B3

Terminal 3  Pin    Pin_name C3

Terminal 4  Buffer Pin_name C3

[End ISS Model]

[Begin ISS Model] IOA3

Language IBIS_ISS

File Value io.iss

Subckt io

Parameter Length Value 10.   | 10mm

Number_of_Terminals 2

Terminal 1  Pin    Pin_name A3

Terminal 2  Buffer Pin_name A3

[End ISS Model]

[Begin ISS Model] DQS

Language Touchstone

File Value DQS.s4p

Number_of_Terminals 4

Terminal 1  Pin    Model_name DQS Diff_pos

Terminal 2  Buffer Model_name DQS Diff_pos

Terminal 3  Pin    Model_name DQS Diff_neg

Terminal 4  Buffer Model_name DQS Diff_neg

[End ISS Model]

[Begin ISS Model] VDDQ

Language IBIS_ISS

File Value vddq.iss

Subckt vddq

Number_of_Terminals 2

Terminal 1  Pin    Signal_name VDDQ

Terminal 2  Buffer Signal_name VDDQ

[End ISS Model]

[Begin ISS Model] VDDQ_A3

Language IBIS_ISS

File Value vddq_a3.iss

Subckt vddq_A3

Number_of_Terminals 2

Terminal 1  Pin    Signal_name VDDQ

Terminal 2  Buffer Pin_name    A3   Pullup_Reference

[End ISS Model]

[Begin ISS Model] IOA3

Language Touchstone

File Value ioA3.s10p

Number_of_Terminals 10

Terminal 1  Pin     Pin_name   A3

Terminal 2  Buffer  Pin_name   A3

Terminal 3  Pin     Model_name DQ  NA       1 Aggressor

Terminal 4  Buffer  Model_name DQ  NA       1 Aggressor

Terminal 5  Pin     Model_name DQ  NA       2 Aggressor

Terminal 6  Buffer  Model_name DQ  NA       2 Aggressor

Terminal 7  Pin     Model_name DQS Diff_pos 3 Aggressor

Terminal 8  Buffer  Model_name DQS Diff_pos 3 Aggressor

Terminal 9  Pin     Model_name DQS Diff_neg 3 Aggressor

Terminal 10 Buffer  Model_name DQS Diff_neg 3 Aggressor

[End ISS Model]

[End ISS Model Data]

[End Package Model]