BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	Draft 8 August, , 2014

ISSUE TITLE:   		Interconnect Modeling Using IBIS-ISS

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:	{date you sent the original document, for new BIRDs}

DATE REVISED:	{date(s) you sent any revisions to the document}

DATE ACCEPTED BY IBIS OPEN FORUM: {leave blank; for administrative use only}

STATEMENT OF THE ISSUE:

This BIRD enhances IBIS interconnect models to supterminal Broadband and Coupled package and on-die interconnect using IBIS-ISS and Touchstone models.

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

Definitions:

IBIS Interconnect modeling makes several assumptions:

- Interconnect Models can either be IBIS-ISS subckts or Touchstone Files

- If two points are “Connected” then there is either a low resistance DC electrical path between the two points, or a small insertion loss at Nyquist frequency between the two points.

- For each I/O Pin, there is a Die Pad and Buffer I/O that are “Connected”.

- For each POWER or GND Signal_name, all pins, die pads and buffer supply terminals that use that Signal_name are “Connected”

- The Terminals (or Terminals) of Interconnect Models are Pins, Die Pads, Buffer I/O or Buffer supply terminals.

- An Interconnect Model may represent a single connection between Pins and Buffers, Pins and Die Pads, or Die Pads and Buffers. An Interconnect Model may also represent multiple connections between Pins and Buffers, Pins and Die Pads, or Die Pads and Buffers.

ANY OTHER BACKGROUND INFORMATION:

{These documents will be archived, so use this section to add any detail that is not part of the section above or the changed text itself , but should not be lost.}

Keyword:	[Interconnect Model Selector]

Required:	No

Description:	Used to pick an interconnect model for this component.

Usage Rules:	Interconnet Models are IBIS-ISS subckts or Toucshstone files that are interconnect models between the Pins, Die Pads and Buffers of a Component.

A component may have none, one or more [Interconnect Model]. If there are any [Interconnect Model]s, they must be listed in this section.

The section under the [Interconnect Model Selector] keyword must have two fields.  The fields must be separated by at least one white space.  The first field lists the [Interconnect Model] name (up to 40 characters long).  The second field is the name of the file containing the [Interconnect Model]. If the [Interconnect Model] is in this IBIS file, then the second field must be “*”.

The first entry under the [Interconnect Model Selector] keyword shall be considered the default by the EDA tool.

Example:

[Interconnect Model Selector]

QS-SMT-cer-8-pin-pkgs_iss *

QS-SMT-cer-8-pin-pkgs_sNp QS-SMT-cer-8-pin-pkgs_sNp.ipkg

[End Interconnect Model Selector]

Keyword:	[Interconnect Model]

Required:	No

Description:	Marks the beginning of an interconnect model description.

Usage Rules:	The length of the package model name must not exceed 40 characters in length.  Blank characters are not allowed.

Example:

[Interconnect Model]     QS-SMT-cer-8-pin-pkgs_iss

Keyword:	[Manufacturer] Allow or Require

Keyword:	[Description] Allow or Require

Same requirements as in IBIS if separate file.

Keyword:	[Begin Interconnect Model] <Interconnect Model Name>

Subparameter: Source <IBIS-ISS | Touchstone>

Subparameter: File  <file name>  {<file name> <file name>}

Subparameter: Subckt  <subckt name>  {< subckt name> < subckt name>}

Subparameter: Parameter <name>  <param value>  {<param value > <param value >}

Subparameter: Unused_Terminal_Termination <resistance>

Subparameter: Number_of_Terminals <# terminals>         |  Made into a Subparameter

Subparameter:  Terminal  <Field 1> <Field 2> <Field 3> <Field 4> {<Field 5> <Field 6> <Field 7>}

Subparameter:  Terminals  <Terminal 1> < Terminal 2> < Terminal 3> < Terminal 4> …

Keyword: [End Interconnect Model]

Keyword: [End Interconnect Model]

Required: Yes, to end the [Interconnect Model] keyword

Description: Indicates the end of the interconnect model data.

Other Notes: In between the [Interconnect Model] and [End Interconnect Model] keywords is the package model data itself. The data is any number of interfaces to either IBIS-ISS models or Touchstone files.

Example:

[End Interconnect Model]

We need a careful discussion on how Pin Mapping is used in conjunction with Terminals that have Signal_name.

We need a carefull discussion on when package models are Pre-Layout only.

We need a carefull discussion on precedence rules if more than one model can be used to represent interconnect.

Terminal/Terminal/Node

Reference Node in Definition of Touchstone Data

If Touchstone then # terminals =N+1 for an sNp, and reference will always be last terminal.

S2p would have terminals 1 2 Ref

Interaction with Circuit Call and External Circuit?

Interaction with Define Package Model, or are they mutually exclusive.

Precedence Rules?

Subparameter: Source <IBIS-ISS | Touchstone>

Required: Yes for each [Begin Interconnect Model]/[End Interconnect Model] group

Description: Indicates if the model is an IBIS-ISS subckt or a Touchstone file.

Other Notes:

Example:

Source IBIS-ISS

Subparameter: File  Typ|Min|Max <file name>

Required: Yes for each [Begin Model]/[End Model] group

Description: Defines the file(s) containing the model.

Other Notes: The Files must be either IBIS-ISS files or Touchstone files.

Example:

File my_file.iss

File my_file_typ.iss my_file_min.iss my_file_max.iss

Subparameter: Subckt Typ|Min|Max <subckt name>

Required: Yes if Source IBIS-ISS.

Description: Defines the subckt in the File.

Other Notes:

Example:

Subckt my_subckt

Subckt my_subckt_typ my_subckt_slow my_subckt_fast

Subparameter: Parameter <name> Typ|Min|Max <param value>

Required: No, but legal only if Language is IBIS-ISS.

Description: Defines the parameters that are to be passed into an instance of the IBIS-ISS subckt. <name> is the name of the parameter. String parameters shall be enclosed in “’”.

Other Notes: Number shall use IBIS number notation. The EDA tool is repsponsible for converint numbers using IBIS scale factors to sumbers using IBIS-ISS scale factors when instantiating subckts.

Parameters are not passed into a Touchstone file; however, there are two optional reserved parameters that are used in conjunction with Language Touchstone. They are FBASE and FMAX. They must have one value. See the IBIS-ISS manual to understand how FBASE and FMAX should be used in conjunction with Touchstone files.

Examples:

Parameter Length 11. 12. 9.

Parameter Tstonefile ‘abc.s2p’

Keyword: Unused_Terminal_Termination <resistance>

Required:  No

Description: Defines the termination that is to be applied to the Terminals of a subckt or Touchstone file that are not being used in each [Begin Interconnect Model]/[End Interconnect Model] group.

Other Notes: If this subparameter is defined the EDA should connect the unused Terminals to GND through a <resistance> ohm resistor.

If this parameter is not defined and if Language is IBIS-ISS, then the EDA tool should connect the unused Terminals to GND through a 1Meg ohm resistor. If Language is Touchstone, then the EDA tool should connect the unused Terminals to GND through a resistor with the Touchstone File reference resistance of the Terminal.

Example:

[Unused_Terminal_Termination] 50

Keyword: Number_of_Terminals <# terminals>         |  Made into a Subparameter

Required:  Yes, for each [Begin Interconnect Model]/[End Interconnect Model] group

Description: The number of terminals (terminals) of the IBIS-ISS subckt or Touchstone file.

Other Notes:

Example:

Number_of_Terminals 2

Subparameter:  Terminal Terminal_number Location ID {Qualifiers}

Required: An Interconnect Model must have Terminal subparameter records for each [Begin Interconnect Model]/[End Interconnect Model] group.

Description: Each Terminal record contains information on a terminal of an IBIS-ISS subckt (or Touchstone file).

Terminal_number must be a positive integer number greater or equal to one and less than or equal to the number of terminals ([Number of Terminals], of the IBIS-ISS subckt (or Toucshtone file). Two Terminal records may not have the same Terminal_number. If a Terminal Number does not exist in any of the [Terminal] records then the terminal is unused, and should be terminated according to the Unused_Terminal_Termination_ Rules.

Location shall be Pin, Pad, Buf, Pin_Sig, Pad_Sig, Buf_Sig, Buf_PURef, Buf_PDRef, Buf_PCRef, Buf_GCRef or Buf_XRef.

- Pin indicates this terminal is at a specific pin, ID must be a Pin_name, Model_name or Default.

- Pad indicates this terminal is at a specific die pad, ID must be a Pin_name, Model_name or Default.

- Buf indicates this terminal is at a specific buffer model I/O or signal terminal, ID must be a Pin_name, Model_name or Default.

- Pin_Sig indicates that this terminal is connected to all pins that have Signal_name ID. ID must be a Signal_name on a Pin that has Model_name Power or GND. All pins that have Signal_name ID are considered shorted together at the pin side of the package model.

- Pad_Sig indicates that this terminal is connected to all die pads that have Signal_name ID. ID must be a Signal_name on a Pin that has Model_name Power or GND. All die pads that have Signal_name ID are considered shorted together at the die pad side of the package model.

- Buf_Sig indicates that this terminal is connected to all buffer model terminals Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference or External Reference that have a Signal_name ID. ID must be a Signal_name on a Pin that has Model_name Power or GND. All Buffer terminal nodes that have Signal_name ID are considered shorted together at the buffer side of the package model.

- Buf_PURef indicates this terminal is at a specific buffer model pullup reference, ID must be a Pin_name, Model_name or Default.

- Buf_PDRef indicates this terminal is at a specific buffer model pulldown reference, ID must be a Pin_name, Model_name or Default.

- Buf_PCRef indicates this terminal is at a specific buffer model power clamp reference, ID must be a Pin_name, Model_name or Default.

- Buf_GCRef indicates this terminal is at a specific buffer model ground clamp reference, ID must be a Pin_name, Model_name or Default.

- Buf_XRef indicates this terminal is at a specific buffer model external reference, ID must be a Pin_name, Model_name or Default.

ID shall be a Pin_name, Signal_name, Model_name or Default.

Qualifiers may have the values Aggressor, Model_name, Default, Inverting, Non-Inverting and Connection(n). Qualifiers are optional, there may be zero, one or several qualifiers on each Terminal record. Qualifiers may appear in any order.

- Aggressor, any Terminal may have the qualifier aggressor. It means that terminal does not have coupling from all aggressor sources, so can be treated as an aggressor and should not be treated as a victim.

- Model_name, means that the ID on this terminal is a Model_name

- Default, means that the ID on this terminal must be Default.

- A terminal cannot have both Default and Model_name qualifiers.

- If a terminal either qualifier Default or Model_name then the terminal is considered a “Pre-Layout” terminal.

- If a “Pre-Layout” terminal is connected to a differential model, then the terminal must have either the Inverting or Non-Inverting qualifier.

- All terminals that have the same Connection(n) (where n is a positive integer) are electrically connected. A single ended connection will have two terminals with Connection(n). A differential connection will have four terminals with Connection(n).` Connection(n) qualifiers are required if there are two or more Pre-Layout connections.

- Special differential rules for Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference.

  - There can be only one terminal for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference on a true differential [External Model]. These can be referenced by either the Non-Inverting or Inverting Pin_name.

  - There may be only one terminal for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference for each side of a legacy differential model that consists of two independent single ended models. These can be referenced by either the Non-Inverting or Inverting Pin_name.

  - There may be two terminals for each Pullup Reference, Power Reference, Power Clamp Reference, Ground Clamp Reference and External Reference for each side of a legacy differential model that consists of two independent single ended models.

Other Notes:

More than one [Interconnect Model] may be available for a specific simulation. The EDA tool may choose any of the available models but, in general, should choose a model preferring a model that matches by Pin_name, then Model_name and finally Default.

An Interconnect Model will Source Touchstone will reference a Touchstone File with N Ports. N is either determined from the N in the .sNp file name extension for a Touchstone I file or from the [Number of Ports] record in a Touchstone II file. The [Number of Terminals] in the Interconnect Model shall either be N+1. The Terminal Rules is described below:

- [

  - The EDA tool shall use the Pin_name or Signal_name specified in the Terminal “N+1” record as the reference node for each of the N ports.

  - Terminal/Port Mapping

    - Terminal              Port

    - 1                              1

    - 2                              2

    - …

    - N                             N

  - If a Port is not connected, then it shall be terminated with a resistor to the node on Terminal N+1. The resistance shall be the Port Reference Impedance.

  - If Terminal N+1 is not connected, then the Port Reference shall be Global Ground.

Examples:

IBIS File

[Pin] signal_name model_name      R_pin   L_pin   C_pin

A1    DQ1         DQ

A2    DQ2         DQ

A3    DQ3         DQ

D1    DQS         DQS

D2    DQS         DQS

P1    VDD         POWER

P2    VDD         POWER

P3    VDD         POWER

P4    VDD         POWER

P5    VDD         POWER

G1    VSS         GND

G2    VSS         GND

G3    VSS         GND

G4    VSS         GND

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

[Diff Pin]  inv_pin  vdiff  tdelay_typ tdelay_min tdelay_max

D1          D2       NA     NA         NA         NA

[Die Supply Pads]

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

[Pin Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref ext_ref

A1            VSS          VDD        NC            NC              NC

A2            VSS          VDD        NC            NC              NC

A3            VSS          VDD        NC            NC              NC

D1            VSS          VDD        NC            NC              NC

D2            VSS          VDD        NC            NC              NC

- Single DQ (A1)

  - Terminal 1 Pin A1

  - Terminal 2 Buf A1

- Single DQS | There is a [Diff Pin] record “D1 D2 …”

  - Terminal 1 Pin D1

  - Terminal 2 Pin D2

  - Terminal 3 Buf D1

  - Terminal 4 Buf D2

- One DQ (A2) victim, two DQ (A1 and A3) aggressors

  - Terminal 1 Pin A1 Aggressor

  - Terminal 2 Buf A1 Aggressor

  - Terminal 3 Pin A2

  - Terminal 4 Buf A2

  - Terminal 5 Pin A3 Aggressor

  - Terminal 6 Buf A3 Aggressor

- Single DQ (A1) Pin to Die Pad

  - Terminal 1 Pin A1

  - Terminal 2 Pad A1

- Single ended model that can be used for all I/O pins

  - Terminal 1 Pin Default Default

  - Terminal 2 Buf Default Default

- Madel that can connect all Pins with Signal_name VDD to all Buffer supply terminals that are connected to Signal_name VDD as described in Pin_mapping. All Pins with Signal_name VDD are shorted together. All Buffer supply terminals that are connected to Signal_name VDD are shorted together

  - Terminal 1 Pin_Sig VDD

  - Terminal 2 Buf_Sig VDD

- VDD: Pins connected to board “bed spring” model, all buffers connected to VDD shorted

  - Terminal 1 Pin P1

  - Terminal 2 Pin P2

  - Terminal 3 Pin P3

  - Terminal 4 Pin P4

  - Terminal 5 Pin P5

  - Terminal 6 Buf_Sig VDD

- VDD: Interconnect between VDD Pins and individual buffer Pullup Reference.

  - Terminal 1 Pin P1

  - Terminal 2 Pin P2

  - Terminal 3 Pin P3

  - Terminal 4 Pin P4

  - Terminal 5 Pin P5

  - Terminal 6 Buf_PURef A1

  - Terminal 7 Buf_PURef A2

  - Terminal 8 Buf_PURef A3

  - Terminal 9 Buf_PURef D1

- VDD: Interconnect between VDD Pins and die VDD pads.

  - Terminal 1 Pin P1

  - Terminal 2 Pin P2

  - Terminal 3 Pin P3

  - Terminal 4 Pin P4

  - Terminal 5 Pin P5

  - Terminal 6 Pad VDD1

  - Terminal 7 Pad VDD2

  - Terminal 8 Pad VDD3

- VDD: Interconnect between die VDD pads and individual buffer Pullup Reference.

  - Terminal 1 Pad VDD1

  - Terminal 2 Pad VDD2

  - Terminal 3 Pad VDD3

  - Terminal 4 Buf_PURef A1

  - Terminal 5 Buf_PURef A2

  - Terminal 6 Buf_PURef A3

  - Terminal 7 Buf_PURef D1

- Single DQ

  - Terminal 1 Pin DQ Model_name

  - Terminal 2 Buf DQ Model_name

- Single DQS

  - Terminal 1 Pin DQS Model_name Non-Inverting

  - Terminal 2 Pin DQS Model_name Inverting

  - Terminal 3 Buf DQS Model_name Non-Inverting

  - Terminal 4 Buf DQS Model_name Inverting

- Single DQ victim, two DQ aggressors

  - Terminal 1 Pin DQ Model_name Aggressor  Connection(1)

  - Terminal 2 Buf DQ Model_name Aggressor  Connection(1)

  - Terminal 3 Pin DQ Model_name Connection(2)

  - Terminal 4 Buf DQ Model_name Connection(2)

  - Terminal 5 Pin DQ Model_name Aggressor Connection(3)

  - Terminal 6 Buf DQ Model_name Aggressor Connection(3)

- One DQ victim, two DQ aggressors, one DQS aggressor

  - Terminal 1 Pin DQ Model_name Aggressor  Connection(1)

  - Terminal 2 Buf DQ Model_name Aggressor  Connection(1)

  - Terminal 3 Pin A2

  - Terminal 4 Buf A2

  - Terminal 5 Pin DQ Model_name Aggressor Connection(2)

  - Terminal 6 Buf DQ Model_name Aggressor Connection(2)

  - Terminal 7 Pin DQS Model_name Aggressor Connection(3) Non-Inverting

  - Terminal 8 Buf DQS Model_name Aggressor Connection(3) Inverting

  - Terminal 9 Pin DQS Model_name Aggressor Connection(3) Non-Inverting

  - Terminal 10 Buf DQS Model_name Aggressor Connection(3) Inverting

- One single ended victim, two single ended aggressors, one differential aggressor

  - Terminal 1 Pin Default Default Aggressor  Connection(1)

  - Terminal 2 Buf Default Default Aggressor  Connection(1)

  - Terminal 3 Pin Default Default

  - Terminal 4 Buf Default Default

  - Terminal 5 Pin Default Default Aggressor  Connection(2)

  - Terminal 6 Buf Default Default Aggressor  Connection(2)

  - Terminal 7 Pin Default Default Aggressor Connection(3) Non-Inverting

  - Terminal 8 Buf Default Default Aggressor Connection(3) Inverting

  - Terminal 9 Pin Default Default Aggressor Connection(3) Non-Inverting

  - Terminal 10 Buf Default Model_name Aggressor Connection(3) Inverting

Keyword:	[Die Supply Pads]

Required:	No

Description:	This begins a section in [Component] that contains one line of data for die pads supply nodes. IBIS assumes that for I/O pins (pins that have a Model_name that is not POWER, GND or NC), there is a one to one correspondence between a Pin, Die Pad and Buffer I/O. There are no such assumptions for POWER and GND pins. A POWER or GND Signal_name may have a different number of Pin nodes, die pad nodes and buffer nodes. If the model maker chooses to make separate package and on-die power distribution networks (PDN), then he must supply a list of nodes (and their associated Signal_name) that can be used to mate the package and on-die PDN models.

Sub-Params:	?

Usage Rules:	 TBD

Other Notes:	The data in this section consists of  a list of die pad node names and their corresponding Signal_names that can be used to mate package and on-die PDN networks.

Example:

[Die Supply Pads]

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

Keyword: [End Die Supply Pads]

Required: Yes.

Description: Indicates the end of the [Die Supply Pads] data.

Other Notes:

Example:

[End Die Supply Pads]

Examples

[Define Package Model]

[ISS Model Data]

[Begin ISS Model] IOA3

Language Touchstone

File Value ioA3.s2p

Number_of_Terminals 2

Terminal 1  Pin    Pin_name A3

Terminal 2  Buffer Pin_name A3

Terminals Pin:A3 Buf:A3

Nodes Pin:A3 Buf:A3

[End ISS Model]

[Begin ISS Model] IOA7

| This model uses I/O pin A7

Language Touchstone

File Value ioA7.s2p

Number_of_Terminals 2

Terminals  Pin.A7  Buf.A7

[End ISS Model]

[Begin ISS Model] IOB3C3

Language Touchstone

File Value ioB3C3.s4p

Number_of_Terminals 4

Terminal 1  Pin    Pin_name B3

Terminal 2  Buffer Pin_name B3

Terminal 3  Pin    Pin_name C3

Terminal 4  Buffer Pin_name C3

[End ISS Model]

[Begin ISS Model] IOA3

Language IBIS_ISS

File Value io.iss

Subckt io

Parameter Length Value 10.   | 10mm

Number_of_Terminals 2

Terminal 1  Pin    Pin_name A3

Terminal 2  Buffer Pin_name A3

[End ISS Model]

[Begin ISS Model] DQS

Language Touchstone

File Value DQS.s4p

Number_of_Terminals 4

Terminal 1  Pin    Model_name DQS Diff_pos

Terminal 2  Buffer Model_name DQS Diff_pos

Terminal 3  Pin    Model_name DQS Diff_neg

Terminal 4  Buffer Model_name DQS Diff_neg

[End ISS Model]

[Begin ISS Model] VDDQ

Language IBIS_ISS

File Value vddq.iss

Subckt vddq

Number_of_Terminals 2

Terminal 1  Pin    Signal_name VDDQ

Terminal 2  Buffer Signal_name VDDQ

[End ISS Model]

[Begin ISS Model] VDDQ_A3

Language IBIS_ISS

File Value vddq_a3.iss

Subckt vddq_A3

Number_of_Terminals 2

Terminal 1  Pin    Signal_name VDDQ

Terminal 2  Buffer Pin_name    A3   Pullup_Reference

[End ISS Model]

[Begin ISS Model] IOA3

Language Touchstone

File Value ioA3.s10p

Number_of_Terminals 10

Terminal 1  Pin     Pin_name   A3

Terminal 2  Buffer  Pin_name   A3

Terminal 3  Pin     Model_name DQ  NA       1 Aggressor

Terminal 4  Buffer  Model_name DQ  NA       1 Aggressor

Terminal 5  Pin     Model_name DQ  NA       2 Aggressor

Terminal 6  Buffer  Model_name DQ  NA       2 Aggressor

Terminal 7  Pin     Model_name DQS Diff_pos 3 Aggressor

Terminal 8  Buffer  Model_name DQS Diff_pos 3 Aggressor

Terminal 9  Pin     Model_name DQS Diff_neg 3 Aggressor

Terminal 10 Buffer  Model_name DQS Diff_neg 3 Aggressor

[End ISS Model]

[End ISS Model Data]

[End Package Model]