The Terminals of an Interconnect Model may be located at Pins and Pads, Pins and Buffers, or Pads and Buffers. A single Interconnect Model shall not have Terminals at Pins, Pads and Buffers simultaneously.

Example:

[Pin] signal_name model_name      R_pin   L_pin   C_pin

A1    DQ1         DQ

A2    DQ2         DQ

A3    DQ3         DQ

D1    DQS+        DQS

D2    DQS-        DQS

P1    VDD         POWER

P2    VDD         POWER

P3    VDD         POWER

P4    VDD         POWER

P5    VDD         POWER

G1    VSS         GND

G2    VSS         GND

G3    VSS         GND

G4    VSS         GND

| Full Package/Die Model Complex Power Distribution

[Begin Interconnect Model]

Number_of_Terminals 29

1  Pin_I/O     Pin_name A1  |  DQ1         DQ

2  Pin_I/O     Pin_name A2  |  DQ2         DQ

3  Pin_I/O     Pin_name A3  |  DQ3         DQ

4  Pin_I/O     Pin_name D1  |  DQS+        DQS

5  Pin_I/O     Pin_name D2  |  DQS-        DQS

6  Pin_I/O     Pin_name P1  |  VDD         POWER

7  Pin_I/O     Pin_name P2  |  VDD         POWER

8  Pin_I/O     Pin_name P3  |  VDD         POWER

9  Pin_I/O     Pin_name P4  |  VDD         POWER

10 Pin_Rail    Pin_name P5  |  VDD         POWER

11 Pin_Rail    Pin_name G1  |  VSS         GND

12 Pin_Rail    Pin_name G2  |  VSS         GND

13 Pin_Rail    Pin_name G3  |  VSS         GND

14 Pin_Rail    Pin_name G4  |  VSS         GND

15 Buffer_I/O  Pin_name A1  |  DQ1         DQ

16 Buffer_I/O  Pin_name A2  |  DQ2         DQ

17 Buffer_I/O  Pin_name A3  |  DQ3         DQ

18 Buffer_I/O  Pin_name D1  |  DQS+        DQS

19 Buffer_I/O  Pin_name D2  |  DQS-        DQS

20 PUref       Pin_name A1  |  DQ1         DQ

21 PUref       Pin_name A2  |  DQ2         DQ

22 PUref       Pin_name A3  |  DQ3         DQ

23 PUref       Pin_name D1  |  DQS+        DQS

24 PUref       Pin_name D2  |  DQS-        DQS

25 PDref       Pin_name A1  |  DQ1         DQ

26 PDref       Pin_name A2  |  DQ2         DQ

27 PDref       Pin_name A3  |  DQ3         DQ

28 PDref       Pin_name D1  |  DQS+        DQS

29 PDref       Pin_name D1  |  DQS+        DQS

[End Interconnect Model]

Keyword: 	[End Interconnect Model]

Required: 	Yes, for each instance of the [Begin Interconnect Model] keyword

Description: 	Indicates the end of the Interconnect Model data.

Other Notes:	Between the [Begin Interconnect Model] and [End Interconnect Model] keywords is the package model data itself. The data describes any number of interfaces to either IBIS-ISS models or Touchstone files.