BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	Draft 2, April 23, 2014

ISSUE TITLE:   		Package Modeling Using IBIS-ISS

REQUESTOR:     	Walter Katz, Signal Integrity Software, Inc.

DATE SUBMITTED:	{date you sent the original document, for new BIRDs}

DATE REVISED:	{date(s) you sent any revisions to the document}

DATE ACCEPTED BY IBIS OPEN FORUM: {leave blank; for administrative use only}

STATEMENT OF THE ISSUE:

This BIRD enhances IBIS Packaging models to support Broadband and Coupled package and on-die interconnect using IBIS-ISS and Touchstone models (ISS Models).

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

Definitions:

ISS

ISS package and on-die modeling makes several assumptions:

- ISS Models can either be IBIS-ISS subckts or Touchstone Files

- If two points are “Connected” then there is either a low resistance DC electrical path between the two points, or a small insertion loss at Nyquist frequency between the two points.

- For each I/O Pin, there is a Die Pad and Buffer I/O that are “Connected”.

- For each POWER or GND Signal_name, all pins, die pads and buffer supply terminals that use that Signal_name are “Connected”

- The Ports (or Terminals) of ISS Models are Pins, Die Pads, Buffer I/O or Buffer supply terminals.

- An ISS Model may represent a single connection between Pins and Buffers, Pins and Die Pads, or Die Pads and Buffers. An ISS Model may also represent multiple connections between Pins and Buffers, Pins and Die Pads, or Die Pads and Buffers.

ANY OTHER BACKGROUND INFORMATION:

{These documents will be archived, so use this section to add any detail that is not part of the section above or the changed text itself , but should not be lost.}

Keyword:	[ISS Model Data]

Required:	Sometimes.

Description:	The begins a section in [Define Package Model] that contains the interfaces to IBIS-ISS subckts and Touchstone Files

Sub-Params:	Lots, TBD

Usage Rules:	[Define Package Model]

TBD

Other Notes:

Following on page 137/138 needs to change.

Either the [Number Of Sections] or the [Model Data]/[End Model Data] keywords are required. Note that [Number of Sections] and the [Model Data]/[End Model Data] keywords are mutually exclusive.

To

Either the [Number Of Sections], [ISS Model Data] or the [Model Data]/[End Model Data] keywords are required. Note that [Number of Sections], [ISS Model Data]  and the [Model Data]/[End Model Data] keywords are mutually exclusive.

There a number of Editorial Changes with this new [Define Package Model] keyword.

I see no need for [Number Of Pins] or [Pin Numbers],

Example:

[ISS Model Data]

Keyword: [End ISS Model Data]

Required: Yes, if there is a |

Description: Indicates the end of the formatted ISS model data.

Other Notes: In between the [ISS Model Data] and [End ISS Model Data] keywords is the package model data itself. The data is any number of interfaces to either IBIS-ISS models or Touchstone files.

Example:

[End ISS Model Data]

Keyword:	[Begin ISS Model] <ISS Model Name>

Required:	Yes.

Description:	The begins a section in [ISS Model Data] that contains the data required to interface to IBIS-ISS subckts or Touchstone Files

Sub-Params:	Lots, TBD

Usage Rules:	[Begin ISS Model]  <ISS Model Name>

TBD

Other Notes:

<ISS Model Name> is not used in simulation, it is just a name to describe this model interface. Two [Begin ISS Model]  keywords may not have the same <ISS Model Name>

Example:

[Begin ISS Model] DQ1

Keyword: [End ISS Model]

Required: Yes.

Description: Indicates the end of the formatted ISS model interface.

Other Notes: In between the [Begin ISS Model] and [End ISS Model] keywords is the interface to either IBIS-ISS models or Touchstone files.

Example:

[End ISS Model]

Subparameter: [Language] IBIS-ISS|Touchstone

Required: Yes.

Description: Indicates if the model is an IBIS-ISS subckt or a Touchstone file.

Other Notes:

Example:

[Language] IBIS-ISS

Subparameter: [File] <format> <file name>  {<file name> <file name>}

Required: Yes.

Description: Defines the file(s) containing the model.

Other Notes: The Files must be either IBIS-ISS files or Touchstone files.

<format>  need to points to a <format>   section describing Value, DelayCorner, …

Example:

File Value my_file.iss

Subparameter: [Subckt] <format> <subckt name>  {< subckt name> < subckt name>}

Required: Yes if Language IBIS-ISS.

Description: Defines the subckt(s) in the [File]

Other Notes:

Example:

Subckt Value my_subckt

Subparameter: [Parameter] <name> <format> <param value>  {<param value > <param value >}

Required: Sometimes if Language IBIS-ISS.

Description: Defines the parameters that are to be passed into an instance of the IBIS-ISS subckt. <name> is the name of the parameter. <format> is described below. Depending on the <format> used there will be one or more than one <param value>. String parameters shall be enclosed in “’”.

Other Notes: One must watch there m and M’s when entering parameter value scale factors. Please consider the following table of parameter values and how IBIS and IBIS-ISS evaluate them:

Param	IBIS	IBIS-ISS

1m	1e-3	1e-3

1M	1e6	1e-3

1meg	1e-3	1e6

1Meg	1e6	1e6

Parameter values shall assume the IBIS interpretation. It is recommended that when generating these parameter records, that model makers use the 1m and 1Meg constructs to avoid any possible confusion by an EDA tool or User.

Parameters are not passed into a Touchstone file; however, there are two optional reserved parameters that are used in conjunction with Language Touchstone. They are FBASE and FMAX. They must be of Format Value. See the IBIS-ISS manual to understand how FBASE and FMAX should be used in conjunction with Touchstone files.

<format> may be

Value <value>

DelayCorner <Typ> <Fast> <Slow>

XtalkCorner <Typ> <Min Crosstalk> <Max Crosstalk>

DelayXtalkCorner <nine corners need to be listed>

Gaussian <Mean> <Gaussian>

Integer Range <Typ> <Min> <Max>

Real Range <Typ> <Min> <Max>

PDF <probability> <value> <probability> <value> <probability> <value> …

List <typical value> <value> <value> <value> <value> <value> …

Examples:

Parameter Length Value 11.

Parameter Tstonefile Value ‘abc.s2p’

Subparameter: [Unused Port Termination] <resistance>

Required:  No

Description: Defines the termination that is to be applied to the Ports of a subckt or Touchstone file that are not being used.

Other Notes: If this subparameter is defined the EDA should connect the unused Ports to GND through a <resistance> ohm resistor.

If this parameter is not defined and if Language is IBIS-ISS, then the EDA tool should connect the unused Ports to GND through a 1Meg ohm resistor. If Language is Touchstone, then the EDA tool should connect the unused Ports to GND through a resistor with the Touchstone File reference resistance of the Port.

Example:

[Unused Port Termination] 50

Subparameter:  [Port] <Field 1>  <Field 2> <Field 3> <Field 4> (<Field 5> <Field 6>)

Required: Port is required.

Description: Each Port record contains information on a port (or terminal) of an IBIS-ISS subckt (or Touchstone file).

<Field 1>  Port Number. This must be an integer number greater or equal to 1 and less than or equal to the number of ports (aka terminals) of the IBIS-ISS subckt (or Toucshtone file). Two [Port] records may not have the same Port Number. If a Port Number does not exist in any of the [Port] records then the port is unused, and should be terminated according to the [Unused Port Termination Rules].

<Field 2> Shall be either Pin Pad or Buffer. This describes if the Port is at a Pin, Die Pad, or at the Buffer.

<Field 3> Shall be either Pin_name, Signal_name, Model_name or Default.

<Field 4>  If <Field 3> is Pin_name, Signal_name or Model_name then <Field 4> shall be either a legal Pin_name, Signal_name or  Model_name respectively. If <Field 3> is Default then <Field 4> shall be NA.

<Field 5> If the connection on this port is to a Buffer signal terminal then <Field 3> is Model_name or Default <Field 5> shall be either Diff_pos, Diff_neg or SE, otherwise it shall be NA. If the connection on this port is to a Buffer supply terminal, then <Field 5> shall be either Pullup_Reference, Pulldown_Reference , Power_Clamp_Reference , Ground_Clamp_Reference  or External_Reference.

<Field 6> Is the channel number. A channel is either a single connection or two connections that form a differential pair.  If there is only one channel in the model then <Field 6> may be NA.

<Field 7> Shall contain Aggressor or NA. If NA, then this channel accurately represents that channel through path and crosstalk from all of the other channels. If Aggressor than the channel only accurately represents the crosstalk to all of the NA (Victim) channels.

Other Notes:

Fields at then end of a Port record that are NA are optional

If <Field 2> is Buffer, and the port connects to ths

Example:

Port 1 Pin    Pin_name    M8    NA       NA NA

Port 1 Pin    Pin_name    M8

Port 1 Pad    Pin_name    M8

Port 1 Buffer Pin_name    M8

Port 1 Buffer Pin_name    M8    Pullup_Reference

Port 1 Pin    Model_name  DQ

Port 1 Pin    Model_name  DQS   Diff_pos

Port 1 Pin    Default     NA    SE

Port 1 Pin    Model name  DQ    SE

Port 1 Buffer Pin_name    M8    NA       2

Port 1 Buffer Pin_name    M8    NA       3  Aggressor

Port 1 Buffer Signal_name VDDQ

Keyword:	[Die Supply Pads]

Required:	Sometimes.

Description:	This begins a section in [Component] that contains one line of data for die pads supply nodes. IBIS assumes that for I/O pins (pins that have a Model_name that is not POWER, GND or NC), there is a one to one correspondence between a Pin, Die Pad and Buffer I/O. There are no such assumptions for POWER and GND pins. A POWER or GND Signal_name may have a different number of Pin nodes, die pad nodes and buffer nodes. If the model maker chooses to make separate package and on-die power distribution networks (PDN), then he must supply a list of nodes (and their associated Signal_name) that can be used to mate the package and on-die PDN models.

Sub-Params:	?

Usage Rules:	 TBD

Other Notes:	The data in this section consists of  a list of die pad node names and their corresponding Signal_names that can be used to mate package and on-die PDN networks.

Example:

[Die Supply Pads]

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

Keyword: [End Die Supply Pads]

Required: Yes.

Description: Indicates the end of the [Die Supply Pads] data.

Other Notes:

Example:

[End Die Supply Pads]

Examples

[Define Package Model]

[ISS Model Data]

[Begin ISS Model] IOA3

Language Touchstone

File Value ioA3.s2p

Port 1  Pin    Pin_name A3

Port 2  Buffer Pin_name A3

[End ISS Model]

[Begin ISS Model] IOA7

| This model uses I/O pin A7

Language Touchstone

File Value ioA7.s2p

Ports  Pin.A7  Buf.A7

[End ISS Model]

[Begin ISS Model] IOB3C3

Language Touchstone

File Value ioB3C3.s4p

Port 1  Pin    Pin_name B3

Port 2  Buffer Pin_name B3

Port 3  Pin    Pin_name C3

Port 4  Buffer Pin_name C3

[End ISS Model]

[Begin ISS Model] IOA3

Language IBIS_ISS

File Value io.iss

Subckt io

Parameter Length Value 10.   | 10mm

Port 1  Pin    Pin_name A3

Port 2  Buffer Pin_name A3

[End ISS Model]

[Begin ISS Model] DQS

Language Touchstone

File Value DQS.s4p

Port 1  Pin    Model_name DQS Diff_pos

Port 2  Buffer Model_name DQS Diff_pos

Port 3  Pin    Model_name DQS Diff_neg

Port 4  Buffer Model_name DQS Diff_neg

[End ISS Model]

[Begin ISS Model] VDDQ

Language IBIS_ISS

File Value vddq.iss

Subckt vddq

Port 1  Pin    Signal_name VDDQ

Port 2  Buffer Signal_name VDDQ

[End ISS Model]

[Begin ISS Model] VDDQ_A3

Language IBIS_ISS

File Value vddq_a3.iss

Subckt vddq_A3

Port 1  Pin    Signal_name VDDQ

Port 2  Buffer Pin_name    A3   Pullup_Reference

[End ISS Model]

[Begin ISS Model] IOA3

Language Touchstone

File Value ioA3.s2p

Port 1  Pin     Pin_name   A3

Port 2  Buffer  Pin_name   A3

Port 3  Pin     Model_name DQ  NA       1 Aggressor

Port 4  Buffer  Model_name DQ  NA       1 Aggressor

Port 5  Pin     Model_name DQ  NA       2 Aggressor

Port 6  Buffer  Model_name DQ  NA       2 Aggressor

Port 7  Pin     Model_name DQS Diff_pos 3 Aggressor

Port 8  Buffer  Model_name DQS Diff_pos 3 Aggressor

Port 9  Pin     Model_name DQS Diff_neg 3 Aggressor

Port 10  Buffer Model_name DQS Diff_neg 3 Aggressor

[End ISS Model]

[End ISS Model Data]

[End Package Model]