Proposed New IBIS Terminal Syntax

Draft 1, 2 Sep 2015

Walter Katz, SiSoft

Terminal rules:

Terminal records shall appear after the Number_of_Terminals subparameter and before the [End Interconnect Model] keyword.

Each Terminal line contains information on a terminal of an IBIS-ISS subckt (or Touchstone file).

Terminal records are of the form

<Terminal_number> <Terminal_type> <Terminal_type_qualifier> Aggressor

Terminal_number

Terminal_number is an identifier for a specific terminal.  Terminal_number shall be a positive non-zero integer less than or equal to the value of the Number_of_Terminals argument. The same Terminal_number shall not appear more than once for a given Interconnect Model.  If any Terminals are not present for a given Interconnect Model, then those terminals are unused, and shall be terminated according to the Unused_Terminal_Termination rules.

There are three classes of pins in a component, Signal Pins, Supply Pins and No Connect Pins. Supply Pins have model_name POWER or GND. No Connect Pins have model_name NC. All other pins are classified as Signal Pins. Package models defined in this section assume that there is one Buffer_I/O Terminal and one Die Pad for each Signal Pin.

The model of an I/O Buffer has supply terminals in addition to the Buffer_I/O. These supply (or rail) terminals can be PUref, PDref, PCref, GCref and/or EXTref. The association of the PUref, PDref, PCref, GCref and/or EXTref terminal of a buffer are associate with either bus_label or signal_name in the [Pin Mapping] section. These terminals can be connected to interconnect models one of two ways:

- By specifying a unique interconnect terminal for each I/O Buffer PUref, PDref, PCref, GCref and/or EXTref

- By assuming that all I/O Buffer supply terminals connected to a supply signal_name or bus_label are shorted together. This is done by specifying a unique terminal for all I/O Buffer terminals that are connected to a specific signal_name or bus_label on at least one Supply Pin.

Pads are the location of the interface between the die and the package. Interconnect models can either be between the Pins of a component and the I/O Buffers, or they can be split into models between the Pins of a component and the Pads of the die, and model between the Pads of the die and the I/O Buffer models. There is exactly one Pad (Pad_I/O) for each Signal Pin. There can be any number of Pads (Pad_Rail) for each signal_name or buf_label on Supply Pins. If interconnect models of supply (rail) networks are split between Pin/Pad and Pad/Buffer models, then the interface of supply connections at the die package interface can be handled in one of two ways:

- By defining a list of Die Supply Pads, and specifying terminals for some or all of the Die Supply Pads that are connected to a bus_label or signal_name on at least one Supply Pin.

- By assuming that all supply Pads connected to a supply signal_name or bus_label are shorted together. This is done by specifying a unique terminal for all Pads that are connected to a specific signal_name on at least one Supply Pin.

Pins can be terminals of the interconnect model that connect directly to a PCB board or other type of system connection to an IBIS component. Pins can be Signal Pins (Pin_I/O), or Supply Pins (Pin_Rail). An interconnect model can connect supply pins in one of two ways:

- By specifying terminals for some or all of the Supply Pins.

- By assuming that all supply Pins connected to a supply signal_name or bus_label are shorted together. This is done by specifying a unique terminal for all Pins that are connected to a specific signal_name on at least one Supply Pin.

Terminal_type must be one of the following: Buffer_I/O, PUref, PDref, PCref, GCref, EXTref, Buffer_Rail, Pad_I/O, Pad_Rail, Pin_I/O or Pin_Rail. Buffer_I/O, PUref, PDref, PCref, GCref, EXTref and Buffer_Rail are terminals of an Interconnect Model that connect directly to I/O Buffers. Pin_I/O and Pin_Rail are terminals that are at the Die/Package interface. Pin_I/O and Pin_Rail are terminals that are at the Component PCB interface. The Terminal_type_qualifier for Terminal_types Buffer_I/O, PUref, PDref, PCref, GCref and EXTref must be pin_name. The Terminal_type_qualifier for Terminal_type Buffer_Rail may be signal_name or bus_label.

The Terminal_type_qualifier for Terminal_type Pad_I/O must be pin_name.

The Terminal_type_qualifier for Terminal_type Pad_Rail must be either pad_name, signal_name or bus_label.

The Terminal_type_qualifier for Terminal_type Pin_I/O must be pin_name.

The Terminal_type_qualifier for Terminal_type Pin_Rail must be either pin_name, signal_name or bus_label.

The optional Aggressor field is only allowed allowed on Buffer_I/O records. Connections to Buffer_I/O terminals may be missing coupling to connects that are not included in this interconnect model.

The following table summarized the rules described above.

| Terminal_Type | pin_name | signal_name | bus_label | pad_name | Aggressor |
| --- | --- | --- | --- | --- | --- |
| Buffer_I/O | X |  |  |  | A |
| Puref | X |  |  |  |  |
| Pdref | X |  |  |  |  |
| Pcref | X |  |  |  |  |
| Gcref | X |  |  |  |  |
| EXTref | X |  |  |  |  |
| Buffer_rail |  | Y | Y |  |  |
| Pad_I/O | X |  |  |  |  |
| Pad_rail |  | Y | Y | Z |  |
| Pin_I/O | X |  |  |  |  |
| Pin_rail | Y | Y | Y |  |  |

For an Interconnect Model using File_TS with N ports, N is determined from the [Number of Ports] field in a Touchstone 2 file. The [Number of Terminals] in the Interconnect Model shall be N+1.  Terminal rules are described below:

- The EDA tool shall use the Pin_name or Signal_name specified for the associated Terminal “N+1” entry as the reference node for each of the N ports.

- Terminal/Port Mapping

  - Terminal              Port

  - 1                              1

  - 2                              2

  - …

  - N                             N

  - N+1		reference

- If a Port is not connected, then it shall be terminated by the EDA tool with a resistor to the node on Terminal N+1. The resistance shall be the Port Reference Impedance.

- It shall be an error if Terminal N+1 is not specified to be connected to a Pin, a Pad, or a Buffer that is not part of a connection to a Signal_name that is POWER or GND

Examples:

[Pin] signal_name model_name      R_pin   L_pin   C_pin

A1    DQ1         DQ

A2    DQ2         DQ

A3    DQ3         DQ

D1    DQS+        DQS

D2    DQS-        DQS

P1    VDD         POWER

P2    VDD         POWER

P3    VDD         POWER

P4    VDD         POWER

P5    VDD         POWER

G1    VSS         GND

G2    VSS         GND

G3    VSS         GND

G4    VSS         GND

[Diff Pin] inv_pin  vdiff  tdelay_typ tdelay_min tdelay_max

D1          D2       NA     NA         NA         NA

[Die Supply Pads]  signal_name

VDD1 VDD

VDD2 VDD

VDD3 VDD

VSS1 VSS

VSS2 VSS

[Pin Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref ext_ref

Bus_label_signal_name

A1            VSS          VDD        NC            NC              NC

A2            VSS          VDD        NC            NC              NC

A3            VSS          VDD        NC            NC              NC

D1            VSS          VDD        NC            NC              NC

D2            VSS          VDD        NC            NC              NC

| Full Package/Die Model Complex Power Distribution

Number_of_Terminals 29

1  Pin_I/O     Pin_name A1  |  DQ1         DQ

2  Pin_I/O     Pin_name A2  |  DQ2         DQ

3  Pin_I/O     Pin_name A3  |  DQ3         DQ

4  Pin_I/O     Pin_name D1  |  DQS+        DQS

5  Pin_I/O     Pin_name D2  |  DQS-        DQS

6  Pin_I/O     Pin_name P1  |  VDD         POWER

7  Pin_I/O     Pin_name P2  |  VDD         POWER

8  Pin_I/O     Pin_name P3  |  VDD         POWER

9  Pin_I/O     Pin_name P4  |  VDD         POWER

10 Pin_Rail    Pin_name P5  |  VDD         POWER

11 Pin_Rail    Pin_name G1  |  VSS         GND

12 Pin_Rail    Pin_name G2  |  VSS         GND

13 Pin_Rail    Pin_name G3  |  VSS         GND

14 Pin_Rail    Pin_name G4  |  VSS         GND

15 Buffer_I/O  Pin_name A1  |  DQ1         DQ

16 Buffer_I/O  Pin_name A2  |  DQ2         DQ

17 Buffer_I/O  Pin_name A3  |  DQ3         DQ

18 Buffer_I/O  Pin_name D1  |  DQS+        DQS

19 Buffer_I/O  Pin_name D2  |  DQS-        DQS

20 PUref       Pin_name A1  |  DQ1         DQ

21 PUref       Pin_name A2  |  DQ2         DQ

22 PUref       Pin_name A3  |  DQ3         DQ

23 PUref       Pin_name D1  |  DQS+        DQS

24 PUref       Pin_name D2  |  DQS-        DQS

25 PDref       Pin_name A1  |  DQ1         DQ

26 PDref       Pin_name A2  |  DQ2         DQ

27 PDref       Pin_name A3  |  DQ3         DQ

28 PDref       Pin_name D1  |  DQS+        DQS

29 PDref       Pin_name D1  |  DQS+        DQS

| Full Package/Die Model Simple Power Distribution

Number_of_Terminals 14

1  Pin_I/O     Pin_name A1         |  DQ1         DQ

2  Pin_I/O     Pin_name A2         |  DQ2         DQ

3  Pin_I/O     Pin_name A3         |  DQ3         DQ

4  Pin_I/O     Pin_name D1         |  DQS+        DQS

5  Pin_I/O     Pin_name D2         |  DQS-        DQS

6  Pin_Rail    signal_name   VDD   |  VDD         POWER

7  Pin_Rail    signal_name   VSS   |  VSS         GND

8  Buffer_I/O  Pin_name A1         |  DQ1         DQ

9  Buffer_I/O  Pin_name A2         |  DQ2         DQ

10 Buffer_I/O  Pin_name A3         |  DQ3         DQ

11 Buffer_I/O  Pin_name D1         |  DQS+        DQS

12 Buffer_I/O  Pin_name D2         |  DQS-        DQS

13 Buffer_Rail signal_name   VDD   |  VDD         POWER

14 Buffer_Rail signal_name   VSS   |  VSS         GND

| Single DQ (A1)

Number_of_Terminals 2

1 Pin_I/O     Pin_name A1

2 Buffer_I/O  Pin_name A1

| Single DQ (A1) , Split into package and on-die models

Number_of_Terminals 2

1 Pin_I/O     Pin_name A1

2 Pad_I/O     Pin_name A1

Number_of_Terminals 2

1 Pad_I/O     Pin_name A1

2 Buffer_I/O  Pin_name A1

Full VDD Power Supply Model

Number_of_Terminals 9

1 Pin_Rail   Pin_name P1  |  VDD         POWER

2 Pin_Rail   Pin_name P2  |  VDD         POWER

3 Pin_Rail   Pin_name P3  |  VDD         POWER

4 Pin_Rail   Pin_name P4  |  VDD         POWER

5 Pin_Rail   Pin_name P5  |  VDD         POWER

6 PDref Pin_name A1  |  DQ1         DQ

7 PDref Pin_name A2  |  DQ2         DQ

8 PDref Pin_name A3  |  DQ3         DQ

9 PDref Pin_name D1  |  DQS+        DQS

Full VDD Power Supply Model split into package and on-die

Number_of_Terminals 8

1 Pin_Rail Pin_name P1   |  VDD         POWER

2 Pin_Rail Pin_name P2   |  VDD         POWER

3 Pin_Rail Pin_name P3   |  VDD         POWER

4 Pin_Rail Pin_name P4   |  VDD         POWER

5 Pin_Rail Pin_name P5   |  VDD         POWER

6 Pad_Rail Pad_name VDD1 |  VDD         POWER

7 Pad_Rail Pad_name VDD2 |  VDD         POWER

8 Pad_Rail Pad_name VDD3 |  VDD         POWER

Number_of_Terminals 7

1 Pad_Rail Pad_name VDD1 |  VDD         POWER

2 Pad_Rail Pad_name VDD2 |  VDD         POWER

3 Pad_Rail Pad_name VDD3 |  VDD         POWER

4 PDref    Pin_name A1   |  DQ1         DQ

5 PDref    Pin_name A2   |  DQ2         DQ

6 PDref    Pin_name A3   |  DQ3         DQ

7 PDref    Pin_name D1   |  DQS+        DQS

Power supply model assuming pins shorted, pads shorted, and buffer rail shorted

Number_of_Terminals 2

1 Pin_Rail     signal_name VDD  |  VDD         POWER

2 Buffer_Rail  signal_name VDD  |  VDD         POWER

Power supply model assuming pins shorted, pads shorted, and buffer rail shorted, split between package and die

Number_of_Terminals 2

1 Pin_Rail      signal_name VDD  |  VDD         POWER

2 Pad_Rail      signal_name VDD  |  VDD         POWER

Number_of_Terminals 2

1 Pad_Rail      signal_name VDD  |  VDD         POWER

2 Buffer_Rail   signal_name VDD  |  VDD         POWER

| Single DQ Crosstalk Model

Number_of_Terminals 6

1 Pin_I/O     Pin_name A1

2 Buffer_I/O  Pin_name A1 Aggressor

3 Pin_I/O     Pin_name A2

4 Buffer_I/O  Pin_name A2

5 Pin_I/O     Pin_name A3

6 Buffer_I/O  Pin_name A3 Aggressor

Example with signal_name split into bus_labels

Examples:

[Pin] signal_name model_name      R_pin   L_pin   C_pin

A1    DQ1         DQ

A2    DQ2         DQ

A3    DQ3         DQ

A4    DQ4         DQ

P1    VDD         POWER

P2    VDD         POWER

G1    VSS         GND

G2    VSS         GND

[Pin Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref ext_ref

Bus_label_signal_name

A1            VSS           VDD1        NC            NC              NC

A2            VSS           VDD1        NC            NC              NC

A3            VSS           VDD2        NC            NC              NC

A4            VSS           VDD2        NC            NC              NC

P1            NC            VDD1        NC            NC              NC

P2            NC            VDD2        NC            NC              NC

G1            VSS           NC          NC            NC              NC

G2            VSS           NC          NC            NC              NC

Power supply model assuming pins shorted, pads shorted, and buffer rail shorted

Number_of_Terminals 2

1 Pin_Rail     signal_name VDD  |  VDD         POWER

2 Pin_Rail     signal_name VSS  |  VSS         GND

3 Buffer_Rail  bus_label   VDD1 |  VDD         POWER

4 Buffer_Rail  bus_label   VDD2 |  VDD         POWER

5 Buffer_Rail  signal_name VSS  |  VDD         POWER

EDA tool hooks up the following terminals to …

1 Pins P1 and P2

2 Pins G1 and G2

3 PUref of buffers A1 and A2

4 PUref of buffers A3 and A4

5 PDref of buffers A1, A2, A3 and A4