BUFFER ISSUE RESOLUTION DOCUMENT (BIRD)

BIRD NUMBER: 	189.5_draft15

ISSUE TITLE:   		Interconnect Modeling Using IBIS-ISS and Touchstone

REQUESTOR:     		Walter Katz, Signal Integrity Software (SiSoft); Radek Biernacki, Keysight Technologies; Justin Butterfield, Micron Technology; Curtis Clark, ANSYS; Mike LaBonte, Signal Integrity Software (SiSoft); Arpad Muranyi, Mentor Graphics; Michael Mirmak, Intel Corp.; Bob Ross, Teraspeed Labs; Randy Wolff, Micron Technology

DATE SUBMITTED:	January 27, 2017

DATE REVISED:	March 29, 2017; April 19, 2017; April 26, 2017; June 22, 2017

DATE ACCEPTED:

STATEMENT OF THE ISSUE:

This BIRD enhances IBIS with interconnect modeling features to support broadband, coupled package, and on-die interconnect using IBIS-ISS and Touchstone data.

The BIRD also adds a keyword for buffer rail mapping, to link to new terminal definitions defined for buffers.

ANALYSIS PATH/DATA THAT LED TO SPECIFICATION:

This BIRD has resulted from several years of discussion regarding the need for more flexible description of interconnects in IBIS. It was decided to avoid a keyword-based approach, in favor of a circuit language approach. IBIS-ISS was developed for this purpose, and a means to instantiate IBIS-ISS models from IBIS became the logical next step.

SOLUTION REQUIREMENTS:

The IBIS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - The model maker must be able to provide interconnect models representing die and package, using a combination of IBIS-ISS and Touchstone formats. | Might replace BIRD 125.1 |
| - Touchstone models without an IBIS-ISS wrapper circuit must be supported. | Might replace BIRD 158.1 |
| - An interconnect model may connect buffers to pins directly or separate models may be used for the buffer to pad and pad to pin connections (die and package portions). | Die is buffer to pad. Package is pad to pin. |
| - An interconnect model may connect one pin or any combination of pins on one [Component]. | Coupled models are supported. |
| - The buffer I/O, pad, and pin terminals associated with I/O pins must be assignable to interconnect model terminals directly by pin name. |  |
| - The buffer supply, pad, and pin terminals associated with POWER and GND rail pins must be assignable to interconnect model terminals directly by pin name, or indirectly by [Pin] signal_name or [Pin Mapping] bus_label. |  |
| - The model maker must be able to provide alternative interconnect models for any given set of pins. | For example for a given pin pair it must be possible to provide both coupled and uncoupled models, high and low bandwidth models, or both IBIS-ISS and Touchstone models. |
| - The model maker may use new interconnect models for some pins and legacy package models for other pins. | Legacy Package Models are “Package models defined in IBIS 6.1”. |
| - The model user must be able, given a pin or set of pins it must analyze, to locate all interconnect models that include the pin(s), if any. | Simulation netlisting begins with a list of pins that must be simulated. |
| - The model user must be able to determine all of the pins that a given interconnect model includes. | Once a model is chosen, it may add more pins to the simulation. |
| - The model user must be able to determine how to terminate any terminals of an interconnect model not necessary for a particular analysis. | May need to handle s-parameter and circuit models differently. |
| - For any pin having an interconnect model, models encompassing the full path from buffer to pin must be present and identifiable by the user. | The full path may be described using BIRD 189 Buffer to Pad models and Legacy Pad to Pin Models. |
| - The model user must have useful information needed to make the choice between alternative interconnect models that differ only in characteristics other than the model format and the set of pins included. | For example: coupled/uncoupled, low/high bandwidth. This will be used to choose which alternative model set to use. |
| - The order of precedence for new interconnect models and legacy forms of package models must be specified. | Probably will take precedence over [Package Model], [Pin] RLC, and [Package]. |
| - The model user must be informed which pins of an interconnect model have been modeled with coupling to other pins, sufficient for the former to represent the victim pins and the latter all of the aggressor pins in a crosstalk simulation. | Pins near one “end” of the model will be coupled to pins on one side but probably not enough pins on the other side. |

BACKGROUND INFORMATION/HISTORY:

This BIRD was originally submitted to the IBIS Interconnect Task Group by Walter Katz in April, 2014.  Subsequent revisions were created and reviewed in the Interconnect Task Group with contributions from Radek Biernacki, Justin Butterfield, Curtis Clark, Mike LaBonte, Arpad Muranyi, Michael Mirmak, Bob Ross, and Randy Wolff.

Parameter is shortened to Param (.param is legal in IBIS-ISS) to differentiate it further from Parameters in the multi-lingual syntax (Parameter has several meanings in IBIS and the Algorithmic Modeling Interface.)

File_names are not quoted, to be consistent with Corner in the multi-lingual syntax. Multiple file names for corners are not supported here, however.

Entries for strings in Param are surrounded by double quotes to be consistent with string_literal Parameters in the multi-lingual syntax (or where the AMI string_literal parameter surrounded by double quotes is passed into the multi-lingual Parameters reference).  The EDA tool needs to convert string_literals into the parameter string syntax in IBIS-ISS.

Interaction of Param entries was not discussed.  For example, for a transmission line, TD and Z0 could each have max and min entries, but the EDA tool could make available combinations of min/min, min/max, max/min or max/max for any corner.  Due to parameter interactions, some mixing of corner combinations might not be realistic. (E.g., Z0min or Z0max might not correlate with TDmin or TDmax values, where TDmin=sqrt(LminCmin), Z0min=sqrt(Lmin/Cmax), etc.).

How corners of File_IBIS-ISS and Params are processed might be based on vendor supplied documentation.  For example, some, but not all, combinations are shown below:

- One file_name for all corners, one .subckt name, and all corner settings controlled by Param settings

- One file_name, three .subckts (with internal default .param settings), additional corner settings controlled by Param settings or Param is not used

- Three file_names with the same .subckt name, but with distinct default .param settings, additional settings controlled by Param settings or Param is not used

- Three file_names with three distinct .subckt name and with distinct default .param settings, additional corner settings controlled by Param settings or Param is not used

No interpretation is given for Param typ, min, and max values.  It is possible to independently use typ, min, or max values for any of the Param names that have been defined (e.g., the max value of one parameter may be used with the min value of another parameter).

Some concern has been noted that EDA tools may not be able to clearly define a complete interconnect path from separate Interconnect Models that specify only part of the electrical path.  While several methods to do this are possible, an example flow for an EDA tool to assemble a complete interconnect path from separate Interconnect Models is as follows:

The suggested example flow outlined below and sentence following it may no longer be relevant due to some additions and changes in BIRD189:

- Read in the list of I/O buffers; this must contain:

  - Pin_I/O nodes

  - Buffer_I/O nodes

  - Buffer_Rail nodes; this also defines the respective rail attributes, including:

    - Signal_names

    - Bus_labels

    - Rail pin_names

- Search models to find the smallest number of models that contain all Buffer_I/O (this is List A)

  - If List A contains all Pin_I/O then

    - Done with I/O

  - Else search models to find the smallest number of models that contain the Pad_I/O and Pin_I/O that are missing in List A (this is List B)

- If a power delivery network model is required

  - If the models in List A and List B have connections to all Buffer_Rail terminals and pins for each of the signal names in the Buffer_Rail list, then

    - Done with PDN

  - Else search models that contain the Pad_Rail and Pin_Rail that are missing in List A and List B (this is List C)

- Verify that no pin or buffer terminal is connected to two or more models

- Verify that all pin and buffer I/O terminals are connected to a single interconnect model terminal

- If a power delivery network is defined, then

  - Verify that all Buffer_Rail terminals are connected to a single interconnect model terminal

  - Verify that there is at least one Pin_Rail terminal with a signal_name defined in 3.a

The user may direct the EDA tool to use models from the interconnect model sets in an interconnect model group

BIRD189 was submitted to the IBIS Open Forum January 27, 2017.

BIRD189.1 was created to correct several minor editorial issues, to clarify Unused_port_termination rules and the meaning of Aggressor_Only, to remove a figure, and to update three other figures for clarity.

BIRD189.2 was created to update the list of authors, to correct the capitalization of “Aggressor_Only”, to selectively change “IO” to “I/O”, and to change “Buf_I/O” to “Buffer_I/O” and “Buf_Rail” to “Buffer_Rail” (with appropriate re-formatting for the longer strings) to better match usage elsewhere in IBIS.  A clarification of the meaning of “I/O” in the context of terminals was also added.

BIRD189.3 was created to correct a Param example, and to change “filename” to “base name” in the .ims file rules, for consistency with BIRD186.

BIRD189.4 was created to update the file name rules for compliance with the new terminology defined in BIRD186.3.  Minor editorial corrections were made.  A comment line was added under the [Interconnect Model Set Selector] keyword.

BIRD189.5 contains a number of updates based on a review by Arpad Muranyi, sent July 4, 2017, and several Interconnect Task group reviews. The 60 character limit for [Description] is removed for both [Interconnect Model Set] and [Define Package Model].

[Interconnect Model Set Selector] is replaced with [Interconnect Model Group]s.  Unused_port_termination leaves the termination to EDA tools.

The Unused_port_termination subparameter is restored with options Open, Reference, and Resistance.  Rigid rules are established related to Unused_port termination usage.  Terminal_type

Terminal_type A_gnd is added to connect a terminal to the EDA tool’s ground (or node 0 or equivalent) at any interface.

More statements are given to show how bus_label names can be entered with any or all of the [Pin Mapping], [Pin], [Bus Label] and [Die Supply Pads] keywords

PROPOSED CHANGES:

The following keyword should be added to Chapter 5, COMPONENT DESCRIPTION, after the [Alternate Package Models] keyword:

Keyword:	[Interconnect Model Group]

Required:	No

Description:	 [Interconnect Model Group] has a single argument, which is the name of the associated Interconnect Model Group.  The length of the Interconnect Model Group name shall not exceed 40 characters in length.  Blank characters are not allowed.  The [Interconnect Model Group]/[End Interconnect Model Group] keyword pair is hierarchically scoped by the [Component] keyword. The [Interconnect Model Group] keyword is used to define a list of [Interconnect Model Set]s by name that shall be used together to define interconnect models to be used in a simulation. A simulation may contain Interconnect Models from the Interconnect Model Sets listed in only one Group.

Usage Rules:	[Component] may contain zero or more [Interconnect Model Group] keywords (identified by a name). Each [Interconnect Model Group] must contain at least one [Interconnect Model Set] name. Interconnect Model Sets contain Interconnect Models used to describe pin, die pad or buffer terminal connections to IBIS-ISS subcircuits or Touchstone files.

Interconnect Model Sets that exist for the component shall be listed in one or more Interconnect Model Groups.  An Interconnect Model Group is required even if it references only one Interconnect Model Set.  If there are no Interconnect Model Sets, the [Interconnect Model Group] keyword is illegal

The section under the [Interconnect Model Group] keyword shall have two entries per line, with each line identifying one Interconnect Model Set associated with the component.  The entries shall be separated by at least one white space.  The first entry lists the Interconnect Model Set name (up to 40 characters long).  The second entry is the file reference of the file containing the Interconnect Model Set and shall have the extension “ims”. This file reference shall conform to the rules given in Section 3, ‘GENERAL SYNTAX RULES AND GUIDELINES’.  If the Interconnect Model Set is in the same IBIS file as [Component], then the second entry shall be “NA”.

The files containing the Interconnect Model Sets with the ims extension shall be located in the same directory as the .ibs file or in a specified directory under the .ibs file as determined by the directory path according to the file name rules given in Section 3, ’GENERAL SYNTAX RULES AND GUIDELINES’ (i.e., a file reference containing a relative path to a directory below that of the referencing .ibs file is permitted).  An [Interconnect Model Set] with matching name shall be found in the stated location for each Interconnect Model Set named in the [Interconnect Model Group].

Each Interconnect Model Set name and its file_reference may only appear once under each [Interconnect Model Group] keyword for a given component.

As discussed in Section XXX, three interface locations exist: pin, die pad, and buffer.  These interfaces are identified in the Terminal lines under the [Interconnect Model] keyword and by their Terminal_type column entries (shown in Table 41) as follows:

pin:		Pin_I/O, Pin_Rail

die pad:	Pad_I/O, Pad_Rail

buffer:	Buffer_I/O, Buffer_Rail, Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref, Ext_ref

ground	A_gnd (connection to global ground at any interface)

Identifiers associated with these Termimal_type *_I/Os are pin_name entries.  In addition, some *_I/O terminals may have the optional Aggressor_Only column.  If any *_I/O pin is marked as Aggressor_Only, then all *_I/O pins with the same pin_name entry shall be considered as Aggressor_Only. Any *_I/O Terminal_type without the Aggressor_Only column may be considered as an aggressor or a victim.

The remaining terminals are used for POWER or GND and are referred to as “rails”.  The rail identifiers are pin_name, signal_name, bus_label (described below) and pad_name entries (described below) according the the allowable association rules summarized in Section XXX (Connecting Pins, Pads and Buffer Terminals) and Table 41.

An Interconnect Model Group contains of a list of Interconnect Model Sets which in turn contains a list of Interconnect Models. There are a number of rules that apply to this combined list of Interconnect Models in an Interconnect Model Group.

- I/O pin_name rules

  - I/O terminals use pin_name identifiers

  - All *_I/O pin_names may omit the Aggressor_Only column (may be aggressors or victims)

  - No I/O pin_name in a component may appear as a Pin_I/O terminal without the Aggressor_Only column in more than one Interconnect Model in the Interconnect Model Group.

  - No I/O pin_name in a component may appear as a Buffer_I/O terminal without the Aggressor_Only column in more than one Interconnect Model in the Interconnect Model Group.

  - An I/O pin_name may appear in Interconnect Models with the following interface combinations:

    - pin to buffer

    - pin to die pad (in one Interconnect Model) and die pad to buffer (in another Interconnect Model)

    - pin to die pad

    - die pad to buffer

  - A *_I/O pin_name may not appear in Interconnect Models of Interconnect Model Sets that are listed in one Interconnect Model Group with the following interface combinations:

    - pin to buffer (in one Interconnect Model) and pin to die pad (in another Interconnect Model)

    - pin to buffer  (in one Interconnect Model) and die pad to buffer (in another Interconnect Model)

    - pin to buffer and pin to die pad and die pad to buffer in three separate Interconnect Models

  - A *_I/O pin_name which does not appear in any of the Interconnect Models without the Aggressor_Only column may still be used in simulations, however, the user should be aware that that Interconnect Model will not include all of the crosstalk effects to that Aggressor_Only *_I/O pin_name.

  - If a *_I/O pin_name which does not appear in any of the Interconnect Models without the Aggressor_Only column but appears in more than one Interconnect Models with the Aggressor_Only column, then the model maker should assume that the simulation will use the first Interconnect Model that contains a terminal with this pin_name in the first Interconnect Model Set in the Interconnect Model Group that contains an Interconnect Model that contains a terminal with this pin_name.

    - In order to avoid leaving the choice of Interconnect Model selection to the EDA tool, the model maker can put these models into separate Interconnect Model Sets that can be referenced by separate Interconnect Model Groups.

    - However, the best approach for the model maker is to create an Interconnect Model Group that contains one or more Interconnect Models without the Aggressor_Only column for each pin.

- General description of rail terminals

  - At the pin interface, a terminal whose terminal_type is Pin_Rail  can be identified by a pin_name, signal_name or bus_label entry.  An pin_name maps directly into a Pin_Rail pin_name entry or the pin_name can be mapped into a bus_label or a signal_name with the information given in the [Pin] keyword or by the [Pin Mapping], [Bus Label], or [Die Supply Pads] keywords described later in this section.

    - Note that a terminal whose terminal_type is Pin_Rail  may be associated with one pin_name or a list of pin_names on a rail that is associated with a single signal_name or bus_label.  If it is associated with more than one pin_name then these pin_names are shorted together.

  - At a die pad interface, a terminal whose terminal_type is Pad_Rail can be identified by a pad_name, signal_name or bus_label entry.  Connections between die pad interfaces in different Interconnect models can be made by using identical pad_names or identifying a common bus_label or signal_name that is available in the [Pin], [Pin Mapping], [Die Supply Pads], or [Bus Label] keywords.

    - Note that a terminal whose terminal_type is Pad_Rail  may be associated with one pad_name or a list of pad_names on a rail that is associated with a single signal_name or bus_label.  If it is associated with more than one pad_name then these pad_names are shorted together.

  - At the buffer interface, a terminal whose terminal_type is Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref or Buffer_Rail can be identified by a signal_name or bus_label entry, or directly by the *_ref entries that are associated with Buffer_I/O pin_names.

    - Note that a terminal whose terminal_type is Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref or Buffer_Rail  may be associated with one buffer terminal or a list of buffer terminals on a rail that is associated with a single signal_name or bus_label.  If it is associated with more than one buffer terminals then these buffer terminals are shorted together.

  - A Power Delivery Network (PDN) has one or more connections of rail signal_names between Pin and Buffer, Pin and Pad or Pad and Buffer.

  - An Interconnect Model with only rail terminals (no I/O terminals) can be used for a PDN.

  - A PDN structure can also exist in an Interconnect Model with I/O terminals.

  - Also, rail terminals or A_gnd can be used in Interconnect Models to provide reference node for the electrical interconnections associated with *_I/O terminals.

- Rail terminal rules

  - At the pin interface, a rail pin_name may not appear on a terminal line whose terminal_type is Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref or Buffer_Rail in more than one PDN Interconnect Model in the Interconnect Model Group.   A rail_pin_name may appear on a terminal line whose terminal_type is Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref or Buffer_Rail in multiple Interconnect Models if the terminal is used as a reference node for I/O paths.

  - At the buffer interface, a rail_pin_name may appear on a terminal line whose terminal_type is Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref or Buffer_Rail in more than one PDN Interconnect Model in the Interconnect Model Group.   A rail_pin_name may appear on a terminal line whose terminal_type is Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref or Buffer_Rail in multiple Interconnect Models if the terminal is used as a reference node for I/O paths.

  - A rail terminal may be in Interconnect Models with the following interface combinations:

    - pin to buffer

    - pin to die pad (in one Interconnect Model) and die pad to buffer (in another Interconnect Model)

    - pin to die pad

    - die pad to buffer

  - A rail terminal may not be in Interconnect Models with the following interface combinations:

    - pin to buffer (in one Interconnect Model) and pin to die pad (in another Interconnect Model)

    - pin to buffer (in one Interconnect Model) and die pad to buffer (in another Interconnect Model)

    - pin to buffer, pin to die pad, and die pad to buffer in three separate Interconnect Models

Note that these rules apply to the complete list of Interconnect Models that are included in each Interconnect Model Group. It is necessary but not sufficient condition that the Interconnect Models in one Interconnect Model Set satisfy these rules in order for the rules to be satisfied in any Interconnect Model Group containing the Interconnect Model Set.

If an *_I/O pin_name appears on terminal lines of Interconnect Model(s) in the Interconnect Model Group with the interface combinations: pin to buffer, or pin to die pad and die pad to buffer, then the Interconnect Model(s) in the Interconnect Model Group define the full interconnect electrical path between the pin and buffer interfaces.  If this is not he case then:

- If an *_I/O pin_name appears only in a pin to die pad Interconnect Model in the Interconnect Model Group, then the *_I/O pin_name electrical path from the die pad to buffer  shall be shorted.

- If an *_I/O pin_name appears only in a buffer to die pad Interconnect Model in the Interconnect Model Group, then the *_I/O pin_name electrical path from die pad to buffer shall be connected using any other existing package model in this document including those under [Package] R_pkg, L_pkg, and C_pkg entries; [Pin] R_pin, L_pin, and  C_pin entries in this section; or entries under the [Define Package Model] keyword described in Section 7.  Note, if several [Define Package Model] keywords exist, the EDA tool may have to select which on one to use.  EDA tools may provide the option to ignore any of the other package model formats and to use shorted paths instead.

- If an *_I/O pin_name does not appear on a terminal line in any Interconnect Model in an Interconnect Model Group, then the EDA tool should use any other existing package model in this document.

If a PDN structure has terminals in an Interconnect Model(s) in the Interconnect Model Group with the interface combinations: pin to buffer, or pin to die pad and die pad to buffer, then the Interconnect Model(s) in the Interconnect Model Group define the full PDN electrical path between the pin and buffer interfaces.  If this is not he case then:

- If rail terminals describe a PDN structure with only a pin to die pad Interconnect Model in the Interconnect Model Group, then the rail electrical path from the die pad to buffer shall be shorted.  Note, the shorted connections from the die pad terminal names to the buffer interface rail terminal names might require using the information under the [Pin], [Pin Mapping], [Die Supply Pads] or [Bus Label] keywords in this section.

- If rail terminals describe a PDN structure with only a buffer to die pad Interconnect Model in the Interconnect Model Group, then the rail electrical path from die pad to Pin_Rail pin_name entry shall be connected using any other existing package model in this document including those with [Package] R_pkg, L_pkg, and C_pkg entries or [Pin] R_pin, L_pin, and  C_pin entries in this section; or entries under the [Define Package Model] keyword described in Section 7.  Note, if several [Define Package Model] keywords exist, the EDA tool may have to select which on one to use.  Also note, the Pad_Rail terminals have pad_name bus_label, or signal_name entries that may short the electrical connections at the die pad interface based on the information under the [Pin], [Pin Mapping], [Die Supply Pads] or [Bus Label] keywords in this section.  If there are more rail pad_names than Pin_Rail pin_names, the EDA tool will have to short some pad_names to support existing package model formats.

Examples:

| Some [Interconnect Model Set] names used in Examples from Section 12 are

| referenced below:

|

| Example 1

|

[Interconnect Model Group] Full_ISS_PDN_1

| Interconnect Model Set   file_reference

Full_ISS_PDN_1             NA                | The [Interconnect Model Set] is

| present in the .ibs file for

| all pins

[End Interconnect Model Group]

|

| Example 2

|

[Interconnect Model Group] Full_ISS_PDN_sn_2

| Interconnect Model Set   file_reference

Full_ISS_PDN_sn_2          NA                | The [Interconnect Model Set] is

| present in the .ibs file for

| all I/O pins and PDN described

| by signal_names (sn)

[End Interconnect Model Group]

|

| Example 3

|

[Interconnect Model Group] A1_TS

| Interconnect Model Set   file_reference

A1_TS                      touchstone/ts_sets.ims | [Interconnect Model Set] is

| in ts_sets.ims under the

| touchstone directory for A1

[End Interconnect Model Group]

|

| Example 4

|

[Interconnect Model Group] A1_ISS_buf_pad_TS_pad_pin

| Interconnect Model Set   file_reference

A1_ISS_buf_pad             NA     | Interconnect Model Sets combined from

A1_TS_pad_pin              NA     | buffer to pad and pad to pin Sets with

| different file formats for A1

[End Interconnect Model Group]

|

| Example 5

|

[Interconnect Model Group] Full_ISS_split_IO_PDN_3

| Interconnect Model Set   file_reference

Full_ISS_buf_pin_IO_1      NA     | IO paths with common sn reference

Full_ISS_buf_pin_PDN_1     NA     | Detailed (by pin) PDN paths

| PDN terminals G1-G4 get shorted

[End Interconnect Model Group]

***** ALL OTHER EXAMPLES NEED CAREFUL REVIEW FOR REFERENCING *****

Keyword: 	[End Interconnect Model Group]

Required: 	Yes, for each instance of the [Interconnect Model Group] keyword

Description: 	Indicates the end of the data for one [Interconnect Model Group].

Example:

[End Interconnect Model Group]

The following keywords should be placed in section 5, COMPONENT DESCRIPTION, after the [Pin Mapping] keyword.

Keyword:	[Bus Label]

Required:	No

Description:	Defines bus_label names and associates a POWER or GND signal_name with one or more bus_label names within a Component. The bus_label names can be used to define connection points for Interconnect Model terminals.

Sub-Params:	signal_name

Usage Rules:	The first column shall contain a bus_label.  The second column, signal_name, shall be a corresponding signal_name entry for a pin under the [Pin] keyword that uses the model_name POWER or GND.

The [Bus Label] keyword shall be followed by the string “signal_name” as a column heading.

Duplicate bus_labels are not permitted. A bus_label may be defined also by the [Pin Mapping] keyword, by a signal_name under the [Pin] keyword, and/or by the [Die Supply Pads] keyword below.

Column length limits are:

[Bus Label]	15 characters max

signal_name	40 characters max

Example:

[Bus Label]	signal_name

VDD1 		VDD

VDD2 		VDD

VDD3 		VDD

VSS1 		VSS

VSS2 		VSS

Keyword:	[Die Supply Pads]

Required:	No

Description:	Defines supply rail die pads and associates signal_names and bus_labels with those die pads.

Sub-Params:	signal_name, bus_label

Usage Rules:	 Only die pads with signal_names that occur on POWER or GND pins are allowed.  Each line shall contain either two or three columns.  The first column shall contain the supply die pad name (the column entry is also referred to as “pad_name” elsewhere in this document).  The second column, signal_name, shall contain the signal name as given under the [Pin] keyword.  The third column is optional.  If it exists, it is a bus_label.  If the third column does not exist, then the bus_label shall be the signal_name.

The [Die Supply Pads] keyword shall be followed by the strings “signal_name” and “bus_label” as column headings.

Other Notes:	The data in this section consists of a list of pad_names and their corresponding signal_names and bus_labels, which can be used to mate package and on-die power delivery networks.

The keywords described above ([Pin Mapping], [Pin], [Bus Label], and [Die Supply Pads]) describe several ways to name the bus_label entries.  Briefly, they are listed here:

[Pin Mapping] associates each rail pin_name with a bus_label for all rail pin_names.  For the listed buffer I/O pin_names (in the first column), the bus_label entries are listed under the pulldown_ref, pullup_ref, gnd_clamp_ref, power_clamp_ref, and ext_ref columns of [Pin Mapping]. This listing of any or all POWER and/or GND pin_names (also referred to as rails) is optional.

[Pin] associates each pin_name with a signal_name.   The signal_name can be used as a bus_label for rail pin_names that are not listed under [Pin Mapping] or not described by the [Bus Label] and [Die Supply Pads] keywords.

[Bus Label] also associates signal_names with bus_labels.

[Die Supply Pads] is used to define rail pad_names and to associate them with signal_name, but the second and third columns can provide another way to associate signal_names with bus_labels in a manner that may not be covered above.

Such entries can be used as terminals at designated locations in [Interconnect Model] terminal lines described later in Section XXX.  The keywords can also be used to describe how different terminal_type_qualifiers (described later) can be associated with each other.  For example, a POWER or GND pin_name with a bus_label entry in [Pin Mapping] would find its corresponding signal_name from the [Pin] keyword for the same pin_name.

With these four keywords, it is possible to create bus_label names for rails in four different ways, and any or all of the four ways can be used at once.

These keywords also support using each rail terminal individually or for creating a single terminal that connects terminals that connects rails with the same bus_label or signal_name, or to designate  rail pad_names that might be different than rail pin_names.  With these keywords, the number of rail nets can be reduced.  Also, a different number of rail terminals can be entered at each boundary to support few-to-many or many-to-few connection terminals.

Example:

[Die Supply Pads] signal_name bus_label

VDDQ 		      VDDQ

VDD1 		      VDD	      VDDa

VDD2 		      VDD	      VDDa

VDD3 		      VDD	      VDDb

VSS1 		      VSS

VSS2 		      VSS

The following text should be added at the beginning of Chapter 7, PACKAGE MODELING, after the chapter title.

7.1 INTRODUCTION

Several package modeling formats are available in IBIS.  These include:

- Lumped [Component]-level models for the entire [Component], using the [Package] keyword.

- Lumped [Component]-level modeling per-pin, using the [Pin] keyword.

- [Package Model] (including [Alternate Package Models] and [Define Package Model]).

- [Interconnect Model Group] and the keywords associated with it.

The lumped formats are described in the [Package] and [Pin] keyword definitions in Chapter 5.  Keywords for use with the [Package Model] format are described in this chapter, while keywords for use with [Interconnect Model Group] are described in Chapter 12.

7.2 RULES OF PRECEDENCE

The order of precedence for package model data to be used by EDA tools in simulation is defined below, in ascending order.  If a package data format at a numerically higher position on the list is available in an IBIS or related file, that data shall be considered by the EDA tool to be more detailed and is therefore preferred.

- [Component]/[Package]

- [Component]/[Pin]

- [Package Model] (including [Alternate Package Models] and [Define Package Model])

- [Interconnect Model Group]

Note that [External Circuit] and [Node Declarations] are mutually exclusive with [Interconnect Model Group] within the same [Component].  [Package Model] and [Interconnect Model Group] may both be present for the same [Component] but should not both be used at the same time.

7.3 KEYWORDS FOR USE WITH [PACKAGE MODEL]

< Insert Existing Text Here>

Page 141 for the [Description] keyword, Replace:

Usage Rules: The description must be less than 60 characters in length, must fit on a single line, and may contain spaces.

With:

Usage Rules: The description shall fit on a single line, and may contain spaces.

The following new Chapter 12 should be added after Chapter 11.

12  INTERCONNECT MODELING

12.1 INTRODUCTION

IBIS supports broadband interconnect models describing connections between the pins of a component and its I/O buffers. These interconnect models may include descriptions of frequency-dependent losses, interconnect coupling and/or complex supply rail distributions.

Interconnect is defined between up to three interface locations:

- pin, where a component connects to a printed circuit board

- die pad, where a component die connects to the routing on a package substrate

- buffer, where the buffer itself connects to the die substrate and routing

The relationship between the terminals at the buffer, die pad, and pin interfaces is shown in the figure below.

Figure 47 – Example Interconnect Model Structure

The connection between the pin and die pad is generally called “package interconnect”, while the connection between the die pad and the buffer is generally called “on-die interconnect.”  The die pad is distinct from the buffer terminal; the buffer includes the circuitry that would be described through the [Model] keyword and related keywords, and would not include transmission line behavior.

Interconnect models may be supplied separately for on-die interconnect and package interconnect, or may be supplied as a single model for the entire connection between the package pins and buffers.

The electrical behavior of an interconnect is described through either IBIS-ISS subcircuits or Touchstone network parameters.  An Interconnect Model defines the connections to either an IBIS-ISS subcircuit or a Touchstone file. An Interconnect Model may describe the connection between the pins of the package and the buffers, the pins of the package and the die pads, or the die pads and buffers.

Interconnect Models are organized into Interconnect Model Sets. An [Interconnect Model Set] keyword consists of one or more [Interconnect Model] keywords. One Interconnect Model Set may contain groups of similar interconnect models or different interconnect models to describe the complete connections from the buffer to pin interface.

Each I/O pin is associated with one I/O buffer terminal and optionally one I/O die pad. By contrast, there is no required one-to-one relationship between rail pins, (optional) rail die pads, and buffer rail terminals.

Figure 48 below shows the [Interconnect Model] terminals for an I/O path on both package and on-die substrates.

Figure 48 – Package Substrate I/O Paths

The figure also shows on-die interconnect routes that may experience crosstalk effects.  This example assumes that only a few routes out of a larger bus are shown.  In such a model, the crosstalk on any one route in the model could only be caused by its nearest neighbors.

While each of the inner two routes in the figure may have all potential crosstalk represented in the model, the outer signals would not.  The model maker would therefore indicate that connections to the outer routes’ terminals do not include all of their aggressors by adding the optional argument “Aggressor_Only” to their terminals.  The descriptions of the associated terminals would not use the “Aggressor_Only” designation for the inner routes. The EDA tool may therefore assume in simulation that the inner routes have all (or more practically most of) the coupling to their aggressor connections represented in the model.

Crosstalk simulations require Interconnect Models to have connections to multiple I/O pin_names.

Figure 49 of a package and die shows graphically the potential [Interconnect Model] terminals for a rail connection. A single pin terminal for a supply rail connects to multiple die pads.  Note that these terminals may be collapsed to a single terminal at the pin (as shown), or alternately at the die pad interface and/or the buffer rail terminals.

Figure 49 – Package Substrate Rail Terminals

The terminal section of an [Interconnect Model] describes how the terminals of an Interconnect Model subcircuit or Touchstone file instance are connected at a buffer terminal, die pad interface or pin/board interface.

12.2 GENERAL INTERCONNECT SYNTAX REQUIREMENTS

Terminal lines under the [Interconnect Model] keyword describe connections.

I/O terminals shall be connected using only the pin_name qualifier at these locations:

- pins: I/O pin_name

- die pads: I/O pin_name

- buffer: I/O pin_name

Rail terminal connections have more options to support direct connections to terminals or to groups of terminals using signal_name, bus_label, or pad_name entries at the pin, die pad or buffer locations. For the following locations the rail terminal can connect to:

- pins

- a specific rail pin_name

- all of the pins of a rail signal_name

- all of the pins of a bus_label

- die pads

- all of the die pads with a rail signal_name

- all of the die pads with a rail bus_label

- a specific die pad pad_name

- buffer rail terminals

- all of the buffer rail terminals of a rail signal_name

- all of the buffer rail terminals of a bus_label

- a specific buffer rail terminal for an I/O buffer pin_name

One or more Interconnect Model Sets may be included in a separate Interconnect Model Set file, using a file name with the extension “ims”, or within the .ibs file where [Interconnect Model Group] is used.  The [Interconnect Model Set] keyword can contain the optional [Manufacturer] and [Description] keywords and one or more [Interconnect Model] keywords and the [Interconnect Model] associated subparameters, as is listed in Table 40.

Table 40 – Interconnect Modeling Keywords and Subparameters

| Keyword or Subparameter | Notes |
| --- | --- |
| [Interconnect Model Set] |  |
| [Manufacturer] | (note 1) |
| [Description] | (note 1) |
| [Interconnect Model] | (note 2) |
| Param |  |
| File_TS | (note 3) |
| File_IBIS-ISS | (note 3) |
| Unused_port_termination | (note 4) |
| Number_of_terminals | (note 5) |
| <terminal line> | (note 6) |
| [End Interconnect Model] | (note 7) |
| [End Interconnect Model Set] | (note 8) |
| Note 1 [Manufacturer] and [Description] are each optional keywords within any [Interconnect Model Set]. Note 2 At least one [Interconnect Model] is required for each [Interconnect Model Set]. Note 3 One of either the File_TS or File_IBIS-ISS subparameters is required. Note 4 Required for Touchstone files where ports are unused, illegal if there are no unused ports or for IBIS-ISS file Note 5 This subparameter shall be followed by the “=” character and an integer value, with both optionally surrounded by whitespace. Note 6 See text below. Note 7 Required when the [Interconnect Model] keyword is used Note 8 Required when the [Interconnect Model Set] keyword is used |  |

When Interconnect Model Set definitions occur within a .ibs file, their scope is “local”— they are known only within that .ibs file and no other .ibs file.  In addition, within that .ibs file, they override any interconnect package models defined using the [Package], [Pin], or [Define Package Model] keywords.  Interconnect Models in separate .ims files referenced by the [Interconnect Model Group] keyword in a .ibs file also override any interconnect package models defined in the same .ibs file using the [Package], [Pin], or [Define Package Model] keywords.

Usage Rules for the .ims file:

Interconnect models are stored in a file whose file name uses the format:

<stem>.ims

The <stem> provided shall adhere to the rules given for the [File Name] keyword.  Use the “ims” extension to identify files containing Interconnect Models.  The .ims file shall contain the [IBIS Ver], [File Name], [File Rev], and the [End] keywords.  Optional elements include the [Date], [Source], [Notes], [Disclaimer], [Copyright], and [Comment Char] keywords. All of these keywords and associated subparameters follow the same rules as those for a normal .ibs file.

Note that the [Component] and [Model] keywords are not allowed in the .ims file.  The .ims file is for Interconnect Models only.

Keyword:	[Interconnect Model Set]

Required:	No

Description:	Used to contain Interconnect Models

Usage Rules:	[Interconnect Model Set] has a single argument, which is the name of the Interconnect Model Set.  The length of the Interconnect Model Set name shall not exceed 40 characters in length.  Blank characters are not allowed.  The [Interconnect Model Set]/[End Interconnect Model Set] keyword pair is hierarchically equivalent in scope to [Component] and [Model].

The section under the [Interconnect Model Set] keyword may contain a [Manufacturer] keyword section and [Description] keyword section and shall contain one or more Interconnect Models.  See the section [Interconnect Model] for a description of the content of each Interconnect Model.

An [Interconnect Model Set] contains a list of [Interconnect Model]s that have a logical association such as:

- All models in a bus (e.g.. DDR4, or PCIeG3)

- Full PDN structure from buffer to pin

- On-die PDN structure from buffers to die pads

- Package only PDN structure from die pads to pins

- All I/O models between die pad and pin

- All I/O models between buffer and die pad

- All I/O models between buffer and pin

- Combined I/O and PDN models

- All uncoupled models

- Coupled models

- Touchstone electrical models

- IBIS-ISS electrical models

Example:

[Interconnect Model Set] Signal_Integrity

[Manufacturer] Acme Packaging, Inc.

[Description] This set contains one model for each I/O buffer

[Interconnect Model] DQ1

…

[End Interconnect Model]

[Interconnect Model] DQ2

…

[End Interconnect Model]

[Interconnect Model] DQS

…

[End Interconnect Model]

[End Interconnect Model Set]

Keyword:	[Manufacturer]

Required:	No

Description:	Specifies the name of the [Interconnect Model Set] manufacturer.

Usage Rules:	The length of the manufacturer’s name shall not exceed 40 characters (blank characters are allowed, e.g., Oklahoma Instruments).

Example:

[Manufacturer]  NoName Corp.

Keyword:	[Description]

Required:	No

Description:	Provides a concise yet easily human-readable description of what kind of interconnect the [Interconnect Model Set] represents.

Usage Rules:	The description shall fit on a single line, and may contain spaces.

Example:

[Description]   220-Pin Quad Ceramic Flat Pack

Keyword: 	[End Interconnect Model Set]

Required: 	Yes, for each instance of the [Interconnect Model Set] keyword.

Description: 	Indicates the end of the Interconnect Model Set data.

Example:

[End Interconnect Model Set]

Keyword:	[Interconnect Model]

Required:	No

Description:	Marks the beginning of an Interconnect Model description that is used to define the interfaces to IBIS-ISS subcircuit or Touchstone files.

Sub-Params:	Param, File_TS, File_IBIS-ISS, Number_of_terminals

Usage Rules:	[Interconnect Model] has a single argument, which is the name of the associated Interconnect Model.  The length of the Interconnect Model name shall not exceed 40 characters in length.  Blank characters are not allowed.  The [Interconnect Model]/[End Interconnect Model] keyword pair is hierarchically scoped by the [Interconnect Model Set]/[End Interconnect Model Set] keywords.

The [Interconnect Model]/[End Interconnect Model] section defines both the association between a Touchstone file or IBIS-ISS subcircuit and an Interconnect Model, as well as defining the terminals and terminal usage for the Interconnect Model in the context of the given [Component].

An [Interconnect Model] shall contain one and only one of the following combinations:

- pins and buffer terminals (full package model)

- pins and die pads (package only model)

- die pads and buffer terminals (on-die interconnect model)

Other Notes:	If a full package model contains an I/O pin terminal for a pin_name then it shall also contain an I/O buffer terminal for the same pin_name. If a package only model contains an I/O pin terminal for a pin_name then it shall also contain an I/O die pad for the same pin_name. If an on-die interconnect model contains an I/O buffer terminal for a pin_name then it shall also contain an I/O die pad for the same pin_name.

An [Interconnect Model] may contain:

- only power rail models

- one or more I/O signal models

- both power rail models and one or more I/O signal models

Each terminal of an Interconnect Model is connected to a node and has a “voltage”. This, as stated, is imprecise. Voltage, by definition, is a potential difference between two points. It is common to probe and plot the potential difference between simulator nodes at a terminal and the simulator ideal Node 0. This is valid for non-power aware simulations when the local ground (or return path) node is forced to Node 0 by the simulator, or for “ground referenced” power aware simulations that lump the effect of the ground interconnect into the power rails. However, this is not valid when the local ground nodes are “floating”. In this case it is important that the actual rail node that is the reference node for measurements at the I/O buffer is included as a terminal in the Interconnect Model. If this is not done, then the Interconnect Model will not correctly account for all return currents, particularly from capacitive elements. If an Interconnect Model does not contain a reference terminal, then the user of these models should be aware that using these models in non-ground referenced power aware simulations will introduce potential errors in simulations.

The following subparameters are defined:

Param

File_IBIS-ISS

File_TS

Unused_port_termination

Number_of_terminals = <value>

In addition to these subparameters, the [Interconnect Model]/[End Interconnect Model] section may contain lines describing terminals and their connections.  No specific subparameter name, token, or other string is used to identify terminal lines.

Unless noted below, no Interconnect Model subparameter requires the presence of any other subparameter.

Param rules:

The subparameter Param is optional and only legal with the File_IBIS-ISS subparameter documented below.  Param is illegal with the File_TS subparameter documented below.  Param shall be followed by three arguments: an unquoted string argument giving the name of the parameter to be passed into the IBIS-ISS subcircuit, a reserved word for the parameter format, and one numerical value or one string value (surrounded by double quotes) for the parameter value to be passed into the IBIS-ISS subcircuit.

The numerical value rules follow the scaling conventions in Section 3.2, “SYNTAX RULES”.  The EDA tool is responsible for translating IBIS specified parameters into IBIS-ISS parameters.  For example, 1 megaohm, would be represented as 1M in Param value according to the Section 3 rules, but would be converted by the EDA tool to case-insensitive 1meg (1X is not recommended) or 1E6 for IBIS-ISS use.  Quoted string parameters in IBIS are converted to the string parameter syntax in IBIS-ISS subcircuits.  For example, the Param value "typ.s2p" would be converted to str('typ.s2p') in IBIS-ISS subcircuits.

Examples:

| Param   name     format   value

Param     abc      Value    2m        | 2E-3 in IBIS

Param     def      Value    4k        | 4E3 in IBIS

Param     ts_file  Value    "typ.s2p" | file name string passed

| into IBIS-ISS

File_IBIS-ISS rules:

Either File_IBIS-ISS or File_TS is required for a [Interconnect Model]/[End Interconnect Model] group.  The File_IBIS-ISS subparameter is followed by two unquoted string arguments consisting of the file_reference and circuit_name (.subckt name) for an IBIS-ISS file.  The IBIS-ISS file under file_reference shall be located in the same directory as the referencing .ibs file or .ims file or in a specified directory under the referencing file as determined by the directory path (i.e., a file reference containing a relative path to a directory below that of the referencing .ibs or .ims file is permitted).

Example:

| file_type    file_reference    circuit_name(.subckt name)

File_IBIS-ISS  net.iss           netlist_typ

File_TS rules:

Either File_TS or File_IBIS-ISS is required for a [Interconnect Model]/[End Interconnect Model] group.  File_TS is followed by one unquoted string argument, which is the file_reference for a Touchstone file.  The Touchstone file under file_reference shall be located in the same directory as the referencing .ibs file or .ims file or in a specified directory under the referencing file as determined by the directory path (i.e., a file reference containing a relative path to a directory below that of the referencing .ibs or .ims file is permitted).

Example:

| file_type    file_reference

File_TS        typ.s8p

Unused_port_termination rules:

The Unused_port_termination subparameter is required under this condition:

File_TS is used and the number of terminal lines (described below) is less than N+1

Unused_port_termination_directive is illegal under these conditions:

File_IBIS-ISS is used.

File_TS is used and the number of terminal lines is N+1

If required, only one Unused_port_termination_directive subparameter may appear for a given [Interconnect Model] keyword.

The Unused_port_termination subparameter is followed by white space and one of these arguments:

Open

Reference

Resistance

“Open” declares that the unused ports remain unterminated (open-circuited).

“Reference” declares that the EDA tool terminates all the unused ports with resistors whose resistance values are equal to the reference impedances provided in the Touchstone file for the respective unused ports.

“Resistance” declares that the EDA tool will terminate all unused ports with resistors, all having the same value.  The “Resistance” entry is followed by a third column entry with the (non-negative) numerical resistance value.

Examples:

Unused_port_termination		Open

Unused_port_termination		Reference

Unused_port_termination		Resistance	43.5

Number_of_terminals rules:

The Number_of_terminals subparameter is required and defines the number of terminals associated with the Interconnect Model.  The subparameter name shall be followed by a single integer argument on the same line.  The argument shall be separated from the subparameter name by the “=” character. The subparameter name, “=” character, and argument may optionally be separated by whitespace.

Only one Number_of_terminals subparameter may appear for a given [Interconnect Model] keyword.  The Number_of_terminals subparameter shall appear before any terminal lines and after all other subparameters for a given Interconnect Model.

For File_IBIS-ISS, the Number_of_terminals value shall be equal to the number of subcircuit terminals for an IBIS-ISS subcircuit.  Because an IBIS-ISS subcircuit requires at least one terminal the Number_of_terminals value shall be 1 or greater.  The IBIS-ISS subcircuit terminals shall not contain the ideal ground node (node 0 or its synonyms).

For File_TS, the Number_of_terminals value shall be a value equal to N+1 (number of ports plus one) in the Touchstone file.  Because a Touchstone file requires at least one port, the Number_of_terminals value shall be 2 or greater.

Example:

Number_of_terminals = 3

Terminal line rules:

The terminal lines shall appear after the Number_of_terminals subparameter and before the [End Interconnect Model] keyword.

Terminal lines are of the following form, with each identifier separated by whitespace:

<Terminal_number> <Terminal_type> <Terminal_type_qualifier> <Qualifier_entry> [Aggressor_Only]

Terminal_number

The Terminal_number is the identifier for a specific terminal. The value shall be 1 or greater and less than or equal to the Number_of_terminals. The same Terminal_number shall not appear more than once for a given Interconnect Model.

For File_IBIS-ISS, the Terminal_number entry shall match the IBIS-ISS terminal (node) position.  The Terminal_number entries may be listed in any order as long as there are no duplicate entries. Each IBIS-ISS terminal shall have terminal line entry.

For File_TS, the Terminal_number entry shall match the Touchstone file port number or reference terminal line, as shown below.  The Terminal_number entries may be listed in any order as long as there are no duplicate entries.  The terminal line for Terminal_number N+1 is required as a reference terminal for each port and shall be connected to a rail terminal in the Interconnect Model.  At least one other terminal line entry is required.

- Terminal_number	Port

- 1                    		1

- 2                         	2

- …

- N                       	N

- N+1			Reference terminal for the Touchstone file

For Touchstone files, each unused port and its corresponding Terminal_number shall be terminated simulation with a resistor whose value corresponds to the Unused_port_termination subparameter entry. The resistor is connected to the model’s reference terminal.

Terminal_type
The Terminal_type is a string that identifies whether the terminal is a reference, supply or I/O terminal and whether the terminal is connected at the buffer, die pad, or pin level.  (Note that “I/O” in this context is a synonym for “signal”, as opposed to “supply” or “”rail”; it is not intended to imply model type as used in the “Model_type” subparameter).  Furthermore, if the terminal is connected to a buffer supply rail, the Terminal_type identifies to which specific buffer rail the terminal is connected.  The Terminal_type shall be one of the following:

- Pin_I/O

- Pad_I/O

- Buffer_I/O

- Pin_Rail

- Pad_Rail

- Buffer_Rail

- Pullup_ref

- Pulldown_ref

- Power_clamp_ref

- Gnd_clamp_ref

- Ext_ref

- A_gnd

Buffer_I/O, Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref, Ext_ref and Buffer_Rail are terminals of an Interconnect Model that connect directly to I/O buffers.

Pad_I/O and Pad_Rail are terminals that are at the die pad interface.

Pin_I/O and Pin_Rail are terminals that are at the pin interface that can connect the package to the PCB.

The Terminal_types Buffer_I/O, Pad_I/O and Pin_I/O are used only for any single terminal of a buffer described by the [Model] keyword and for any Model_type subparameter listed in Section 5, Table 1.  The Model_types Series and *_diff are used for two-terminal configurations, and their terminals require two separate Buffer_I/O, Pad_I/O or Pin_I/O Terminal_type lines.

Terminal_type A_gnd defines a connection to the EDA tool’s ideal ground node (also known as node 0 in SPICE).  The A_gnd can be at any interface.

Terminal_type A_gnd is not required under File_TS or File_IBIS-ISS.

If present under File_TS, Terminal_type A_gnd may be used only once on the N+1th terminal line.

If present under File_IBIS-ISS, Terminal_type A_gnd may be used any number of times on any of the terminal lines.

Important:  Power-aware simulations may require that Terminal_type A_gnd NOT be used as reference for interconnect models, or that ideal ground (node 0) NOT be used inside IBIS-ISS subcircuits.

Terminal_type_qualifier 
The Terminal_type_qualifier is a string that identifies the association between a terminal and a specific pin_name, signal_name, bus_label, or pad_name.  Only certain Terminal_types may be used with pad_names, pin_names, signal_names, or bus_labels respectively, as outlined in the Connecting Pins, Pads, and Buffer Terminals section below and summarized in Table 41.

Qualifier_entry 
The <Qualifier_entry>, shown in angle brackets, is the name required for the following Terminal_type_qualifiers:

pin_name <pin_name_entry>

signal_name <signal_name_entry>

bus_label <bus_label_entry>

pad_name <pad_name_entry>

Aggressor_Only

The Aggressor_Only entry is optional and is indicated by the string “Aggressor_Only” without the quotation marks.

Multi-line models may describe only a subset of a coupled structure (e.g., a 64-line bus may be described by a four-line model).  As a result, while the interconnects at the edges of the model may induce crosstalk onto other interconnects nearby, being on the edge of the model, they may not themselves experience the full crosstalk impact that the corresponding interconnect experiences in the real, full structure.  The Aggressor_Only column entry is allowed on all terminal locations for I/O terminals to indicate such incomplete coupling. Terminals that include the Aggressor_Only entry may not be suitable to be simulated as victims, as they do not experience the full coupling present in the real physical structure.  If an I/O terminal is not identified as Aggressor_Only, then the interconnect to that I/O terminal includes coupling to all interconnections deemed necessary for coupled signal analysis.  If a particular terminal is identified as Aggressor_Only, then the entire path of the associated pin_name is to be considered Aggressor_Only.

Connecting Pins, Pads and Buffer Terminals

Terminal lines describe the IBIS-ISS node or Touchstone port that each terminal should be connected to. Terminals may be at pins, die pads or the buffer.  The arrangement of the terminal line entries (columns) is described below.

- The first column, Terminal_number, contains an integer between 1 and the Number_of_terminals that describes the ordinal (positional) number of the IBIS-ISS node in the [Interconnect Model] subcircuit or Touchstone file port. The second column is Terminal_type, the third column is Terminal_type_qualifier, the fourth column is Qualifier_entry and there is an optional fifth column “Aggressor_Only”

- The second column, Terminal_type, determines if the terminal is at a pin, die pad or buffer.

  - For I/O connections

    - At pins, die pads or buffers

      - Terminal_type can be Pin_I/O, Pad_I/O and Buffer_I/O

      - Terminal_type_qualifier shall be pin_name.

      - Qualifier_entry shall be the pin_name of an I/O pin.

  - For rail connections

    - At pins

      - Terminal_type shall be Pin_Rail

      - Terminal_type_qualifier shall be one of the following

        - pin_name

          - Qualifier_entry shall be a rail pin_name

        - signal_name

          - Qualifier_entry shall be a rail signal_name

        - bus_label

          - Qualifier_entry shall be a bus_label

    - At die pads

      - Terminal_type shall be Pad_Rail

      - Terminal_type_qualifier shall be

        - signal_name

          - Qualifier_entry shall be a rail signal_name

        - bus_label

          - Qualifier_entry shall be a bus_label

        - pad_name

          - Qualifier_entry shall be the pad_name of a rail pad

    - At buffers

      - Terminal_type shall be Buffer_Rail or any of the five *_ref terminals associated with an I/O buffer below

      - Buffer_Rail Terminal_type_qualifier shall be

        - signal_name

          - Qualifier_entry shall be a rail signal_name

        - bus_label

          - Qualifier_entry shall be a bus_label

      - Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref or Ext_ref Terminal_type_qualifiers shall be

        - pin_name

          - Qualifier_entry shall be the I/O buffer pin_name

    - At any interface

      - Terminal_type A_gnd is available at any interface and without any Terminal_type qualifier

Table 41 summarizes the rules described above.

Table 41 – Allowed Terminal_type Associations1

| Terminal_type | Terminal_type_qualifier |  |  |  | Aggressor_Only |
| --- | --- | --- | --- | --- | --- |
|  | pin_name | signal_name | bus_label | pad_name |  |
| Pin_I/O | X |  |  |  | A |
| Pad_I/O | X |  |  |  | A |
| Buffer_I/O | X |  |  |  | A |
| Pin_Rail | Y | Y | Y |  |  |
| Pad_Rail |  | Y | Y | Z |  |
| Buffer_Rail |  | Y | Y |  |  |
| Pullup_ref | X |  |  |  |  |
| Pulldown_ref | X |  |  |  |  |
| Power_clamp_ref | X |  |  |  |  |
| Gnd_clamp_ref | X |  |  |  |  |
| Ext_ref | X |  |  |  |  |
| A_gnd |  |  |  |  |  |

Notes

- In the table, “X” refers to I/O pin names.  “Y” and “Z” are POWER and GND names. The letter “A” designates "Aggressor_Only".

Three classes of pins are defined for a component: signal pins, supply pins and no-connect pins. Supply pins have a model_name of either POWER or GND. No-connect pins have model_name NC. All other pins are classified as signal pins. Interconnect Models defined in this section assume that there is one Buffer_I/O terminal and one die pad for each signal pin.  Pins are assumed to use the names listed under the first column of the [Pin] keyword (the pin_name column).

The model of an I/O buffer has supply terminals in addition to the Buffer_I/O. These supply (or rail) terminals can be Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref and/or Ext_ref.  The Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref and/or Ext_ref terminals of a buffer are associated either with a bus_label under the [Pin Mapping] keyword or a signal_name under the [Pin] keyword. These terminals can be connected to Interconnect Models one of two ways:

- By specifying a unique interconnect terminal for each I/O buffer Pullup_ref, Pulldown_ref, Power_clamp_ref, Gnd_clamp_ref and/or Ext_ref terminal

- By assuming that all I/O buffer supply terminals connected to a supply signal_name or bus_label are shorted together. This is done by specifying a unique terminal (of Terminal_type Buffer_Rail) for all I/O buffer terminals that are connected to a specific signal_name or bus_label on at least one supply pin.

Pads are the location of the interface between the die and the package (also called the die pad interface). Interconnect Models can either be between the pins of a component and the I/O buffers, or they can be split into models between the pins of a component and the pads of the die and models between the pads of the die and the I/O buffers. There is exactly one Pad (of Terminal_type Pad_I/O) for each signal pin. There can be any number of pads (of Terminal_type Pad_Rail) for each signal_name or bus_label on supply pins. If Interconnect Models of supply (rail) networks are split between Pin/Pad and Pad/Buffer models, then the interface of supply connections at the die pad interface can be handled in one of two ways:

- By defining a list of die supply pads, and specifying terminals for some or all of the die supply pads that are connected to a bus_label or signal_name on at least one supply pin.

- By assuming that all supply pads connected to a supply signal_name or bus_label are shorted together. This is done by specifying a unique terminal (of Terminal_type Pad_Rail) for all pads that are connected to a specific signal_name on at least one supply pin.

Pins may be terminals of the Interconnect Model that connect directly to a printed circuit board or other type of system connection to an IBIS component. Pins can be signal pins (Pin_I/O), or supply pins (Pin_Rail). An Interconnect Model can connect supply pins in one of two ways:

- By specifying terminals for some or all of the supply pins.

- By assuming that all supply pins connected to a supply signal_name or bus_label are shorted together. This is done by specifying a unique terminal (of Terminal_type Pin_Rail) for all pins that are connected to a specific signal_name on at least one supply pin.

The terminals of an Interconnect Model may be located at pins and die pads, pins and buffers, or die pads and buffers. A single Interconnect Model shall not have terminals at pins, die pads and buffers simultaneously.

Any one pin name shall not be included in more than one terminal of an Interconnect Model.

Any one die pad name shall not be included in more than one terminal of an Interconnect Model.

Any one buffer terminal name shall not be included in more than one terminal of an Interconnect Model.

Examples:

| All examples show a [Interconnect Model Set] for grouping of the

|  [Interconnect Model] descriptions that can be referenced

|

| Naming convention for [Interconnect Model Set] examples is below

|  ([Interconnect Model] may show additional details)

|

| Full        – Includes all I/O pins

| A1 or A1_A3 – Designated pin or pins

| TS          - Touchstone representation

| ISS         - IBIS-ISS representation

| PDN         - Includes power delivery network, can also be PU and PD

| IO          - Only if modified differently than PDN below for buf_pad_pin

| buf_pad_pin – Includes models for buf_pad, pad_pin; if missing, buf_pad

| sn          - Uses signal_name; if missing assumes pin_name

| bl          - Uses bus_label; if missing assumes pin_name

| pn          - Uses pad_name; if missing assumes pin_name

| XTALK       - Cross talk analysis (coupled nets may include Aggressor_Only)

| Examples 1 – 11 apply to the configuration below:

[Pin] signal_name model_name      R_pin   L_pin   C_pin

A1    DQ1         DQ

A2    DQ2         DQ

A3    DQ3         DQ

D1    DQS+        DQS

D2    DQS-        DQS

P1    VDD         POWER

P2    VDD         POWER

P3    VDD         POWER

P4    VDD         POWER

P5    VDD         POWER

G1    VSS         GND

G2    VSS         GND

G3    VSS         GND

G4    VSS         GND

[Diff Pin] inv_pin  vdiff  tdelay_typ tdelay_min tdelay_max

D1         D2       NA     NA         NA         NA

[Die Supply Pads]  signal_name bus_label

VDD1               VDD

VDD2               VDD

VDD3               VDD

VSS1               VSS

VSS2               VSS

[Pin Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref ext_ref

A1            VSS          VDD        NC            NC              NC

A2            VSS          VDD        NC            NC              NC

A3            VSS          VDD        NC            NC              NC

D1            VSS          VDD        NC            NC              NC

D2            VSS          VDD        NC            NC              NC

| Pins below are optional per [Pin Mapping] rules

P1            NC           VDD

P2            NC           VDD

P3            NC           VDD

P4            NC           VDD

P5            NC           VDD

G1            VSS          NC

G2            VSS          NC

G3            VSS          NC

G4            VSS          NC

|******************************************************************************

| Example 1: Terminals for full IBIS-ISS component with PDN, as depicted below.

|

[Interconnect Model Set]      Full_ISS_PDN_1

|-----

[Interconnect Model]          Full_ISS_buf_pin_1

File_IBIS-ISS   full_buf_pin_1.iss           full_buf_pin_typ

Number_of_terminals = 29

1  Pin_I/O      pin_name      A1    |  DQ1         DQ

2  Pin_I/O      pin_name      A2    |  DQ2         DQ

3  Pin_I/O      pin_name      A3    |  DQ3         DQ

4  Pin_I/O      pin_name      D1    |  DQS+        DQS

5  Pin_I/O      pin_name      D2    |  DQS-        DQS

6  Pin_Rail     pin_name      P1    |  VDD         POWER

7  Pin_Rail     pin_name      P2    |  VDD         POWER

8  Pin_Rail     pin_name      P3    |  VDD         POWER

9  Pin_Rail     pin_name      P4    |  VDD         POWER

10 Pin_Rail     pin_name      P5    |  VDD         POWER

11 Pin_Rail     pin_name      G1    |  VSS         GND

12 Pin_Rail     pin_name      G2    |  VSS         GND

13 Pin_Rail     pin_name      G3    |  VSS         GND

14 Pin_Rail     pin_name      G4    |  VSS         GND

15 Buffer_I/O   pin_name      A1    |  DQ1         DQ

16 Buffer_I/O   pin_name      A2    |  DQ2         DQ

17 Buffer_I/O   pin_name      A3    |  DQ3         DQ

18 Buffer_I/O   pin_name      D1    |  DQS+        DQS

19 Buffer_I/O   pin_name      D2    |  DQS-        DQS

20 Pullup_ref   pin_name      A1    |  DQ1         DQ

21 Pullup_ref   pin_name      A2    |  DQ2         DQ

22 Pullup_ref   pin_name      A3    |  DQ3         DQ

23 Pullup_ref   pin_name      D1    |  DQS+        DQS

24 Pullup_ref   pin_name      D2    |  DQS-        DQS

25 Pulldown_ref pin_name      A1    |  DQ1         DQ

26 Pulldown_ref pin_name      A2    |  DQ2         DQ

27 Pulldown_ref pin_name      A3    |  DQ3         DQ

28 Pulldown_ref pin_name      D1    |  DQS+        DQS

29 Pulldown_ref pin_name      D2    |  DQS+        DQS

[End Interconnect Model]

[End Interconnect Model Set]

Figure 51 – Electrical Connections for Full Buffer Pin Model with Power Routing

Figure 52 – Electrical Terminals for Full Buffer Pin Model with Power Routing

|******************************************************************************

| Example 2: Same as Example 1 except the PDN networks are simplified with

|   signal_name qualifiers to create a pair of POWER terminals and a pair

|   of GND terminals

[Interconnect Model Set]      Full_ISS_PDN_sn_2

|-----

[Interconnect Model]          Full_ISS_buf_pin_2

File_IBIS-ISS   full_buf_pin.iss             full_buf_pin_2_typ

Number_of_terminals = 14

1  Pin_I/O      pin_name      A1    |  DQ1         DQ

2  Pin_I/O      pin_name      A2    |  DQ2         DQ

3  Pin_I/O      pin_name      A3    |  DQ3         DQ

4  Pin_I/O      pin_name      D1    |  DQS+        DQS

5  Pin_I/O      pin_name      D2    |  DQS-        DQS

|

| POWER and GND terminals with signal_names

|

6  Pin_Rail     signal_name   VDD   |  VDD         POWER

7  Pin_Rail     signal_name   VSS   |  VSS         GND

|

8  Buffer_I/O   pin_name      A1    |  DQ1         DQ

9  Buffer_I/O   pin_name      A2    |  DQ2         DQ

10 Buffer_I/O   pin_name      A3    |  DQ3         DQ

11 Buffer_I/O   pin_name      D1    |  DQS+        DQS

12 Buffer_I/O   pin_name      D2    |  DQS-        DQS

|

| POWER and GND terminals with signal_names

|

13 Buffer_Rail  signal_name   VDD   |  VDD         POWER

14 Buffer_Rail  signal_name   VSS   |  VSS         GND

|

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 3: Single I/O Touchstone connection with one extra terminal for the

|   N+1 .s2p reference connection terminal; [Interconnect Model Set] keyword

|   stored in touchstone/ts_sets.ims

[Interconnect Model Set]      A1_TS

|-----

[Interconnect Model]          A1_TS_buf_pin

File_TS         dq_ts_buf_pin.s2p

Number_of_terminals = 3

1  Pin_I/O      pin_name      A1

2  Buffer_I/O   pin_name      A1

3  Pulldown_ref pin_name      A1  | VSS reference for .s2p file

| Rail connections to Buffer_I/O through

| [Pin Mapping] or a [Model] reference

| voltage used if no external rails

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 4: Single I/O pin documenting both IBIS-ISS and Touchstone files and

|   showing that the File_TS Touchstone N+1 reference connection is to the VSS

|   rail

[Interconnect Model Set]      A1_TS_pad_pin

|-----

[Interconnect Model]          A1_TS_pad_pin

File_TS         dq_ts_pad_pin.s2p

Number_of_terminals = 3

1  Pin_I/O      pin_name      A1

2  Pad_I/O      pin_name      A1

3  Pin_Rail     signal_name   VSS   | VSS is reference for .s2p file

|                                   | Requires Pin_Rail VSS connection

[End Interconnect Model]

[End Interconnect Model Set]

[Interconnect Model Set]      A1_ISS_buf_pad

|-----

[Interconnect Model]          A1_ISS_buf_pad

File_IBIS-ISS   dq_iss_buf_pad.iss           DQ_buf_pad_typ

Number_of_terminals = 3

1  Pad_I/O      pin_name      A1

2  Buffer_I/O   pin_name      A1

3  Pulldown_ref pin_name      A1

|

| [Pin Mapping] connections used to connect external rails; or default

|   internal [Model] rails used if no external rails

|

[End Interconnect Model]

[End Interconnect Model Set]

| As an alternative formulation, the [Interconnect Model]s in two

| Interconnect Model Set]s could be combined into one [Interconnect Model

| Set] describing the full connection of A1 from buffer to pin

|

|******************************************************************************

| Example 5: Full I/O IBIS-ISS configuration with PDN terminals in a separate

|   [Interconnect Model Set]; when connected the individual Pin_Rail

|   terminals G1-G4 become shorted together with common VSS reference

[Interconnect Model Set]     Full_ISS_buf_pin_IO_1

|-----

[Interconnect Model]         Full_ISS_buf_pin_IO

File_IBIS-ISS   full_buf_pin.iss            full_buf_pin_typ

Number_of_terminals = 13

1  Pin_I/O      pin_name      A1   |   DQ1         DQ

2  Pin_I/O      pin_name      A2   |   DQ2         DQ

3  Pin_I/O      pin_name      A3   |   DQ3         DQ

4  Pin_I/O      pin_name      D1   |   DQS+        DQS

5  Pin_I/O      pin_name      D2   |   DQS-        DQS

6  Buffer_I/O   pin_name      A1   |   DQ1         DQ

7  Buffer_I/O   pin_name      A2   |   DQ2         DQ

8  Buffer_I/O   pin_name      A3   |   DQ3         DQ

9  Buffer_I/O   pin_name      D1   |   DQS+        DQS

10 Buffer_I/O   pin_name      D2   |   DQS-        DQS

11 Pin_Rail     signal_name   VSS  |   VSS         GND

[End Interconnect Model]

[End Interconnect Model Set]

[Interconnect Model Set]      Full_ISS_buf_pin_PDN_1

|-----

[Interconnect Model]          Full_ISS_buf_pin_PDN_1

File_IBIS-ISS   full_ISS_buf_pin_pdn.iss      full_buf_pin_PDN_typ

Number_of_terminals = 19

1  Pin_Rail     pin_name      P1    |  VDD         POWER

2  Pin_Rail     pin_name      P2    |  VDD         POWER

3  Pin_Rail     pin_name      P3    |  VDD         POWER

4  Pin_Rail     pin_name      P4    |  VDD         POWER

5  Pin_Rail     pin_name      P5    |  VDD         POWER

6  Pullup_ref   pin_name      A1    |  DQ1         DQ

7  Pullup_ref   pin_name      A2    |  DQ2         DQ

8  Pullup_ref   pin_name      A3    |  DQ3         DQ

9  Pullup_ref   pin_name      D1    |  DQS+        DQS

10 Pullup_ref   pin_name      D2    |  DQS-        DQS

11 Pin_Rail     pin_name      G1    |  VSS         GND

12 Pin_Rail     pin_name      G2    |  VSS         GND

13 Pin_Rail     pin_name      G3    |  VSS         GND

14 Pin_Rail     pin_name      G4    |  VSS         GND

15 Pulldown_ref pin_name      A1    |  DQ1         DQ

16 Pulldown_ref pin_name      A2    |  DQ2         DQ

17 Pulldown_ref pin_name      A3    |  DQ3         DQ

18 Pulldown_ref pin_name      D1    |  DQS+        DQS

19 Pulldown_ref pin_name      D2    |  DQS-        DQS

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 6: Full IBIS-ISS IOs and separate PDNs, all with buf_pad and

|   pad_pin [Interconnect Model]s in separate [Interconnect Model]s

[Interconnect Model Set]      Full_ISS_buf_pad_pin_PDN_4

|-----

[Interconnect Model]          Full_ISS_pad_pin_IO

File_IBIS-ISS   full_pad_pin_io.iss          full_pad_pin_IO_typ

Number_of_terminals = 11

1  Pin_I/O      pin_name      A1    |  DQ1         DQ

2  Pin_I/O      pin_name      A2    |  DQ2         DQ

3  Pin_I/O      pin_name      A3    |  DQ3         DQ

4  Pin_I/O      pin_name      D1    |  DQS+        DQS

5  Pin_I/O      pin_name      D2    |  DQS-        DQS

|

6  Pad_I/O      pin_name      A1    |  DQ1         DQ

7  Pad_I/O      pin_name      A2    |  DQ2         DQ

8  Pad_I/O      pin_name      A3    |  DQ3         DQ

9  Pad_I/O      pin_name      D1    |  DQS+        DQS

10 Pad_I/O      pin_name      D2    |  DQS-        DQS

11 Pin_Rail     signal_name   VSS

[End Interconnect Model]

[Interconnect Model]          Full_ISS_buf_pad_IO

File_IBIS-ISS   full_buf_pad_io.iss          full_buf_pad_IO_typ

Number_of_terminals = 11

1  Pad_I/O      pin_name      A1    |  DQ1         DQ

2  Pad_I/O      pin_name      A2    |  DQ2         DQ

3  Pad_I/O      pin_name      A3    |  DQ3         DQ

4  Pad_I/O      pin_name      D1    |  DQS+        DQS

5  Pad_I/O      pin_name      D2    |  DQS-        DQS

|

6  Buffer_I/O   pin_name      A1    |  DQ1         DQ

7  Buffer_I/O   pin_name      A2    |  DQ2         DQ

8  Buffer_I/O   pin_name      A3    |  DQ3         DQ

9  Buffer_I/O   pin_name      D1    |  DQS+        DQS

10 Buffer_I/O   pin_name      D2    |  DQS-        DQS

11 Buffer_Rail  signal_name   VSS

[End Interconnect Model]

[Interconnect Model]          Full_ISS_pad_pin_PDN

File_IBIS-ISS   full_iss_pad_pin_pdn.iss     full_iss_pad_pin_PDN_typ

Number_of_terminals = 14

1  Pin_Rail     pin_name      P1    |  VDD         POWER

2  Pin_Rail     pin_name      P2    |  VDD         POWER

3  Pin_Rail     pin_name      P3    |  VDD         POWER

4  Pin_Rail     pin_name      P4    |  VDD         POWER

5  Pin_Rail     pin_name      P5    |  VDD         POWER

|

6  Pad_Rail     pad_name      VDD1  |  VDD         POWER

7  Pad_Rail     pad_name      VDD2  |  VDD         POWER

8  Pad_Rail     pad_name      VDD3  |  VDD         POWER

|

9  Pin_Rail     pin_name      G1    |  VSS         GND

10 Pin_Rail     pin_name      G2    |  VSS         GND

11 Pin_Rail     pin_name      G3    |  VSS         GND

12 Pin_Rail     pin_name      G4    |  VSS         GND

|

13 Pad_Rail     pad_name      VSS1  |  VSS         GND

14 Pad_Rail     pad_name      VSS2  |  VSS         GND

[End Interconnect Model]

[Interconnect Model]          Full_ISS_buf_pad_PDN

File_IBIS-ISS   full_iss_buf_pad_pdn.iss     full_iss_buf_pad_PDN_typ

Number_of_terminals = 15

1  Pad_Rail     pad_name      VDD1  |  VDD         POWER

2  Pad_Rail     pad_name      VDD2  |  VDD         POWER

3  Pad_Rail     pad_name      VDD3  |  VDD         POWER

|

4  Pullup_ref   pin_name      A1    |  DQ1         DQ

5  Pullup_ref   pin_name      A2    |  DQ2         DQ

6  Pullup_ref   pin_name      A3    |  DQ3         DQ

7  Pullup_ref   pin_name      D1    |  DQS+        DQS

8  Pullup_ref   pin_name      D2    |  DQS-        DQS

|

9  Pad_Rail     pad_name      VSS1  |  VSS         GND

10 Pad_Rail     pad_name      VSS2  |  VSS         GND

|

11 Pulldown_ref pin_name      A1    |  DQ1         DQ

12 Pulldown_ref pin_name      A2    |  DQ2         DQ

13 Pulldown_ref pin_name      A3    |  DQ3         DQ

14 Pulldown_ref pin_name      D1    |  DQS+        DQS

15 Pulldown_ref pin_name      D2    |  DQS-        DQS

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 7: Full IBIS-ISS model with I/O only [Interconnect Model] and a

|   separate PDN [Interconnect Model] with signal_name qualifiers

[Interconnect Model Set]     Full_ISS_PDN_sn_5

|-----

[Interconnect Model]         Full_ISS_buf_pin_IO

File_IBIS-ISS   full_buf_pin.iss            full_buf_pin_typ

Number_of_terminals = 11

1  Pin_I/O      pin_name      A1   |   DQ1         DQ

2  Pin_I/O      pin_name      A2   |   DQ2         DQ

3  Pin_I/O      pin_name      A3   |   DQ3         DQ

4  Pin_I/O      pin_name      D1   |   DQS+        DQS

5  Pin_I/O      pin_name      D2   |   DQS-        DQS

6  Buffer_I/O   pin_name      A1   |   DQ1         DQ

7  Buffer_I/O   pin_name      A2   |   DQ2         DQ

8  Buffer_I/O   pin_name      A3   |   DQ3         DQ

9  Buffer_I/O   pin_name      D1   |   DQS+        DQS

10 Buffer_I/O   pin_name      D2   |   DQS-        DQS

11 Pin_Rail     signal_name   VSS

[End Interconnect Model]

[Interconnect Model]          Full_ISS_buf_pin_PDN_2

File_IBIS-ISS   full_iss_buf_pin_pdn_2.iss   full_iss_buf_pad_PDN_2

Number_of_terminals = 4

1  Pin_Rail     signal_name   VDD   |  VDD         POWER

2  Buffer_Rail  signal_name   VDD   |  VDD         POWER

3  Pin_Rail     signal_name   VSS   |  VSS         GND

4  Buffer_Rail  signal_name   VSS   |  VSS         GND

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 8: Same full IBIS-ISS model with PDN as in Example 7, but with the

|   [Interconnect Model]s describing buf_pad and pad_pin connections

|   separately

[Interconnect Model Set]      Full_ISS_buf_pad_pin_PDN_sn_6

|-----

[Interconnect Model]          Full_ISS_pad_pin_IO

File_IBIS-ISS   full_pad_pin_io.iss          full_pad_pin_IO_typ

Number_of_terminals = 11

1  Pin_I/O      pin_name      A1    |  DQ1         DQ

2  Pin_I/O      pin_name      A2    |  DQ2         DQ

3  Pin_I/O      pin_name      A3    |  DQ3         DQ

4  Pin_I/O      pin_name      D1    |  DQS+        DQS

5  Pin_I/O      pin_name      D2    |  DQS-        DQS

|

6  Pad_I/O      pin_name      A1    |  DQ1         DQ

7  Pad_I/O      pin_name      A2    |  DQ2         DQ

8  Pad_I/O      pin_name      A3    |  DQ3         DQ

9  Pad_I/O      pin_name      D1    |  DQS+        DQS

10 Pad_I/O      pin_name      D2    |  DQS-        DQS

11 Buffer_Rail  signal_name   VSS

[End Interconnect Model]

[Interconnect Model]          Full_ISS_buf_pad_IO

File_IBIS-ISS   full_buf_pad_io.iss          full_buf_pad_IO_typ

Number_of_terminals = 11

1  Pad_I/O      pin_name      A1    |  DQ1         DQ

2  Pad_I/O      pin_name      A2    |  DQ2         DQ

3  Pad_I/O      pin_name      A3    |  DQ3         DQ

4  Pad_I/O      pin_name      D1    |  DQS+        DQS

5  Pad_I/O      pin_name      D2    |  DQS-        DQS

|

6  Buffer_I/O   pin_name      A1    |  DQ1         DQ

7  Buffer_I/O   pin_name      A2    |  DQ2         DQ

8  Buffer_I/O   pin_name      A3    |  DQ3         DQ

9  Buffer_I/O   pin_name      D1    |  DQS+        DQS

10 Buffer_I/O   pin_name      D2    |  DQS-        DQS

11 Pin_Rail     signal_name   VSS

[End Interconnect Model]

[Interconnect Model]          Full_ISS_pad_pin_PDN_3

File_IBIS-ISS   full_iss_pad_pin_pdn_3.iss   full_iss_pad_pin_pdn_3

Number_of_terminals = 4

1  Pin_Rail     signal_name   VDD   |  VDD         POWER

2  Pad_Rail     signal_name   VDD   |  VDD         POWER

3  Pin_Rail     signal_name   VSS   |  VSS         GND

4  Pad_Rail     signal_name   VSS   |  VSS         GND

[End Interconnect Model]

[Interconnect Model]          Full_ISS_buf_pad_PDN_3

File_IBIS-ISS   full_iss_buf_pad_pdn_3.iss   full_iss_buf_pad_pdn_3

Number_of_terminals = 4

1  Buffer_Rail  signal_name   VDD   |  VDD         POWER

2  Pad_Rail     signal_name   VDD   |  VDD         POWER

3  Buffer_Rail  signal_name   VSS   |  VSS         GND

4  Pad_Rail     signal_name   VSS   |  VSS         GND

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 9: Same full IBIS-ISS configuration with PDN as in Example 8, except

|   that I/O connections are direct from buf_pin while the PDN connections are

|   from buf_pad and pad_pin using the signal_name qualifier

[Interconnect Model Set]     Full_ISS_IO_buf_pad_pin_PDN_sn_7

|-----

[Interconnect Model]         Full_ISS_buf_pin_IO

File_IBIS-ISS   full_buf_pin.iss            full_buf_pin_typ

Number_of_terminals = 11

1  Pin_I/O      pin_name      A1   |   DQ1         DQ

2  Pin_I/O      pin_name      A2   |   DQ2         DQ

3  Pin_I/O      pin_name      A3   |   DQ3         DQ

4  Pin_I/O      pin_name      D1   |   DQS+        DQS

5  Pin_I/O      pin_name      D2   |   DQS-        DQS

6  Buffer_I/O   pin_name      A1   |   DQ1         DQ

7  Buffer_I/O   pin_name      A2   |   DQ2         DQ

8  Buffer_I/O   pin_name      A3   |   DQ3         DQ

9  Buffer_I/O   pin_name      D1   |   DQS+        DQS

10 Buffer_I/O   pin_name      D2   |   DQS-        DQS

11 Pin_Rail     signal_name   VSS

[End Interconnect Model]

[Interconnect Model]          Full_ISS_pad_pin_PDN_3

File_IBIS-ISS   full_iss_pad_pin_pdn_3.iss   full_iss_pad_pin_pdn_3

Number_of_terminals = 4

1  Pin_Rail     signal_name   VDD   |  VDD         POWER

2  Pad_Rail     signal_name   VDD   |  VDD         POWER

3  Pin_Rail     signal_name   VSS   |  VSS         GND

4  Pad_Rail     signal_name   VSS   |  VSS         GND

[End Interconnect Model]

[Interconnect Model]          Full_ISS_buf_pad_PDN_3

File_IBIS-ISS   full_iss_buf_pad_pdn_3.iss   full_iss_buf_pad_pdn_3

Number_of_terminals = 4

1  Buffer_Rail  signal_name   VDD   |  VDD         POWER

2  Pad_Rail     signal_name   VDD   |  VDD         POWER

3  Buffer_Rail  signal_name   VSS   |  VSS         GND

4  Pad_Rail     signal_name   VSS   |  VSS         GND

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 10: Terminals A1_A3 set up for and IBIS-ISS connections with coupling

|   for cross-talk analysis – Aggressor_Only terminals at the Buffer are

|   designated

[Interconnect Model Set]      A1_A3_DQ_TS_XTALK

|-----

[Interconnect Model]          A1_A3_DQ_TS_buf_pin_XTALK

File_TS  dq_buf_pin_xtalk.s6p

Number_of_terminals = 7

1  Pin_I/O      pin_name      A1    Aggressor_Only

2  Buffer_I/O   pin_name      A1    Aggressor_Only

3  Pin_I/O      pin_name      A2

4  Buffer_I/O   pin_name      A2

5  Pin_I/O      pin_name      A3    Aggressor_Only

6  Buffer_I/O   pin_name      A3    Aggressor_Only

7  Pulldown_ref pin_name      A1

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 11: Same as Example 10, but with a PDN network added

[Interconnect Model Set]      A1_A3_DQ_TS_XTALK_ISS_PDN

|-----

[Interconnect Model]          A1_A3_DQ_TS_buf_pin_XTALK

File_TS  dq_buf_pin_xtalk.s6p

Number_of_terminals = 7

1  Pin_I/O      pin_name      A1    Aggressor_Only

2  Buffer_I/O   pin_name      A1    Aggressor_Only

3  Pin_I/O      pin_name      A2

4  Buffer_I/O   pin_name      A2

5  Pin_I/O      pin_name      A3    Aggressor_Only

6  Buffer_I/O   pin_name      A3    Aggressor_Only

7  Pulldown_ref pin_name      A1

[End Interconnect Model]

[Interconnect Model]          Full_ISS_buf_pin_PDN_2

File_IBIS-ISS   full_iss_buf_pin_pdn_2.iss   full_iss_buf_pad_PDN_2

Number_of_terminals = 4

1  Pin_Rail     signal_name   VDD   |  VDD         POWER

2  Buffer_Rail  signal_name   VDD   |  VDD         POWER

3  Pin_Rail     signal_name   VSS   |  VSS         GND

4  Buffer_Rail  signal_name   VSS   |  VSS         GND

[End Interconnect Model]

[End Interconnect Model Set]

|******************************************************************************

| Example 12 applies to the configuration below

[Pin] signal_name model_name      R_pin   L_pin   C_pin

A1    DQ1         DQ

A2    DQ2         DQ

A3    DQ3         DQ

A4    DQ4         DQ

P1    VDD         POWER

P2    VDD         POWER

G1    VSS         GND

G2    VSS         GND

[Bus Label] signal_name

VDD1        VDD

VDD2        VDD

[Pin Mapping] pulldown_ref pullup_ref gnd_clamp_ref power_clamp_ref ext_ref

A1            VSS           VDD1        NC            NC              NC

A2            VSS           VDD1        NC            NC              NC

A3            VSS           VDD2        NC            NC              NC

A4            VSS           VDD2        NC            NC              NC

| Entries below may optionally be deleted and replaced with [Bus Label] per

| [Bus Label] and [Pin Mapping] rules

P1            NC            VDD1        NC            NC              NC

P2            NC            VDD2        NC            NC              NC

G1            VSS           NC          NC            NC              NC

G2            VSS           NC          NC            NC              NC

|******************************************************************************

| Example 12: Full IBIS-ISS configuration with PDN described using both

|   bus_label and signal_name qualifiers for the Rails

[Interconnect Model Set]      Full_ISS_IO_PDN_bl_sn_6

|-----

[Interconnect Model]          Full_ISS_buf_pin_IO_4

File_IBIS-ISS   full_iss_buf_pin_io_4.iss    full_iss_buf_pin_IO_4_typ

Number_of_terminals = 9

1  Pin_I/O      pin_name      A1    |  DQ1         DQ

2  Pin_I/O      pin_name      A2    |  DQ2         DQ

3  Pin_I/O      pin_name      A3    |  DQ3         DQ

4  Pin_I/O      pin_name      A4    |  DQ4         DQ

5  Buffer_I/O   pin_name      A1    |  DQ1         DQ

6  Buffer_I/O   pin_name      A2    |  DQ2         DQ

7  Buffer_I/O   pin_name      A3    |  DQ3         DQ

8  Buffer_I/O   pin_name      A4    |  DQ4         DQ

9  Pin_Rail     signal_name   VSS

[End Interconnect Model]

[Interconnect Model]          Full_ISS_PDN_bl_sn

File_IBIS-ISS   buf_pin_pdn.iss      buf_pin_PDN_typ

Number_of_terminals = 5

1  Pin_Rail     signal_name   VDD   |  VDD         POWER

2  Pin_Rail     signal_name   VSS   |  VSS         GND

|

3  Buffer_Rail  bus_label     VDD1  |  VDD         POWER

4  Buffer_Rail  bus_label     VDD2  |  VDD         POWER

5  Buffer_Rail  signal_name   VSS   |  VSS         GND

[End Interconnect Model]

[End Interconnect Model Set]

| The EDA tool connects the terminals and pins as follows:

|

| 1 Pins P1 and P2

| 2 Pins G1 and G2

| 3 Pullup_ref of buffers A1 and A2

| 4 Pullup_ref of buffers A3 and A4

| 5 Pulldown_ref of buffers A1, A2, A3 and A4

|******************************************************************************

Keyword: 	[End Interconnect Model]

Required: 	Yes, for each instance of the [Interconnect Model] keyword

Description: 	Indicates the end of the Interconnect Model data.

Example:

[End Interconnect Model]