13.7  ADDITIONAL EMD MODEL EXAMPLES

Examples:

The example below for a simplified DIMM includes pins at the EMD interface and at the designator interfaces of two memory components.  Three EMD Groups provide EMD Model options including one option with no crosstalk and two options with crosstalk included.  The EMD Groups with crosstalk included show use of IBIS-ISS or Touchstone files, and the rail connections are modeled in separate EMD Sets that are included in each EMD Group.  The rail terminals are connected by either bus_label or signal_name.  Bus_labels are used to split the VDD rail into VDD1 and VDD2 buses.  While only one VSS rail is shown, separate VSS rails could exist (for example, VSS1 and VSS2) and would be included by using bus_label syntax.

[Begin EMD] DIMM

[Number of EMD Pins] 9

[EMD Pin List] signal_name  signal_type  bus_label

A1             	DQ0

A2             	DQ1

A3             	DQ2

A4             DQ3

P1             	VDD          POWER        VDD1

P2             	VDD          POWER        VDD2

G1             	VSS          GND

[End EMD Pin List]

[EMD Parts]

ACME_MEM  mem.ibs  MEMx4

[End EMD Parts]

[EMD Designator List]

U1  ACME_MEM

U2  ACME_MEM

[End EMD Designator List]

[Designator Pin List] signal_name  signal_type  bus_label

U1.1                  		VDD          		POWER        	VDD1

U1.2                  		VDD          		POWER        VDD2

U1.3                  		VSS          		GND

U1.4                  		VSS          		GND

U1.5                  		DQ0

U1.6                  		DQ1

U1.7                  		DQ2

U1.8                  		DQ3

|

U2.1                  VDD          POWER        VDD1

U2.2                  VDD          POWER        VDD2

U2.3                  VSS          GND

U2.4                  VSS          GND

U2.5                  DQ0

U2.6                  DQ1

U2.7                  DQ2

U2.8                  DQ3

[End Designator Pin List]

| EMD Group has no crosstalk modeled and includes the

| rails in the same IBIS-ISS subcircuit

[EMD Group]  All_DQs_No_Coupling_Rails

All_DQs_Uncoupled     NA

[End EMD Group]

| EMD Group models crosstalk with IBIS-ISS subcircuits

[EMD Group]  All_DQs_Aggressor_Options_ISS

All_DQs_Crosstalk_ISS NA

Rails_ISS             NA

[End EMD Group]

| EMD Group models crosstalk with Touchstone files

[EMD Group]  All_DQs_Aggressor_Options_TS

All_DQs_Crosstalk_TS  NA

Rails_TS              NA

[End EMD Group]

[End EMD]             | End of [Begin EMD]

|*************************** EMD Sets *************************

[EMD Set]       All_DQs_Uncoupled

[EMD Model]     DQ0_3

File_IBIS-ISS   DQ.iss        DQ

Number_of_terminals = 20

1  Pin_I/O      pin_name      A1        | DQ0

2  Pin_I/O      pin_name      A2        | DQ1

3  Pin_I/O      pin_name      A3        | DQ2

4  Pin_I/O      pin_name      A4        | DQ3

5  Pin_Rail     signal_name   VDD       | EMD Pins P1 and P2

6  Pin_Rail     signal_name   VSS       | EMD Pin G1

|

7  Pin_I/O      pin_name      U1.5      | DQ0

8  Pin_I/O      pin_name      U1.6      | DQ1

9  Pin_I/O      pin_name      U1.7      | DQ2

10 Pin_I/O      pin_name      U1.8      | DQ3

11 Pin_Rail     bus_label     U1.VDD1   | U1 Pin 1

12 Pin_Rail     bus_label     U1.VDD2   | U1 Pin 2

13 Pin_Rail     signal_name   U1.VSS    | U1 Pins 3 and 4

|

14 Pin_I/O      pin_name      U2.5      | DQ0

15 Pin_I/O      pin_name      U2.6      | DQ1

16 Pin_I/O      pin_name      U2.7      | DQ2

17 Pin_I/O      pin_name      U2.8      | DQ3

18 Pin_Rail     bus_label     U2.VDD1   | U2 Pin 1

19 Pin_Rail     bus_label     U2.VDD2   | U2 Pin 2

20 Pin_Rail     signal_name   U2.VSS    | U2 Pins 3 and 4

[End EMD Model]

[End EMD Set]

[EMD Set]       All_DQs_Crosstalk_ISS

| EMD Model includes all crosstalk contributions for DQ1.

| Crosstalk contributions are incomplete for other nets

| marked as Aggressor_Only.

[EMD Model]     DQ1_Victim

File_IBIS-ISS   DQ.iss        DQ1_Victim

Number_of_terminals = 15

1  Pin_I/O      pin_name      A1        Aggressor_Only  | DQ0

2  Pin_I/O      pin_name      A2                        | DQ1

3  Pin_I/O      pin_name      A3        Aggressor_Only  | DQ2

4  Pin_I/O      pin_name      A4        Aggressor_Only  | DQ3

5  Pin_Rail     signal_name   VSS

|

6  Pin_I/O      pin_name      U1.5      | DQ0

7  Pin_I/O      pin_name      U1.6      | DQ1

8  Pin_I/O      pin_name      U1.7      | DQ2

9  Pin_I/O      pin_name      U1.8      | DQ3

10 Pin_Rail     signal_name   U1.VSS    | U1 Pins 3 and 4

|

11 Pin_I/O      pin_name      U2.5      | DQ0

12 Pin_I/O      pin_name      U2.6      | DQ1

13 Pin_I/O      pin_name      U2.7      | DQ2

14 Pin_I/O      pin_name      U2.8      | DQ3

15 Pin_Rail     signal_name   U2.VSS    | U2 Pins 3 and 4

[End EMD Model]

| EMD Model includes all crosstalk contributions for DQ2.

| Crosstalk contributions are incomplete for other nets

| marked as Aggressor_Only.

[EMD Model]     DQ2_Victim

File_IBIS-ISS   DQ.iss        DQ2_Victim

Number_of_terminals = 15

1  Pin_I/O      pin_name      A1        Aggressor_Only  | DQ0

2  Pin_I/O      pin_name      A2        Aggressor_Only  | DQ1

3  Pin_I/O      pin_name      A3                        | DQ2

4  Pin_I/O      pin_name      A4        Aggressor_Only  | DQ3

5  Pin_Rail     signal_name   VSS

|

6  Pin_I/O      pin_name      U1.5      | DQ0

7  Pin_I/O      pin_name      U1.6      | DQ1

8  Pin_I/O      pin_name      U1.7      | DQ2

9  Pin_I/O      pin_name      U1.8      | DQ3

10 Pin_Rail     signal_name   U1.VSS    | U1 Pins 3 and 4

|

11 Pin_I/O      pin_name      U2.5      | DQ0

12 Pin_I/O      pin_name      U2.6      | DQ1

13 Pin_I/O      pin_name      U2.7      | DQ2

14 Pin_I/O      pin_name      U2.8      | DQ3

15 Pin_Rail     signal_name   U2.VSS    | U2 Pins 3 and 4

[End EMD Model]

[End EMD Set]

[EMD Set]       Rails_ISS

[EMD Model]     Power_Rails

File_IBIS-ISS   Power_Rails.iss  Rails

Number_of_terminals = 8

1  Pin_Rail     signal_name   VDD       | EMD Pins P1 and P2

2  Pin_Rail     signal_name   VSS       | EMD Pin G1

|

3  Pin_Rail     bus_label     U1.VDD1   | U1 Pin 1

4  Pin_Rail     bus_label     U1.VDD2   | U1 Pin 2

5  Pin_Rail     signal_name   U1.VSS    | U1 Pins 3 and 4

|

6  Pin_Rail     bus_label     U2.VDD1   | U2 Pin 1

7  Pin_Rail     bus_label     U2.VDD2   | U2 Pin 2

8  Pin_Rail     signal_name   U2.VSS    | U2 Pins 3 and 4

[End EMD Model]

[End EMD Set]

[EMD Set]       All_DQs_Crosstalk_TS

| EMD Model includes all crosstalk contributions for DQ1.

| Crosstalk contributions are incomplete for other nets

| marked as Aggressor_Only.

[EMD Model]     DQ1_Victim

File_TS         DQ1_Victim.ts

Unused_port_termination       Reference

Number_of_terminals = 25

1  Pin_I/O      pin_name      A1        Aggressor_Only  | DQ0

3  Pin_I/O      pin_name      A2                        | DQ1

5  Pin_I/O      pin_name      A3        Aggressor_Only  | DQ2

7  Pin_I/O      pin_name      A4        Aggressor_Only  | DQ3

|

9  Pin_I/O      pin_name      U1.5      | DQ0

11 Pin_I/O      pin_name      U1.6      | DQ1

13 Pin_I/O      pin_name      U1.7      | DQ2

15 Pin_I/O      pin_name      U1.8      | DQ3

|

17 Pin_I/O      pin_name      U2.5      | DQ0

19 Pin_I/O      pin_name      U2.6      | DQ1

21 Pin_I/O      pin_name      U2.7      | DQ2

23 Pin_I/O      pin_name      U2.8      | DQ3

|

25 A_gnd        | Reference for all ports

[End EMD Model]

| EMD Model includes all crosstalk contributions for DQ2.

| Crosstalk contributions are incomplete for other nets

| marked as Aggressor_Only.

[EMD Model]     DQ2_Victim

File_TS         DQ2_Victim.ts

Unused_port_termination       Reference

Number_of_terminals = 25

1  Pin_I/O      pin_name      A1        Aggressor_Only  | DQ0

3  Pin_I/O      pin_name      A2        Aggressor_Only  | DQ1

5  Pin_I/O      pin_name      A3                        | DQ2

7  Pin_I/O      pin_name      A4        Aggressor_Only  | DQ3

|

9  Pin_I/O      pin_name      U1.5      | DQ0

11 Pin_I/O      pin_name      U1.6      | DQ1

13 Pin_I/O      pin_name      U1.7      | DQ2

15 Pin_I/O      pin_name      U1.8      | DQ3

|

17 Pin_I/O      pin_name      U2.5      | DQ0

19 Pin_I/O      pin_name      U2.6      | DQ1

21 Pin_I/O      pin_name      U2.7      | DQ2

23 Pin_I/O      pin_name      U2.8      | DQ3

|

25 A_gnd        | Reference for all ports

[End EMD Model]

[End EMD Set]

[EMD Set]       Rails_TS

[EMD Model]     Power_Rails

File_TS         Power_Rails_TS.s8p

Number_of_terminals = 9

1  Pin_Rail     signal_name   VDD       | EMD Pins P1 and P2

2  Pin_Rail     signal_name   VSS       | EMD Pin G1

|

3  Pin_Rail     bus_label     U1.VDD1   | U1 Pin 1

4  Pin_Rail     bus_label     U1.VDD2   | U1 Pin 2

5  Pin_Rail     signal_name   U1.VSS    | U1 Pins 3 and 4

|

6  Pin_Rail     bus_label     U2.VDD1   | U2 Pin 1

7  Pin_Rail     bus_label     U2.VDD2   | U2 Pin 2

8  Pin_Rail     signal_name   U2.VSS    | U2 Pins 3 and 4

|

9  A_gnd        | Reference for all ports

[End EMD Model]

[End EMD Set]