Table XX summarizes the rules described above.

Table XX – Allowed Terminal_type Associations1

| Terminal_type | Terminal_type_qualifier |  |  |  | aggressor |
| --- | --- | --- | --- | --- | --- |
|  | pin_name | signal_name | bus_label | pad_name |  |
| Buf_I/O | X |  |  |  | A |
| Pullup_ref | X |  |  |  |  |
| Pulldown_ref | X |  |  |  |  |
| Power_clamp_ref | X |  |  |  |  |
| Gnd_clamp_ref | X |  |  |  |  |
| Ext_ref | X |  |  |  |  |
| Buf_Rail |  | Y | Y |  |  |
| Pad_I/O | X |  |  |  | A |
| Pad_Rail |  | Y | Y | Z |  |
| Pin_I/O | X |  |  |  | A |
| Pin_Rail | Y | Y | Y |  |  |

Notes

- In the table, “X” refers to I/O pin names.  “Y” and “Z” are POWER and GND names. The letter “A” designates the word "Aggressor"

Table XYX summarizes the rules described above.

Table XYZ – Allowed Terminal_type Associations1-6

| Terminal_type | Terminal_type_qualifier |  |  |  | aggressor |
| --- | --- | --- | --- | --- | --- |
|  | pin_name | signal_name | bus_label | pad_name |  |
| Buf_I/O | B |  |  |  | A |
| Pad_I/O | B |  |  |  | A |
| Pin_I/O | B |  |  |  | A |
| Pullup_ref | B |  |  |  |  |
| Pulldown_ref | B |  |  |  |  |
| Power_clamp_ref | B |  |  |  |  |
| Gnd_clamp_ref | B |  |  |  |  |
| Ext_ref | B |  |  |  |  |
| Buf_Rail |  | R | R |  |  |
| Pad_Rail |  | R | R | D |  |
| Pin_Rail | R | R | R |  |  |

Notes

- Except for the aggressor column, each table entry is for the designated Terminal_type_qualifier and its corresponding name (two columns).

- “B” refers to I/O pin names, as declared by the [Pin] keyword.

- “A” designates the optional column entry “Aggressor” (a single column entry).

- “B” also associates Rails named by Terminal_type  *_ref terminals to their corresponding buffers identified by Buf_I/O pin_name.  The [Pin Mapping] keyword is not needed nor used.

- Buf_Rail uses the [Pin Mapping] keyword information to associate the Rails with Buf_I/O terminals, and the associations cannot co-exist with the *_ref  Terminal_types when the Terminal_type_qualifiers define overlapping terminals.

- “R” and “D” show allowable Terminal_type_qualifiers for POWER and GND terminals, where the pad_name for “D” is declared by with [Die Supply Pads]

Table WXYZ – Allowed Terminal_type Associations

| Terminal_type | Terminal_type_qualifier |  |  |  | Aggressor2 |
| --- | --- | --- | --- | --- | --- |
|  | pin_name1 | signal_name1 | bus_label1 | pad_name1 |  |
| Buf_I/O3 | I |  |  |  | A |
| Pad_I/O3 | I |  |  |  | A |
| Pin_I/O3 | I |  |  |  | A |
| Pullup_ref4 | I |  |  |  |  |
| Pulldown_ref4 | I |  |  |  |  |
| Power_clamp_ref4 | I |  |  |  |  |
| Gnd_clamp_ref4 | I |  |  |  |  |
| Ext_ref4 | I |  |  |  |  |
| Buf_Rail5,6 |  | R | R |  |  |
| Pad_Rail6 |  | R | R | D |  |
| Pin_Rail6 | R | R | R |  |  |

Notes

- Each Terminal_type_qualifier and its corresponding argument occupies two columns

- “A” designates an optional, single-column entry “Aggressor”

- “I” refers to I/O pin names, as declared by the [Pin] keyword

- “I” also associates Rails named by Terminal_type  *_ref terminals to their corresponding buffers identified by Buf_I/O pin_name.  The [Pin Mapping] keyword is not needed nor used.

- Buf_Rail uses the [Pin Mapping] keyword information to associate the Rails with Buf_I/O terminals, and the associations cannot co-exist with the *_ref  Terminal_types when the Terminal_type_qualifiers define overlapping terminals.

- “R” and “D” show allowable Terminal_type_qualifiers for POWER and GND terminals, where the pad_name entry for “D” is declared by the [Die Supply Pads] keyword