IBIS-ISS ISSUE RESOLUTION DOCUMENT (ISSIRD)

ISSIRD NUMBER: 	1.0 {draft 1}

ISSUE TITLE:   		Enable Cascading of S-parameters Through W-element Wrapping

REQUESTOR:     	Michael Mirmak, Intel Corp.

DATE SUBMITTED:	{July 27, 2022}

DATE REVISED:

DATE ACCEPTED:

DEFINITION OF THE ISSUE:

IBIS-ISS omits a critical option available in several EDA tools today: the capability to use S-parameter data within the W-element.  In these commercial implementations, the W-element defines the nodal connections for an interconnect model but uses an S-parameter file or model definition as the behavioral data, instead of using an RLGC file or model definition.

S-parameters are often used to represent unitary circuit elements (e.g., a solder pad), but can be used in these commercial implementations to represent scalable structures (e.g., a portion of PCB trace modeled as a transmission line).  Without support in IBIS-ISS for S-parameter data within the W-element, S-parameters cannot be scaled for use in PCB traces, cables, or similar structures.  The proposed text adds this support.

SOLUTION REQUIREMENTS:

The IBIS-ISS specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Modify the W-element to point to the S Model | The W-element definitions will be expanded to support S-parameter data through existing model data structures |
| - Use consistent terminology throughout the document | Instances of “S-Model” and “S-Element Model” will be modified to ensure clarity |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

Table 2: IBIS-ISS Structures Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |
| Rename “S-Element Model” and “S-Model” to “S Model” for consistency with usage elsewhere in the document | Modified |  |
| Add parameters to the W-element definition to support the S Model | New |  |

PROPOSED CHANGES:

Page 37 of the IBIS-ISS Specification is modified as is shown in the highlighted text below:

The W-element is a versatile transmission line model that may be used to describe a variety of transmission line structures, from a simple lossless line to complex frequency-dependent lossy-coupled lines.

Syntax

Wxxx i1 i2 ... in ir o1 o2 ... on or N=val L=val

+ RLGCMODEL=name | TABLEMODEL=name | SMODEL=name [FGD=val]

Page 38 of the IBIS-ISS Specificaiton is modeified as is shown in the highlighted text below:

| RLGCMODEL=name | Defines string name as the name of the RLGC model |
| --- | --- |
| TABLEMODEL=name | Defines string name as the name of the frequency-dependent tabular model |
| SMODEL=name | Defines string name as the name of the scattering parameter S model |
| FGD=val | Specifies the cut-off frequency of dielectric loss, in hertz, as positive real number val. Zero is permitted (if set to zero, dielectric loss is assumed to maintain a linear dependence on frequency). |

Specify the number of signal conductors, N, after the list of nodes. The W-element does not limit the number of coupled conductors.

The W-element supports three formats to specify transmission line properties:

- Format 1: RLGC specification, specified in a .MODEL statement

- Format 2: Frequency-dependent tabular specification, specified in a .MODEL statement

- Format 3: Scattering parameter specification using Touchstone data, specified in .MODEL statement

The RLGCMODEL. TABLEMODEL, and SMODEL arguments associate a W-element definition with a .MODEL using either Format 1, Format 2 or Format 3, respectively.  A W-element shall use either RLGCMODEL, TABLEMODEL or SMODEL, but no more than one in the same W-element.

In Formats 1 and 2, the characteristics of the W-element are expressed in per-unit-length matrices: Ro (DC resistance), L, G, C, Rs (skin effect), and Gd (dielectric loss). In Format 3, the characteristics of the W-element are expressed in scattering parameters, using Touchstone formatted data in a separate file.

Page 39 of the IBIS-ISS Specificaiton is modeified as is shown in the highlighted text below:

Format 3: Scattering Parameter S Model

The S model may be used as a method to scale a set of frequency-dependent scattering parameters according to length.

The W-element syntax in this approach supports data provided in a Touchstone file (using a .MODEL statement of type S). The .MODEL statement uses the "type" S and names the Touchstone file containing the appropriate scattering parameters.

The following requirements apply when using an S model with a W-element:
• If the W-element input model is SMODEL, an S model definition must accompany that
input model.
• The associated S-parameters must have even number of terminals.
• The associated S-parameters must be symmetric.
• The associated S-parameters must be passive.
• Transmission-line based S-parameters can be used with different lengths of a system
when the varying length parameter (L) in a W-element instance statement is present.
• The XLINELENGTH keyword shall be set when the associated S model will be used in a W-element.

Page 47 of the IBIS-ISS Specificaiton is modeified as is shown in the highlighted text below:

S Model Syntax

Use the following syntax to describe specific S models:

.MODEL Smodel_name S N=val TSTONEFILE=filename [XLINELENGTH=val]

Table 22: S Model Definition Arguments

| Argument | Description |
| --- | --- |
| Smodel_name | Name of the S model. |

Page 48 of the IBIS-ISS Specificaiton is modeified as is shown in the highlighted text below:

| N=val | Sets positive, non-zero integer val as the number of ports for the S model. This value must match the number of ports defined in the associated Touchstone file. |
| --- | --- |
| TSTONEFILE=filename | Sets the string filename as the name of a Touchstone file. Note that string parameters are supported for TSTONEFILE Example: .subckt sparam n1 n2 tsfile=str('ss_ts.s2p') S1 n1 n2 0 mname=s_model .model s_model S TSTONEFILE=str(tsfile) .ends x1 A B sparam tsfile=str('ss_ts.s2p') … For details, see Touchstone® File Format Specification by the IBIS Open Forum (http://www.ibis.org). |
| XLINELENGTH | Sets the length of the transmission line from which the referenced S-parameters in the Touchstone file were extracted. |

BACKGROUND INFORMATION/HISTORY: