TOUCHSTONE ISSUE RESOLUTION DOCUMENT (TSIRD)

TSIRD NUMBER:	Y_draft_09

ISSUE TITLE:		Standardized Port Mapping

REQUESTOR:		Arpad Muranyi, Siemens EDA

Walter Katz, MathWorks

DATE SUBMITTED:	{for administrative use}

DATE REVISED:	{for administrative use}

DATE ACCEPTED:	{for administrative use}

DEFINITION OF THE ISSUE:

When using Touchstone models, it is essential to know how their ports should be connected in a design.  This implies that the “port ordering” or “port mapping” information must be supplied along with or inside the Touchstone file.  Currently, this is information is sometimes provided in a separate file, but increasingly more often it is included in the Touchstone file as a commented “header” section.

When a Touchstone file contains many ports, connecting its ports manually becomes a tedious and error-prone job.  Consequently automation is highly desired.  EDA vendors developed several different formats throughout the years, but the problem is that they are all different with various degrees of features and capabilities, making it difficult to parse them with software.  Additionally, in most cases this information is provided as a commented header section in the Touchstone file, and strictly speaking, comments are non-parsable, arbitrary text.

The goal of this proposal is to provide a standardized port mapping format for Touchstone files so that all relevant information could be included in it, and all EDA tools could parse them reliably.

SOLUTION REQUIREMENTS:

Port Mapping data must be able to support:

- Reliably, hooking up a Touchstone File in a simulation.

- Automate creating a schematic symbol.

- Automate generating/verifying [Interconnect Model]s in .ibs files.

- Automate generating/verifying [EMD Model]s in .emd files.

- Automate generating/verifying [C Comp Model]s in .ibs files.

- Enable Touchstone File viewers to generate mixed-mode S-parameters for differential ports.

- Sij Status (Measured | Simulated | TBD | Placeholder)

- Automatic generation of test probe locations for test equipment.

- Swathing

- Ability to add new user defined parameters

The Touchstone specification must meet these requirements:

Table 1: Solution Requirements

| Requirement | Notes |
| --- | --- |
| - Define two new keywords | [Begin Port Map] / [End Port Map] |

SUMMARY OF PROPOSED CHANGES:

For review purposes, the proposed changes are summarized as follows:

Table 2: Touchstone Keywords Affected

| Specification Item | New/Modified/Other | Notes |
| --- | --- | --- |

PROPOSED CHANGES:

Add the following keywords to the Touchstone specification in the appropriate location:

Keyword:	[Begin Port Map] / [End Port Map]

Required:	Optional; illegal prior to Version 3.0.

Description:	Identifies and associates physical, schematic, and optionally measurement locations with the port numbers specified in a Touchstone file

Sub-Params:	IBIS_file, EMD_file, C_comp_model_file, Ts4file, Source,  Swathing, Group, Symbol_leftside, Symbol_topside, Symbol_rightside, Symbol_bottomside, Sij_status, User_defined, Port

Usage rules:	If present in the file, the [Begin Port Map] / [End Port Map] keyword pair shall immediately follow the [Number of Ports] keyword and its associated data.  Each subparameter shall start on a new line.

The following optional subparameters identify what the Touchstone file port map is describing; only one of these subparameters is permitted per [Begin Port Map]/[End Port Map] keyword pair.

The IBIS_file subparameter is optional.  It is followed by an IBIS file name and component.  If present, this Touchstone file represents an [Interconnect Model] that is present in the named IBIS file.

The EMD_file subparameter is optional.  It is followed by an EMD file name.  If present, this Touchstone file represents an [EMD Model] that is present in the named EMD file.

The C_comp_model_file subparameter is optional.  It is followed by a C Comp Model file name.  If present, this Touchstone file represents an IBIS C Comp Model that is present in named model file.

The 	Ts4file subparameter is optional.  It is followed by an IBIS

The Source subparameter is optional.  It is followed by a file name identifying the original file where the Touchstone data was obtained or derived.  The file name may be an ODB++ board data base, a schematic file, a simulation model, or any other data file.  As the subparameter is purely informational, the data file is not required to be present on the computer system containing the associated Touchstone file.

The Swathing subparameter is optional.  It is followed by a string, defining a Schema name.  There are several ways that connector companies create Touchstone files for a slice (e.g., 3 wafers of a connector) of a connector that can be combined algorithmically to create a virtual Touchstone file that represents all the pins of the connector.  Separately, both Reserved Schemas, and other, model-maker-defined Schema values will be supported.

The SijStatus subparameter is optional.  SijStatus is followed by a row number, a column number and a status string.  The row and column numbers shall be between 1 and Number_of_Ports, inclusive.  Allowed argument values are Measured, Simulated, TBD, and Placeholder.

The Group subparameter is optional.  Group may appear multiple times for a [Begin Port Map]/[End Port Map] pair, with each Group subparameter appearing on a separate line.  These lines are placed after the last Port subparameter (see below) and the [End Port Map] keyword.

The Group subparameter is followed by a white space and the name of the Group

whose port is described by the line.  This Group <name> is followed by a white space, a “(“, followed by a list of Physical names, and terminated by a “)”.  Any Physical port names (see below) cannot be used as Group names.  An EOL (end of line character) without a “)” will continue the list of Physical names on the next line.

The Symbol_leftside  subparameter is optional.  These lines are placed after the last Port subparameter and the [End Port Map] keyword. The Symbol_left subparameter string is followed by a list of Port numbers that can be terminals on the left side of a schematic symbol ordered top to bottom.

The Symbol_rightside  subparameter is optional.  These lines are placed after the last Port Subparameter and the [End Port Map] keyword. The Symbol_right subparameter string is followed by a list of Port numbers that can be terminals on the right side of a schematic symbol ordered top to bottom.

The Symbol_topside  subparameter is optional.  These lines are placed after the last Port Subparameter and the [End Port Map] keyword. The Symbol_top subparameter string is followed by a list of Port numbers that can be terminals on the top side of a schematic symbol ordered left to right.

The Symbol_bottomside  subparameter is optional.  These lines are placed after the last Port Subparameter and the [End Port Map] keyword. The Symbol_bottomside subparameter string is followed by a list of Port numbers that can be terminals on the bottom side of a schematic symbol ordered left to right.

If any Symbol_leftside, Symbol_rightside, Symbol_topside or Symbol_bottomside subparameters are present then any Port number shall appear once and only once in these subparameters.

The User_defined subparameter is optional.  It shall be followed by one or more strings within the same pair of parentheses.  These subparameters are effectively ignored by the parser and EDA tools, but may be used by recipients to track whether physical measurements have been conducted on a Port, and/or how the Port data was measured.

Examples:

(Status Measured)

(Probe_Angle 15)

The Port subparameter is required.  The [Begin Port Map] keyword shall be followed by as many Port subparameter lines as the number of ports defined by the [Number of Ports] keyword.  The entire content of each Port subparameter shall be on a single line.

The Port subparameter is followed by a white space and an integer number, indicating which port is described by the line.  Note that the Port number shall be larger than zero and shall be smaller than or equal to the Number of Ports entry in the same Touchstone file.  The integer port number is followed by a white space, then (<name> <value>) pairs enclosed in parentheses.  Unless otherwise noted, all Port subparameter pairs are optional.  Subparameter names are case-sensitive.

Several names are reserved and shall not be used as values or arguments to Port subparameters.  These are listed below:

Type

Physical

Logical

Net

Diff_Port

Logical

Net

Physical

Reference

Side

Type

The Port subparameter and its value shall appear first, at the beginning of each line.  All other name/value pairs may appear in any order thereafter.

SijStatus is optional. SijStatus is followed by a row number, a column number and a status. Row and column number shall be between 1 and Number_of_Ports. Allowed value status are Measured, Simulated, TBD, Placeholder.

Examples:

Port 1 (Physical U7.1)

Port 2 (Physical U7.3) (Logical DQ5+) (Diff_Port 3)

Port 3 (Physical U7.4) (Logical DQ5-) (Diff_Port 2)

Port Subparameter Name Rules

Type:	The value is either S or P. S stands for Signal, P stands for Power (in other words, rail).  If not specified, the Type default is S.

Physical:	The value is a string that describes the physical location of the terminal used to generate the Port network data. This can be a pin on a component, a set of coordinates identifying the location of a probe, or a sheet and node location for a schematic.  The string should clearly indicate to a human where, for instance, the probe used to measure the S-parameter data is placed, or how to connect to the described network element in a circuit simulator.  For a printed circuit board (PCB), the argument to “Physical” should either be a reference designator (also called a “refdes”), followed by a dot and a pin number (e.g., U7.3), or an X,Y, Layer.. There are special rules that can be followed to automatically connect the element described by the Touchstone network data in IBIS and EMD [Interconnect Model]s and  in IBIS [C_comp_Model]s. If the value is provided in the Bus_Label:<name> format, then <name> shall be either defined in the IBIS or EMD file, or defined in the Bus_Labels subparameter section.

Allowed Physical names for C Comp Models:

Buffer_I/O

Buffer_I

Pullup_ref

Pulldown_ref

Power_clamp_ref

Gnd_clamp_ref

Ext_Ref

A_gnd

Rules for [EMD Model]s:

For Type S Ports, the Physical name shall be either the EMD Pin_name or a designator.pin_name

For Type P Ports, the Physical name shall be either the EMD Pin_name, a designator.pin_name, a Bus_Label.name or Group.name

Rules for IBIS [Interconnect Model]s:

For Type S Ports, the Physical name shall be either pin.<pin_name>, pad.<pin_name> or buf.<pin_name>

For Type P Ports, the Physical name shall be either pin.pin_name, pad.pin_name The list of Bus_Label pins can be determined from the associated .ibs file.  Rail connections to the buffer shall be in one of the following formats:

Pullup_ref.<pin_name>

Pulldown_ref.pin_name

Power_clamp_ref.pin_name

Gnd_clamp_ref.pin_name

Pullup_ref.Bus_label:<name>

Pulldown_ref. Bus_label:<name>

Power_clamp_ref. Bus_label:<name>

Gnd_clamp_ref. Bus_label:<name>

Pullup_ref.Group:<name>

Pulldown_ref.Group:<name>

Power_clamp_ref. Group:<name>

Gnd_clamp_ref. Group:<name>

ext_ref?

Logical:		The Logical value identifies a grouping of ports to be used in a schematic symbol for the Touchstone file.

Net:		The Net value identifies a grouping for generation of simulation or schematic topologies.  All ports that have the same Net value are part of the same signal path (this does not mean they share the same node, as in a short).

Side:		The Side value identifies a grouping, for connecting elements described by Touchstone network data in larger structures across multiple components.  A Touchstone-described interconnect file often has two sides (e.g., a connector between a mainboard and daughterboard). A memory DQ route may have a Controller Side, and separate Sides for each memory device connected to that route.  A cable assembly for a car may have four Sides (e.g., CPU, Brake, Engine, and Camera).

Diff_Port:		The Diff_Port value identifies the given Port as one-half of a differential pair, and states the Port number of the complementary port. Polarity is not directly indicated, since this will be established by how the port is used by the differential models in simulation.	Each port declared as a Diff_port shall have a corresponding (complementary) Diff_port declaration.

Reference: The Reference value identifies the physical location of the reference terminals used for probing (measurement). If more than one reference is needed, the value shall be in the format Bus_Label:<Bus_Label name> or Group:<Group name>.

Example 1:   4 pin cable from CPU to Sensor.

[Begin Port Map]

Port 1 (Physical A.1)  (Side CPU) (Net 1)

Port 2 (Physical A.2)  (Side CPU) (Net 2)

Port 3 (Physical A.3)  (Side CPU) (Net 3)

Port 4 (Physical A.4)  (Side CPU) (Net 4)

Port 5 (Physical B.1)  (Side Sensor) (Net 1)

Port 6 (Physical B.2)  (Side Sensor) (Net 2)

Port 7 (Physical B.3)  (Side Sensor) (Net 3)

Port 8 (Physical B.4)  (Side Sensor) (Net 4)

Symbol_left 1 2 3 4

Symbol_right 5 6 7 8

(Add Short Cuts e.g., NFNF NNFF, Gonzalez, Bogatin, IEEE)

[End Port Map]

Example 2:   Transistor

[Begin Port Map]

Port 1 (Logical Emitter)

Port 2 (Logical Base)

Port 3 (Logical Collector)

Symbol_left 1

Symbol_right 3

Symbol_bottom 2

[End Port Map]

Example 3:   A 8” coplanar wave guide. (probe has 3 connections: 1 signal and 2 references)

[Begin Port Map]

Port 1 (Physical   0.:0.:Top)  (Side Left)  (Net 1) (Reference Group:GND_L)

Port 2 (Physical  8.:0.:Top)  (Side Right) (Net 1) (Reference Group:GND_R)

Group GND_L (0.:.1:Top 0.:-.1:Top)

Group GND_R (8.:.1:Top 8.:-.1:Top)

[End Port Map]

Example 4:  Single ended IBIS package model between pin and pad on pin 7.

[Begin Port Map]

Port 1 (Physical pin.7)  (Side Pin)  (Net 7) (Logical DQ3pin)

Port 2 (Physical pad.7)  (Side Pad) (Net 7) (Logical DQ3pad)

[End Port Map]

Example 5:  Single ended IBIS package model between pin and buffer on pin 7, including VDD voltage port.

[Begin Port Map]

Port 1 (Physical   pin.7) (Type S) (Side Pin)  (Net 7) (Logical DQ3pin)

| Type S is redundant, as this is the default

Port 2 (Physical   buffer.7) (Type S) (Side buffer) (Net 7) (Logical DQ3buffer)

| Type S is redundant, as this is the default

Port 3 (Physical Pin.Bus_label:VDD) (Type P) (Side Pin)  (Net VDD) (Logical VDDpin)

Port 4 (Physical Pullup_ref.7) (Type P) (Side Buffer)  (Net VDD) (Logical VDDbuffer)

[End Port Map]

Example 6:  Single-ended connection between U1.7 and U3.5 for a PCB.

[Begin Port Map]

Port 1 (Physical   U1.7)  (Side U1)  (Net DQ3) (Logical CPU)

Port 2 (Physical   U3.5)  (Side U3) (Net DQ3) (Logical SDRAM)

[End Port Map]

Example 7:  EMD of a 4 bit DQ nibble in a 2 rank DIMM

[Begin Port Map]

Port 1 (Physical   20)  (Side EMD)  (Net DQ0) (Logical DQ0)

Port 2 (Physical   21)  (Side EMD)  (Net DQ1) (Logical DQ1)

Port 3 (Physical   22)  (Side EMD)  (Net DQ2) (Logical DQ2)

Port 4 (Physical   23)  (Side EMD)  (Net DQ3) (Logical DQ3)

Port 5 (Physical   25)  (Side EMD)  (Net DQS+) (Logical DQS+) (Diff_port 6)

Port 6 (Physical   26)  (Side EMD)  (Net DQS-) (Logical DQS-) (Diff_port 5)

Port 7 (Physical   27)  (Side mem1)  (Net DQ0) (Logical mem1_DQ0)

Port 8 (Physical   28)  (Side mem1)  (Net DQ1) (Logical mem1_DQ1)

Port 9 (Physical   29)  (Side mem1)  (Net DQ2) (Logical mem1_DQ2)

Port 10 (Physical   30)  (Side mem1)  (Net DQ3) (Logical mem1_DQ3)

Port 11 (Physical   31)  (Side mem1)  (Net DQS+) (Logical mem1_DQS+) (Diff_port 12)

Port 12 (Physical   32)  (Side mem1)  (Net DQS-) (Logical mem1_DQS-) (Diff_port 11)

Port 13 (Physical   33)  (Side mem2)  (Net DQ0) (Logical mem2_DQ0)

Port 14 (Physical   34)  (Side mem2)  (Net DQ1) (Logical mem2_DQ1)

Port 15 (Physical   35)  (Side mem2)  (Net DQ2) (Logical mem2_DQ2)

Port 16 (Physical   36)  (Side mem2)  (Net DQ3) (Logical mem2_DQ3)

Port 17 (Physical   37)  (Side mem2)  (Net DQS+) (Logical mem2_DQS+) (Diff_port 18)

Port 18 (Physical   38)  (Side mem2)  (Net DQS-) (Logical mem2_DQS-) (Diff_port 17)

[End Port Map]

BACKGROUND INFORMATION/HISTORY:

TBD