IBIS Open Forum Minutes

Meeting Date: November 16, 2018

Meeting Location: Taipei, Taiwan

VOTING MEMBERS AND 2018 PARTICIPANTS

ANSYS				Curtis Clark, Miyo Kawata

Applied Simulation Technology	(Fred Balistreri)

Broadcom				(Yunong Gan)

Cadence Design Systems		Brad Brim, Ken Willis, Ambrish Varma, Zhen Mu

Morihiro Nakazato, Jinsong Hu, Skipper Liang*

Zuli Qin, Haisan Wang, Hui Wang, Yitong Wen

Clark Wu, Zhangmin Zhong, Jessica Yen*, Nemo Hsu*

Cisco Systems			Stephen Scearce, Cassie Yan, Baosh Xu

CST					Stefan Paret

Ericsson				Anders Ekholm*, Zilwan Mahmod*, Guohua Wang

Wenyan Xie, Amy Zhang*

GLOBALFOUNDRIES		Steve Parker

Huawei Technologies			(Hang (Paul) Yan)

IBM	Greg Edlund, Luis Armenta, Hubert Harrer

Michael Cohen

Infineon Technologies AG 		(Christian Sporrer)

Intel Corporation	Hsinho Wu, Michael Mirmak, Nilesh Dattani

Fernando Mendoza Hernandez, Varun Gupta

Subas Bastola, Hansel Dsilva, Gianni Signorini

Kai Yuan, Denis Chen*, Jimmy Hsu*, Cucumber Lin*

IO Methodology			Lance Wang

Keysight Technologies	Radek Biernacki, Ming Yan, Heidi Barnes

Pegah Alavi, Toshinori Kageura, Satoshi Nakamizo

Umekawa Mitsuharu

Maxim Integrated			Joe Engert, Yan Liang

Mentor, A Siemens Business		Arpad Muranyi, Weston Beal, Raj Raghuram

Carlo Bleu, Mikael Stahlberg, Yasushi Kondou

Vladimir Dmitriev-Zdorov, Nitin Bhagwath

Kazuhiro Kadota, Terence Guo

Micron Technology			Randy Wolff, Justin Butterfield

Micron Memory Japan		Masayuki Honda, Tadaaki Yoshimura, Toshio Oki

Mikio Sugawara

NXP					(John Burnett)

Raytheon				Joseph Aday

SiSoft					Mike LaBonte*, Walter Katz, [Todd Westerhoff]

Synopsys				Ted Mido, Adrien Auge, Scott Wedge, Xuefeng Chen

Jinghua Huang, Yuyang Wang

Teraspeed Labs			Bob Ross

Xilinx					Ravindra Gali

ZTE Corporation			Shunlin Zhu, Liqiang Meng, Yonghui Ren, Bi Yi

Zuken					Michael Schaeder, Takayuki Shiratori

OTHER PARTICIPANTS IN 2018

A&D Print Engineering Co.		Ryu Murota

Abeism Corporation			Nobuyuki Kiyota, Noboru Kobayashi

Alpine Electronics			Norio Mashiko

AMD Japan				Tadashi Arai

Apollo Giken Co.			Naoya Iisaka, Satoshi Endo

ASRock Rack				Eric Chien*, Timmy Kao*

ASUSTek Computer			Eric Hsieh*, Nick KH Huang*, Jenyung Li*, Eric Tsai*

Avnet					Shinya Ishizuka

BasiCAE				Kiki Li, Darcy Liu, Linda Zhang

Canon Components			Takeshi Nagata

Canon					Syoji Matsumoto, Yusuke Matsudo, Manabu Sakakibara

Tadashi Aoki, Hitoshi Matsuoka, Ryuta Kusaka

Masaaki Ohishi, Satoru Ishikawa

Casio Computer Co.			Yasuhisa Hayashi

Celestica				Sophia Feng, Bowen Shi

CMK Products Corp.			Hiroyasu Miura

Credo					Anyun Liu

Cybernet Systems			Takayuki Tsuzura

Denso Corp.				Yukiya Fukunaga

Eizo Corp.				Tokimitsu Eso

Finnhan				Yuan Xu

Fuji Xerox Manufacturing Co.		Rumi Maeda

Fujitsu Advanced Technologies	Tendo Hirai, Kumiko Teramae, Hidenobu Shiihara

Fujitsu Interconnect Technologies	Masaki Kirinaka, Akiko Tsukada

Fujitsu Ltd.				Takashi Kobayashi

Fujitsu Optical Components		Masaki Kunii

Genesis Technology			TF Chiang*

Gifu University				Toshikazu Sekine

Global Unichip Japan			Masafumi Mitsuishi

Google					Zhiping Yang

Hamamatsu Photonics		Akihiro Inoguchi, Shigenori Fujita, Hidetoshi Nakamura

Haskware				David Banas

Hewlett Packard Enterprise		Passor Ho*, Corey Huang*, Hellen Lo*, Edward Pan*

Hitachi ULSI Systems Co.		Sadahiro Nonoyama

Hitachi Ltd.				Yasuhiro Ikeda

Hoei Co.				Tatsuya Chiba

Huawei Technologies			Haiping Cao, Longfang Lv, Shengli (Victory) Wang

Hang (Paul) Yan, Chen (Jeff) Yu, Zhengyi Zhu

Peng Huang

IB-Electronics				Matsumuro Makoto

Independent				Hiroshi Ishikawa, Fumiyo Kawafuji

Inspur Technologies			Josh Chen*, Steven Ho*, Dane Huang*, Nieves Lee*

Eric Lee*, Rock Wang*

Institute for Information Industry	Joseph Yang*

Japan Radio Co.			Hiroto Katakura

JEITA					Yukio Masuko

John Baprawski, Inc.			John Baprawski

JVC Kenwood Corp.			Yasutoshi Ojima, Masayuki Kurihara

KEI Systems				Shinichi Maeda

Keihin Corp.				Takayuki Ota

Lapis Semiconductor Co.		Satoshi Tachi

Lattice Semiconductor		Dinh Tran, Maryam Shahbazi

Lenovo				Mark Zheng, Alex Chu*, Alan Sun*, Simon Yeh*

Marvell					Jianping Kong, Fang Lv, Banglong Qian

Songjie (Jacky) Wang, Liang Wu

Megachips Corp.			Tomochika Kitamura

Mitsubishi Electric Corp.		Yusuke Suzuki

Mobile Techno Corp.			Kazuhiro Kamegawa

Molex Japan				Nobumasa Motohashi

Murata Manufacturing Co.		Kazutaka Mukaiyama

Nanya Technology Corp.		Ching-Feng Chen*, Chi-Wei Chen*

Taco (Changqun) Hsieh*, Benson Hsu*, George Lee*

Linda*, Allen Ye*

NEC Magnus Communications	Toshio Saito

New H3C Group			Xinyi Hu, Zixiao Yang

Nikon Corp.				Manabu Matsumoto

Nvidia Corp.				Norman Chang*, Chiayuan Hsieh*, Rich Lu*

Chihwei (Jason) Tsai*

Oki Electric Industry Co.		Kenichi Saito

OmniVision				Sirius Tsang

Panasonic Corp.			Minori Harada, Tomohiro Tsuchiya, Naoyuki Aoki

Atsushi Nakano

Panasonic Industrial Devices,	Kazuki Wakabayashi

Systems and Technology Co.

Politecnico di Milano			Flavia Grassi, Xinglong Wu

Politecnico di Torino			Tommaso Bradde, Marco De Stefano, Paulo Manfredi

Riccardo Trinchero, Stefano Grivet-Talocia

PWB Corp.				Toru Ohisa

Qualcomm				Kevin Roselle, Tim Michalka, Zhiguang Li

Razer					Irwin (Zhilong) Xue*

Renesas Electronics Corp.		Masayasu Koumyou, Kazunori Yamada, Kenzo Tan

Hiroyoshi Kuge, Masato Suzuki

Ricoh Company			Kazuki Murata, Yasuhiro Akita, Kazumasa Aoki

Toshihiko Makino, Koji Kurose

RITA Electronics Ltd.			Kenichi Higashiura, Hiroyuki Motoki

Rohm Co.				Noboru Takizawa, Ryosuke Inagaki, Nobuya Sumiyoshi

Ryosan Co.				Takahiro Sato, Takumi Ito

SAE ITC				(Jose Godoy)

Sanwa Denki Kogyo Co.		Yutaka Takasaki

Shanghai IC R&D Center (ICRD)	Huijie Yan, Hailing Yang

Shanghai Zhaoxin Semiconductor	Chuanyu (Liam) Li

Shinewave				Nike Yang*

Shinko Electric Industries Co.		Takumi Ikeda

Signal Metrics				Ron Olisar

Silvaco Japan Co.			Yoshihiko Yamamoto, Kaoru Kashimura

SMK Corp.				Norihide Taguchi

Socionext				Megumi Ono, Yumiko Sugaya, Mitsuhiro Tomita

Katsuya Ogata, Yoshihiko Sumimoto, Yuji Nakagawa

Takashi Araki

Sohwa & Sophia Technologies	Tomoki Yamada

Sony Global Manufacturing &		Takashi Mine, Toshio Murayama, Taichi Hirano

Operations Corp.			  Takashi Mizoroki

Sony LSI Design			Toru Fujii

Sony Semiconductor Solutions	Takeshi Ogura

SPISim				Wei-hsing Huang*, Wei-Kai Shih*

Stanford University			Tom Lee

STMicroelectronics			Aurora Sanna, Olivier Bayet

Syswave				Kazuo Ogasawara

Tatung Technology			Barry Chen*

TDK Corp.				Kotaro Suzuki

Technopro Design Co.		Mai Fukuoka

Teledyne LeCroy			Denny Li, Nan Son, Suping Wu, Sherry

Telepower				Kenji Kobayashi

TFF Tektronix Co.			Katsuhiko Suzuki

Thine Electronics			Takafumi Nakamori

Tomen Devices Corp.			Kinji Mitani

Toshiba Corp.				Yasuki Torigoshi

Toshiba Development & 		Nobuyuki Kasai

Engineering Corp.

Toshiba Electronic Devices &		Atsushi Tomishima, Yasunobu Umemoto

Storage Corp.			  Yoshinori Fukuba, Hitoshi Imi, Motochika Okano

Tetsuya Nakamura

Toshiba Memory Corp.		Masato Kanie, Takayuki Mizogami

Toshiba Memory Systems Co.	Eiji Kozuka, Tomomichi Takahashi

Toshiba Microelectronics Corp.	Jyunya Masumi

Unisoc					Junyong Deng, Nikki Xie

Université de Bretagne Occidentale	Mihai Telescu, Charles Canaff

University of Illinois			José Schutt-Aine
University of Siegen			Elmar Griese
University of Technology Hamburg	Torben Wendt
Xpeedic				Suxiang Zhou

Xrossvate				Toshiyuki Kaneko

Yamaha Corp.				Tetsuya Kakimoto

Zhejiang Uniview Technologies	Fang Yang

Zheijiang YUSHI Technology		E. Deng

In the list above, attendees at the meeting are indicated by *.  Principal members or other active members who have not attended are in parentheses. Participants who no longer are in the organization are in square brackets.

UPCOMING MEETINGS

The bridge numbers for future IBIS teleconferences are as follows:

Date				Meeting Number		Meeting Password

November 30, 2018		624 227 121			IBISfriday11

For teleconference dial-in information, use the password at the following website:

http://tinyurl.com/y7yt7buz

All teleconference meetings are 8:00 a.m. to 9:55 a.m. US Pacific Time.  Meeting agendas are typically distributed seven days before each Open Forum.  Minutes are typically distributed within seven days of the corresponding meeting.

NOTE: "AR" = Action Required.

-------------------------------------------------------------------------------------------------------------------------------

OFFICIAL OPENING

The Asian IBIS Summit took place on Friday, November 16, 2018 at the Sherwood Hotel in Taipei.  About 47 people representing 17 organizations attended.

The notes below capture some of the content and discussions.  The meeting presentations and other documents are available at:

http://www.ibis.org/summits/nov18c/

Mike LaBonte opened the summit by welcoming everyone and thanking the sponsors Cadence Design Systems, KairosTech Innovation (on behalf of SPISim), Synopsys, and IO Methodology.  He noted that minutes of the meeting would be posted.  There would be two breaks for refreshments and vendor interaction, and free lunch.

IBIS UPDATE

Mike LaBonte (SiSoft, USA)

The status and activities of the IBIS Open Forum were described.  Mike showed the progress on development of the IBIS 7.0 specification, which he estimated might be ratified in March of 2019, if all goes well.  Mike noted that few BIRDs were currently in the pipeline for further development, encouraging the audience to consider submitting their own ideas for IBIS.  He planned to give a short walk-through of the BIRD submission and adoption process during final discussion, if time would permit.

A PRACTICAL METHODOLOGY FOR SERDES DESIGN

Amy Zhang*, Guohua Wang*, David Zhang*, Zilwan Mahmod*, Anders Ekholm** (Ericsson, *PRC, **Sweden)

[Presented by Amy Zhang (Ericsson, PRC)]

Analyzing a SerDes channel to find the best case operation involves not only many combinations of driver and receiver settings, but also a significant number of interconnect characteristics that must be explored under typ/min/max conditions.  Simulating 1 million bits across all possible variations for one example would take 506.25 days.  The challenge is to make satisfactory design decisions without running many simulations.  Ideally, we would create an equation that quickly models system performance, given all of the system condition inputs that can vary.  This can be done using Design of Experiments (DOE) methods to statistically sample the parameter space, producing a Response Surface Model (RSM) using relatively few simulations.  A CEI-28g example was shown, with the quality of the RSM fit evaluated.  Sensitivity analysis was used to assign a different sampling distribution to each factor.  Millions of conditions were then evaluated very quickly.  Increasingly, we will find that the best/worst case analysis supported by typ/min/max data will not suffice for design closure.  Adding an option for IBIS-AMI to represent a full distribution of data would help with DOE analysis, and would allow for predicting performance confidence intervals.

CHARACTERIZING AND MODELING OF A CLAMPED NON-LINEAR CTE/AGC

Skipper Liang (Cadence Design Systems, ROC)

Skipper summarized his 2017 presentation describing a Thevenin equivalent circuit method for characterizing a Continuous Time Equalizer (CTE).  The Device Under Test (DUT) could include an Automatic Gain Control (AGC), as long as the condition of linearity was met.  The technique involved scaling voltages, so it was important that the DUT have the same characteristics at any voltage, since scaled results would be normalized to actual.  In reality, clamping diodes were usually present, causing non-linear effects.  One approach is to use a small signal input of 20mV, which captures the high frequency behavior well, but misses the DC behavior.  The large signal approach would be to use a 100mV input, but that fails to capture the high frequency response while successfully capturing the DC behavior.  To produce a single model that performs well in all cases, a model was constructed in which separate code blocks captured the small signal and large signal behaviors.  A method to find the linear voltage range was described.  It was found that a hyperbolic tangent function could be used to model the non-linear behavior of clamping diodes, and iterating to find the best coefficients would yield acceptable results for mapping input voltage to output voltage.  This was suitable for use in AMI_GetWave.  However, the frequency response of the hyperbolic tangent function model could not be directly derived, because the function fails one of the criteria to be Fourier transformable.  A solution to that involved using an equivalent hyperbolic cosecant function to derive the Fourier transform, for use in statistical analysis.  Two examples using CTE and AGC were shown, each with good correlation to measurement.

MODEL CORRELATION FOR IBIS-AMI

Wenyan Xie*, Guohua Wang*, David Zhang*, Anders Ekholm** (Ericsson, *PRC, **Sweden)

[Presented by Anders Ekholm (Ericsson, Sweden)]

Once simulation results are correlated to corresponding measured results, simulation can be used to verify cases that are beyond the scope of measurement.  A method for correlating IBIS-AMI Tx models was described.  A slow clock pattern made it easier to compare edges and amplitudes, and also to compare FFE tap action.  Each tap setting was swept across all values, and correlation evaluated for each.  It is necessary to achieve good correlation for the Tx first, because it will be used to drive the Rx for Rx correlation.  Sweeping Tx FFE tap settings again, the Rx eye at the decision point was monitored.  Some devices have an internal ability to report the internal Rx eye.  Example correlation results for eye width and height were shown.  Time domain waveforms can also be compared using Figure of Merit (FOM) or Feature Selective Validation (FSV).  More than the usual 5 FSV metrics should be used, each weighted differently.  The example was correlated against only typical silicon.  We do not know the span of real silicon performance that IBIS-AMI min and max corners captures.  Having models with statistical distributions for each parameter would be better.

STUDY OF DDR ASYMMETRIC RT/FT IN EXISTING IBIS-AMI FLOW

Wei-kai Shih*, Wei-hsing Huang** (SPISim, *Japan, **USA)

[Presented by Wei-hsing Huang (SPISim, USA)]

Wei-hsing described how statistical and time domain IBIS-AMI flows worked.  For asymmetric rise/fall, Wei-kai described a method for deriving a fall edge from the rise edge, or vice-versa, using a transfer function.  He noted it would still be necessary for IBIS-AMI to know the common mode DC offset for single-ended signals.  Wei-kai showed example pseudo-code for the transfer function to recover a fall response from the rise response.  This could be used to construct eye diagrams with rise/fall asymmetry.  A recursive algorithm for calculating eye PDF with asymmetric rise/fall was described.  Simulating bit-by-bit with Tx and Rx AMI_GetWave should work well, but there would be glitches if convolution was involved at the Rx.  Pseudo-code for the AMI_GetWave process was shown.

OPEN DISCUSSION

During the time for open discussion, Mike LaBonte gave two ad-hoc presentations.  The first included a subset of slides from Mike's presentation from the September 2017 EDICON IBIS Summit in Boston.  Mike noted that previous speakers had described the challenges of proving low bit error rates using time domain simulations, and a method by which statistical analysis could be performed to calculate very low probability events.  He showed the numbers of error free bits required to prove 1e-12 BER with various confidence levels, which were impractical to achieve in simulators.  In one example, statistical analysis was able to predict 6e-12 BER.  Extrapolation of time domain results using statistical results could provide the benefits of both time domain simulation, to model adaptive behavior, and statistical simulation, to prove lower BER requirements.

Mike also showed the IBIS website to explain the process by which the IBIS specification is developed.  BIRD documents are written by people from one or more organizations using a template, and submitted to the IBIS Chair.  Often there are multiple authors, from different companies.  The BIRD includes a statement of the issue to be addressed, particular requirements for the solution, and proposed changes to the current IBIS specification to meet the requirements.  Anyone may submit a BIRD, but only official IBIS members may vote.  The BIRD is discussed in meetings, sometimes over an extended period, and finally a vote to accept it is taken in an IBIS Open Forum meeting.  If it passes, the BIRD eventually will be incorporated into a future IBIS specification produced by the IBIS Editorial Task Group.

CLOSING REMARKS

In closing, Mike LaBonte thanked the sponsors, authors, presenters, and all participants for their support of IBIS and for good discussions.  The meeting was adjourned.

NEXT MEETING

The next IBIS Open Forum teleconference meeting will be held on November 30, 2018.  The following IBIS Open Forum teleconference meeting is tentatively scheduled on December 21, 2018.

========================================================================

NOTES

IBIS CHAIR: Mike LaBonte

mlabonte@sisoft.com

IBIS-AMI Modeling Specialist, SiSoft

6 Clock Tower Place, Suite 250

Maynard, MA 01754

VICE CHAIR: Lance Wang (978) 633-3388

lwang@iometh.com

President/CEO, IO Methodology, Inc.

PO Box 2099

Acton, MA  01720

SECRETARY: Randy Wolff (208) 363-1764

rrwolff@micron.com

Principal Engineer, Silicon SI Group Lead, Micron Technology, Inc.

8000 S. Federal Way

P.O. Box 6, Mail Stop: 01-711

Boise, ID  83707-0006

TREASURER: Bob Ross (503) 246-8048

bob@teraspeedlabs.com

Engineer, Teraspeed Labs

10238 SW Lancaster Road

Portland, OR 97219

LIBRARIAN: Anders Ekholm (46) 10 714 27 58, Fax: (46) 8 757 23 40

ibis-librarian@ibis.org

Digital Modules Design, PDU Base Stations, Ericsson AB

BU Network

Färögatan 6

164 80 Stockholm, Sweden

WEBMASTER: Mike LaBonte

mlabonte@sisoft.com

IBIS-AMI Modeling Specialist, SiSoft

6 Clock Tower Place, Suite 250

Maynard, MA 01754

POSTMASTER: Curtis Clark

curtis.clark@ansys.com

ANSYS, Inc.

150 Baker Ave Ext

Concord, MA 01742

This meeting was conducted in accordance with ANSI guidance.

All inquiries may be sent to info@ibis.org.  Examples of inquiries are:

- To obtain general information about IBIS.

- To ask specific questions for individual response.

- To subscribe to the official ibis@freelists.org and/or ibis-users@freelists.org email lists (formerly ibis@eda.org and ibis-users@eda.org).

- To subscribe to one of the task group email lists: ibis-macro@freelists.org, ibis-interconn@freelists.org, or ibis-quality@freelists.org.

- To inquire about joining the IBIS Open Forum as a voting Member.

- To purchase a license for the IBIS parser source code.

- To report bugs or request enhancements to the free software tools: ibischk6, tschk2, icmchk1, s2ibis, s2ibis2 and s2iplt.

The BUG Report Form for ibischk resides along with reported BUGs at:

http://www.ibis.org/bugs/ibischk/ 
http://www.ibis.org/ bugs/ibischk/bugform.txt

The BUG Report Form for tschk2 resides along with reported BUGs at:

http://www.ibis.org/bugs/tschk/ 
http://www.ibis.org/bugs/tschk/bugform.txt

The BUG Report Form for icmchk resides along with reported BUGs at:

http://www.ibis.org/bugs/icmchk/ 
http://www.ibis.org/bugs/icmchk/icm_bugform.txt

To report s2ibis, s2ibis2 and s2iplt bugs, use the Bug Report Forms which reside at:

http://www.ibis.org/bugs/s2ibis/bugs2i.txt 
http://www.ibis.org/bugs/s2ibis2/bugs2i2.txt 
http://www.ibis.org/bugs/s2iplt/bugsplt.txt

Information on IBIS technical contents, IBIS participants and actual IBIS models are available on the IBIS Home page:

http://www.ibis.org/

Check the IBIS file directory on ibis.org for more information on previous discussions and results:

http://www.ibis.org/directory.html

Other trademarks, brands and names are the property of their respective owners.

SAE STANDARDS BALLOT VOTING STATUS

| Organization | Interest Category | Standards Ballot Voting Status | November 2, 2018 | November 12, 2018 | November 14, 2018 | November 16, 2018 |
| --- | --- | --- | --- | --- | --- | --- |
| ANSYS | User | Inactive | X | X | - | - |
| Applied Simulation Technology | User | Inactive | - | - | - | - |
| Broadcom Ltd. | Producer | Inactive | - | - | - | - |
| Cadence Design Systems | User | Active | X | X | X | X |
| Cisco Systems | User | Inactive | - | - | - | - |
| CST | User | Inactive | - | - | - | - |
| Ericsson | Producer | Active | X | X | X | X |
| GLOBALFOUNDRIES | Producer | Inactive | X | - | - | - |
| Huawei Technologies | Producer | Inactive | - | - | - | - |
| IBM | Producer | Inactive | X | - | - | - |
| Infineon Technologies AG | Producer | Inactive | - | - | - | - |
| Intel Corp. | Producer | Active | X | - | X | X |
| IO Methodology | User | Inactive | - | - | X | - |
| Keysight Technologies | User | Inactive | X | X | - | - |
| Maxim Integrated | Producer | Inactive | - | - | - | - |
| Mentor, A Siemens Business | User | Active | X | X | X | - |
| Micron Technology | Producer | Inactive | X | X | - | - |
| NXP | Producer | Inactive | - | - | - | - |
| Raytheon | User | Inactive | - | - | - | - |
| SiSoft | User | Active | X | X | X | X |
| Synopsys | User | Active | X | X | X | - |
| Teraspeed Labs | General Interest | Inactive | X | - | - | - |
| Xilinx | Producer | Inactive | - | - | - | - |
| ZTE Corp. | User | Inactive | - | - | X | - |
| Zuken | User | Inactive | - | X | - | - |

Criteria for SAE member in good standing:

- Must attend two consecutive meetings to establish voting membership

- Membership dues current

- Must not miss two consecutive meetings

Interest categories associated with SAE standards ballot voting are:

- Users - members that utilize electronic equipment to provide services to an end user.

- Producers - members that supply electronic equipment.

- General Interest - members are neither producers nor users. This category includes, but is not limited to, government, regulatory agencies (state and federal), researchers, other organizations and associations, and/or consumers.