AMI Check List

Edited by Greg Edlund

08/16/11

Revision history:

08/16/11	Categorized Section 7.0, “Table of Requirements,” from “Opal: Best Practices for IBIS-AMI Modeling,” SiSoft, June 15, 2010.

08/29/11	added some comments and more points , eckhard lenski

- Syntax Checks

  - Parameter names in model same as model names in .ami file

  - Labels consistent with List

  - All parameters in dependency table declared before table

  - Column header and all rows in dependency table have same length

  - Dependency row value type convertible to all column types

  - Fully IBIS compliant analog model available

- Usage Checks

  - All files distributed in a single archive

  - Model installation directory independent of execution directory

  - Support Windows and Linux

  - Multiple instances of one model in one simulation/analysis

  - Multiple instance of multiple models in one simulation/analysis

  - Multiple simultaneous simulations/analyses

  - Unrecognized parameters do not cause failure

  - Useful parameter Description
The Description branch of a parameter is optional. However, it is very important for users to fully understand each parameter. Descriptions should be present and written to be as informative as possible.

- Accuracy Checks

  - Model correlated to another behavior description

  - Correlation conditions defined

  - Correlation method defined

  - Correlation criteria defined

  - Package models not counted twice

- Documentation Checks

  - Minimum documentation requirements

  - S parameter file ports and organization

  - Node map consistent with S parameter file

  - overview table init&getwave

  - information about datarate delivered

  - Info, which other software is needed

- Completeness Checks

  - package model delivered for TX as sNp

  - package model delivered for RX as sNp

  - ibis ami TX delivered with dll & ami

  - ibis ami RX delivered with dll & ami

  - ibis classic model delivered ( refer to Syntax point 8)