| IBIS EVOLUTION (IBIS, PACKAGE, ELECTRICAL BOARD DESCRIPTION, EMI, SSO, ALGORITHMIC MODELING INTERFACE) Bob Ross, July 25, 2006, updated September 14, 2007, September 8, 2013, April 20, 2014 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| HEADER SECTION (.ibs, .pkg, .ebd) |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
| [IBIS Ver] | [Copyright] |  | x |  |  |
| [Comment Char] (re-defined anywhere in file) |  |  |  |  |  |
| [File Name] |  |  |  |  |  |
| [File Rev] |  |  |  |  |  |
| [Date] |  |  |  |  |  |
| [Source] |  |  |  |  |  |
| [Notes] |  |  |  |  |  |
| [Disclaimer] |  |  |  |  |  |
| TOP-LEVEL KEYWORDS (with END Keywords) |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
| [Component] | [Define Package Model] (.ibs, .pkg) | [Model Selector] | [External Circuit] | [Test Load] (defined under [Model] in 4.2) |  |
| [Model] | [End Package Model] (.ibs, .pkg) | [Submodel] | [End External Circuit] | [Test Data] (defined under [Test Load] in 4.2) |  |
| [End] | [End] (.pkg) | [Begin Board Description] (.ebd) |  |  |  |
|  |  | [End Board Description] (.ebd) |  |  |  |
|  |  | [End] (.ebd) |  |  |  |
| [Component] PINOUT AND PACKAGE SECTION |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
| [Component] | [Pin Mapping] | [Series Pin Mapping] | [Alternate Package Models] | [Begin EMI Component] | [Repeater Pin] |
| [Manufacturer] | [Diff Pin] | [Series Switch Groups] | [End Alternate Package Models] | [End EMI Component] |  |
| [Package] | [Package Model] |  | [Node Declarations] |  |  |
| [Pin] |  |  | [End Node Declarations] |  |  |
|  |  |  | [Circuit Call] |  |  |
|  |  |  | [End Circuit Call] |  |  |
| [Model] SECTION |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
| [Model] | [Temperature Range] | [Model Spec] | [Receiver Thresholds] | [Begin EMI Model] |  |
| [Voltage Range] | [Pullup Reference] | [Driver Schedule] | [External Reference] | [End EMI Model] |  |
| [Pulldown] | [Pulldown Reference] | [TTgnd] | [Test Data] | [Algorithmic Model] |  |
| [Pullup] | [POWER Clamp Reference] | [TTpower] | [Rising Waveform Near] | [End Algorithmic Model] |  |
| [GND Clamp] | [GND Clamp Reference] | [On] | [Falling Waveform Near] | [ISSO PU] |  |
| [POWER Clamp] | [Rgnd] | [Off] | [Rising Waveform Far] | [ISSO PD] |  |
| [Ramp] | [Rpower] | [R Series] | [Falling Waveform Far] | [Composite Current] (under [Rising Waveform]) |  |
|  | [Rac] | [L Series] | [Diff Rising Waveform Near] | [Composite Current] (under Falling Waveform]) |  |
|  | [Cac] | [Rl Series] | [Diff Falling Waveform Near] | [C Comp Corner] |  |
|  | [Rising Waveform] | [C Series] | [Diff Rising Waveform Far] |  |  |
|  | [Falling Waveform] | [Lc Series] | [Diff Falling Waveform Far] |  |  |
|  |  | [Rc Series] | [Test Load] |  |  |
|  |  | [Series Current] | [External Model] |  |  |
|  |  | [Series MOSFET] | [End External Model] |  |  |
|  |  | [Add Submodel] |  |  |  |
| [Submodel] SECTION |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  | [Submodel] |  |  |  |
|  |  | [Pullup] (1.1) |  |  |  |
|  |  | [Pulldown] (1.1) |  |  |  |
|  |  | [GND Clamp] (1.1) |  |  |  |
|  |  | [POWER Clamp] (1.1) |  |  |  |
|  |  | [Ramp] (1.1) |  |  |  |
|  |  | [Rising Waveform] (2.1) |  |  |  |
|  |  | [Falling Waveform] (2.1) |  |  |  |
|  |  | [Submodel Spec] |  |  |  |
|  |  | [GND Pulse Table] |  |  |  |
|  |  | [POWER Pulse Table] |  |  |  |
| PACKAGE MODEL DEFINITION (within .ibs or as .pkg) |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  | [Define Package Model] | [Number Of Sections] |  |  |  |
|  | [Manufacturer] (1.1) |  |  |  |  |
|  | [OEM] |  |  |  |  |
|  | [Description] |  |  |  |  |
|  | [Number Of Pins] |  |  |  |  |
|  | [Pin Numbers] |  |  |  |  |
|  | [Model Data] |  |  |  |  |
|  | [End Model Data] |  |  |  |  |
|  | [Resistance Matrix] |  |  |  |  |
|  | [Inductance Matrix] |  |  |  |  |
|  | [Capacitance Matrix] |  |  |  |  |
|  | [Row] |  |  |  |  |
|  | [Bandwidth] |  |  |  |  |
|  | [End Package Model] |  |  |  |  |
| ELECTRICAL BOARD DESCRIPTION (.ebd) |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  | [Begin Board Description] |  |  |  |
|  |  | [Manufacturer] (1.1) |  |  |  |
|  |  | [Number Of Pins] |  |  |  |
|  |  | [Pin List] |  |  |  |
|  |  | [Path Description] |  |  |  |
|  |  | [Reference Designator Map] |  |  |  |
|  |  | [End Board Description] |  |  |  |
| [Model] SUBPARAMETERS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
| Polarity | Cref |  | C_comp_pullup |  |  |
| Enable | Rref |  | C_comp_pulldown |  |  |
| Model_type | Vref |  | C_comp_power_clamp |  |  |
| C_comp | Vmeas |  | C_comp_gnd_clamp |  |  |
| Vinh |  |  | Rref_diff |  |  |
| Vinl |  |  | Cref_diff |  |  |
| Model_type SELECTIONS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
| Input | Open_sink | 3-state_ECL | Input_diff |  |  |
| Output | Open_source | Series | Output_diff |  |  |
| 3-state | I/O_open_drain | Series_switch | 3-state_diff |  |  |
| Open_drain | I/O_open_sink |  | I/O_diff |  |  |
| I/O | I/O_open_source |  |  |  |  |
|  | Input_ECL |  |  |  |  |
|  | Output_ECL |  |  |  |  |
|  | I/O_ECL |  |  |  |  |
|  | Terminator |  |  |  |  |
| [Model Spec] SUBPARAMETERS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  | Vinh | Cref | Weak_R |  |
|  |  | Vinl | Rref | Weak_I |  |
|  |  | Vinh+ | Vref | Weak_V |  |
|  |  | Vinh- | Cref_rising | D_overshoot_area_h |  |
|  |  | Vinl+ | Cref_falling | D_overshoot_area_l |  |
|  |  | Vinl- | Rref_rising | D_overshoot_ampl_h |  |
|  |  | S_overshoot_high | Rref_falling | D_overshoot_ampl_l |  |
|  |  | S_overshoot_low | Vref_rising |  |  |
|  |  | D_overshoot_high | Vref_falling |  |  |
|  |  | D_overshoot_low | Vmeas_rising |  |  |
|  |  | D_overshoot_time | Vmeas_falling |  |  |
|  |  | Pulse_high | Rref_diff |  |  |
|  |  | Pulse_low | Cref_diff |  |  |
|  |  | Pulse_time |  |  |  |
|  |  | V_meas |  |  |  |
| [Submodel] SUBPARMETER |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  | Submodel_type |  |  |  |
| Submodel_type SELECTIONS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  | Dynamic_clamp | Fall_back |  |  |
|  |  | Bus_hold |  |  |  |
| [Submodel Spec] SUBPARAMETERS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  | V_trigger_r |  |  |  |
|  |  | V_trigger_f |  |  |  |
|  |  | Off_delay |  |  |  |
| Test_data_type and Test_load_type SELECTIONS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  |  | Single_ended |  |  |
|  |  |  | Differential |  |  |
| [Begin EMI Component] KEYWORDS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  |  |  | [Pin EMI] |  |
|  |  |  |  | [Pin Domain EMI] |  |
| Model_emi_type SELECTIONS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
|  |  |  |  | Ferrite |  |
|  |  |  |  | Not_a_Ferrite |  |
| (SUBPARAMETERS) FOR OTHER KEYWORDS |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
| [Component] |  | [Component] (Si_location, Timing_location) |  | [Begin EMI Component] (Domain, Cpd, C_Heatsink_gnd, C_Heatsink_float) | [Repeater Pin] (tx_non_inv_pin) |
| [Package] (R_pkg, L_pkg, C_pkg) |  |  |  | [Pin EMI] (domain_name, clock_div) |  |
| [Pin] (signal_name, model_name, R_pin, L_pin, C_pin) |  |  |  | [Pin Domain EMI} (percentage) |  |
|  | [Pin Mapping] (pulldown_ref, pullup_ref, gnd_clamp_ref, power_clamp_ref) |  | [Pin Mapping] (ext_ref) | [Begin EMI Model] (Model_emi_type, Model_Domain) |  |
|  | [Diff Pin] (inv_pin, vdiff, t_delay, tdelay_min, tdelay_max) |  |  | [Algorithmic Model] (Executable) |  |
|  |  | [Series Pin Mapping] (pin_2, model_name, function_table_group) |  | [C Comp Corner] (C_comp, C_comp_pullup, C_comp_pulldown, C_comp_power_clamp, C_comp_gnd_clamp |  |
|  |  | [Series Switch Groups] (On, Off) |  |  |  |
|  |  |  | [Circuit Call] (Signal_pin, Diff_signal_pins, Series_pins, Port_map) |  |  |
|  |  |  | [Receiver Thresholds] (Vth, Vth_min, Vth_max, Vinh_ac,Vinh_dc, Vinl_ac, Vinl_dc, Threshold_sensitivity, Reference_supply, Vcross_low, V_cross_high, Vdiff_ac, Vdiff_dc, Tslew_ac, Tdiffslew_ac) |  |  |
|  |  | [Series MOSFET] (Vds) |  |  |  |
| [Ramp] (dV/dt_r, dV/dt_f) | [Ramp] (R_load) |  |  |  |  |
|  | [Rising Waveform] (R_fixture, V_fixture, V_fixture_min, V_fixture_max, C_fixture, L_fixture, R_dut, L_dut, C_dut) |  |  |  |  |
|  | [Falling Waveform] (R_fixture, V_fixture, V_fixture_min, V_fixture_max, C_fixture, L_fixture, R_dut, L_dut, C_dut) |  |  |  |  |
|  |  |  | [Test Data] (Test_data_type, Driver_model, Driver_model_inv, Test_load) (moved to top-level in 5.1) |  |  |
|  |  |  | [Test Load] (Test_load_type, C1_near, Rs_near, Ls_near, C2_near, Rpl_near, Rp2_near, Td, Zo, Rpl_far, Rp2_far, C1_far, Ls_far, Rs_far, V_term1, V_term2, Receiver_model, Receiver_model_inv, R_diff_near, R_diff_far) (moved to top-level in 5.1) |  |  |
|  |  |  | [External Model] (Language, Corner, Parameters, Ports, D_to_A, A_to_D) |  | [External Model] (Converter_Parameters) |
|  |  |  | [External Circuit] (Language, Corner, Parameters, Ports, D_to_A, A_to_D) |  | [External Circuit] (Converter_Parameters) |
|  | [Pin Numbers] | [Pin Numbers] (Len, L, C, R, Fork, Endfork) |  |  |  |
|  | [Resistance Matrix] (Banded_matrix, Sparce_matrix, Full_matrix) |  |  |  |  |
|  | [Inductance Matrix] (Banded_matrix, Sparce_matrix, Full_matrix) |  |  |  |  |
|  | [Capacitance Matrix] (Banded_matrix, Sparce_matrix, Full_matrix) |  |  |  |  |
|  |  | [Pin List] (signal_name) |  |  |  |
|  |  | [Begin Board Description] (Len, L, C, R, Fork, Endfork, Node, Pin) |  |  |  |
| OTHER NOTABLE CHANGES |  |  |  |  |  |
| Version 1.1 | Version 2.1 | Version 3.2 | Version 4.2 | Version 5.1 | Version 6.0 |
| File width = 80 |  |  | File width=120 |  |  |
| Filename length = 8 |  | Filename length = 20 | Filename length = 40 |  |  |
| Model name length = 20 |  |  | Model name length = 40 |  |  |
|  |  | Submodel name length = 20 | Submodel name length = 40 |  |  |
|  | [Rising Waveform], [Falling Waveform] rows = 100 |  | [Rising Waveform], [Falling Waveform] rows = 1000 |  |  |
| Vinh, Vinl optional for Input and I/O | Vinh, Vinl required for Input and I/O |  |  |  |  |
| Reserved words NC POWER, GND, NA |  |  | Added reserved word CIRCUITCALL |  |  |
| Fixed keyword space and underbar convention | Space and underbar equivalent in keywords |  |  |  |  |
| Original comment characters | +, - removed as comment characters |  |  |  |  |
| Multipliers M, k, m, u, n, p | Multipliers T, G and f added |  |  |  |  |
| Buffers by extraction |  | By construction added | By language linkage added | EMI, SSO, and Algorithmic Modeling Interface added, [Test Load] and [Test Data] hierarchy changed |  |